3GPP TSG-RAN WG4 Meeting #116	R4-2511634

# Bengaluru, India, 25th - 29th Aug, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on LP-WUR requirements for FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LPWUS-Core |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of FR2 requirements for LR and MR |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduced Rx beam sweeeping scaling factor for LR and measurement relaxation requirements for MR in FR2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | FR2 requirements will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.x.2.2.2, 4.x.2.2.3, 4.x.2.3.1, 4.x.2.3.2, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <<Start of Change1>>

#### 4.x.2.2.2	LP-WUR measurement and evaluation requirements for PSS/SSS

The UE shall measure the LP-SSS-RSRP and LP-SSS-RSRQ level once every LO cycle and evaluate whether one or more of the following conditions defined in TS 38.304 [1], if configured, are met within Tevaluate-LP-WUR-PSS/SSS

-	[entry condition for LP-WUS monitoring]

-	[entry condition for RRM relaxation]

-	[entry condition for RRM offloading]

-	exit condition for LP-WUS monitoring

-     exit condition for RRM offloading

-	exit condition for RRM relaxation

The UE shall filter the SS-RSRP and SS-RSRQ measurements of the serving cell using at least 2 measurement samples. Within the set of measurements used for the filtering, at least two measurement samples shall be spaced by LO-periodicity/2.

Table 4.x.2.2-1: Tevaluate-LP-WUR-PSS/SSS

| LO periodicity [s] Note 1 | Scaling Factor (N1WUS) |  |  | Tevaluate-LP-WUR-PSS/SSS [s] (number of LO Cycles) |
| --- | --- | --- | --- | --- |
|  | FR1 | FR2 |  |  |
| 0.32 | 1 | 8 |  | 0.32 x 2 x [y] x N1WUS (2 x [y] x N1WUS) |
| 0.64 |  | 5 |  | 0.64 x 2 x [y] x N1WUS (2 x [y] x N1WUS) |
| 1.28 |  | 4 |  | 1.28 x 2 x [y] x N1WUS (2 x [y] x N1WUS) |
| 2.56 |  | 3 |  | 2.56 x 2 x [y] x N1WUS (2 x [y] x N1WUS) |
| Note 1: The LO periodicity is the same as the configured DRX cycle length |  |  |  |  |

The UE shall evaluate and consider the corresponding entry criteria fulfilled within Tevaluate-LP-WUR-PSS/SSS, provided that the criteria is met by a margin of [±5.5 or ±6] dB in FR1 and by a margin of [±7 or ±7.5] dB in FR2 when SSB Ês/Iot ≥ -3dB

The UE shall evaluate and consider the corresponding exit criteria fulfilled within Tevaluate-LP-WUR- PSS/SSS, provided that the criteria is met by a margin of [±5.5 or ±6] dB in FR1 and by a margin of [±7 or ±7.5] dB in FR2 when SSB Ês/Iot ≥ -3dB.

Note: When a configured entry or exit condition is fulfilled, the UE shall perform corresponding actions as defined in clause XX in TS 38.304 [1].

#### 4.x.2.2.3	LP-WUR measurement and evaluation requirements for LP-SS

The UE shall measure the LP-RSRP and LP-RSRQ level once every LP-SS cycle and evaluate whether one or more of the following conditions defined in TS 38.304 [1] are met within Tevaluate-LP-WUR-LP-SS

-	[entry condition for LP-WUS monitoring]

-	[entry condition for RRM relaxation]

-	[entry condition for RRM offloading]

-	exit condition for LP-WUS monitoring

-     exit condition for RRM offloading

The UE shall filter the LP-RSRP and LP-RSRQ measurements of the serving cell using at least 2 measurement samples.

Table 4.x.2.3-1: Tevaluate-LP-WUR-LP-SS

| LP-SS periodicity [s] | Scaling Factor (N1WUS) |  | Tevaluate-LP-WUR-LP-SS [s] (number of LP-SS Cycles) |
| --- | --- | --- | --- |
|  | FR1 | FR2 |  |
| 0.16 | 1 | 8 | 0.16 x 2 x [x] x N1WUS (2 x [x] x N1WUS) |
| 0.32 |  | 8 | 0.32 x 2 x [x] x N1WUS (2 x [x] x N1WUS) |

The UE shall evaluate and consider the corresponding entry criteria fulfilled within Tevaluate-LP-WUR-LP-SS, provided that the criteria is met by a margin of [±5.5 or ±6] dB in FR1 and by a margin of [±7 or ±7.5] dB in FR2 when LP-SS Ês/Iot ≥ -3dB

The UE shall evaluate and consider the corresponding exit criteria fulfilled within Tevaluate-LP-WUR- LP-SS, provided  that the criteria is met by a margin of [±5.5 or ±6] dB in FR1 and by a margin of [±7 or ±7.5] dB in FR2 when LP-SS Ês/Iot ≥ -3dB.

Note: When a configured entry or exit condition is fulfilled, the UE shall perform corresponding actions as defined in clause XX in TS 38.304 [1].

### <<End of Change1>>

### <<Start of Change2>>

### 4.X.2.3	Measurement and evaluation of serving cell by MR

The requirements in this clause apply for UE measurement and evaluation of serving cell using MR, when LP-WUS UE is not in [RRM offloading mode] as defined in [1].

#### 4.X.2.3.1	Requirements for evaluation of cell selection criterion

When LP-WUS UE is not in [RRM relaxation mode] as defined in [1], the requirements in clause 4.2.2.2 shall apply.

When LP-WUS UE is in [RRM relaxation mode] as defined in [1], the UE shall measure the SS-RSRP and SS-RSRQ level of the serving cell and evaluate the cell selection criterion S defined in TS 38.304 [1] for the serving cell at least once every N1WUS*16 DRX cycle.

The LP-WUS UE shall filter the SS-RSRP and SS-RSRQ measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by 8 DRX cycles.

If the LP-WUS UE has evaluated according to table 4.X.2.3.1-1 in 16*Nserv consecutive DRX cycles that the serving cell does not fulfil the cell selection criterion S, the LP-WUS UE shall initiate the measurements of all neighbour cells indicated by the serving cell, regardless of the measurement rules currently limiting LP-WUS UE measurement activities.

If the LP-WUS UE in RRC_IDLE has not found any new suitable cell based on searches and measurements using the intra-frequency, inter-frequency and inter-RAT information indicated in the system information during the time T, the LP-WUS UE shall initiate cell selection procedures for the selected PLMN as defined in TS 38.304 [1], where T= 10s.

Table 4.X.2.3.1-1: Nserv

| DRX cycle length [s] | Scaling Factor (N1WUS) |  | Nserv [number of 16* N1WUS*DRX cycles] |
| --- | --- | --- | --- |
|  | FR1 | FR2 |  |
| 0.32 | 1 | 8 | 4 |
| 0.64 |  | 5 | 4 |
| 1.28 |  | 4 | 2 |
| 2.56 |  | 3 | 2 |

#### 4.X.2.3.2	Requirements for evaluation of LP-WUS related conditions

When LP-WUS UE is not in [RRM relaxation mode] as defined in [1], the LP-WUS UE shall measure the SS-RSRP and SS-RSRQ level of the serving cell and evaluate the following LP-WUR related conditions defined in TS 38.304 [1], if configured,

-	[entry condition for LP-WUS monitoring]

-	[entry condition for RRM relaxation]

-	[entry condition for RRM offloading]

-	FFS: exit condition for RRM relaxation

for the serving cell at least once every M1*N1WUS DRX cycle; where:

-	M1=2 if SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤ 0.64 second,

-	otherwise M1=1.

The LP-WUS UE shall filter the SS-RSRP and SS-RSRQ measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by DRX cycle/2.

If the LP-WUS UE has evaluated according to table 4.X.2.3.2-1 in Nserv consecutive DRX cycles that the serving cell fulfils the entry condition for [LP-WUR monitoring, RRM relaxation or RRM offloading], the LP-WUS UE shall perform corresponding actions as defined in clause XX in [1].

FFS: If the UE has evaluated according to table 4.X.2.3.2-1 in Nserv consecutive DRX cycles that the serving cell fulfils the exit condition for [RRM relaxation], the UE shall perform corresponding actions as defined in clause YY in [1].

Table 4.X.2.3.2-1: Nserv When UE is not in [RRM relaxation mode]

| DRX cycle length [s] | Scaling Factor (N1WUS) |  | Nserv [number of DRX cycles] |
| --- | --- | --- | --- |
|  | FR1 | FR2 |  |
| 0.32 | 1 | 8 | M1*4 |
| 0.64 |  | 5 | M1*4 |
| 1.28 |  | 4 | 2 |
| 2.56 |  | 3 | 2 |

When LP-WUS UE is in [RRM relaxation mode] as defined in [1], the LP-WUS UE shall measure the SS-RSRP and SS-RSRQ level of the serving cell and evaluate the following LP-WUR related conditions defined in TS 38.304 [1], if configured,

-	[entry condition for LP-WUS monitoring]

-	[entry condition for RRM offloading]

-	FFS: exit condition for RRM relaxation

for the serving cell at least once every 16 DRX cycle.

The LP-WUS UE shall filter the SS-RSRP and SS-RSRQ measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by 8 DRX cycles.

If the LP-WUS UE has evaluated according to table 4.X.2.3.2-2 in 16*Nserv consecutive DRX cycles that the serving cell fulfils the entry condition for [LP-WUR monitoring or RRM offloading], the LP-WUS UE shall perform corresponding actions as defined in clause XX in [1].

FFS: If the LP-WUS UE has evaluated according to table 4.X.2.3.2-2 in 16*Nserv consecutive DRX cycles that the serving cell fulfils the exit condition for [RRM relaxation], the LP-WUS UE shall perform corresponding actions as defined in clause YY in [1].

Table 4.X.2.3.2-2: Nserv when UE is in [RRM relaxation mode]

| DRX cycle length [s]; Scaling Factor (N1WUS); Nserv [number of 16* N1WUS*DRX cycles]. FR1; FR2. 0.32; 1; 8; 4. 0.64; 5; 4. 1.28; 4; 2. 2.56; 3; 2. |  |  |
| --- | --- | --- |
| DRX cycle length [s] | Scaling Factor (N1WUS) |  |  | Nserv [number of 16* N1WUS*DRX cycles] |  |
|  | FR1 | FR2 |  |  |  |
| 0.32 | 1 | 8 |  | 4 |  |
| 0.64 |  | 5 |  | 4 |  |
| 1.28 |  | 4 |  | 2 |  |
| 2.56 |  | 3 |  | 2 |  |

### <<End of Change2>>