3GPP TSG-RAN WG4 Meeting #116	R4-25xxxxx

Bengaluru, IN, August 25th – 29th 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.104 | CR | draftCR | rev | -- | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 38.104 on adding SBFD BS Types and Configuration Constraints |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_duplex_evo-Core |  |  |  |  |  | Date: |  |  | 2025-08-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Adding SBFD BS type and configuration constraints |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 12.1.2, 12.1.3 are introduced to capture General SBFD Base Station Operating Requirements and SBFD Base Station Types and Configuration Constraints. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The RF requirements for SBFD-capable BS are not completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.1.2, 12.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------------------Start of change-----------------------------------------------------

### 12.1.2 General SBFD Base Station Operating Requirements

A SBFD-capable BS shall operate only SBFD subband configurations (i.e., DL/UL subbands) that have been declared in accordance with the provisions in this specification.

### 12.1.3 SBFD Base Station Types and Configuration Constraints

#### 12.1.3.1 General

SBFD-capable BSs are categorized as either Type-A or Type-B according to the classification defined in Table 12.1.3-1. The classification is based on BS class and rated output power specified in clause 12.5.2.

Table 12.1.3.1-1: SBFD-capable BS classification

| BS Class | Rated Output Power (Prated,c,sys) | SBFD BS type |
| --- | --- | --- |
| FR1 WA BS | -- | Type-A |
| FR1 MR BS | ≥ 36 dBm | Type-A |
| FR1 MR BS | < 36 dBm | Type-B |
| FR1 LA BS | -- | Type-B |
| FR2 WA BS | -- | Type-B |
| FR2 LA BS | -- | Type-B |
| NOTE 1: A power level threshold of Prated,c,sys = 36 dBm is used to distinguish between Type-A and Type-B SBFD-capable BSs in the FR1 MR BS class. |  |  |

#### 12.1.3.2 Type-A SBFD-Capable BS (Configuration-Limited)

For Type-A SBFD-capable BSs, the following requirements apply:

- For each channel bandwidth supported for SBFD operation, a maximum of three SBFD subband configurations may be declared.

  - The three configurations shall cover all supported SBFD patterns, including both DUD and DU/UD cases.

  - A single UL subband size used in both DU and UD configurations shall be counted as two configurations. For example, a UD 20-80 MHz and a DU 80-20 MHz configuration shall be counted as two SBFD subband configurations.

- For each supported channel bandwidth and for each supported UL subband size:

  - Only one DL subband size may be declared and tested for the DU/UD pattern.

  - Only one DL subband size may be declared and tested for the DUD pattern.

- A Type-A SBFD-capable BS shall not operate in any declared SBFD configuration (i.e., DL/UL subband) unless it has been tested.

#### 12.1.3.3 Type-B SBFD-Capable BS (Configuration-Flexible)

For Type-B SBFD-capable BSs, the following requirements apply:

- There is no restriction on the number of UL subband sizes that may be declared.

- There is no restriction on the number of DL subband sizes that may be declared.

- For each declared channel bandwidth and for each declared SBFD pattern (i.e., DUD or DU/UD), the following configurations shall be tested:

  - The configuration with the narrowest UL subband size, and the corresponding worst-case SBFD DL transmission bandwidth:

    - For DU/UD: the configuration with the largest NRB,SBFD,DL.

    - For DUD: the configuration with the largest NRB,SBFD,DL,1.

  - The configuration with the widest UL subband size, and the corresponding worst-case SBFD DL transmission bandwidth:

    - For DU/UD: the configuration with the largest NRB,SBFD,DL.

    - For DUD: the configuration with the largest NRB,SBFD,DL,1.

------------------------------------------End of change------------------------------------------------------