3GPP TSG-RAN WG4 Meeting #116   											          R4-2511454

Bangaluru, India, August 25 - 29, 2025

Agenda item:			7.24.1

Source:	Moderator (Huawei)

Title:	Topic summary for [116][308] NR_LPWUS

Document for:	Information

Introduction

This contribution summarizes the documents that are submitted to agenda 7.24.4 for RAN4#116 and it concentrates on the following aspect.

- Topic #1: On BS RF requirements for LP-WUS/WUR

Topic #1: On the BS RF requirement for LP-WUS

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2509192 | Nokia | Proposal 1: There is no need to specify the EVM requirement for LP-WUS/LP-SS. Proposal 2: There is no need to test the EVM requirement for LP-WUS/LP-SS even if it is specified. Proposal 3: There is no restriction of LP-WUS operation with higher order modulation. |
| R4-2509382 | CATT | Observation 1: Two different methods have been used for selecting EVM values, one from system level performance perspective for a low order modulation scheme, the other from link level performance perspective for a high order modulation scheme. Observation 2: The two methods cannot be directly used for determining EVM requirement for LP-WUS due to non-applicability of throughput as the performance metric. Proposal 1: RAN4 to decide whether or not to introduce EVM requirement for LP-WUS based on system level instead of link level simulations. Proposal 2: RAN4 to consider a new method to determine EVM value for LP-WUS by modifying the system-level performance based method and checking CDF loss at the required SNR for different arbitrary EVM values. Proposal 3: RAN4 to determine EVM value for LP-WUS at 1% CDF loss at the required SNR compared with the case of 0% EVM. Proposal 4: RAN4 to set EVM requirement as 28.5% for LP-WUS. Proposal 5: No restriction of LP-WUS operation, this is BS implementation issue. |
| R4-2509383 | CATT | draftCR to TS 38.141-1 on declaration of support for LP-WUS |
| R4-2509384 | CATT | draftCR to TS 38.141-2 on declaration of support for LP-WUS |
| R4-2510135 | CMCC | Proposal 1: No restriction of LP-WUS operation. |
| R4-2510399 | ZTE | Observation 1: Frequency-domain EVM on LP-WUS has minor impact on the receiving MDR. Proposal 1: It is proposed not to define EVM requirements for LP-WUS, but BS still needs to satisfy the NR EVM requirements on each NR carrier for different modulation schemes on PDSCH if the BS supports LP-WUS operation. Proposal 2: Support Option 2: No restriction of LP-WUS operation. |
| R4-2510414 | Qualcomm | draftCR to TS 38.104 on introduction of LP-WUS FR2-1 BS RF considerations Change #1 Change #2 Change #3 Change #4 Change #5 Change #6 |
| R4-2511000 | Huawei, HiSilicon | Proposal 1: No need to define EVM requirements for LP-WUS. Observation: LP-WUS could introduce noticeable EVM degradation in certain cases for 256QAM. Proposal 2: It is proposed to restrict in-band LP-WUS operation to cases where the NR signal employs a maximum modulation order of 64QAM. This limitation should be explicitly specified to ensure robust system performance. |
| R4-2511004 | Huawei, HiSilicon | draft CR to TS 38.104 on clarification of transmitted signal quality for LP-WUS operation |
| R4-2511005 | Huawei, HiSilicon | draft CR to TS 38.141-1 on clarification of transmitted signal quality for LP-WUS operation |
| R4-2511006 | Huawei, HiSilicon | draft CR to TS 38.141-2 on clarification of transmitted signal quality for LP-WUS operation |
| R4-2511425 | Ericsson | CR to 38.104 for transmit signal quality of LP-WUS Change #1 Change #2 |
| R4-2511426 | Ericsson | Observation 1 WID objective mandates the BS new requirement subject to the declaration and use the legacy NR requirement as baseline. Proposal-1: 17.5% EVM requirement for LP-WUS. Proposal-2: The OOK4 signal quality test can be dependent on the 64QAM EVM test. Proposal-3: No restriction on the LP-WUS operation. |

Open issues summary

Sub-topic 1-1 Remaining issues for LP-WUS

Issue 1-1: Whether/How to define EVM requirements for LP-WUS

- Options

  - Option 1: No EVM requirement specified for LP-WUS.

  - Option 2: Specify EVM requirement for LP-WUS.

    - Option 2-1: Set it as 28.5% for LP-WUS, which is based on SLS by evaluating SNR loss under different EVM values.

    - Option 2-2: Set it as 17.5% for LP-WUS, which is the EVM requirement for NR OFDM signal with QPSK modulation order.

- Recommended WF

  - TBA

Issue 1-2: Whether/how to test EVM requirements for LP-WUS

- Options

  - Option 1: No EVM test for LP-WUS/LP-SS.

    - Option 1a: The BS still needs to satisfy the NR EVM requirements on each NR carrier for different modulation schemes on PDSCH if the BS supports LP-WUS.

  - Option 2: The OOK4 signal quality test can be dependent on the 64QAM EVM test.

- Recommended WF

  - Pending on the outcome of Issue 1-1.

Issue 1-3: Whether to introduce restriction for LP-WUS operation with higher order modulation

- Proposals

  - Proposal 1: No restriction of LP-WUS operation with higher modulation order. (Nokia, CATT, CMCC,  ZTE, Ericsson)

  - Proposal 2: Restrict in-band LP-WUS operation to cases where the NR signal employs a maximum modulation order of 64QAM. This limitation should be explicitly specified to ensure robust system performance. (Huawei)

- Recommended WF

  - Check if RAN4 can agree on no restriction of LP-WUS operation with higher modulation order in this meeting.

Issue 1-4: Submitted TP

- TPs

  - R4-2511426 TP to TR38.774: BS RF requirement overview for LP-WUS. (Ericsson)

- Recommended WF

  - Proponent is encouraged to check companies’ view regarding the TP.

Issue 1-5: Submitted draft CR

- Draft CRs

  - TS 38.104

    - R4-2510414 Draft CR to TS 38.104 on introduction of LP-WUS FR2-1 BS RF considerations. (Qualcomm)

    - R4-2511004 Draft CR to TS 38.104 on clarification of transmitted signal quality for LP-WUS operation. (Huawei)

    - R4-2507087 CR to 38.104 for transmit signal quality of LP-WUS. (Ericsson)

  - TS 38.141-1/2

    - R4-2509383 Draft CR to TS 38.141-1 on declaration of support for LP-WUS. (CATT)

    - R4-2509384 Draft CR to TS 38.141-2 on declaration of support for LP-WUS. (CATT)

    - R4-2511005 Draft CR to TS 38.141-1 on clarification of transmitted signal quality for LP-WUS operation. (Huawei)

    - R4-2511006 Draft CR to TS 38.141-2 on clarification of transmitted signal quality for LP-WUS operation. (Huawei)

- Recommended WF

  - Proponent is encouraged to check companies’ view regarding the draft CR.