3GPP TSG-RAN4 Meeting #119	R4-2606162

Dalian, China, May 18-22, 2026

| CR-Form-v13.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 6635 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for maintenance on R19 LPWUR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo,oppo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LPWUS-Core |  |  |  |  |  | Date: |  |  | 2026-05-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | Clarification on the applicability of LR requirements on some entry conditions |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the applicability of LR requirements on some entry conditions |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements for LR-WUR is not accurate |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.8.2.2.2; 4.8.2.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| Forge related attachments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

4.8.2.2.2	LP-WUR measurement and evaluation requirements for PSS/SSS

Upon meeting the entry conditions for LP-WUS monitoring, RRM offloading or RRM relaxation, the UE shall measure the SS-RSRP and SS-RSRQ level once every LO cycle and evaluate whether one or more of the following conditions defined in TS 38.304 [1], if configured, are met within Tevaluate-LP-WUR-PSS/SSS

-	exit condition for LP-WUS monitoring

-	exit condition for RRM offloading

-	condition for exiting RRM relaxation mode

The UE shall filter the SS-RSRP and SS-RSRQ measurements of the serving cell using at least 2 measurement samples. Within the set of measurements used for the filtering, at least two measurement samples shall be spaced by LO-periodicity/2.

Table 4.8.2.2.2-1: Tevaluate-LP-WUR-PSS/SSS for FR1 and FR2

| LO periodicity [s] Note 1 | Scaling Factor (NLP-WUS) |  | Tevaluate-LP-WUR-PSS/SSS [s] (number of LO Cycles) |
| --- | --- | --- | --- |
|  | FR1 | FR2 |  |
| 0.32 | 1 | 8 | 1.28s x NLP-WUS (4 x NLP-WUS) |
| 0.64 |  | 5 | 2.56s x NLP-WUS (4 x NLP-WUS) |
| 1.28 |  | 4 | 5.12s x NLP-WUS (4 x NLP-WUS) |
| 2.56 |  | 3 | 10.24s x NLP-WUS (4 x NLP-WUS) |
| Note 1: The LO periodicity is the same as the configured DRX cycle length |  |  |  |

The UE shall evaluate and consider an exit criteria is fulfilled within Tevaluate-LP-WUR-PSS/SSS, provided that the corresponding criteria is met by a margin of 6 dB for SS-RSRP and/or 3.5 dB for SS-RSRQ in FR1 and by a margin of 7.5 dB for SS-RSRP and/or 3.5 dB for SS-RSRQ in FR2 when SSB Ês/Iot ≥ -3dB

The UE shall evaluate and consider an entry criteria is fulfilled provided that the corresponding criteria is met by a margin of 6 dB for SS-RSRP and/or 3.5 dB for SS-RSRQ in FR1 and by a margin of 7.5 dB for SS-RSRP and/or 3.5 dB for SS-RSRQ in FR2 when SSB Ês/Iot ≥ -3dB

Upon fulfilling a configured entry or exit condition, the UE shall perform corresponding actions as defined in clause 5.2.4.12 in TS 38.304 [1].

The requirements in this clause apply for UE which supports lpwus-OFDM-r19 and measures PSS/SSS.

4.8.2.2.3	LP-WUR measurement and evaluation requirements for LP-SS

Upon meeting the entry conditions for LP-WUS monitoring, RRM offloading or RRM relaxation, the UE shall measure the LP-RSRP and LP-RSRQ level once every LP-SS cycle and evaluate whether one or more of the following conditions defined in TS 38.304 [1] are met within Tevaluate-LP-WUR-LP-SS

-	exit condition for LP-WUS monitoring

-	exit condition for RRM offloading

-	condition for exiting RRM relaxation mode

The UE shall filter the LP-SS measurements of the serving cell using at least 2 measurement samples.

Table 4.8.2.2.3-1: Tevaluate-LP-WUR-LP-SS

| LP-SS periodicity [s] | Tevaluate-LP-WUR-LP-SS [s] (number of LP-SS Cycles) |
| --- | --- |
| 0.16 | 0.96s (6) |
| 0.32 | 1.92s (6) |

The UE shall evaluate and consider an exit criteria is fulfilled within Tevaluate-LP-WUR-LP-SS, provided that the criteria is met by a margin of 6 dB for LP-RSRP and/or 3.5 dB for LP-RSRQ in FR1 when LP-SS Ês/Iot ≥ -3dB.

The UE shall evaluate and consider an entry criteria is fulfilled provided that the criteria is met by a margin of 6 dB for LP-RSRP and/or 3.5 dB for LP-RSRQ in FR1 when LP-SS Ês/Iot ≥ -3dB.

Upon fulfilling a configured entry or exit condition, the UE shall perform corresponding actions as defined in clause 5.2 in TS 38.304 [1].

The requirements in this clause apply for UE which supports lpwus-OOK-r19, or UE which supports lpwus-LP-SS-r19 and measures only LP-SS.

Note:	Higher margin may be required for UE to consider an entry or exit criteria fulfilled when LP-RSRP and LP-RSRQ thresholds are configured, if the serving cell and neighbour cell are using opposite binary sequences for LP-SS, and LP-SS are transmitted on the same time and frequency resource in the two cells.

<End of Change 1>