3GPP TSG-RAN WG4 Meeting #119                                                         R4-2607701

Dalian, CHINA, May 18 - 22, 2026

Agenda item:			7.1.1

Source:	Moderator (Nokia), Skyworks

Title:	Way Forward for [119][340] NR_UE_RF_Ph5

Document for:	Approval

# 4Tx with up to 32 dBm MOP

Agreements:

‘txDiversity4Tx-r18’ shall be used to indicate 4Tx 32dBm HPUE

Regarding DPPowerClass use 3dB for ‘t2rN’ and 6 dB for ‘t1rN’ in SRS transmission with ‘antennaSwitching’.

  - Regarding DTRxSRS, reuse the R18 framework 4Tx framework.

  - Modify the LCRB,edge to 8 and fix RBedge,start to 0 for PC1.

  - MPR listed in Table 1 is agreed

    - For edge allocations, the listed values in square brackets are the agreed upper bound.

    - Comeback in RAN4#120 regarding edge allocations MPR values.

Table 1: MPR for 32dBm with 4Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ [9.0] | ≤ 3.0 | ≤ 1.5 |
|  | QPSK | ≤ [9.5] | ≤ 3.5 | ≤ 1.5 |
|  | 16 QAM | ≤ [9.5] | ≤ 4.5 | ≤ 2.5 |
|  | 64 QAM | ≤ [9.5] | ≤ 5.0 | ≤ 4.5 |
|  | 256 QAM | ≤ [9.5] | ≤ 7.0 | ≤ 7.0 |
| CP-OFDM | QPSK | ≤ [9.5] | ≤ 5.0 | ≤ 3.0 |
|  | 16 QAM | ≤ [9.5] | ≤ 5.5 | ≤ 3.5 |
|  | 64 QAM | ≤ [9.5] | ≤ 6.0 | ≤ 6.0 |
|  | 256 QAM | ≤ [10.0] | ≤ 9.0 | ≤ 9.0 |
| NOTE 1: This table is targeted to devices with 15 dB or above antenna isolation. |  |  |  |  |

# NSs work split

Following work split is agreed.

| NS | objective | Company name |
| --- | --- | --- |
| NS_04 | PC1/n41 | Qualcomm |
| NS_47 | NS_41 is not required as PC1 operation is not permitted. |  |
| NS_03 | PC1.5/n25 | Apple |
| NS_05 | PC1.5/n1 | Apple |
| NS_48 | PC1.5/n1 | Huawei |
| NS_49 | PC1.5/n1 | Qualcomm |
| NS_03U | Not needed/No request |  |
| NS_05U | Not needed/No request |  |
| NS_100 | PC1.5/n1/n25 | Apple |