3GPP TSG RAN WG4 Meeting #116-bis	R4-2513013

Prague, Czech, 13 - 17 Oct, 2025

3GPP TSG RAN WG2 Meeting #131              	                	                          R2-2506470

Bangalore, India  Aug 25th – 29th , 2025

Title:	LS on candidate data collection

Release:	Release 19

Work Item:	NR_AIML_air-Core

Source:	RAN2

To:	RAN1

Cc:	RAN4

Contact person:	Ziyi Li

liziyi5@xiaomi.com

Andra Mihaela Voicu

andra.mihaela.voicu@ERICSSON.COM

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

Overall description

For AI/ML based beam management and CSI prediction UE-side data collection, before UE is configured with CSI-ReportConfig for UE-side data collection measurement, RAN2 agreed to allow UE to indicate its preferred UE-side data collection configuration(s) based on the candidate UE-side data collection configuration(s) provided by network.

RAN2 also agreed that UE is NOT expected to perform measurement(s) solely based on those candidate UE-side data collection configuration(s) and this UE behavior will be captured in RRC specification, i.e., no RAN1 specification impact.

For AI/ML based beam management, according to RAN1 agreement on UE-side data collection, for each candidate UE-side data configuration, RAN2 agreed at least the following IEs are included:

- An identifier of the candidate configuration

- CSI-ResourceConfigId for Set A

- CSI-ResourceConfigId for Set B

- Related associated IDs

In addition, RAN2 agreed the same framework of candidate/preferred UE-side data collection configuration is also applicable to CSI prediction. However, RAN2 wonders what IEs are needed for CSI prediction as part of candidate UE-side data collection configuration.

Action

To RAN1

ACTION:

RAN2 respectfully asks RAN1 if there’s any concern on the above agreements and provide the necessary higher layer parameters for candidate UE-side data collection configuration for CSI prediction and for AI/ML beam management if any additional IE is missing.

Dates of the next TSG RAN WG2 meetings

TSG RAN2 Meeting #131bis 	13th – 17th Oct, 2025	Prague, CZ

TSG RAN2 Meeting #132 	17th – 21st Nov, 2025	Dallas, US

Annex of RAN2 agreements on candidate UE-side data collection configuration

RAN2 #129bis

- The UE can request measurement configuration for data collection of AI/ML based beam management. The request can contain one or more of the following: • An indication on start/stop of data collection • Preferred configuration from a list of candidate configurations provided by NW. Details of signaling are FFS. It is up to network what it configures at the end. - Introduce UAI message for UE request of data collection measurement configuration. And it is up to UE implementation when to send the request.

RAN2 #130

The UE doesn’t need to measure the candidate data collection configuration(s). This will be specified in RAN2 specs. For beam management, candidate data collection configuration includes at least: - CSI-ResourceConfigId of Set A - CSI-ResourceConfigId of Set B - One/two associated IDs (up to whether Set B is equal/subset of Set A or not) according to RAN1 agreements FFS the details of how this is signalled (e.g. CSIReport config or simplified signaling)

RAN2 #131

Agreements on UE candidate data collection 1 Multiple preferred configurations within the list of candidate configurations provided by NW can be indicated by the UE via UAI. 2 No prohibit timer is needed for UE indicating its preferred data collection configuration. 3 On stop/start indication - The UE can send start indication (without a preferred list) to indicate preference to start data collection - The UE can send preferred list implying that it would like to start data collection on those configuration - The UE can send stop indication for all or a given actual CSI report config ID. - Rapporteur will determine best way of signaling. This doesn’t preclude merging 1 and 2. - Adopt below text in the field description of dataCollectionCandidateConfig: The UE is not expected to perform measurements solely based on the configurations provided by this IE. - Adopt the following solution: OtherConfig contains a list of candidate configurations as a list of a new IE, where each candidate configuration contains at least an identifier of the candidate configuration, CSI-ResourceConfigId for Set A, CSI-ResourceConfigId for Set B, and related associated IDs, as agreed in RAN2#130. Each candidate configuration is associated with a cell ID. We will also include individual IEs for CSI prediction case. - Ask RAN1 what IEs are needed for CSI prediction and inform them of our agreements on BM and confirm if anything else is needed.