3GPP TSG-RAN5 Meeting #108	R5-253982

# Bengaluru, India, 25th August 2025 – 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3729 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR ATG: Initial Test Model |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF 160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The test model for the ATG network is not specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the definition of the ATG nework test model |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The definition of the ATG network test model will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

7.6	Air to Ground Network

### 7.6.1	ATG cell configuration

An NR cell to be used in an Air-To-Ground (ATG) Network test environment is an NR cell configured with ATG-specific configuration parameters. It is called an ATG cell.

In particular an ATG cell broadcasts a system information block combination, defined in TS 38.508-1 [5] clause 4.4.3.1.2, that contains SIB22.

7.6.2	ATG timing considerations

#### 7.6.2.1	General

DL and UL are frame aligned at the ground based gNB. To accommodate the propagation delays in ATG environment and to provide ATG UE enough processing time for the uplink transmission and downlink reception, a cell-specific offset: Koffset  with Koffset ≥ TTA is configured for the ATG cell. The actual TTA is calculated to integrate the compensation of the two-way propagation delay between the ATG UE and the gNB, which based on the GNSS position of the ATG UE and the gNB location parameters.

The ATG timing relationships are illustrated in Figure 7.6.2.1-1.

Figure 7.6.2.1-1: ATG timing relationship parameters

#### 7.6.2.2	ATG timing advance

The Timing Advance (TA) formula, specified in TS 38.211 [19] clause 4.3.1, can be summarized as follows:

TTA = TTA,legacy + TTA,ATG

with:

-	TTA,legacy = (NTA + NTA,offset) * Tc: the (legacy) part of the TA formula that applies also to Terrestrial Networks,

-	TTA,ATG = NTA,adjUE * Tc: the TA corresponding to a two-way propagation delay between the ATG UE and the gNB.

The time unit TC is defined in TS 38.211 [19] clause 4.1.

The ATG test environment in TS 38.508-1 [5] specifies that both the ATG UE and the gNB under test have a static position during the lifetime of a test case execution. Therefore, the corresponding TTA is also static for a given ATG cell. TTCN provides to the SS the following inputs to enable the SS to calculate TTA:

Table 7.6.2.2-1: ATG timing advance - input parameters

| TTA element | Associated parameter(s) |  | Relationship |
| --- | --- | --- | --- |
| (in sec) | Protocol parameter | ASP field | (TS 38.213 [21] clause 4.2) |
| TTA,legacy |  | InitialValue integer (0..3846) | NTA = InitialValue * 16∙642μ |
| TTA,ATG | NTA,adjUE | AtgUeSpecificTA integer (0..3846) | NTA,adjUE= AtgUeSpecificTA * 16∙642μ |

NTA,adjUE is calculated using the gNB static position specified in TS 38.508-1 [5] SIB22's reference location and height and in TS 38.508-1 [5] clause 4.5D1 specified static position of the ATG UE.  The end result is provided to the SS by TTCN. The calculation is provided hereafter:

The two-way propagation delay in sec between the gNB and the non-moving ATG UE is assumed to be constant at any time and is calculated according to the formula:

t=2*d/c

where c is the speed of light (299792458 m/sec) and d is the distance in meters between the gNB and the ATG UE.

For two points in Euclidian geometry d in meters is derived by:

d=∆x+∆y+∆z

with

∆x=(xUE-xgNB)2

∆y= (yUE-ygNB)2

∆z=(zUE-zgNB)2

and

xgNB, ygNB, zgNB being gNB coordinates, in m

xUE, yUE, zUE being ATG UE coordinates, in m

As a consequence of TS 38.508-1 [5] specifying an identical geographical position of the ATG UE and the gNB, the actual distance between them can be reduced to a difference of the corresponding altitudes. Thus

d=zUE-zgNB=10000 m-25 m=9975 m

After completing the above calculations, the final value t results in 0.00001984706366429005 sec that yields AtgUeSpecificTA to have a value of 127.76839102470015, that is rounded to 128.

#### 7.6.2.3	ATG scheduling offsets

The ATG timing parameters listed in Table 7.6.2.3-1 come into play when scheduling data in an ATG cell.

Table 7.6.2.3-1: ATG timing parameters in an ATG cell

| ATG timing parameter | Definition | Reference |
| --- | --- | --- |
| Koffset | cellSpecificKoffset - Differential Koffset | TS 38.213 [21] clause 4.2 |

TTCN configures the ATG Koffset in the SS as part of the ATG cell configuration. The SS shall use those parameters to adjust the timing & scheduling of DL & UL.

#### 7.6.2.4	RRC connection release sequence

The sequence specified in clause 7.3.5.2 applies, the delay on the link between the ATG UE and gNB is negligible and is not considered.