3GPP TSG-RAN5 Meeting #108	R5-254067

# Bengaluru, India, 25th - 29th August, 2025

Agenda item:	5.3.13.9

Source:	Keysight Technologies UK Ltd

Title:	Discussion on MU and TT for ATG RF testing

Document for:	Discussion and Endorsement

# Introduction

This contribution aims to continue defining the MU and TT for those ATG test cases where the analysis is still pending. MU and TT analysis [1] and [2] made in previous meetings will be used as reference. The RF test cases with MU/TT analysis pending are the next:

- 6.4J.2.1a Error Vector Magnitude including symbols with transient period for ATG

- 6.4J.2.2	 Carrier leakage for ATG (still undefined)

- 6.4J.2.3	 In-band emissions for ATG (still undefined)

- 6.4J.2.4	 EVM equalizer spectrum flatness for ATG

- 6.4J.2.5	 EVM equalizer spectrum flatness for Pi/2 BPSK for ATG

In addition, AP#105.23 was closed in previous meeting clarifying that “expected implementation is 2Tx, 2Rx conveyed” for ATG UEs, regarding the number of omni-directional antennas or, in case of using an antenna array, the number of TAB connectors. This feedback will be analysed to evaluate whether changes would be required in the test specs.

# Discussion

## Test Cases Analysis

#### Error Vector Magnitude including symbols with transient period for ATG

- In the equivalent non-ATG test case 6.4.2.1a, as per agreed proposals in [3], UL power control is used to limit UL power to 11.5dBm in order to avoid the influence of the MPR the UE can apply. However, in the case of ATG, there is no MPR defined. Therefore, if the UE power is set to maximum, as per MU analysis made in [4], when evaluating the maximum UE power variance, aggregate power tolerance and relative power tolerance still will need to be applied due to PRB reallocation and test time duration;, however, the size of the power control window does not need to be considered. Reducing the UE power dynamic range in this test helps to reduce a little bit the required MU. However, as per proposals endorsed in [2], the presence of extra attenuation required to protect the test system, will increase the MU. We have evaluated the performance of our test system in these circumstances and concluded that if UE power is set to maximum power, then MU and TT defined for ATG test case 6.4J.2.1a can be reused from non-ATG test case 6.4.2.1a.

- The previous statement is assuming the EVM in transient period is evaluated per UL antenna/TAB connector, using the same arguments used for general EVM test case for ATG and endorsed proposals made in [1]. In addition, this has already been clarified in latest version of core specs [6] for all transmit signal quality tests under section 6.4J.2.

Proposal 1. In ATG test case 6.4J.2.1a, set UE to maximum power in the test procedure.

Proposal 2. In ATG test case 6.4J.2.1a, update test definition to apply the requirement per UL antenna/TAB connector.

Proposal 3. If previous proposals are endorsed, in ATG test 6.4J.2.1a, define MU and TT same as defined for equivalent non-ATG test 6.4.2.1a.

#### Carrier leakage for ATG

- As previously stated, core specs [6] has clarified that for all transmit signal quality tests under section 6.4J.2, the requirements are defined per UL antenna/TAB connector.

- In that situation and considering the extra attenuation that may be required for ATG devices, we have evaluated the performance of our test system and concluded that MU and TT defined for non-ATG test case 6.4.2.2 can be leveraged for ATG test case 6.4J.2.2. However, as in the EVM test case 6.4.J.2.1, UL power control is used and power configured quite low, it is proposed to limit the MU and TT definition for ATG UEs with no more than 2 transmit antenna connectors/ transceiver array boundary (TAB) connectors.

Proposal 4. For ATG test 6.4J.2.2, for ATG UEs with no more than 2 transmit antenna connectors/ transceiver array boundary (TAB) connectors, define MU and TT same as defined for equivalent non-ATG test 6.4.2.2.

#### In-band emissions for ATG

- Similar to analysis done for test case 6.4J.2.2, we conclude that MU and TT defined for non-ATG test case 6.4.2.3 can be leveraged for ATG test case 6.4J.2.3 for ATG UEs with no more than 2 transmit antenna connectors/ transceiver array boundary (TAB) connectors.

Proposal 5. For ATG test 6.4J.2.3, for ATG UEs with no more than 2 transmit antenna connectors/ transceiver array boundary (TAB) connectors, define MU and TT same as defined for equivalent non-ATG test 6.4.2.3.

#### EVM equalizer spectrum flatness for ATG tests

- Same situation for EVM equalizer spectrum flatness test cases. However, these tests have already been defined before the core specs clarification regarding the applicability of the requirements per UL antenna/TAB connector. Therefore, test definition should be updated accordingly. In this case the UE is set to maximum output power without using any power control window, therefore there is no need to limit the MU and TT depending on the number of antenna/TAB connectors.

Proposal 6. For ATG tests 6.4J.2.4 and 6.4J.2.5, define MU and TT same as defined for equivalent non-ATG tests 6.4.2.4 and 6.4.2.5.

Proposal 7. In ATG test cases 6.4J.2.4 and 6.4J.2.5, update test definition to apply the requirement per UL antenna/TAB connector.

## ATG UE antenna assumption

AP#105.23 was closed in previous meeting clarifying that “expected implementation is 2Tx, 2Rx conveyed” for ATG UEs, regarding the number of omni-directional antennas or, in case of using an antenna array, the number of TAB connectors.

Current MU/TT analysis for ATG has been limited for ATG UEs with up to 2 transmit antenna connectors / transceiver array boundary (TAB) connectors. Based on the expected implementation of ATG UEs, at the moment there is no need to extend the MU/TT analysis beyond 2 antenna/TAB connectors case.

However, core requirements for ATG do not clarify or limit the number of antennas/TAB connectors an ATG UE may have. Even right now it could be 1 or 2 antennas/TAB connectors, based on current expected implementation, and it cannot be discarded to be extended beyond 2 connectors in future. Therefore, we consider it is necessary to define a manufacturer declaration.

Many ATG test cases in the test definition and in the annex F clarify that MU and TT are only defined for ATG UEs with no more than 2 transmit antenna connectors/ transceiver array boundary (TAB) connectors. We propose to keep those clarifications. However, defining the manufacturer declaration is still convenient to help both test case implementation and test system operation to run and measure accordingly to the antennas declared by the UE.

Observation 1. Current MU/TT analysis for ATG has been limited for ATG UEs with up to 2 transmit antenna connectors / transceiver array boundary (TAB) connectors, what is consistent with the expected implementation of ATG UEs. So, at the moment, there is no need to extend the MU/TT analysis beyond the 2 antenna/TAB connectors case.

Observation 2. Core requirements for ATG UEs do not clarify or limit the number of antennas/TAB connectors of an ATG UE. Therefore, test specs need to clarify their current scope in terms of number of antennas.

Proposal 8. Keep current editor’s notes and descriptions in annex F clarifying that currently MU and TT are only defined for ATG UEs with no more than 2 transmit antenna/TAB connectors.

Proposal 9. For ATG UEs, define a manufacturer declaration regarding the number antenna connectors / transceiver array boundary (TAB) connectors implemented by the UE in order to help ATG test cases being implemented and operated accordingly.

# Conclusion

The following observations and proposals were made in this contribution.

Observation 1. Current MU/TT analysis for ATG has been limited for ATG UEs with up to 2 transmit antenna connectors / transceiver array boundary (TAB) connectors, what is consistent with the expected implementation of ATG UEs. So, at the moment, there is no need to extend the MU/TT analysis beyond the 2 antenna/TAB connectors case.

Observation 2. Core requirements for ATG UEs do not clarify or limit the number of antennas/TAB connectors of an ATG UE. Therefore, test specs need to clarify their current scope in terms of number of antennas.

Proposal 1. In ATG test case 6.4J.2.1a, set UE to maximum power in the test procedure.

Proposal 2. In ATG test case 6.4J.2.1a, update test definition to apply the requirement per UL antenna/TAB connector.

Proposal 3. If previous proposals are endorsed, in ATG test 6.4J.2.1a, define MU and TT same as defined for equivalent non-ATG test 6.4.2.1a.

Proposal 4. For ATG test 6.4J.2.2, for ATG UEs with no more than 2 transmit antenna connectors/ transceiver array boundary (TAB) connectors, define MU and TT same as defined for equivalent non-ATG test 6.4.2.2.

Proposal 5. For ATG test 6.4J.2.3, for ATG UEs with no more than 2 transmit antenna connectors/ transceiver array boundary (TAB) connectors, define MU and TT same as defined for equivalent non-ATG test 6.4.2.3.

Proposal 6. For ATG tests 6.4J.2.4 and 6.4J.2.5, define MU and TT same as defined for equivalent non-ATG tests 6.4.2.4 and 6.4.2.5.

Proposal 7. In ATG test cases 6.4J.2.4 and 6.4J.2.5, update test definition to apply the requirement per UL antenna/TAB connector.

Proposal 8. Keep current editor’s notes and descriptions in annex F clarifying that currently MU and TT are only defined for ATG UEs with no more than 2 transmit antenna/TAB connectors.

Proposal 9. For ATG UEs, define a manufacturer declaration regarding the number antenna connectors / transceiver array boundary (TAB) connectors implemented by the UE in order to help ATG test cases being implemented and operated accordingly.

# References

- R5-252136, “Discussion on MU and TT for ATG RF testing”, Keysight, RAN5#107

- R5-246445, “Discussion on MU and TT for ATG RF testing”, Keysight, RAN5#105

- R5-234178, “Discussion on FR1 EVM testing including symbols with transient period”, Keysight, RAN5#100

- R5237859, “On the MU for FR1 EVM including symbols with transient period”, R&S, RAN5#101

- TS 38.521-1, “User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone”, v19.0.0 (2025-06)

- TS 38.101-1, “User Equipment (UE); Radio transmission and reception; Part 1: Range 1 Standalone”, v18.10.0 (2025-06)