3GPP TSG-RAN5 Meeting #108	R5-254240

# Bengaluru, India, 25th - 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 4045 | rev |  | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Annex F correction for NRU Inter-Frequency accuracy test cases Including TT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_unlic-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test Tolerance for NRU Inter-Frequency accuracy test cases is missing in Annex F |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test Tolerance parameters have been added to Annex F |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test tolerance parameters will remain not recorded in Annex F and test cases cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | F.1.1.2, F.1.3.2. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TR 38.903 CR 1058, 1059 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RRM TT. Depends on R4-2509740 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

### F.1.1.2	Measurement of RRM requirements

# < Unchanged sections omitted >

Table F.1.1.2-13: Maximum test system uncertainty for RRM requirements for SA FR1 test cases for shared spectrum

| Subclause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 11.1.1.1 NR SA FR1 Cell reselection to FR1 intra-frequency NR cells when subject to CCA on the serving and target cell | Noc ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 11.1.1.2 NR SA FR1 Cell reselection to FR1 inter-frequency NR case when subject to CCA on the serving and target cell | Noc1 ±1.5 dB Ês1 / Noc1 ±0.3 dB Noc2 ±1.5 dB Ês2 / Noc2 ±0.3 dB | Note: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 11.1.2.1 Cell re-selection to NR with source NR carrier frequency under CCA | Same as 11.1.1.2 | Same as 11.1.1.2 |
| 11.1.3.1 NR SA FR1 Cell re-selection to NR with source NR carrier frequency under CCA | Same as 11.1.1.2 | Same as 11.1.1.2 |
| 11.1.4.1 NR SA FR1 Cell reselection to higher priority E-UTRAN when serving cell is subject to CCA | Noc1 ±1.5 dB Ês1 / Noc1 ±0.3 dB Noc2 ±1.5 dB Ês2 / Noc2 ±0.3 dB | Note: Noc1 is the AWGN on cell 1 (NR) frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 (E-UTRAN) frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 11.1.4.2 NR SA FR1 Cell reselection to lower priority E-UTRAN when serving cell is subject to CCA | Same as 11.1.4.1 | Same as 11.1.4.1 |
| 11.2.1.1 NR SA FR1 Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; known target cell | Average Noc ±1.5 dB Average Ês1 / Noc ±0.3 dB Average Ês2 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês1 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 11.2.1.2 NR SA FR1 Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell | Same as 11.2.1.1 | Same as 11.2.1.1 |
| 11.2.1.3 NR SA FR1 Inter-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell | Noc1 ±1.5 dB Ês1 / Noc1 ±0.3 dB Noc2 ±1.5 dB Ês2 / Noc2 ±0.3 dB | Note: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 11.2.1.4 NR SA FR1 Inter-frequency handover from FR1 carrier under CCA to FR1; known target cell | Same as 11.2.1.3 | Same as 11.2.1.3 |
| 11.2.1.5 NR SA FR1 Inter-frequency handover from FR1 carrier under CCA to FR1; unknown target cell | Same as 11.2.1.3 | Same as 11.2.1.3 |
| 11.2.1.6 NR SA FR1 Inter-frequency handover from FR1 to FR1 carrier under CCA; unknown target cell | Same as 11.2.1.3 | Same as 11.2.1.3 |
| 11.2.1.7 NR SA FR1 carrier under CCA - E-UTRAN handover with known target cell | Same as 11.1.4.1 | Same as 11.1.4.1 |
| 11.2.1.8 NR SA FR1 carrier under CCA - E-UTRAN handover with unknown target cell | Same as 11.1.4.1 | Same as 11.1.4.1 |
| 11.2.2.1.1 NR SA FR1 Intra-frequency RRC Re-establishment with CCA in FR1 | Same as 11.1.1.1 | Same as 11.1.1.1 |
| 11.2.2.1.2 NR SA FR1 Inter-frequency RRC Re-establishment with CCA in FR1 | Same as 11.1.1.2 | Same as 11.1.1.2 |
| 11.2.2.1.3 NR SA FR1 Intra-frequency RRC Re-establishment with CCA without serving cell timing | Same as 11.1.1.1 | Same as 11.1.1.1 |
| 11.2.2.1.4 NR SA FR1 Inter-frequency RRC Re-establishment from NR FR1 carrier without CCA to NR FR1 carrier under CCA | Same as 11.1.1.2 | Same as 11.1.1.2 |
| 11.2.2.2.1 NR SA FR1 4-step RA type contention-based random access for NR PCell with CCA | Same as 10.1.1.1.1 | Same as 10.1.1.1.1 |
| 11.2.2.2.2 NR SA FR1 4-step RA type non-contention based random access for NR PSCell with CCA | Same as 10.1.1.1.1 | Same as 10.1.1.1.1 |
| 11.2.2.2.3 NR SA FR1 2-step RA type contention-based random access for NR PCell with CCA | Same as 10.1.1.1.1 | Same as 10.1.1.1.1 |
| 11.2.2.2.4 NR SA FR1 2-step RA type non-contention-based random access for NR PCell with CCA | Same as 10.1.1.1.1 | Same as 10.1.1.1.1 |
| 11.2.2.3.1 NR SA FR1 Redirection from NR FR1 carrier under CCA to NR FR1 carrier under CCA | Same as 11.2.2.3.2 | Same as 11.2.2.3.2 |
| 11.2.2.3.2 NR SA FR1 Redirection from NR FR1 carrier without CCA to NR FR1 carrier under CCA | Noc1 ±1.5 dB Ês1 / Noc1 ±0.3 dB Noc2 ±1.5 dB Ês2 / Noc2 ±0.3 dB | Note: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 11.3.1.1 NR SA FR1 UE Transmit Timing Test with PCell under DL CCA | Same as 6.4.1.1 | Same as 6.4.1.1 |
| 11.3.2.1 NR SA FR1 UE Timing Advance Adjustment Accuracy with PCell under DL CCA | Same as 6.4.3.1 | Same as 6.4.3.1 |
| 11.4.1.2 NR SA FR1 Radio link monitoring out-of-sync test for PCell under CCA configured with SSB-based RLM-RS in non-DRX mode | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 11.4.1.3 NR SA FR1 Radio link monitoring in-sync test for PCell under CCA configured with SSB-based RLM-RS in non-DRX mode | Same as 6.5.1.2 | Same as 6.5.1.2 |
| 11.4.3.1 NR SA FR1 SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 160 ms SCell measurement cycle | Same as 6.5.3.1 | Same as 6.5.3.1 |
| 11.4.3.2 NR SA FR1 SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 640 ms SCell measurement cycle | Same as 6.5.3.1 | Same as 6.5.3.1 |
| 11.4.3.3 NR SA FR1 SCell Activation and Deactivation of unknown SCell with PCell and SCell under CCA | Same as 6.5.3.1 | Same as 6.5.3.1 |
| 11.4.4.1 NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell under CCA configured with SSB-based BFD and LR in non-DRX mode | Same as 6.5.5.1 | Same as 6.5.5.1 |
| 11.4.4.2 NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell under CCA configured with SSB-based BFD and LR in DRX mode | Same as 6.5.5.1 | Same as 6.5.5.1 |
| 11.4.5.1 NR SA FR1 UL active BWP switch delay with consistent UL LBT failure on PCell subject to UL CCA | 1.5dB | Measurement uncertainty for OCNG noise as defined in Table F.1.1.2-1 |
| 11.4.5.2.1 NR SA FR1 - NR FR1 DL active BWP switch of PCell under CCA with non-DRX | Same as 6.5.6.1.1 | Same as 6.5.6.1.1 |
| 11.4.5.2.2 NR SA FR1 DCI-based DL active BWP switch with non-DRX under CCA | Same as 6.5.6.1.1 | Same as 6.5.6.1.1 |
| 11.4.5.3.1 NR SA FR1 DL active BWP switch with non-DRX | Same as 6.5.6.1.1 | Same as 6.5.6.1.1 |
| 11.5.1.1 SA FR1 Event-triggered reporting tests on PCC without gaps under non-DRX | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 11.5.1.2 SA FR1 Event-triggered reporting tests on PCC without gaps under DRX | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 11.5.2.3 NR SA FR1-FR1 Event triggered reporting tests with CCA without SSB time index detection when DRX is not used | Same as 10.4.2.3 | Same as 10.4.2.3 |
| 11.5.2.4 NR SA FR1-FR1 Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used | Same as 10.4.2.3 | Same as 10.4.2.3 |
| 11.5.2.5 NR SA FR1-FR1 Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used | Same as 10.4.2.3 | Same as 10.4.2.3 |
| 11.5.2.6 NR SA FR1-FR1 Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used | Same as 10.4.2.3 | Same as 10.4.2.3 |
| 11.5.2.7 NR SA FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used | Same as 10.4.2.3 | Same as 10.4.2.3 |
| 11.5.2.8 NR SA FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is used | Same as 10.4.2.3 | Same as 10.4.2.3 |
| 11.5.2.9 NR SA FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used | Same as 10.4.2.3 | Same as 10.4.2.3 |
| 11.5.2.10 NR SA FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is used | Same as 10.4.2.3 | Same as 10.4.2.3 |
| 11.5.3.1 NR SA FR1 - E-UTRAN event-triggered reporting in non-DRX | Same as 6.6.3.1 | Same as 6.6.3.1 |
| 11.5.3.2 NR SA FR1 - E-UTRAN event-triggered reporting in DRX | Same as 6.6.3.1 | Same as 6.6.3.1 |
| 11.5.4.1 NR SA FR1 SSB based L1-RSRP measurement when DRX is not used | Same as 6.6.4.1 | Same as 6.6.4.1 |
| 11.5.4.2 NR SA FR1 SSB based L1-RSRP measurement when DRX is used | Same as 6.6.4.1 | Same as 6.6.4.1 |
| 11.5.4.3 NR SA FR1 SSB based L1-RSRP measurement on SCC under CCA when DRX is not used | Same as 6.6.4.1 | Same as 6.6.4.1 |
| 11.5.4.4 NR SA FR1 SSB based L1-RSRP measurement on SCC under CCA when DRX is used | Same as 6.6.4.1 | Same as 6.6.4.1 |
| 11.6.1.1 NR SA FR1 intra-frequency SS-RSRP measurement accuracy on a carrier frequency with CCA | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 |
| 11.6.1.2 NR SA FR1 Intra-frequency measurement accuracy on SCC on a carrier frequency with CCA | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 |
| 11.6.2.1 NR SA FR1 intra-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA | Same as 6.7.2.1 | Same as 6.7.2.1 |
| 11.6.2.2 NR SA FR1 inter-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA | Same as 6.7.2.2.1 | Same as 6.7.2.2.1 |
| 11.6.2.3 NR SA FR1 Intra-frequency measurement accuracy on SCC | Same as 10.5.2.1 | Same as 10.5.2.1 |
| 11.6.2.4 NR SA FR1 Inter-frequency measurement accuracy | Same as 6.7.2.2.1 | Same as 6.7.2.2.1 |
| 11.6.3.1 NR SA FR1 Intra-frequency SS-SINR measurement accuracy on a carrier frequency with CCA | Same as 10.5.3.1 | Same as 10.5.3.1 |
| 11.6.3.2 NR SA FR1 Inter-frequency SS-SINR measurement accuracy on a carrier frequency with CCA | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB meas PRB Ês1 / Noc1 ±0.8 dB meas PRB Ês2 / Noc2 ±0.8 dB | Noc1 is the AWGN on NR freq1 Noc2 is the AWGN on NR freq2 Ês1 / Noc1 is the cell 2 SNR Ês2 / Noc2 is the cell 3 SNR meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 11.6.3.3 NR SA FR1 Intra-frequency SS-SINR measurement accuracy on SCC on a carrier frequency with CCA | Same as 10.5.3.1 | Same as 10.5.3.1 |
| 11.6.3.4 NR SA FR1 Intra-frequency SS-SINR measurement accuracy on SCC on a carrier frequency with CCA | Same as 11.6.3.2 | Same as 11.6.3.2 |
| 11.6.4.1 NR SA FR1 SSB based L1-RSRP measurement accuracy with CCA | Same as 10.5.4.1 | Same as 10.5.4.1 |
| 11.6.5.1 NR SA FR1 Intra-frequency RSSI measurement accuracy on PCC with CCA | Average Noc ±1.5 dB | Noc is the AWGN on NR freq1 |
| 11.6.5.2 NR SA FR1 Intra-frequency RSSI measurement accuracy on SCC with CCA | Same as 11.6.5.1 | Same as 11.6.5.1 |
| 11.6.5.3 NR SA FR1 Inter-frequency RSSI measurement accuracy on a carrier with CCA | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB | Noc1 is the AWGN on NR freq1 Noc2 is the AWGN on NR freq2 |
| 11.6.6.1 NR SA FR1 Intra-frequency channel occupancy measurement accuracy on PCC with CCA | Average Noc ±1.5 dB | Noc is the AWGN on NR freq1 |
| 11.6.6.2 NR SA FR1 Intra-frequency channel occupancy measurement accuracy on SCC with CCA | Same as 11.6.6.1 | Same as 11.6.6.1 |
| 11.6.6.3 NR SA FR1 Inter-frequency channel occupancy measurement accuracy on a carrier with CCA | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB | Noc1 is the AWGN on NR freq1 Noc2 is the AWGN on NR freq2 |

# < Unchanged sections omitted >

### F.1.3.2	Measurement of RRM requirements

# < Unchanged sections omitted >

Table F.1.3.2-11: Derivation of test requirements for SA FR1 RRM tests for shared spectrum

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 11.1.1.1 NR SA FR1 Cell reselection to FR1 intra-frequency NR cells when subject to CCA on the serving and target cell | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +16dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: +13dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0.45dB During T3: 0dB 0.45dB 0dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +16.45dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +16.45dB Ês2 / Noc: +13dB |
| 11.1.1.2 NR SA FR1 Cell reselection to FR1 inter-frequency NR case when subject to CCA on the serving and target cell | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -4dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: +12dB | During T1: 0dB -2dB 1.6dB 0.4dB During T2: 0dB 0dB 1.6dB 0dB During T3: 0dB 0dB 1.6dB 1.6dB | During T1: Noc1: -98dBm/15kHz Noc2: -100dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: -3.6dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: 13.6dB |
| 11.1.2.1 Cell re-selection to NR with source NR carrier frequency under CCA | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -4dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: +12dB | During T1: 0dB -2dB 1.6dB 0.4dB During T2: 0dB 0dB 1.6dB 0dB During T3: 0dB 0dB 1.6dB 1.6dB | During T1: Noc1: -98dBm/15kHz Noc2: -100dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: -3.6dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: 13.6dB |
| 11.1.3.1 NR SA FR1 Cell re-selection to NR with source NR carrier frequency under CCA | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -4dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: +12dB | During T1: 0dB -2dB 1.6dB 0.4dB During T2: 0dB 0dB 1.6dB 0dB During T3: 0dB 0dB 1.6dB 1.6dB | During T1: Noc1: -98dBm/15kHz Noc2: -100dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: -3.6dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: 13.6dB |
| 11.1.4.1 NR SA FR1 Cell reselection to higher priority E-UTRAN when serving cell is subject to CCA | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: +12dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -4dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 1.6dB 1.6dB During T3: 0dB -2dB 1.6dB 0.4dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: 13.6dB During T3: Noc1: -98dBm/15kHz Noc2: -100dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: -3.6dB |
| 11.1.4.2 NR SA FR1 Cell reselection to lower priority E-UTRAN when serving cell is subject to CCA | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -4dB Ês2 / Noc2: +14dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +12dB Ês2 / Noc2: +14dB | During T1: -2dB 0dB 0.4dB 1.6dB During T2: 0dB 0dB 1.6dB 0dB | During T1: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -3.6dB Ês2 / Noc2: +15.6dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +13.6dB Ês2 / Noc2: +14dB |
| 11.2.1.1 NR SA FR1 Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; known target cell | During T1: Noc: -98dBm/15kHz Ês1 / Noc1: 8dB Ês2 / Noc2: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +11dB During T3: Noc: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +11dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB -1dB During T3: 0dB 0dB -1dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc1: 8dB Ês2 / Noc2: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +10dB During T3: Noc: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +10dB |
| 11.2.1.2 NR SA FR1 Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell | During T1: Noc: -98dBm/15kHz Ês1 / Noc: 8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +8dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: 8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +8dB |
| 11.2.1.3 NR SA FR1 Inter-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +5dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +5dB |
| 11.2.1.4 NR SA FR1 Inter-frequency handover from FR1 carrier under CCA to FR1; known target cell | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 8dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +11dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +11dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB -1dB During T3: 0dB 0dB 0dB -1dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 8dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +10dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +10dB |
| 11.2.1.5 NR SA FR1 Inter-frequency handover from FR1 carrier under CCA to FR1; unknown target cell | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +5dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +5dB |
| 11.2.1.6 NR SA FR1 Inter-frequency handover from FR1 to FR1 carrier under CCA; unknown target cell | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +5dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +5dB |
| 11.2.1.7 NR SA FR1 carrier under CCA - E-UTRAN handover with known target cell | During T1: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +12dB Ês2 / Noc2: -infinity During T2: Noc1: -104dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 0dB Ês2 / Noc2: +8dB During T3: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 0dB Ês2 / Noc2: +8dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 1.55dB During T3: 0dB 0dB 0dB 1.55dB | During T1: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +12dB Ês2 / Noc2: -infinity During T2: Noc1: -104dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 0dB Ês2 / Noc2: +9.55dB During T3: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 0dB Ês2 / Noc2: +9.55dB |
| 11.2.1.8 NR SA FR1 carrier under CCA - E-UTRAN handover with unknown target cell | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 0dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 0dB Ês2 / Noc2: +7dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 0dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 0dB Ês2 / Noc2: +7dB |
| 11.2.2.1.1 NR SA FR1 Intra-frequency RRC Re-establishment with CCA in FR1 | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +7dB Ês2 / Noc: +4dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB During T3: 0dB 0dB 0dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +7dB Ês2 / Noc: +4dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB |
| 11.2.2.1.2 NR SA FR1 Inter-frequency RRC Re-establishment with CCA in FR1 | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: +4dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -infinity Ês2 / Noc2: +4dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -infinity Ês2 / Noc2: +4dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: +4dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -infinity Ês2 / Noc2: +4dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -infinity Ês2 / Noc2: +4dB |
| 11.2.2.1.3 NR SA FR1 Intra-frequency RRC Re-establishment with CCA without serving cell timing | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: -infinity During T3: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB During T3: 0dB 0dB 0dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: -infinity During T3: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB |
| 11.2.2.1.4 NR SA FR1 Inter-frequency RRC Re-establishment from NR FR1 carrier without CCA to NR FR1 carrier under CCA | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -infinity Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -infinity Ês2 / Noc2: +7dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -infinity Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -infinity Ês2 / Noc2: +7dB |
| 11.2.2.2.1 NR SA FR1 4-step RA type contention-based random access for NR PCell with CCA | Same as 10.1.1.1.1 | Same as 10.1.1.1.1 | Same as 10.1.1.1.1 |
| 11.2.2.2.2 NR SA FR1 4-step RA type non-contention based random access for NR PSCell with CCA | Same as 10.1.1.1.1 | Same as 10.1.1.1.1 | Same as 10.1.1.1.1 |
| 11.2.2.2.3 NR SA FR1 2-step RA type contention-based random access for NR PCell with CCA | Same as 10.1.1.1.1 | Same as 10.1.1.1.1 | Same as 10.1.1.1.1 |
| 11.2.2.2.4 NR SA FR1 2-step RA type non-contention-based random access for NR PCell with CCA | Same as 10.1.1.1.1 | Same as 10.1.1.1.1 | Same as 10.1.1.1.1 |
| 11.2.2.3.1 NR SA FR1 Redirection from NR FR1 carrier under CCA to NR FR1 carrier under CCA | Same as 11.2.2.3.2 | Same as 11.2.2.3.2 | Same as 11.2.2.3.2 |
| 11.2.2.3.2 NR SA FR1 Redirection from NR FR1 carrier without CCA to NR FR1 carrier under CCA | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +4 dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +4 dB |
| 11.3.1.1 NR SA FR1 UE Transmit Timing Test with PCell under DL CCA | Same as 6.4.1.1 | Same as 6.4.1.1 | Same as 6.4.1.1 |
| 11.3.2.1 NR SA FR1 UE Timing Advance Adjustment Accuracy with PCell under DL CCA | Same as 6.4.3.1 | Same as 6.4.3.1 | Same as 6.4.3.1 |
| 11.4.1.2 NR SA FR1 Radio link monitoring out-of-sync test for PCell under CCA configured with SSB-based RLM RS in non-DRX mode | Same as 10.3.1.2 | Same as 10.3.1.2 | Same as 10.3.1.2 |
| 11.4.1.3 NR SA FR1 Radio link monitoring in-sync test for PCell under CCA configured with SSB-based RLM RS in non-DRX mode | Same as 10.3.1.3 | Same as 10.3.1.3 | Same as 10.3.1.3 |
| 11.4.3.1 NR SA FR1 SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 160 ms SCell measurement cycle | Same as 10.3.3.1 | Same as 10.3.3.1 | Same as 10.3.3.1 |
| 11.4.3.2 NR SA FR1 SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 640 ms SCell measurement cycle | Same as 10.3.3.1 | Same as 10.3.3.1 | Same as 10.3.3.1 |
| 11.4.3.3 NR SA FR1 SCell Activation and Deactivation of unknown SCell with PCell and SCell under CCA | Same as 10.3.3.1 | Same as 10.3.3.1 | Same as 10.3.3.1 |
| 11.4.4.1 NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell under CCA configured with SSB-based BFD and LR in non-DRX mode | Same as 10.3.4.1 | Same as 10.3.4.1 | Same as 10.3.4.1 |
| 11.4.4.2 NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell under CCA configured with SSB-based BFD and LR in DRX mode | Same as 10.3.4.1 | Same as 10.3.4.1 | Same as 10.3.4.1 |
| 11.4.5.1 NR SA FR1 UL active BWP switch delay with consistent UL LBT failure on PCell subject to UL CCA | Same as 10.3.5.1 | Same as 10.3.5.1 | Same as 10.3.5.1 |
| 11.4.5.2.1 NR SA FR1 - NR FR1 DL active BWP switch of PCell under CCA with non-DRX | Same as 10.3.5.2.1 | Same as 10.3.5.2.1 | Same as 10.3.5.2.1 |
| 11.4.5.2.2 NR SA FR1 DCI-based DL active BWP switch with non-DRX under CCA | Same as 10.3.5.2.1 | Same as 10.3.5.2.1 | Same as 10.3.5.2.1 |
| 11.4.5.3.1 NR SA FR1 DL active BWP switch of Cell with non-DRX | Same as 10.3.5.3.1 | Same as 10.3.5.3.1 | Same as 10.3.5.3.1 |
| 11.5.1.1 SA FR1 Event-triggered reporting tests on PCC without gaps under non-DRX | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 11.5.1.2 SA FR1 Event-triggered reporting tests on PCC without gaps under DRX | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 11.5.2.3 NR SA FR1-FR1 Event triggered reporting tests with CCA without SSB time index detection when DRX is not used | Same as 10.4.2.3 | Same as 10.4.2.3 | Same as 10.4.2.3 |
| 11.5.2.4 NR SA FR1-FR1 Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used | Same as 10.4.2.3 | Same as 10.4.2.3 | Same as 10.4.2.3 |
| 11.5.2.5 NR SA FR1-FR1 Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used | Same as 10.4.2.3 | Same as 10.4.2.3 | Same as 10.4.2.3 |
| 11.5.2.6 NR SA FR1-FR1 Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used | Same as 10.4.2.3 | Same as 10.4.2.3 | Same as 10.4.2.3 |
| 11.5.2.7 NR SA FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used | Same as 10.4.2.3 | Same as 10.4.2.3 | Same as 10.4.2.3 |
| 11.5.2.8 NR SA FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is used | Same as 10.4.2.3 | Same as 10.4.2.3 | Same as 10.4.2.3 |
| 11.5.2.9 NR SA FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used | Same as 10.4.2.3 | Same as 10.4.2.3 | Same as 10.4.2.3 |
| 11.5.2.10 NR SA FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is used | Same as 10.4.2.3 | Same as 10.4.2.3 | Same as 10.4.2.3 |
| 11.5.3.1 NR SA FR1 - E-UTRAN event-triggered reporting in non-DRX | During T1: Noc1: -106dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +18dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -2dB Ês2 / Noc2: +17 dB | During T1: 0dB 0dB +1.55dB 0dB During T2: 0dB 0dB 0dB +1.55dB | During T1: Noc1: -106dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +19.55dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -2dB Ês2 / Noc2: +18.55 dB |
| 11.5.3.2 NR SA FR1 - E-UTRAN event-triggered reporting in DRX | Same as 11.5.3.1 | Same as 11.5.3.1 | Same as 11.5.3.1 |
| 11.5.4.1 NR SA FR1 SSB based L1-RSRP measurement when DRX is not used | Same as 10.4.3.1 | Same as 10.4.3.1 | Same as 10.4.3.1 |
| 11.5.4.2 NR SA FR1 SSB based L1-RSRP measurement when DRX is used | Same as 10.4.3.1 | Same as 10.4.3.1 | Same as 10.4.3.1 |
| 11.5.4.3 NR SA FR1 SSB based L1-RSRP measurement on SCC under CCA when DRX is not used | Same as 10.4.3.1 | Same as 10.4.3.1 | Same as 10.4.3.1 |
| 11.5.4.4 NR SA FR1 SSB based L1-RSRP measurement on SCC under CCA when DRX is used | Same as 10.4.3.1 | Same as 10.4.3.1 | Same as 10.4.3.1 |
| 11.6.1.1 NR SA FR1 intra-frequency SS-RSRP measurement accuracy on a carrier frequency with CCA | Same as 10.5.1.1 | Same as 10.5.1.1 | Same as 10.5.1.1 |
| 11.6.1.2 NR SA FR1 Intra-frequency measurement accuracy on SCC on a carrier frequency with CCA | Test 2: Noc: -94.00dBm/30kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±8dB Test 3: NR_CCA_FR1_I: Noc: -110.00dBm/30kHz Ês1 / Noc: +1.0dB Ês2 / Noc: +0.0dB NR_CCA_FR1_J: Noc: -109.50dBm/30kHz Ês1 / Noc: +1.0dB Ês2 / Noc: +0.0dB Normal conditions: Reported RSRP values: ±4.5dB | Test 2: 0dB 0dB +0.4dB Via mapping Test 2: NR_CCA_FR1_I: -1.0dB 0dB +0.2dB NR_CCA_FR1_J: -1.5dB 0dB +0.2dB Via mapping | Test 1: Noc: -91dBm/30kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.4dB RSRP_57 to RSRP_76 Test 2: NR_CCA_FR1_I: Noc: -111dBm/30kHz Ês1 / Noc: +1.0dB Ês2 / Noc: +0.2dB NR_CCA_FR1_J: Noc: -111dBm/30kHz Ês1 / Noc: +1.0dB Ês2 / Noc: +0.2dB NR_CCA_FR1_I: RSRP_43 to RSRP_55 NR_CCA_FR1_J: RSRP_43 to RSRP_55 |
| The derivation of the RSRP values takes into account the uncertainty in Cell 2 SS-RSRP from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRP values given above are for normal conditions. In all cases the RSRP values are 4.5dB wider at each end for extreme conditions. |  |  |  |
| 11.6.2.1 NR SA FR1 intra-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA | Same as 10.5.2.1 | Same as 10.5.2.1 | Same as 10.5.2.1 |
| 11.6.2.2 NR SA FR1 inter-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA | Same as 10.5.2.2 | Same as 10.5.2.2 | Same as 10.5.2.2 |
| 11.6.2.3 NR SA FR1 Intra-frequency measurement accuracy on SCC | Same as 10.5.2.1 | Same as 10.5.2.1 | Same as 10.5.2.1 |
| 11.6.2.4 NR SA FR1 Inter-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA from carrier without CCA | TEST CONFIGURATION 1, 2 |  |  |
|  | Test 1: Noc1: -85 dBm/15kHz Noc2: -86.27 dBm/15kHz Ês1 / Noc1:3dB Ês2 / Noc2: -1.75dB Abs. Reported RSRQ values: ±2.5dB Rel. Reported RSRQ values: ±3dB Test 2: NR_FDD_FR1_A Noc1: -114dBm/15kHz NR_CCA_FR1_I: Noc2: -112dBm/15kHz Ês1 / Noc1: -4dB Ês2 / Noc2: -1.75dB NR_FDD_FR1_H Noc1: -110.5dBm/15kHz NR_CCA_FR1_J: Noc2: -111.5dBm/15kHz Ês1 / Noc1: -4dB Ês2 / Noc2: -1.75dB Abs. Reported RSRQ values: ±2.5dB Rel. Reported RSRQ values: ±3dB | Test 1: 0dB -1.53dB 0dB 0dB Via mapping Test 2: 0dB 0dB 1.3dB 0dB 0dB 0dB 1.3dB 0dB Via mapping | Test 1: Noc1: -85dBm/15kHz Noc2: -87.8dBm/15kHz Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 RSRQ_x-11 to RSRQ_x+2 Test 2: NR_FDD_FR1_A Noc1: -114dBm/15kHz NR_CCA_FR1_I: Noc2: -112dBm/15kHz Ês1 / Noc1: -2.7dB Ês2 / Noc2: -1.75dB NR_FDD_FR1_H Noc1: -110.5dBm/15kHz NR_CCA_FR1_J: Noc2: -111.5dBm/15kHz Ês1 / Noc1: -2.7dB Ês2 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 RSRQ_x-6 to RSRQ_x+8 |
|  | TEST CONFIGURATION 3 |  |  |
|  | Test 1: Noc1: -91 dBm/15kHz Noc2: -86.27 dBm/15kHz Ês1 / Noc1:3dB Ês2 / Noc2: -1.75dB Abs. Reported RSRQ values: ±2.5dB Rel. Reported RSRQ values: ±3dB Test 2: NR_FDD_FR1_A Noc1: -114dBm/15kHz NR_CCA_FR1_I: Noc2: -112dBm/15kHz Ês1 / Noc1: -4dB Ês2 / Noc2: -1.75dB NR_FDD_FR1_H Noc1: -110.5dBm/15kHz NR_CCA_FR1_J: Noc2: -111.5dBm/15kHz Ês1 / Noc1: -4dB Ês2 / Noc2: -1.75dB Abs. Reported RSRQ values: ±2.5dB Rel. Reported RSRQ values: ±3dB | Test 1: 0dB -1.53dB 0dB 0dB Via mapping Test 2: 0dB 0dB 1.3dB 0dB 0dB 0dB 1.3dB 0dB Via mapping | Test 1: Noc1: -91dBm/15kHz Noc2: -87.8dBm/15kHz Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 RSRQ_x-11 to RSRQ_x+2 Test 2: NR_FDD_FR1_A Noc1: -114dBm/15kHz NR_CCA_FR1_I: Noc2: -112dBm/15kHz Ês1 / Noc1: -2.7dB Ês2 / Noc2: -1.75dB NR_FDD_FR1_H Noc1: -110.5dBm/15kHz NR_CCA_FR1_J: Noc2: -111.5dBm/15kHz Ês1 / Noc1: -2.7dB Ês2 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 RSRQ_x-6 to RSRQ_x+8 |
| The derivation of the RSRQ values takes into account the uncertainty in Cell 2 SS-RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRQ values given above are for normal conditions. For extreme conditions, the RSRQ values are 1.5dB wider at each end. |  |  |  |
| 11.6.3.1 NR SA FR1 Intra-frequency SS-SINR measurement accuracy on a carrier frequency with CCA | Same as 10.5.3.1 | Same as 10.5.3.1 | Same as 10.5.3.1 |
| 11.6.3.2 NR SA FR1 Inter-frequency SS-SINR measurement accuracy on a carrier frequency with CCA | Same as 10.5.3.2 | Same as 10.5.3.2 | Same as 10.5.3.2 |
| 11.6.3.3 NR SA FR1 Intra-frequency SS-SINR measurement accuracy on SCC on a carrier frequency with CCA | Same as 10.5.3.1 | Same as 10.5.3.1 | Same as 10.5.3.1 |
| 11.6.3.4 NR SA FR1 Inter-frequency SS-SINR measurement accuracy on a carrier frequency with CCA from carrier without CCA | TEST CONFIGURATION 1, 2 |  |  |
|  | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Abs. Reported SINR values: ±3dB Rel. Reported SINR values: ±3.5dB Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês1 / Noc1: 20dB Ês2 / Noc2: 20dB Abs. Reported SINR values: ±3dB Rel. Reported SINR values: ±3.5dB Test 3: NR_FDD_FR1_A Noc1: -119.5dBm/15kHz NR_CCA_FR1_I: Noc2: -115.5dBm/15kHz Ês1 / Noc1: -4dB Ês2 / Noc2: -4dB NR_FDD_FR1_H Noc1: -116dBm/15kHz NR_CCA_FR1_J: Noc2: -115dBm/15kHz Ês1 / Noc1: -4dB Ês2 / Noc2: -4dB Abs. Reported SINR values: ±3.5dB Rel. Reported SINR values: ±4dB | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0.8dB 0.8dB 0dB 0dB 0.8dB 0.8dB Via mapping | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB SINR_35 to SINR_51 SINR_x-10 to SINR_x+10 Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês1 / Noc1: 20dB Ês2 / Noc2: 20dB SINR_79 to SINR_94 SINR_x-10 to SINR_x+10 Test 3: NR_FDD_FR1_A Noc1: -119.5dBm/15kHz NR_CCA_FR1_I: Noc2: -115.5dBm/15kHz Ês1 / Noc1: -3.2dB Ês2 / Noc2: -3.2dB NR_FDD_FR1_H Noc1: -116dBm/15kHz NR_CCA_FR1_J: Noc2: -115dBm/15kHz Ês1 / Noc1: -3.2dB Ês2 / Noc2: -3.2dB SINR_32 to SINR_49 SINR_x-11 to SINR_x+11 |
|  | TEST CONFIGURATION 3 |  |  |
|  | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Abs. Reported SINR values: ±3dB Rel. Reported SINR values: ±3.5dB Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês1 / Noc1: 20dB Ês2 / Noc2: 20dB Abs. Reported SINR values: ±3dB Rel. Reported SINR values: ±3.5dB Test 3: NR_FDD_FR1_A Noc1: -119.5dBm/15kHz NR_CCA_FR1_I: Noc2: -115.5dBm/15kHz Ês1 / Noc1: -4dB Ês2 / Noc2: -4dB NR_FDD_FR1_H Noc1: -116dBm/15kHz NR_CCA_FR1_J: Noc2: -115dBm/15kHz Ês1 / Noc1: -4dB Ês2 / Noc2: -4dB Abs. Reported SINR values: ±3.5dB Rel. Reported SINR values: ±4dB | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0.8dB 0.8dB 0dB 0dB 0.8dB 0.8dB Via mapping | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB SINR_35 to SINR_51 SINR_x-10 to SINR_x+10 Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês1 / Noc1: 20dB Ês2 / Noc2: 20dB SINR_79 to SINR_94 SINR_x-10 to SINR_x+10 Test 3: NR_FDD_FR1_A Noc1: -119.5dBm/15kHz NR_CCA_FR1_I: Noc2: -115.5dBm/15kHz Ês1 / Noc1: -3.2dB Ês2 / Noc2: -3.2dB NR_FDD_FR1_H Noc1: -116dBm/15kHz NR_CCA_FR1_J: Noc2: -115dBm/15kHz Ês1 / Noc1: -3.2dB Ês2 / Noc2: -3.2dB SINR_32 to SINR_49 SINR_x-11 to SINR_x+11 |
| The derivation of the SINR values takes into account the uncertainty in Cell 2 SS- SINR from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS- SINR values given above are for normal conditions. For extreme conditions, the SS- SINR values are 1dB wider at each for Tests 1 and 2 and 0.5dB wider at each end for Test 3. |  |  |  |
| 11.6.4.1 NR SA FR1 SSB based L1-RSRP measurement accuracy with CCA | Same as 10.5.4.1 | Same as 10.5.4.1 | Same as 10.5.4.1 |
| 11.6.5.1 NR SA FR1 Intra-frequency RSSI measurement accuracy on PCC with CCA | Same as 10.5.5.1 | Same as 10.5.5.1 | Same as 10.5.5.1 |
| 11.6.5.2 NR SA FR1 Intra-frequency RSSI measurement accuracy on SCC with CCA | Same as 10.5.5.1 | Same as 10.5.5.1 | Same as 10.5.5.1 |
| 11.6.5.3 NR SA FR1 Inter-frequency RSSI measurement accuracy on a carrier with CCA | Same as 10.5.5.3 | Same as 10.5.5.3 | Same as 10.5.5.3 |
| 11.6.6.1 NR SA FR1 Intra-frequency channel occupancy measurement accuracy on PCC with CCA | Same as 10.5.6.1 | Same as 10.5.6.1 | Same as 10.5.6.1 |
| 11.6.6.2 NR SA FR1 Intra-frequency channel occupancy measurement accuracy on SCC with CCA | Same as 10.5.6.1 | Same as 10.5.6.1 | Same as 10.5.6.1 |
| 11.6.6.3 NR SA FR1 Inter-frequency channel occupancy measurement accuracy on a carrier with CCA | Same as 10.5.6.3 | Same as 10.5.6.3 | Same as 10.5.6.3 |

< End of changes >