3GPP TSG-RAN5 Meeting #108	R5-254445

# Bengaluru, India, Aug 25th – 29th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-1 | CR | 0656 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introducing new NR RSTD measurement accuracy test case for PRS aggregation in FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | NR RSTD measurement accuracy test case for PRS aggregation in FR2 is absent in the specification |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introducing new test case: NR RSTD measurement accuracy test case for PRS aggregation in FR2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The necessary test case is absent in specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.3.10(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Please implement this CR after implementing CR#0655 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

14.3.8.5	Test requirement

Table 14.3.8.5-3: RSTD accuracy requirements for the reported values

| Test Configuration | Lowest reported value | Highest reported value |
| --- | --- | --- |
| Sub-test 1 | (5898 – 120) Tc converted to RSTD measurement according to clause 4.15.6 | (5898 + 120) Tc converted to RSTD measurement according to clause 4.15.6 |
| Sub-test 2 | (5898 – 86) Tc converted to RSTD measurement according to clause 4.15.6 | (5898 + 86) Tc converted to RSTD measurement according to clause 4.15.6 |

The test tolerances are defined in clauses C.1.6 and C.2.5.

The rate of successful tests during repeated tests shall be at least 90% with a confidence level of 95%.

<Unchanged Skipped >

14.3.10	NR RSTD measurement accuracy test case for PRS aggregation in FR2 SA

Editor’s note: This test case is incomplete. The following aspect is either missing or TBD:

- Test requirements and test tolerance analysis is incomplete

14.3.10.1	Test purpose

The purpose of the test is to verify that the RSTD measurement by aggregating PRS resoureces from multiple positioning frequency layers (PFLs) meets the measurement accuracy requirements specified in TS 38.133 [50] clause 10.1.23A.2 in an environment with AWGN propagation conditions.

14.3.10.2	Test applicability

This test applies to all types of NR UE release 18 onwards that supports DL-TDOA positioning and supportOfDL-PRS-BWA-RRC-Connected-r18.

14.3.10.3	Minimum conformance requirements

Same minimum requirements as in clause 14.3.9.3

14.3.10.4	Test description

14.3.10.4.1	Initial conditions

Table 14.3.10.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 200 MHz bandwidth per PFL, TDD duplex mode |

Test Environment: Normal, as defined in TS 38.508-1 [45] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [45] clause 4.3.1.

Channel bandwidth to be tested: are specified in Table 14.3.10.4.1-1.

1.	Connect the SS and AWGN noise sources to the UE antenna connector or antenna connectors as shown in Annex A, Figure A.14.

2.	The general test parameter settings are set up according to Table 14.3.10.5-1 and Table 14.3.10.5-2.

3.	Propagation conditions are set according to clause 4.15.2.

4.	Message contents are defined in clause 14.3.10.4.3.

5.	In the test there are two synchronous cells: Cell 1 and Cell 2. Cell 1 is the reference as well as the PCell. Cell 2 is a neighbour cells. GP#24 is configured if UE supports GP#24, otherwise, GP#13 is configured for the test.

14.3.10.4.2	Test procedure

Same test procedure as defined in clause 14.3.9.4.2 with following exceptions:

3.	Set the parameters according to Table 14.3.10.5-1 and Table 14.3.10.5-2 as appropriate. Propagation conditions are set according to clause 4.15.2.

10.	The SS shall check the nr-RSTD-r16 value for Cell 2 in the nr-DL-TDOA-SignalMeasurementInformation-r16 according to Table 4.3.10.5-3.

14.3.10.4.3	Message contents

Same message contents as defined in clause 14.3.9.4.3 with following exceptions:

Table 14.3.10.4.3-1 replaces Table 14.3.9.4.3-5; Table 14.3.10.4.3-2 replaces Table 14.3.9.4.3-6.

Table 14.3.10.4.3-1: NR-DL-PRS-AssistanceData

| Derivation Path: 37.355 clause 6.4.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NR-DL-PRS-AssistanceData-r16 ::= SEQUENCE { |  |  |  |
| nr-DL-PRS-ReferenceInfo-r16 SEQUENCE { |  |  |  |
| dl-PRS-ID-r16 | 0 |  |  |
| nr-DL-PRS-ResourceID-List-r16 | Not present |  |  |
| nr-DL-PRS-ResourceSetID-r16 | Not present |  |  |
| } |  |  |  |
| nr-DL-PRS-AssistanceDataList-r16 SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF NR-DL-PRS-AssistanceDataPerFreq-r16 { | 2 entries |  |  |
| NR-DL-PRS-AssistanceDataPerFreq-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-PositioningFrequencyLayer-r16 SEQUENCE { |  |  |  |
| dl-PRS-SubcarrierSpacing-r16 | kHz120 |  |  |
| dl-PRS-ResourceBandwidth-r16 | 3 | 32 PRBs |  |
| dl-PRS-StartPRB-r16 | 0 |  |  |
| dl-PRS-PointA-r16 | absoluteFrequencyPointA + offsetToCarrier, as defined for the DL frequency of the Cell 1 |  |  |
| dl-PRS-CombSizeN-r16 | n2 |  |  |
| dl-PRS-CyclicPrefix-r16 | normal |  |  |
| } |  |  |  |
| nr-DL-PRS-AssistanceDataPerFreq-r16 SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF NR-DL-PRS-AssistanceDataPerTRP-r16{ | 2 entries |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[1] SEQUENCE { |  | entry 1 | Cell 1 |
| dl-PRS-ID-r16 | 0 |  |  |
| nr-PhysCellID-r16 | Cell 1 |  |  |
| nr-CellGlobalID-r16 | Cell 1 |  |  |
| nr-ARFCN-r16 | Cell 1 |  |  |
| nr-DL-PRS-SFN0-Offset-r16 SEQUENCE { |  |  |  |
| sfn-Offset-r16 | 0 |  |  |
| integerSubframeOffset-r16 | 0 |  |  |
| } |  |  |  |
| nr-DL-PRS-ExpectedRSTD-r16 | 0 |  |  |
| nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 | 0 |  |  |
| nr-DL-PRS-Info-r16 | As specified in Table 14.3.10.4.3-2 |  |  |
| } |  |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[2] SEQUENCE { |  | entry 2 | Cell 2 |
| dl-PRS-ID-r16 | 1 |  |  |
| nr-PhysCellID-r16 | Cell 2 |  |  |
| nr-CellGlobalID-r16 | Cell 2 |  |  |
| nr-ARFCN-r16 | Cell 2 |  |  |
| nr-DL-PRS-SFN0-Offset-r16 SEQUENCE { |  |  |  |
| sfn-Offset-r16 | 0 |  |  |
| integerSubframeOffset-r16 | 0 |  |  |
| } |  |  |  |
| nr-DL-PRS-ExpectedRSTD-r16 | 23 | About 3 s |  |
| nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 | 39 | About 5 s |  |
| nr-DL-PRS-Info-r16 | As specified in Table 14.3.10.4.3-2 |  |  |
| } |  |  |  |
| } |  |  |  |
| NR-DL-PRS-AssistanceDataPerFreq-r16[2] SEQUENCE { |  | entry 2 |  |
| nr-DL-PRS-PositioningFrequencyLayer-r16 SEQUENCE { |  |  |  |
| dl-PRS-SubcarrierSpacing-r16 | kHz120 |  |  |
| dl-PRS-ResourceBandwidth-r16 | 3 | 32 PRBs |  |
| dl-PRS-StartPRB-r16 | 0 |  |  |
| dl-PRS-PointA-r16 | absoluteFrequencyPointA + offsetToCarrier, as defined for the DL frequency of the Cell 2 |  |  |
| dl-PRS-CombSizeN-r16 | n2 |  |  |
| dl-PRS-CyclicPrefix-r16 | normal |  |  |
| } |  |  |  |
| nr-DL-PRS-AssistanceDataPerFreq-r16 SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF NR-DL-PRS-AssistanceDataPerTRP-r16{ | 2 entries |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[1] SEQUENCE { |  | entry 1 | Cell 1 |
| dl-PRS-ID-r16 | 0 |  |  |
| nr-PhysCellID-r16 | Cell 1 |  |  |
| nr-CellGlobalID-r16 | Cell 1 |  |  |
| nr-ARFCN-r16 | Cell 1 |  |  |
| nr-DL-PRS-SFN0-Offset-r16 SEQUENCE { |  |  |  |
| sfn-Offset-r16 | 0 |  |  |
| integerSubframeOffset-r16 | 0 |  |  |
| } |  |  |  |
| nr-DL-PRS-ExpectedRSTD-r16 | 0 |  |  |
| nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 | 0 |  |  |
| nr-DL-PRS-Info-r16 | As specified in Table 14.3.10.4.3-2 |  |  |
| } |  |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[2] SEQUENCE { |  | entry 2 | Cell 2 |
| dl-PRS-ID-r16 | 1 |  |  |
| nr-PhysCellID-r16 | Cell 2 |  |  |
| nr-CellGlobalID-r16 | Cell 2 |  |  |
| nr-ARFCN-r16 | Cell 2 |  |  |
| nr-DL-PRS-SFN0-Offset-r16 SEQUENCE { |  |  |  |
| sfn-Offset-r16 | 0 |  |  |
| integerSubframeOffset-r16 | 0 |  |  |
| } |  |  |  |
| nr-DL-PRS-ExpectedRSTD-r16 | 23 | About 3 s |  |
| nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 | 39 | About 5 s |  |
| nr-DL-PRS-Info-r16 | As specified in Table 14.3.10.4.3-2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nr-SSB-Config-r16 | Not present |  |  |
| nr-DL-PRS-AggregationInfo-r18 SEQUENCE (SIZE (1..nrMaxNumPRS-BandWidthAggregation-r18)) OF NR-linkedDL-PRS-ResourceSetID-PRS-AggregationList-r18 { | 2 entries |  |  |
| NR-linkedDL-PRS-ResourceSetID-PRS-AggregationList-r18[1] SEQUENCE (SIZE (2..3)) OF NR-DL-PRS-AggregationElement-r18 { | 2 entries | entry 1 |  |
| NR-DL-PRS-AggregationElement-r18[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-FrequencyLayerIndex-r18 | 0 |  |  |
| nr-DL-PRS-TRP-Index-r18 | 0 |  |  |
| nr-DL-PRS-ResourceSetIndex-r18 | 0 |  |  |
| } |  |  |  |
| NR-DL-PRS-AggregationElement-r18[2] SEQUENCE { |  | entry 2 |  |
| nr-DL-PRS-FrequencyLayerIndex-r18 | 1 |  |  |
| nr-DL-PRS-TRP-Index-r18 | 0 |  |  |
| nr-DL-PRS-ResourceSetIndex-r18 | 0 |  |  |
| } |  |  |  |
| NR-linkedDL-PRS-ResourceSetID-PRS-AggregationList-r18[2] SEQUENCE (SIZE (2..3)) OF NR-DL-PRS-AggregationElement-r18 { | 2 entries | entry 2 |  |
| NR-DL-PRS-AggregationElement-r18[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-FrequencyLayerIndex-r18 | 0 |  |  |
| nr-DL-PRS-TRP-Index-r18 | 1 |  |  |
| nr-DL-PRS-ResourceSetIndex-r18 | 0 |  |  |
| } |  |  |  |
| NR-DL-PRS-AggregationElement-r18[2] SEQUENCE { |  | entry 2 |  |
| nr-DL-PRS-FrequencyLayerIndex-r18 | 1 |  |  |
| nr-DL-PRS-TRP-Index-r18 | 1 |  |  |
| nr-DL-PRS-ResourceSetIndex-r18 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.3.9.4.3-6: NR-DL-PRS-Info

| Derivation Path: 37.355 clause 6.4.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NR-DL-PRS-Info-r16 ::= SEQUENCE { |  |  |  |
| nr-DL-PRS-ResourceSetList-r16 SEQUENCE (SIZE (1..nrMaxSetsPerTrp-r16)) OF NR-DL-PRS-ResourceSet-r16 { | 1 entry |  |  |
| NR-DL-PRS-ResourceSet-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-ResourceSetID-r16 | 0 |  |  |
| dl-PRS-Periodicity-and-ResourceSetSlotOffset-r16 CHOICE { |  |  |  |
| scs120-r16 CHOICE { |  |  |  |
| n160-r16 | 80 |  |  |
| } |  |  |  |
| dl-PRS-ResourceRepetitionFactor-r16 | n2 |  |  |
| dl-PRS-ResourceTimeGap-r16 | s1 |  |  |
| dl-PRS-NumSymbols-r16 | n4 |  |  |
| dl-PRS-MutingOption1-r16 | Not present |  |  |
| dl-PRS-MutingOption2-r16 | Not present |  |  |
| dl-PRS-ResourcePower-r16 | 27 |  |  |
| dl-PRS-ResourceList-r16 SEQUENCE (SIZE (1..nrMaxResourcesPerSet-r16)) OF NR-DL-PRS-Resource-r16 { | 1 entry |  |  |
| NR-DL-PRS-Resource-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-ResourceID-r16 | 0 |  | NR-DL-PRS-AssistanceDataPerFreq-r16[1] |
|  | 1 |  | NR-DL-PRS-AssistanceDataPerFreq-r16[2] |
| dl-PRS-SequenceID-r16 | 0 |  | NR-DL-PRS-AssistanceDataPerTRP-r16[1] |
|  | 1 |  | NR-DL-PRS-AssistanceDataPerTRP-r16[2] |
| dl-PRS-CombSizeN-AndReOffset-r16 CHOICE { |  |  |  |
| n2-r16 | 0 |  |  |
| } |  |  |  |
| dl-PRS-ResourceSlotOffset-r16 | 0 |  | NR Cell 1 |
|  | 4 |  | NR Cell 2 |
| dl-PRS-ResourceSymbolOffset-r16 | 0 |  |  |
| dl-PRS-QCL-Info-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

14.3.10.5	Test requirements

The RSTD measurement accuracy for Cell 2 shall fulfil the absolute requirement in clause 14.3.10.3.

Table 14.3.10.5-1: RSTD accuracy test parameters for PRS aggregation

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| PRS ARFCN |  | freq1, freq2 |  |
| Duplex mode |  | TDD |  |
| TDD configuration |  | TDDConf.3.1 |  |
| BWchannel per carrier | MHz | 200: NRB,c = 132 |  |
| Measurement gap |  | GP#24 or GP#13 |  |
| Downlink initial BWP configuration |  | DLBWP.0.1 | - |
| Downlink dedicated BWP configuration |  | DLBWP.1.1 | - |
| Uplink initial BWP configuration |  | ULBWP.0.1 | - |
| Uplink dedicated BWP configuration |  | ULBWP.1.1 | - |
| DRX cycle configuration |  | Not applicable | - |
| TRS configuration |  | TRS.2.1 TDD | - |
| TCI state |  | TCI.State.0 | - |
| PDSCH Reference measurement channel |  | SR.3.1 TDD | - |
| RMSI CORESET Reference Channel |  | CR.3.1 TDD | - |
| Control channel RMC |  | CCR.3.1 TDD | - |
| OCNG Patterns |  | OP.1 | OP.1 |
| SSB configuration |  | SSB.3 FR2 | SSB.3 FR2 |
| SMTC configuration |  | SMTC.1 | SMTC.1 |
| PRS configuration |  | PRS.1.1 FR2 | PRS.1.1 FR2 |
| PRS Resource slot offset | slot | 0 | 4 |
| Expected RSTD | s | N/A | 3 |
| Expected RSTD uncertainty | s | N/A | 5 |
| Time offset with Cell 1 | s | - | 3 |
| PDSCH/PDCCH subcarrier spacing | kHz | 120 | 120 |
| EPRE ratio of PSS to SSS | dB | 0 | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| EPRE ratio of PRS to SSS | dB | 0 | 0 |
| Propagation conditions |  | AWGN | AWGN |
| Antenna configuration |  | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the slots with transmitted PRS. Note 2: Carriers with PRS resources for aggregation are intra-band and contiguous to each other. |  |  |  |

Table 14.3.10.5-2: RSTD accuracy OTA related test parameters

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| Angle of arrival configuration |  | Setup 1 according to TS 38.133 [50] clause A.3.15.1 |  |
| Assumption for UE beams Note 5 |  | Rough |  |
| NocNote1 | dBm/SCSNote3 | -89 |  |
| PRS EsNoc | dB | -5.7 + TT | -11.9 + TT |
| PRP Note2 | dBm/SCS | -94.7 + TT | -100.9 + TT |
| PRS EsIot | dB | -6 + TT | -13 + TT |
| IoNote2 per PFL | dBm/190.08 MHz Note3 | -55.75 + TT | -55.75 + TT |
| Note 1: Where used, interference from other cells and noise sources not specified in the test is assumed to be constant over sub-carriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 2: PRP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. The Io is calculated based only on the symbols in which PRS is transmitted. Note 3: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone. Note 4: Calculation of Es/Iot includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [19] table 6.2.1.3-4. Note 5: Information about types of UE beam is given in clause B.2.1.3, and does not limit UE implementation or test system implementation. |  |  |  |

The value of the LPP Time IE depends on the UE capabilities. The calculation is FFS

The resulting value shall be rounded up to the next integer and transmitted in the LPP-RequestLocationInformation (see Table 14.3.9.4.3-3).

The RSTD accuracy requirements for the reported values is FFS.

< End of Changes >