3GPP TSG-RAN5 Meeting #108	R5-254562

# Bengaluru, India, Aug.25th – 29rd 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3576 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating default configuration of RRC IEs for 2-step RACH RRM test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Starpoint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_2step_RACH-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - RRC IE interlaceindexFirstPO-msgA-PUSCH and nrofinterlacesPerMSGA-PO are set to “1” in Table 7.3.1-14A, which is directly copied from Table 4.6.3-81B. However, useInterlacePUCCH-PUSCH-r16 is not specified in MSGA RMC (38.133 A.3.20) nor enabled in both common RACH configurations (38.508-1 Table 4.6.3-14) and dedicated RACH configurations (38.508-1 Table 4.6.3-15), which means that interlacePUSCH feature is disabled in RRM tests. As a result, these two IEs won’t take effect no matter what the value is configured. For the sake of simplify, we suggest directly set these two IEs to not present. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - RRC IE interlaceindexFirstPO-msgA-PUSCH and nrofinterlacesPerMSGA-PO are set to not present. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusion are caused. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

Table 7.3.1-14A: MsgA-PUSCH-Config

| Derivation Path: Table 4.6.3-81B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MsgA-PUSCH-Config-r16 ::= SEQUENCE { |  |  |  |
| msgA-PUSCH-ResourceGroupA-r16 SEQUENCE { |  |  |  |
| msgA-PUSCH-TimeDomainOffset-r16 | 1 |  |  |
| nrofPRBs-PerMsgA-PO-r16 | 2 |  |  |
| msgA-IntraSlotFrequencyHopping-r16 | Not present |  |  |
| msgA-HoppingBits-r16 | Not present |  |  |
| msgA-DMRS-Config-r16 SEQUENCE { |  |  |  |
| msgA-DMRS-AdditionalPosition-r16 | pos1 |  |  |
| msgA-PUSCH-NrofPorts-r16 | 0 |  |  |
| } |  |  |  |
| nrofDMRS-Sequences-r16 | 1 |  |  |
| msgA-Alpha-r16 | alpha1 |  |  |
| interlaceIndexFirstPO-MsgA-PUSCH-r16 | not present |  |  |
| nrofInterlacesPerMsgA-PO-r16 | not present |  |  |
| } |  |  |  |
| msgA-TransformPrecoder-r16 | Not present |  |  |
| msgA-DeltaPreamble-r16 | 1 | Actual value = 2dB |  |
| } |  |  |  |

<End of Change 1>