3GPP TSG-RAN5 Meeting #108	R5-254702

# Bengaluru, August 25 - 29 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 4113 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of FR1 NR SA RACH-based LTM PSCell switch test case 6.3.5.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | FR1 RACH-based LTM PSCell switch test case 6.3.5.1 was incomplete |  |  |  |  |  |  |  |  |
| Summary of change: |  | FR1 RACH-based LTM PSCell switch test case is updated with test procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

/// Start of changes

#### 6.3.5.1	NR SA FR1 RACH-based intra-frequency LTM PSCell switch

Editor's Note: This test case is incomplete in following aspects:

- TT analysis is missing.

- Test applicability is missing

- Message contents are TBD

6.3.5.1.1	Test purpose

To verify the UE's ability to perform NR FR1 RACH-based intra frequency PSCell switch specified in 38.133 [6] clause 8.20 for both with and without early TCI state activation.

6.3.5.1.2	Test applicability

This test applies to all types of NR UEs supporting NR LTM for SCG with RACH from Release 18 onwards.

6.3.5.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.3.5.0.

The normative reference for this requirement is TS 38.133 [6] clauses 8.20 and A.6.3.5.1.

6.3.5.1.4	Test description

6.3.5.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

This test shall be tested using any of the test configurations in Table 6.3.5.1.4.1-1.

Table 6.3.5.1.4.1-1: Intra-frequency cell switch from FR1 to FR1 test configurations

| Config | Description |
| --- | --- |
| 6.3.5.1-1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.3.5.1-2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.3.5.1-3 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.3.5.1.4.1-2.

Table 6.3.5.1.4.1-2: Initial conditions for NR SA FR1 handover with known target cell

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.3.5.1.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

1. The general test parameter settings are set up according to Table 6.3.5.1.4.1-3.

2. Message contents are defined in clause 6.3.5.1.4.3.

3. Three cells are deployed in the test, which are FR1 PCell (Cell 1), FR1 PSCell (Cell 2) and a FR1 neighbour cell (Cell 3) on the same frequency as the PSCell. The power levels and settings are according to Table 6.3.5.1.4.1-3 and Table 6.3.5.1.5-1.

Table 6.3.5.1.4.1-3: General test parameters for Intra-frequency PSCell cell switch from FR1 to FR1

| Parameter |  | Unit | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1A | Test 1B | Test 2A | Test 2B |  |
| Initial conditions | Active cells |  | Cell 1 (PCell), Cell 2 (PSCell) |  |  |  |  |
|  | Neighbouring cell |  | Cell 3 |  |  |  | Cell 3 is the candidate cell |
| Final condition | Active cells |  | Cell 1 (PCell), Cell 3 (PSCell) |  |  |  |  |
| A3-Offset |  | dB | -6 |  |  |  |  |
| Hysteresis |  | dB | 0 |  |  |  |  |
| Time To Trigger |  | s | 0 |  |  |  |  |
| Filter coefficient |  |  | 0 |  |  |  | L3 filtering is not used |
| DRX |  |  | OFF |  |  |  | DRX is not used |
| Access Barring Information |  | - | Not Sent |  |  |  | No additional delays in random access procedure. |
| Time offset between Cell 2 and Cell 3 |  |  | 2 s |  |  |  | RTD between Cell 2 and Cell 3 is less than CP |
| deriveSSB-IndexFromCell |  |  | Enabled |  |  |  |  |
| LTM-CSI-ReportConfig | L1-RSRP reporting period | slot | 80 |  |  |  | Periodic L1-RSRP reporting configured |
|  | nrOfReportedCells |  | n1 |  |  |  | Report candidate cell’s (Cell 3) L1-RSRP measurement results. |
|  | nrOfReportedRS-PerCell |  | n1 |  |  |  |  |
|  | spCellInclusion |  | N/A |  |  |  |  |
| ltm-DL-OrJointTCI-StateToAddModList | CandidateTCI-State#1 |  | DLorJoint TCI.State.0 | DLorJoint TCI.State.2 | DLorJoint TCI.State.1 | DLorJoint TCI.State.3 | As specified in TS.38.133 [6] clause A.3.16B. In test 1A and 1B, CandidateTCI-State#1 and/or CandidateTCI-UL-State#1 are configured for early TCI state activation. CandidateTCI-State#2 and/or CandidateTCI-UL-State#1 are configured for TCI state indication in cell switch command. In test 2A and 2B, CandidateTCI-State#1 and/or CandidateTCI-UL-State#1 are configured for TCI state indication in cell switch command. |
|  | CandidateTCI-State#2 |  | DLorJoint TCI.State.1 | DLorJoint TCI.State.3 | N/A | N/A |  |
| ltm-UL-TCI-StatesToAddModList | CandidateTCI-UL-State#1 |  | N/A | UL TCI.State.0 | N/A | UL TCI.State.0 |  |
| ltm-ConfigComplete |  |  | True |  |  |  | Candidate cell’s configuration is complete configuration |
| T1 |  | s | <3 |  |  |  |  |
| T2 |  | s | 0.2 |  |  |  |  |
| T3 |  | s | 0.1 |  |  |  |  |
| T4 |  | s | 0.1 |  |  |  |  |

6.3.5.1.4.2	Test procedure

Three cells are deployed in the test, which are FR1 PCell (Cell 1), FR1 PSCell (Cell 2) and a FR1 neighbour cell (Cell 3) on the same frequency as the PSCell. Test configurations are given in table 6.3.4.3.4.1-1. Both cell switch delay and interruption length are tested by using the parameters in table 6.3.5.1.4.1-2, and 6.3.5.1.4.1-3.

1.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5. Establish SRB2 and DRB in the RRC Reconfiguration message. Cell 1 is the active cell.

2.	Set the parameters according to T1 in Table 6.3.5.1.5-1. Propagation conditions are set according to Annex C clause C.2.2. T1 starts.

3.	SS shall transmit an RRCReconfiguration message, configuring measurement object.

4.	UE shall transmit RRCReconfigurationComplete message.

5.	If the UE transmit an A3 message report, the SS shall switch the power setting from T1 to T2 as specified in Table 6.3.5.1.5-1. T2 starts.

6.	At the start of T2, the SS shall transmit an RRCReconfiguration message with LTM-Candidate-r18 for Cell 3 and SSB-based L1-RSRP measurements and periodic L1-RSRP measurement reports on candidate cell (Cell 3) in PUCCH format 2.

7.	UE shall transmit RRCReconfigurationComplete message.

8.	If the UE transmits a valid L1-RSRP result of Cell 3:

Test 1: The SS shall switch the power setting from T2 to T3 as specified in Table 6.3.5.1.5-1. T3 starts.

Test 2: T3 is skipped. The SS shall switch the power setting from T2 to T4 as specified in Table 6.3.5.1.5-1. T4 starts, go to step10.

9.	At the start of T3, the SS shall transmit the candidate cell TCI activation MAC-CE (Refer TS 38.321 [12], clauses 6.1.3.76) for Cell 3. 50ms after the candidate cell TCI state activation MAC-CE transmission, the SS shall switch the power setting from T3 to T4 as specified in Table 6.3.5.1.5-1. T4 starts. Candidate cell TCI activation MAC-CE: CandidateTCI-State#1 is activated.

10.	The start of T4 is the instant when the last TTI containing the LTM cell switch command MAC CE(Refer TS 38.321 [12], clauses 6.1.3.75) is sent by Cell 1 to the UE.

cell switch MAC-CE:

-	Contention-Free Random-Access Resources are indicated and the field of Timing Advance Command is set to FFF

-	Test 1: CandidateTCI-State#2 is indicated.

-	Test 2: CandidateTCI-State#1 is indicated.

11.	If the UE transmits the uplink PRACH channel to Cell 3 less than DLTM from the beginning of time period T4 then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

12.	After T4 expires, the SS sends an RRCReconfiguration with reconfigurationWithSync, which includes the release of LTM-Config-r18 to cause UE handover back to Cell 1.

13.	If UE is not in state RRC_CONNECTED with generic procedure parameters Connectivity NR, connected without release On according to TS 38.508-1 [14] clause 4.5 on Cell 1, switch off and on the UE. Then ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 on Cell 1

14.	Power off Cell 3 and set Cell 3 physical cell identity = ((current Cell 3 physical cell identity + 3) mod1008) for next iteration of the test procedure loop.

15.	Repeat steps 2-14 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.3.5.1.4.3	Message contents

TBD

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.3.5.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | TBD |

6.3.5.1.5	Test requirements

Table 6.3.5.1.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.3.5.1.5-1: Cell specific test parameters for NR FR1 Intra frequency PSCell switch test case

| Parameter |  |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 ~ T4 | T1 ~ T4 | T1 ~ T4 |
| NR RF Channel Number |  |  |  | 1 | 2 | 2 |
| Duplex mode |  | Config 1, 1A |  | FDD |  |  |
|  |  | Config 2, 3, 2A, 3A |  | TDD |  |  |
| TDD configuration |  | Config 1, 1A |  | Not Applicable |  |  |
|  |  | Config 2, 2A |  | TDDConf.1.1 |  |  |
|  |  | Config 3, 3A |  | TDDConf.2.1 |  |  |
| BWchannel |  | Config 1, 1A | MHz | 10: NRB,c = 52 |  |  |
|  |  | Config 2, 2A | MHz | 10: NRB,c = 52 |  |  |
|  |  | Config 3, 3A | MHz | 40: NRB,c = 106 |  |  |
| BWP BW |  | Config 1, 1A | MHz | 10: NRB,c = 52 |  |  |
|  |  | Config 2, 2A | MHz | 10: NRB,c = 52 |  |  |
|  |  | Config 3, 3A | MHz | 40: NRB,c = 106 |  |  |
| PDSCH Reference |  | Config 1, 1A |  | SR.1.1 FDD |  |  |
|  |  | Config 2, 2A |  | SR.1.1 TDD |  |  |
|  |  | Config 3, 3A |  | SR.2.1 TDD |  |  |
| CORESET Reference Channel |  | Config 1, 1A |  | CR.1.1 FDD |  |  |
|  |  | Config 2, 2A |  | CR.1.1 TDD |  |  |
|  |  | Config 3, 3A |  | CR.2.1 TDD |  |  |
| CP length |  | Config 1, 2, 3, 1A, 2A, 3A |  | Normal |  |  |
| TRS configuration |  | Config 1, 1A |  | TRS.1.1 FDD |  |  |
|  |  | Config 2, 2A |  | TRS.1.1 TDD |  |  |
|  |  | Config 3, 3A |  | TRS.1.2 TDD |  |  |
| OCNG Patterns |  | Config 1, 2, 3, 1A, 2A, 3A |  | OP.1 |  |  |
| SMTC Configuration |  | Config 1, 2, 3, 1A, 2A, 3A |  | SMTC.1 |  |  |
| SSB Configuration |  | Config 1,2, 1A, 2A |  | SSB.1 FR1 |  |  |
|  |  | Config 3, 3A |  | SSB.2 FR1 |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2, 1A, 2A | kHz | 15 |  |  |
|  |  | Config 3, 3A | kHz | 30 |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2, 1A, 2A | kHz | 15 |  |  |
|  |  | Config 3, 3A | kHz | 30 |  |  |
| PRACH configuration |  | Config 1, 2, 3, 1A, 2A, 3A |  | FR1 PRACH configuration 6 |  |  |
| BWP configuration | Initial DL BWP | Config 1, 2, 3, 1A, 2A, 3A |  | DLBWP.0.1 |  |  |
|  | Dedicated DL BWP | Config 1, 2, 3, 1A, 2A, 3A |  | DLBWP.1.1 |  |  |
|  | Initial UL BWP | Config 1, 2, 3, 1A, 2A, 3A |  | ULBWP.0.1 |  |  |
|  | Dedicated UL BWP | Config 1, 2, 3, 1A, 2A, 3A |  | ULBWP.1.1 |  |  |
| EPRE ratio of PSS to SSS |  | Config 1, 2, 3, 1A, 2A, 3A | dB | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | Config 1, 2, 3, 1A, 2A, 3A |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | Config 1, 2, 3, 1A, 2A, 3A |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  | Config 1, 2, 3, 1A, 2A, 3A |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | Config 1, 2, 3, 1A, 2A, 3A |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | Config 1, 2, 3, 1A, 2A, 3A |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  | Config 1, 2, 3, 1A, 2A, 3A |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  | Config 1, 2, 3, 1A, 2A, 3A |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  | Config 1, 2, 3, 1A, 2A, 3A |  |  |  |  |
| Note2 |  | Config 1, 2, 3, 1A, 2A, 3A | dBm/15kHz | -98+TT |  |  |
| Note2 |  | Config 1,2, 1A, 2A | dBm/SCS | -98+TT |  |  |
|  |  | Config 3, 3A |  | -95+TT |  |  |
|  |  | Config 1, 2, 3, 1A, 2A, 3A | dB | 11+TT | -0.64+TT | -0.64+TT |
|  |  | Config 1, 2, 3, 1A, 2A, 3A | dB | 11+TT | 8+TT | 8+TT |
| SSB_RP |  | Config 1,2, 1A, 2A | dBm/SCS | -87+TT | -90+TT | -90+TT |
|  |  | Config 3, 3A | dBm/SCS | -84+TT | -87+TT | -87+TT |
| IoNote3 |  | Config 1,2, 1A, 2A | dBm/ 9.36MHz | -58.7+TT | -58.7+TT | -58.7+TT |
|  |  | Config 3, 3A | dBm/ 38.16MHz | -52.62+TT | -52.6+TT | -52.6+TT |
| Propagation condition |  | Config 1, 2, 3, 1A, 2A, 3A | - | AWGN | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

The UE shall start to transmit the PRACH to Cell 3 in no later than DLTM from the beginning of time period T4.

The rate of correct cell switches observed during repeated tests shall be at least 90%.

NOTE:	The cell switch delay can be expressed as DLTM (= Tcmd + TLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU), where:

Tcmd = THARQ + 3ms and is specified in clause 6.3.1.2

-	Tfirst-RS + TRS-proc= 0 ms for Test 1A and 1B, Tfirst-RS + TRS-proc= 22 ms for Test 2A and 2B

-	TLTM-IU = 20 ms

-	TLTM-RRC-processing = 10 ms if UE does not support ltm-FastProcessingConfig-r18, otherwise TLTM-RRC-processing =0ms

-	TLTM-processing = 10 ms if the UE supports ltm-FastUE-Processing-r18 capability and UE reports 10 ms for FR1-to-FR1 cell switch in the capability

-	TLTM-processing = 15 ms if the UE supports ltm-FastUE-Processing-r18 capability and UE reports 15 ms for FR1-to-FR1 cell switch in the capability

-	TLTM-processing = 20 ms if the UE does not support ltm-FastUE-Processing-r18 capability.

/// Unchanged sections omitted