3GPP TSG-RAN5 Meeting #108	R5-254819

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

Agenda item:	5.3.18.9

Source: 	Apple

Title: 	Impact of beam correspondence test method without UE beam lock

Document for:	Discussion and Endorsement

# Introduction

There have been several discussions in RAN5 meetings regarding the test method for the UE Conformance testing for FR2 beam correspondence during initial access and RRC_INACTIVE.

The goal is to consider the beam lock function during initial access (UBF_IDLE) for beam correspondence test.

This paper provides Apple’s view and highlights the disadvantages associated to the ‘test method without UE beam lock function’ presented in [4] and [5] for beam correspondence test during initial access and RRC_INACTIVE.

# Discussion

## Impact of test method without UE Beam Lock

The UE beam lock function is needed to ensure the UE preamble transmission are from UE optimal Tx beam. The test method without considering the UE beam lock function can have following drawbacks for beam correspondence test during initial access and RRC_INACTIVE:

## 2.1.1	Impact on measurements results

- Beam correspondence test without UE beam lock function violates the basic assumption [1] made by RAN4.

- Without the UE beam lock function, the UE can switch between fine and course beam. This can cause the fluctuation in PRACH Tx power measurements results.

- Without the UE beam lock function, the UE may attempt to change the beam during the measurement period. Consequently, measurement samples may be obtained from sub-optimal and optimal beams. This may lead to inaccuracies in PRACH Tx power measurement results [2].

## 2.1.2	Impact on TE side implementation

- The absence of the UE beam lock function may also lead to inaccurate measurement results if the TE collection of several UE RACH preambles is performed to identify the highest PRACH power among the collected samples. The SS identification of highest PRACH power from the consecutive UE transmitted preambles does not guarantee that the preambles were transmitted from the optimal beam.

- The SS identification of the ‘highest power UE transmitted preamble’ may require TE to enhance its implementation and functionality.

- TE is unable to distinguish preambles from sub-optimal and optimal beams, potentially enabling compromised UE to satisfy the test requirement as depicted in Figure 1.

## 2.1.3	Impact on test time

- It is also recommended by RAN4 [1] to utilize the longest applicable PRACH preamble format. The preamble format B4/143, with a 12-symbol duration satisfies this criterion as presented in [6] with TP analysis.

- The PRACH format/index C2/190 [5] is with 6 symbol duration shorter symbol duration compared to B4.

- The preamble format C2[6 symbol duration] will increase the test time. Therefore, TP analysis is necessary to evaluate its suitability for inclusion in the test.

Figure 1 : PRACH Tx Power at TE

Observation 1: There are several disadvantages of defining the test method without using UE beam lock:

- This violates the fundamental assumption [1] made by RAN4 when deciding the minimum requirement.

- It may lead to unstable test results.

- This approach may result in inaccurate PRACH TX power measurement results due to preambles from the sub-optimal beam during measurement period.

Observation 2: The test method without UE Beamlock_IDLE Function (UBF_IDLE) presented in [4] and [5] may add overhead to the TE implementation.

- TE may need to enhance its functionality to implement the sliding window to identify the highest power UE transmitted preamble among the collected samples.

- This approach may also result in varying implementations among different TE vendors, potentially leading to non-repeatable numerical measurements results.

- TE is unable to distinguish between preambles from sub-optimal and optimal beams, potentially allowing compromised UE to meet the test requirement.

Observation 3: The preamble format C2 proposed in [4] and [5] may also impact the test time.

- PRACH preamble format B4[12-symbol duration] is the longest applicable format [1] compared to format C2[4-symbol duration].

- TP analysis for preamble format C2 is necessary to evaluate its suitability for inclusion in the test.

Based on the above observations, we re-iterate the proposals from [3] and [6]:

Proposal 1: To only allow the UE beam lock function (UBF_IDLE) to ensure reliable and repeatable measurement results for beam correspondence test during initial access and RRC_INACTIVE.

Proposal 2: To configure UE with PRACH configuration format/index: B4/143 for beam correspondence test during initial access to minimize the test time.

# Conclusion

The following observation and proposal were made in this contribution.

Observation 1: There are several disadvantages of defining the test method without using UE beam lock:

- This violates the fundamental assumption [1] made by RAN4 when deciding the minimum requirement.

- It may lead to unstable test results.

- This approach may result in inaccurate PRACH TX power measurement results due to preambles from the sub-optimal beam during measurement period.

Observation 2: The test method without UE Beamlock_IDLE Function (UBF_IDLE) presented in [4] and [5] may add overhead to the TE implementation.

- TE may need to enhance its functionality to implement the sliding window to identify the highest power UE transmitted preamble among the collected samples.

- This approach may also result in varying implementations among different TE vendors, potentially leading to non-repeatable numerical measurements results.

- TE is unable to distinguish between preambles from sub-optimal and optimal beams, potentially allowing compromised UE to meet the test requirement.

Observation 3: The preamble format C2 proposed in [4] and [5] may also impact the test time.

- PRACH preamble format B4[12-symbol duration] is the longest applicable format [1] compared to format C2[4-symbol duration].

- TP analysis for preamble format C2 is necessary to evaluate its suitability for inclusion in the test.

Based on the above observations, we re-iterate the proposals from [3] and [6]:

Proposal 1: To only allow the UE beam lock function (UBF_IDLE) to ensure reliable and repeatable measurement results for beam correspondence test during initial access and RRC_INACTIVE.

Proposal 2: To configure UE with PRACH configuration format/index: B4/143 for beam correspondence test during initial access to minimize the test time.

# References

- R4-2317764, “LS on testability issues of beam correspondence requirements”, RAN4#108-bis, October 9 – October 13, 2023

- R5-247075,“Discussion on FR2 beam correspondence for initial access”, Huawei, RAN5#105, Nov 18 – Nov 22, 2024

- R5-244945, “Discussion on FR2 beam correspondence during initial access”, Apple, RAN4#104, Aug 19 – Aug 23, 2024

- R5-252986, “Discussion paper on FR2 PRACH UE beam correspondence”, Qualcomm, RAN5#107, May 19 – May 23, 2025

- R5-252987, “Updates to TC 6.6.3 Beam correspondence for initial access”, Qualcomm, RAN5#107, May 19 – May 23, 2025

- R5-251738, “On FR2 beam correspondence PRACH configuration Index”, Apple, RAN5#106, Feb 17 – Feb 21, 2025