3GPP TSG-RAN5 Meeting #109	R5-255892

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 5185 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new UAV Test Case 8.1.3.5.6 Test Case for event A3H2 reporting for Intra-Frequency Neighbour |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Atheros, Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UAV_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test case 8.1.3.5.6 needs to be added as per NR UAV_plus_CT1-UEConTest WP |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added test case 8.1.3.5.6 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This aspect of WP will remain incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.3.5.6 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.523-2 CR 0638 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

##### 8.1.3.5.6	NR UAV / Measurement configuration control and reporting / Event A3H2 / Measurement of Neighbour NR cell / Intra-frequency measurements

8.1.3.5.6.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state and intra-frequency measurement configured for event A3H2 }

ensure that {

when { Neighbour cell becomes better than serving cell and the Aerial UE altitude becomes lower than the absolute threshold minus hysteresis }

then { UE sends MeasurementReport with correct measdID for event A3H2}

}

(2)

with { UE in RRC_CONNECTED state and intra-frequency measurement configured for event A3H2 }

ensure that {

when { Neighbour cell becomes better than serving cell and the Aerial UE altitude becomes higher than the absolute threshold plus hysteresis }

then { UE stops sending MeasurementReport }

}

(3)

with { UE in RRC_CONNECTED state and intra-frequency measurement configured for event A3H2 }

ensure that {

when { Neighbour cell becomes lower than serving cell and the Aerial UE altitude becomes higher than the absolute threshold plus hysteresis }

then { UE does not send MeasurementReport }

}

8.1.3.5.6.2	Conformance requirements

Same as 8.1.3.5.3.2 with the following difference:

[TS 38.331, clause 5.5.4.24]

The UE shall:

1>	consider the entering condition for this event to be satisfied when both condition A3H2-1 and condition A3H2-2, as specified below, are fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A3H2-3 or condition A3H2-4, i.e. at least one of the two, as specified below, is fulfilled;

1>	use the SpCell for Mp, Ofp and Ocp.

NOTE:	The cell(s) that triggers the event has reference signals indicated in the measObjectNR associated to this event which may be different from the NR SpCell measObjectNR.

Inequality A3H2-1 (Entering condition 1)

Mn + Ofn + Ocn – Hys1 > Mp + Ofp + Ocp + Off

Inequality A3H2-2 (Entering condition 2)

Ms + Hys2 < Thresh

Inequality A3H2-3 (Leaving condition 1)

Mn + Ofn + Ocn + Hys1 < Mp + Ofp + Ocp + Off

Inequality A3H1-4 (Leaving condition 2)

Ms – Hys2 > Thresh

The variables in the formula are defined as follows:

Mn is the measurement result of the neighbouring cell, not taking into account any offsets.

Ofn is the measurement object specific offset of the reference signal of the neighbour cell (i.e. offsetMO as defined within measObjectNR corresponding to the frequency of the neighbour cell).

Ocn is the cell specific offset of the neighbour cell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the frequency of the neighbour cell, or cellIndividualOffset as defined within reportConfigNR), and set to zero if not configured for the neighbour cell.

Mp is the measurement result of the SpCell, not taking into account any offsets.

Ofp is the measurement object specific offset of the SpCell (i.e. offsetMO as defined within measObjectNR corresponding to the SpCell).

Ocp is the cell specific offset of the SpCell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the SpCell), and is set to zero if not configured for the SpCell.

Hys1 is the hysteresis parameter for this event (i.e. a3-Hysteresis as defined within reportConfigNR for this event).

Off is the offset parameter for this event (i.e. a3-Offset as defined within reportConfigNR for this event).

Ms is the Aerial UE altitude relative to the sea level.

Hys2 is the hysteresis parameter for this event (i.e. h2-Hysteresis as defined within reportConfigNR for this event).

Thresh is the threshold parameter for this event (i.e. h2-Threshold as defined within reportConfigNR for this event).

Mn, Mp are expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Ofn, Ocn, Hys1, Ofp, Ocp, Off are expressed in dB.

Ms, Hys2, Thresh are expressed in meters.

8.1.3.5.6.3	Test description

8.1.3.5.6.3.1	Pre-test conditions

Same as 8.1.3.5.3.3.1

8.1.3.5.6.3.2	Test procedure sequence

Table 8.1.3.5.6.3.2-1 illustrates the simulated aerial height above sea level at various time instants of the test execution along with the downlink power levels to be applied for NR Cell 1 and NR Cell 2 at various time instants of the test execution. Row marked "T0" denotes the conditions after the preamble, while rows marked "T1", "T2" and "T3" are to be applied subsequently at the point indicated in the Main behaviour description in Table 8.1.3.1.2.3.2-2.

Table 8.1.3.5.6.3.2-1: Time instances of cell power level and parameter changes for NR Cell 1 and NR Cell 2 in conducted test environment

| Time | Parameter | Unit | Value (Note 1) | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remark (Note 2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T0 | Simulated aerial height above sea level (Ms) | m | 700 | SS/PBCH SSS EPRE | dBm/ SCS | -85 | -91 | Simulated aerial height above sea level (Ms) and Power levels are such that neither of the entry conditions A3H2-1 and A3H2-2 are satisfied: A3H2-1: Mn + Ofn + Ocn – Hys1 > Mp + Ofp + Ocp + Off A3H2-2: Ms + Hys2 < Thresh |
| T1 |  |  | 300 |  |  | -85 | -79 | Simulated aerial height above sea level (Ms) and Power levels are such that both the entry conditions A3H2-1 and A3H2-2 are satisfied: A3H2-1: Mn + Ofn + Ocn – Hys1 > Mp + Ofp + Ocp + Off A3H2-2: Ms + Hys2 < Thresh |
| T2 |  |  | 1100 |  |  | -85 | -79 | Simulated aerial height above sea level (Ms) and Power levels are such that Leaving Condition conditions A3H2-4 is satisfied: A3H2-4: Ms – Hys2 > Thresh |
| T3 |  |  | 1100 |  |  | -85 | -91 | Simulated aerial height above sea level (Ms) and Power levels are such that Leaving Condition conditions A3H2-3 and A3H2-4 are satisfied: A3H2-3: Mn + Ofn + Ocn + Hys1 < Mp + Ofp + Ocp + Off A3H2-4: Ms – Hys2 > Thresh |
| Note 1: An allowance of +/-90m has been used to generate the aerial height values. This allowance considers both the UE measurement accuracy and the test tolerances in the simulation. Note 2: Value of Hys correspond to values of h2-Hysteresis-r18 in Table 8.1.3.5.6.3.3-4. Value of Thresh corresponds to value of h2-Threshold-r18 in Table 8.1.3.5.6.3.3-4. |  |  |  |  |  |  |  |  |

Table 8.1.3.5.6.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Trigger the UE to reset UTC time in order that the UE accepts the time generated by the GNSS simulator and also uses only GNSS for position calculation in the subsequent test procedure, as defined in TS 38.509 [25] clause 5.6.3. (Note1) | - | - | - | - |
| 2 | The SS configures the GNSS simulator for step 1 of Scenario #1 as defined in TS 36.508 [7] Table 4.12.2-1 to simulate a location with the height above sea level given for time T0 in Table 8.1.3.5.6.3.2-1. | - | - | - | - |
| 3 | The SS transmits a RRCReconfiguration message including MeasConfig to setup intra NR measurement and reporting for event A3H2. | <-- | RRCReconfiguration | - | - |
| 4 | The UE transmits a RRCReconfigrationComplete message. | --> | RRCReconfigurationComplete | - | - |
| 5 | The SS configures GNSS simulator for step 2 of Scenario #1 as defined in TS 36.508 [7] Table 4.12.2-1 to simulate the UE altitude moving to a height above sea level and readjusts the cell-specific reference signal to row "T1" in Table 8.1.3.5.6.3.2-1. | - | - | - | - |
| 6 | Check: Does the UE transmit a MeasurementReport message to report event A3H2 with the measured aerial height value? | --> | MeasurementReport | 1 | P |
| - | EXCEPTION: Step 7 below is repeated until 3 MeasurementReport messages are received from the UE. | - | - | - | - |
| 7 | Check: Does the UE transmit a MeasurementReport message to report event A3H2 with the measured aerial height value? | --> | MeasurementReport | 1 | P |
| 8 | The SS configures GNSS simulator for step 2 of Scenario #1 as defined in TS 36.508 [7] Table 4.12.2-1 to simulate the UE altitude moving to a height above sea level and readjusts the cell-specific reference signal to row "T2" in Table 8.1.3.5.6.3.2-1. | - | - | - | - |
| 9 | Wait and ignore MeasurementReport messages for 50s (as detailed in TS 36.508 [7] Table 4.12.2-1) to allow simulated change of aerial height value to be measured by the UE. | - | - | - | - |
| 10 | Check: Does the UE transmit a MeasurementReport message within the next 10s? | - | - | 2 | F |
| 11 | The SS configures GNSS simulator for step 2 of Scenario #1 as defined in TS 36.508 [7] Table 4.12.2-1 to simulate the UE altitude moving to a height above sea level and readjusts the cell-specific reference signal to row "T3" in Table 8.1.3.5.6.3.2-1. | - | - | - | - |
| 12 | Wait for 50s (as detailed in TS 36.508 [7] Table 4.12.2-1) to allow simulated change of aerial height value to be measured by the UE. | - | - | - | - |
| 13 | Check: Does the UE transmit a MeasurementReport message within the next 10s? | - | - | 3 | F |
| Note 1: The UTC time reset may be performed by MMI or AT command (+CUTCR). |  |  |  |  |  |

8.1.3.5.6.3.3	Specific message contents

Table 8.1.3.5.6.3.3-1: RRCReconfiguration (step 3, Table 8.1.3.5.6.3.2-2)

Derivation Path: TS 38.508-1 [4] Table 4.6.1-13 with condition NR_MEAS and UAV

Table 8.1.3.5.6.3.3-2: MeasConfig (step 3, Table 8.1.3.5.6.3.3-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-69 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 1 entry |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR | Table 8.1.3.5.6.3.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR-EventA3H2 | Table 8.1.3.5.6.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 1 entry |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 1 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.5.6.3.3-3: MeasObjectNR (Table 8.1.3.5.6.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-76 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR for SSB of NR Cell 1 |  |  |
| absThreshSS-BlocksConsolidation | Not present |  |  |
| nrofSS-BlocksToAverage | Not present |  |  |
| } |  |  |  |

Table 8.1.3.5.6.3.3-4: ReportConfigNR-EventA3H2 (Table 8.1.3.5.6.3.3-2)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-142 with condition EVENT_A3H2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA3H2-r18 SEQUENCE { |  |  |  |
| eventA3 SEQUENCE { |  |  |  |
| rsrp | 2 | 1 dB(2*0.5 dB) |  |
| } |  |  |  |
| h2-Threshold-r18 | 500 (500m) |  |  |
| h2-Hysteresis-r18 | 5 (5m) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportAmount | infinity |  |  |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrp | true |  |  |
| rsrq | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.5.6.3.3-5: MeasurementReport (step 6, Table 8.1.3.5.6.3.2-2)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-5A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| measurementReport SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServingMOList SEQUENCE (SIZE (1..maxNrofServingCells)) OF MeasResultServMO { | 1 entry | Measurement report for NR Cell 1 |  |
| MeasResultServMO[1] SEQUENCE { |  | entry 1 |  |
| servCellId | ServCellIndex of NR Cell 1 |  |  |
| measResultServingCell SEQUENCE { |  |  |  |
| physCellId | PCI of NR Cell 1 |  |  |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | Not present |  |  |
| sinr | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListNR SEQUENCE (SIZE (1.. maxCellReport)) OF MeasResultNR { | 1 entry | Measurement report for NR Cell 2 |  |
| MeasResultNR[1] SEQUENCE { |  | entry 1 |  |
| physCellId | Physical layer cell identity of NR Cell 2 |  |  |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | Not present |  |  |
| sinr | Not present |  |  |
| } |  |  |  |
| resultsCSI-RS-Cell | Not present |  |  |
| } |  |  |  |
| rsIndexResults | Not present |  |  |
| } |  |  |  |
| cgi-Info | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| altitudeUE-r18 | (-420..10000) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

==============End of change==============