3GPP TSG-RAN5 Meeting #109	R5-256199

Dallas, USA, 17th – 21st Nov. 2025

Source: 	Huawei, HiSilicon

Title: 	Discussion on the optimization of Out-of-band Blocking (OOBB) testing

Document for:	Discussion and endorsement

Introduction

Out-of-band blocking (OOBB) testing currently suffers from excessive test time due to the large number of test frequencies that need to be tested. In [1], the issue was raised and the necessity to optimize the OOBB testing requirements was highlighted. In [2], further analysis for optimization was proposed from multiple perspectives, including regulatory impact, RF theoretical considerations, influence on spurious response requirements, test points selection, and protocol implementation.

In this contribution, we continue the discussion and focus on the potential approach for optimization of OOBB testing.

Discussion

Two-step measurement approach

With the current OOBB test procedure, a large number of interferer frequencies are measured resulting in high test time and low efficiency. To address the above issue, it is proposed that only the interferer frequencies that may cause RF degradation should be tested. Specifically:

- For NR band OOBB, the interferer frequencies near the passband of Band Filter should be tested to check the characteristics of band filter and LNA is from FDL_low-[100MHz] to FDL_low-[15MHz], and from FDL_high+[15MHz] to FDL_high+[100MHz].

- Execute the test when the IM3/IM5 of wanted signal and interferer fall into FDL_low to FDL_high, and when the ≤1/5~5 times harmonic fall into – (FDL_high - FDL_low) /2 to (FDL_high - FDL_low)/2.

Considering the comments received in RAN5#108 meeting, the potential approach for OOBB test optimization is now proposed to ensure both test efficiency and completeness.

The OOBB testing can be performed following a per-step procedure. Step (a) is considered optional. When performing step (a), the number of allowed exceptions is more stringent, which can be either zero or a threshold defined in clause 7.6.3.5. The test frequencies in Step (a) include: Frequencies where intermodulation and harmonic products of the interfering and wanted signals fall within the operating band, and fixed frequencies near the wanted signal that may exhibit reduced interference suppression due to the wideband characteristics of the band filter and LNA.

These frequencies are the main contributors to test failures. If these frequencies pass the test under the stricter exception number in Step (a), the OOBB requirement for the carrier is considered met. If the number of the exceptions exceeds the threshold in Step (a) under the stricter exception conditions, then the step (b) is executed, which covers a full-range frequency scan.

Step (a): Set the parameters of the CW signal generator for an interfering signal below the wanted signal according to Table 7.6.3.5-2a or Table 7.6.3.5-2b. The number of exceptions should be the threshold (range from 0 to ) defined in clause 7.6.3.5.

Step (b): Set the parameters of the CW signal generator for an interfering signal below the wanted signal according to Table 7.6.3.5-2 or Table 7.6.3.5-4. The number of exceptions is defined in clause 7.6.3.5.

Proposal 1:	Introduce an optional test step for OOBB test optimization to reduce the test time. The OOBB testing procedure for interfering signal setting can be divided into two steps, with Step (a) being optional. Step (a) applies a more stringent exception count and focuses on the test frequencies that are the primary contributors to test failures. If the test fails under the Step (a) scenario, Step (b) is performed, which involves a full-range frequency scan according to the standard procedure.

Proposal 2:	RAN5 to discuss and decide the threshold for the number of the exception in step(a).

Example – Band n3:

Taking Band n3 as an example, according to the spec, OOBB measurements shall be performed with the Lowest, Mid, and Highest test channel bandwidths, and Range 3 shall be tested only with the highest channel bandwidth. When sweeping the interferer frequency from 1 to 12750 MHz with the range of 1–1790 MHz and 1895–12750 MHz and with a step size of 5 MHz, the total number of test points is 2602. By applying the proposed optimized method to Band n3 as followed, the number of interferer frequency points can be reduced to 221.

According to the OOBB optimization method, the scan ranges of the interferer frequencies near the passband of Band Filter can be at 1705 to 1790MHz and 1895 to 1980MHz to check the characteristics of band filter and LNA.

- The frequency components of the downlink signal in Band 3 and interference fall within the passband of the filter.

| product label | Product | funwanted range |
| --- | --- | --- |
| IM3 product1 | 2×funwanted−fwanted | 1823.75 - 1861.25 |
| IM3 product2 | 2×fwanted − funwanted | 1805 - 1880 |
| IM5 product1 | 3×funwanted −2×fwanted | 1830 - 1855 |
| IM5 product2 | 3×fwanted −2×funwanted | 1823.75 - 1861.25 |
| 1/3 harmonic product | funwanted −1/3×fwanted | 576.67 - 651.67 |
| 1/5 harmonic product | funwanted −1/5×fwanted | 331 - 406 |
| 3 harmonic product | funwanted −3×fwanted | 5490 - 5565 |
| 5 harmonic product | funwanted −5×fwanted | 9175 - 9250 |

- The frequency components of the uplink signal in Band 3 and interference fall within the passband of the filter.

| product label | Product | funwanted range |
| --- | --- | --- |
| IM3 product1 | 2×funwanted−fwanted | 1776.25 - 1813.75 |
| IM3 product2 | 2×fwanted − funwanted | 1615 - 1690 |
| IM5 product1 | 3×funwanted −2×fwanted | 1766.67 – 1791.67 |
| IM5 product2 | 3×fwanted −2×funwanted | 1681.25 – 1718.75 |
| 1/3 harmonic product | funwanted −1/3×fwanted | 545 - 620 |
| 1/5 harmonic product | funwanted −1/5×fwanted | 5205 - 5280 |
| 3 harmonic product | funwanted −3×fwanted | 312 - 387 |
| 5 harmonic product | funwanted −5×fwanted | 8700 - 8775 |

Spec Implementation

7.6.3.4.2	Test Procedure

1.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.6.3.4.1-1. The SS sends downlink MAC padding bits on the DL RMC.

2.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 7.6.3.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

Subtest-1:

3.	Set the parameters of the CW signal generator for an interfering signal below the wanted signal according to Table 7.6.3.5-2a or 7.6.3.5-4a. The frequency step size is  MHz.

4.	Set the downlink signal level according to the table 7.6.3.5-1 or 7.6.3.5-3. Send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as -MU to -(MU + Uplink power control window size) dB of the target power level in Table 7.6.3.5-1 or Table 7.6.3.5-3 for at least the duration of the Throughput measurement, where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.3-1 for the carrier frequency f and the channel bandwidth BW

-	Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified for test case 6.3.4.3 in Table F.1.2-1.

-	For UEs supporting Tx diversity, the transmit power is measured as the sum of the output power from both UE antenna connectors.

5.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex H.

6.	Record the frequencies for which the throughput doesn't meet the requirements.

7.	Repeat steps from 3 to 6, using an interfering signal above the wanted signal at step 3.

Subtest-2:

3a.	Set the parameters of the CW signal generator for an interfering signal below the wanted signal according to Table 7.6.3.5-2 or 7.6.3.5-4. The frequency step size is  MHz.

4a.	Set the downlink signal level according to the table 7.6.3.5-1 or 7.6.3.5-3. Send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as -MU to -(MU + Uplink power control window size) dB of the target power level in Table 7.6.3.5-1 or Table 7.6.3.5-3 for at least the duration of the Throughput measurement, where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.3-1 for the carrier frequency and the channel bandwidth BW

-	Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified for test case 6.3.4.3 in Table F.1.2-1.

-	For UEs supporting Tx diversity, the transmit power is measured as the sum of the output power from both UE antenna connectors.

5a.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex H.

6a.	Record the frequencies for which the throughput doesn't meet the requirements.

7a.	Repeat steps from 3a to 6a, using an interfering signal above the wanted signal at step 3a.

If the number of frequencies recorded in step 6 and step 7 in subtest-1 exceed the [TBD] defined in clause 7.6.3.5, subtset-2 is executed. Otherwise, subtest-2 can be skipped.

NOTE:	The purpose of the Uplink power control window is to ensure that the actual UE output power is no greater than the target power level, and as close as possible to the target power level. The relationship between the Uplink power control window, the target power level and the corresponding possible actual UE Uplink power window is illustrated in Annex F.4.3.

7.6.3.5	Test Requirement

For NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz, the throughput measurement derived in test procedure shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.3.2 and A.3.3 with parameters specified in Tables 7.6.3.5-1 and 7.6.3.5-2 or 7.6.3.5-2a.

For NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz, if the total number of spurious response frequencies recorded in the step 6 and 7 does not exceed [TBD] in each assigned frequency channel, the requirements of clause 7.7 Spurious Response are applicable for the exceptions in subtest-1. Otherwise, the number of spurious response frequencies recorded in subtest-2 shall not exceed  in each assigned frequency channel when measured using a  MHz step size, the requirements of clause 7.7 Spurious Response are applicable for the exceptions in subtest-2.

Table 7.6.3.5-1: Out-of-band blocking parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| Channel bandwidth | Power in transmission bandwidth configuration [dBm] |
| --- | --- |
| 3 MHz | REFSENS + 6.0 dB |
| 5 MHz | REFSENS + 6.0 dB |
| 10 MHz | REFSENS + 6.0 dB |
| 15 MHz | REFSENS + 7.0 dB |
| 20 MHz | REFSENS + 9.0 dB |
| 25 MHz | REFSENS + 10.0 dB |
| 30 MHz | REFSENS + 11.0 dB |
| 35 MHz | REFSENS + 11.5 dB |
| 40 MHz | REFSENS + 12.0 dB |
| 45 MHz | REFSENS + 12.5 dB |
| 50 MHz | REFSENS + 13.0 dB |
| 60 MHz | REFSENS + 14.0 dB |
| 70 MHz | REFSENS + 14.5 dB |
| 80 MHz | REFSENS + 15.0 dB |
| 90 MHz | REFSENS + 15.5 dB |
| 100 MHz | REFSENS + 16.0 dB |
| NOTE: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2.3-3 with PCMAX_L,f,c defined in clause 6.2.4. |  |

Table 7.6.3.5-2: Out of-band blocking for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| NR band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
| n1, n2, n3, n5, n7, n8, n12, n13, n14, n20, n24, n25, n26, n28, n30, n31, n34, n38, n39, n40, n41, n485, n50, n51, n538, n54, n65, n66, n70, n71, n72, n74, n85, n91, n92, n93, n94, n100, n101, n105, n106 | Pinterferer | dBm | -44 | -30 | -15 |
|  | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 6000 MHz. NOTE 2: For band 51 the FDL_high of band 50 is applied as FDL_high for band 51. For band 50, the FDL_low of band 51 is applied as FDL_low for band 50. NOTE 3: For band 76 the FDL_high of band 75 is applied as FDL_high for band 76. For band 75, the FDL_low of band 76 is applied as FDL_low for band 75. NOTE 4: For UEs supporting both bands 38 and 41, the FDL_high and FDL_low of band 41 is applied as FDL_high and FDL_low for band 38. NOTE 5: n48 follows the requirement in this frequency range according to the general requirement defined in Clause 7.1. The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 2700 MHz and FInterferer < 4800 MHz. NOTE 6: Void. NOTE 7: For UE supporting both bands 25 and 70, the FDL_high of band 70 is applied as FDL_high for band 25, and the FDL_low of band 25 is applied as FDL_low for band 70. NOTE 8: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer ≥ 2580 MHz and FInterferer < 2775 MHz. NOTE 9: Range 3 shall be tested only with the highest channel bandwidth. |  |  |  |  |  |

Table 7.6.3.5-2a: Out of-band blocking for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| NR band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
| n3 | Pinterferer | dBm | -44 | -30 | -15 |
|  | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 312 - 406 545 - 651.67 1615 - 1720 1965 – 1980 5205 - 5280 5490 – 5565 8700 - 8775 9175 - 9250 |
| NOTE 1: For frequency range 1≤ f < FDL_low – [100] and FDL_high + [100] < f ≤ 12750, execute the test when the IM3/IM5 products of wanted signal and interfering signal fall into [fc - BW/2, fc + BW/2], and the mixer product of interferer and 1/3rd, 1/5th , 3rd, 5th harmonic of wanted signal fall into [-BW/2, BW/2]. |  |  |  |  |  |

For NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, the throughput measurement derived in test procedure shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.3.2 and A.3.3 with parameters specified in Tables 7.6.3.5-3 and 7.6.3.5-4 or 7.6.3.5-4a.

For NR bands with FDL_high  ≥ 3300 MHz and FUL_low ≥ 3300 MHz, if the total number of spurious response frequencies recorded in the step 6 and 7 does not exceed [TBD] in each assigned frequency channel, the requirements of clause 7.7 Spurious Response are applicable for the exceptions in subtest-1. Otherwise, the number of spurious response frequencies recorded in subtest-2 shall not exceed  in each assigned frequency channel when measured using a  MHz step size, the requirements of clause 7.7 Spurious Response are applicable for the exceptions in subtest-2.

Table 7.6.3.5-3: Out-of-band blocking parameters for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| RX parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 10 | 15 | 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| Power in transmission bandwidth configuration | dBm | REFSENS + 6 dB | REFSENS + 7 dB | REFSENS + 9 dB |
| NOTE: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2.3-3 with PCMAX_L,f,c defined in clause 6.2.4. |  |  |  |  |

Table 7.6.3.5-4: Out of-band blocking for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| NR band | Parameter | Unit | Range1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
| n77, n78 (NOTE 3) | Pinterferer | dBm | -44 | -30 | -15 |
|  | Finterferer (CW) | MHz | -60 < f – FDL_low ≤ -3*BWChannel or 3*BWChannel ≤ f – FDL_high < 60 | -200 < f – FDL_low ≤ -MAX(60,3*BWChannel) or MAX(60,3*BWChannel) ≤ f – FDL_high < 200 | 1 ≤ f ≤ FDL_low – MAX(200,3*BWChannel) or FDL_high + MAX(200,3*BWChannel) ≤ f ≤ 12750 |
| n79 (NOTE 4) | Finterferer (CW) | MHz | N/A | -150 < f – FDL_low ≤ -MAX(60,3*BWChannel) or MAX(60,3*BWChannel) ≤ f – FDL_high < 150 | 1 ≤ f ≤ FDL_low – MAX(150,3*BWChannel) or FDL_high + MAX(150,3*BWChannel) ≤ f ≤ 12750 |
| n104 (NOTE 6) | Finterferer (CW) | MHz | N/A | -150 < f – FDL_low ≤ -MAX(60,3*BWChannel) or MAX(60,3*BWChannel) ≤ f – FDL_high < 150 | 1 ≤ f ≤ FDL_low – MAX(150,3*BWChannel) or FDL_high + MAX(150,3*BWChannel) ≤ f ≤ 12750 |
| NOTE 1: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 6000 MHz. NOTE 2: BWChannel denotes the channel bandwidth of the wanted signal NOTE 3: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm, for FInterferer > 2700 MHz and FInterferer < 4800 MHz. For BWChannel > 15 MHz, the requirement for Range 1 is not applicable and Range 2 applies from the frequency offset of 3*BWChannel from the band edge. For BWChannel larger than 60 MHz, the requirement for Range 2 is not applicable and Range 3 applies from the frequency offset of 3*BWChannel from the band edge. NOTE 4: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm, for FInterferer > 3650 MHz and FInterferer < 5750 MHz. For BWChannel ≥ 40 MHz, the requirement for Range 2 is not applicable and Range 3 applies from the frequency offset of 3*BWChannel from the band edge. NOTE 5: Range 3 shall be tested only with the highest channel bandwidth. NOTE 6: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm, for FInterferer > 5175 MHz. For BWChannel > 60 MHz, the requirement for Range 2 is not applicable and Range 3 applies from the frequency offset of 3*BWChannel from the band edge. The power level of the interferer (PInterferer) for Range 2 shall be modified to -33 dBm for the range 5925 – MAX(60,3*CBW) ≤ f < FDL_low - MAX(60,3*CBW). |  |  |  |  |  |

Table 7.6.3.5-4a: Out of-band blocking for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

TBD

The OOBB test optimization for CA/DC

For CA\DC case we suggest the similar optimization method. To be more specific, the test requirements can be specified as per band combination. Taking DC_3-n5 as an example, FUL_low _n3= 1710MHz, FDL_high_n3 = 1785MHz, FDL_low _n3= 1805MHz, FDL_high_n3 = 1880MHz, FUL_low _n5= 824MHz, FDL_high_n5 = 849MHz, FDL_low _n5= 869MHz, FDL_high_n5 = 894MHz.

For Band 3, according to the first OOBB optimization method, the scan ranges can be at 1705 to 1790MHz and 1895 to 1980MHz.

- The frequency components of the downlink signal in Band 3 and interference fall within the passband of the filter.

| product label | Product | funwanted range |
| --- | --- | --- |
| IM3 product1 | 2×funwanted−fwanted | 1823.75 - 1861.25 |
| IM3 product2 | 2×fwanted − funwanted | 1805 - 1880 |
| IM5 product1 | 3×funwanted −2×fwanted | 1830 - 1855 |
| IM5 product2 | 3×fwanted −2×funwanted | 1823.75 - 1861.25 |
| 1/3 harmonic product | funwanted −1/3×fwanted | 576.67 - 651.67 |
| 1/5 harmonic product | funwanted −1/5×fwanted | 331 - 406 |
| 3 harmonic product | funwanted −3×fwanted | 5490 - 5565 |
| 5 harmonic product | funwanted −5×fwanted | 9175 - 9250 |

- The frequency components of the uplink signal in Band 3 and interference fall within the passband of the filter.

| product label | Product | funwanted range |
| --- | --- | --- |
| IM3 product1 | 2×funwanted−fwanted | 1776.25 - 1813.75 |
| IM3 product2 | 2×fwanted − funwanted | 1615 - 1690 |
| IM5 product1 | 3×funwanted −2×fwanted | 1766.67 – 1791.67 |
| IM5 product2 | 3×fwanted −2×funwanted | 1681.25 – 1718.75 |
| 1/3 harmonic product | funwanted −1/3×fwanted | 545 - 620 |
| 1/5 harmonic product | funwanted −1/5×fwanted | 5205 - 5280 |
| 3 harmonic product | funwanted −3×fwanted | 312 - 387 |
| 5 harmonic product | funwanted −5×fwanted | 8700 - 8775 |

- The frequency components of the downlink signal in Band n5 and interference fall within the passband of the filter.

| product label | Product | funwanted range |
| --- | --- | --- |
| IM3 product1 | 2×funwanted−fwanted | 1343.25 – 1380.75 |
| IM3 product2 | 2×fwanted − funwanted | NA |
| IM5 product1 | 3×funwanted −2×fwanted | 1189.3 – 1214.3 |
| IM5 product2 | 3×fwanted −2×funwanted | 382.25 – 419.75 |
| 1/3 harmonic product | funwanted −1/3×fwanted | 256.3 – 331.3 |
| 1/5 harmonic product | funwanted −1/5×fwanted | 2607 - 2682 |
| 3 harmonic product | funwanted −3×fwanted | 138.8 – 213.8 |
| 5 harmonic product | funwanted −5×fwanted | 4370 - 4445 |

- The frequency components of the uplink signal in Band n5 and interference fall within the passband of the filter.

| product label | Product | funwanted range |
| --- | --- | --- |
| IM3 product1 | 2×funwanted−fwanted | 1320.75 - 1358.25 |
| IM3 product2 | 2×fwanted − funwanted | NA |
| IM5 product1 | 3×funwanted −2×fwanted | 1159.3 – 1184.3 |
| IM5 product2 | 3×fwanted −2×funwanted | 314.75 –352.25 |
| 1/3 harmonic product | funwanted −1/3×fwanted | 241.33 – 316.33 |
| 1/5 harmonic product | funwanted −1/5×fwanted | 2742 - 2547 |
| 3 harmonic product | funwanted −3×fwanted | 129.8 – 204.8 |
| 5 harmonic product | funwanted −5×fwanted | 4145 - 4220 |

The same calculation method applies to Band n5. The final scanning frequency range is the union of all the analyzed frequency points above.

Proposal 3:	For CA\DC case, test the interference frequency when the intermodulation and harmonic products of the wanted carrier 1 and interference fall in the frequency range of wanted carrier 2, or vice versa. Only ≤ 5 times intermodulation and ≤ 1/5~5 times harmonic in range 1, range 2, range 3 test frequency points are tested.

Proposal 4:	Introduce an optional test step for OOBB test optimization to reduce the test time for CA and EN-DC band combination. The OOBB testing procedure for interfering signal setting can be divided into two steps, with Step (a) being optional. Step (a) applies a more stringent exception count and focuses on the test frequencies that are the primary contributors to test failures. If the test fails under the Step (a) scenario, Step (b) is performed, which involves a full-range frequency scan according to the standard procedure.

One way to modify the out-of-band blocking configuration for EN-DC and CA can be as follows.

Table 7.6B.3.5-4a: Out of-band blocking for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| Band Combination | NR band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- | --- |
| DC_3-n5, DC_5-n3, CA_n3-n5 |  | Pinterferer | dBm | -44 | -30 | -15 |
|  | n5, Band 5 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 154.8 – 188.8 266.33 – 381 570 – 595 601.67 – 626.67 769 – 854 909 – 994 1308.25 – 1320.75 1355.75 – 1368.25 1454.67 – 1463 1518 – 1526.33 2174.25 – 2186.75 2316.75 – 2329.25 2497 – 2522 2632 – 2657 2791 – 2816 4170 – 4195 4395 – 4420 5230 – 5255 5515 – 5540 8725 – 8750 9200 – 9225 |
|  | n3, Band 3 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 129.8 – 213.8 241.33 – 419.75 545 – 651.67 1159.33 – 1214.33 1320.75-1380.75 1615 – 1790 1776.25 – 1813.75 1895 – 1980 2472 – 2547 2607 – 2682 4145 – 4220 4370 – 4445 5205 – 5280 5490 – 5565 8700 – 8775 9175 – 9250 |
| NOTE 1: For frequency range 1≤ f < FDL_low – [100] and FDL_high + [100] < f ≤ 12750, execute the test when the IM3/IM5 products of wanted signal and interfering signal fall into [fc - BW/2, fc + BW/2], and the mixer product of interferer and 1/3rd, 1/5th , 3rd, 5th harmonic of wanted signal fall into [-BW/2, BW/2]. |  |  |  |  |  |  |

Another way could be as follows.

Table 7.6B.3.5-4a: Out of-band blocking for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| NR band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
| DC_3-n5, DC_5-n3, CA_n3-n5 | Pinterferer | dBm | -44 | -30 | -15 |
|  | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 129.8 – 213.8 241.33 – 419.75 545 – 651.67 769 – 854 909 – 994 1159.33 – 1184.33 1189.33 – 1214.33 1308.25 – 1380.75 1454.67 – 1463 1518 – 1526.33 1615 – 1790 1776.25 – 1813.75 1895 – 1980 2174.25 – 2186.75 2316.75 – 2329.25 2472 – 2547 2607 – 2682 2791 – 2816 4145 – 4220 4370 – 4445 5205 – 5280 5490 – 5565 8700 – 8775 9175 – 9250 |
| NOTE 1: For frequency range 1≤ f < FDL_low – [100] and FDL_high + [100] < f ≤ 12750, execute the test when the IM3/IM5 products of wanted signal and interfering signal fall into [fc - BW/2, fc + BW/2], and the mixer product of interferer and 1/3rd, 1/5th , 3rd, 5th harmonic of wanted signal fall into [-BW/2, BW/2]. |  |  |  |  |  |

Conclusion

Based on the above discussion, following observations can be obtained:

Proposal 1:	Introduce an optional test step for OOBB test optimization to reduce the test time. The OOBB testing procedure for interfering signal setting can be divided into two steps, with Step (a) being optional. Step (a) applies a more stringent exception count and focuses on the test frequencies that are the primary contributors to test failures. If the test fails under the Step (a) scenario, Step (b) is performed, which involves a full-range frequency scan according to the standard procedure.

Proposal 2:	RAN5 to discuss and decide the threshold for the number of the exception in step(a).

Proposal 3:	For CA\DC case, test the interference frequency when the intermodulation and harmonic products of the wanted carrier 1 and interference fall in the frequency range of wanted carrier 2, or vice versa. Only ≤ 5 times intermodulation and ≤ 1/5~5 times harmonic in range 1, range 2, range 3 test frequency points are tested.

Proposal 4:	Introduce an optional test step for OOBB test optimization to reduce the test time for CA and EN-DC band combination. The OOBB testing procedure for interfering signal setting can be divided into two steps, with Step (a) being optional. Step (a) applies a more stringent exception count and focuses on the test frequencies that are the primary contributors to test failures. If the test fails under the Step (a) scenario, Step (b) is performed, which involves a full-range frequency scan according to the standard procedure.

- 4 References

- R5-252669 “Discussion on how to reduce the measurement complexity of Out-of-Band Blocking in RAN5”, Huawei, HiSilicon.

- R5-254633 “Discussion on the optimization of Out-of-band Blocking (OOBB) testing”, Huawei, HiSilicon.