3GPP TSG-RAN5 Meeting #109	R5-256276

# Dallas, US, 17th – 21st November, 2025

Agenda item:	5.4.20

Source:	Keysight Technologies UK Ltd

Title:	Discussion on Throughput Test Requirements

Document for:	Discussion and Endorsement

# Introduction

Upon reviewing Annex G.1.4, a potential discrepancy has been identified in the throughput test limits defined in TS 38.521-4 [1]. These limits may be subject to varying interpretations and implementations by different test equipment vendors, potentially leading to inconsistencies in test result standardization across platforms.

This contribution seeks to establish a unified interpretation of the applicable throughput test limits by performing a detailed analysis of the specification. The objective is to eliminate ambiguity and ensure precision in the definition and application of test limits for PDSCH and PDCCH test cases.

# Background

The potential misalignment could appear from differing interpretations of the test procedures and requirements specified for certain test cases. To illustrate this more clearly, we refer to test case 5.2.2.1.1_1, titled 2Rx FDD FR1 PDSCH Mapping Type A Performance – 2x2 MIMO with Baseline Receiver for both SA and NSA modes in TS 38.521-4 [1].

5.2.2.1.1_1.3.2	Test procedure

1.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Tables 5.2.2.1.1_1.4-1 and 5.2.2.1.1_1.4-2. The SS sends downlink MAC padding bits on the DL RMC.

2.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix and the SNR according to Tables 5.2.2.1.1_1.4-1 and 5.2.2.1.1_1.4-2 as appropriate.

3.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL during each subtest and decide pass or fail according to Table G.1.5-1 in Annex G clause G.1.5.

4.	Repeat steps from 1 to 3 for each subtest in Tables 5.2.2.1.1_1.4-1 and 5.2.2.1.1_1.4-2 as appropriate.

5.2.2.1.1_1.4	Test requirement

Tables 5.2.2.1.1_1.4-1 and 5.2.2.1.1_1.4-2 define the primary level settings.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A 3.2.1 for each throughput test shall meet or exceed the specified value in Table 5.2.2.1.1_1.4-1 and Table 5.2.2.1.1_1.4-2 for the specified SNR including test tolerances for all throughput tests.

Table 5.2.2.1.1_1.4-1: Test Requirements for Rank 1

| Test num. | Reference channel | Modulation format | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-1.1 FDD | QPSK, 0.30 | TDLB100-400 | 2x2, ULA Low | 70 | 0.1 |
| 1-2 | R.PDSCH.1-1.2 FDD | QPSK, 0.30 | TDLC300-100 | 2x2, ULA Low | 70 | 1.1 |
| 1-3 | R.PDSCH.1-4.1 FDD | 256QAM, 0.82 | TDLA30-10 | 2x2, ULA Low | 70 | 25.6 |
| 1-4 | R.PDSCH.1-2.1 FDD | 16QAM, 0.48 | TDLC300-100 | 2x2, ULA Low | 30 | 2 |
| 1-5 | R.PDSCH.1-8.1 FDD | 16QAM, 0.48 | HST-750 | 1x2 | 70 | 7.1 |
| 1-6 | R.PDSCH.1-8.2 FDD | 64QAM, 0.43 | HST-972 | 1x2 | 70 | [10.5] |
| 1-7 | R.PDSCH.1-8.1 FDD | 16QAM, 0.48 | TDLC300-600 | 2x2 | 70 | [9.5] |

Table 5.2.2.1.1_1.4-2: Test Requirements for Rank 2

| Test num. | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | R.PDSCH.1-3.1 FDD | 64QAM, 0.51 | TDLA30-10 | 2x2, ULA Low | 70 | 20.4 |
| 2-2 | R.PDSCH.2-1.1 FDD | 64QAM, 0.51 | TDLA30-10 | 2x2, ULA Low | 70 | 20.7 |

The procedure states that the pass/fail decision should be according to Table G.1.5-1. This table doesn’t include pass fail decision limits but Minimum Test Time.

## G.1.5	Minimum Test time

If a pass fail decision in clause G.1.4 can be achieved earlier than the minimum test time, then the test shall not be decided, but continued until the minimum test time is elapsed.

The tables below contain the minimum number of slots for FDD and TDD.

By simulations the minimum number of active subframes (carrying DL payload) was derived (MNAS), then adding inactive subframes to the active ones. (for TDD additional subframes contain no DL payload), then rounding up to full thousand.

Simulation method to derive minimum test time:

With a level, corresponding a throughput at the test limit (here 30 % or 35 % or 50 % or 70 % of the max. throughput) the preliminary throughput versus time converges towards the final throughput. The allowance of ± 2% throughput variation around the above mentioned level is predefined by RAN5 to find the minimum test time. The allowance of ± 2% maps through the function "final throughput versus level" into a throughput corridor. The minimum test time is achieved when the preliminary throughput escapes the corridor the last time. The two functions "final throughput versus level" and "preliminary throughput versus time" are simulation results, which are done individual for each demodulation scenario.

In the case where the throughput does not converge across all the seeds used in the simulation within a reasonable time duration, then the throughput corridor is expanded to +/- 3.5% of the target throughput to see if the all the seeds converge. In order to account for the additional throughput variation, a SNR uncertainty due to finite test time term is added to the overall MU calculation.

The entire procedure is summarized as below.

### G.1.5.0_1	Minimum Test Time procedure for PDSCH scenarios with 30% or 35% or 50% or 70% Throughput requirement

1.	Run the PDSCH simulations for an extended period of time at the SNR of interest for multiple seeds.

2.	The minimum test time is determined by the time it took the throughput curve to settle within +/- 2% of the target throughput value across all seeds.

3.	If the throughput does not settle within the +/- 2% throughput limit within a reasonable amount of time for any seed, expand the threshold to +/- 3.5% of the target throughput value.

4.	To account for the additional 1.5% throughput variation, determine from the Throughput vs SNR curve, what is the SNR delta corresponding to this 1.5 % throughput increase.

5.	That SNR value is the added SNR uncertainty due to finite test time.

There is another Annex, G.1.4. that define test limit.

## G.1.4	Pass Fail limit

Testing Throughput = 30 %, then the test limit is

Number of successes (ACK) / number of samples ≥ 59 / 233

Testing Throughput = 70 % then the test limit is

Number of fails (NACK and statDTX) / number of samples ≤ 66 / 184

Testing BLER = 1% then the test limit is

Number of fails (NACK and statDTX) / number of samples ≤ 163 / 13135

There are 3 distinct cases:

a)	The duration for the number of samples (233, 184 or 13135) is greater than the minimum test time:

Then the number of samples (233, 184 or 13135) is predefined and the decision is done according to the number of events (59 successes, 66 fails or 163 fails)

b)	Since subframe 0 and 5 contain less bits than the remaining subframes, it is allowed to predefine a number of samples contained in an integer number of frames. In this case test-limit-ratio applies.

c)	The minimum test time is greater than the duration for the number of samples:

The minimum test time is predefined and the decision is done comparing the measured ratio at that instant against the test-limit-ratio.

NOTE:	The test time for most of the tests is governed by the Minimum Test Time.

Based on test procedure, test requirements, Annex G.1.4 and Annex G.1.5 there are two possible interpretations that need to be discussed.

Interpretation 1: Throughput limit based on Annex G.1.4 Pass Fail limit.

- The minimum test time is defined by Annex G.1.5 (“With a level, corresponding a throughput at the test limit (here 30 % or 35 % or 50 % or 70 % of the max. throughput)”), but does not provide explicit pass/fail thresholds.

- In this approach, the pass/fail decision is derived from Annex G.1.4, which provides statistical limits based on ACK/NACK/statDTX counts.

- Ex. For cases with a 70% throughput requirement, the effective threshold is interpreted as 64.13%. Considering that the minimum test time is greater than the duration of the number of samples, the measured ratio must be compared with the test-limit-ratio. Using as an example the Fraction of Maximum Throughput of 70% from the Test Requirements, for Testing Throughput could be considered equal to 1 – 66/184 (1- Number of fails (NACK and statDTX) / number of samples). In this case, the throughput limit would be 64.13%.

Interpretation 2: Throughput limit based on Tables 5.2.2.1.1_1.4-1 and 5.2.2.1.1_1.4-2 of test requirements.

- Throughput limit is directly taken from test requirements tables.

- Annex G.1.5 is used solely to define the minimum test time.

- The pass/fail decision is made by comparing the measured throughput against the fraction of maximum throughput specified in test requirements tables.

- Ex. For cases with a 70% of fraction of maximum throughput requirement, the effective threshold is interpreted as 70%.

As a summary, test limits may be derived from two distinct sections of the specification. While Annex G.1.4 define pass/fail test limit, test requirements impose a throughput limit with specific values in Tables 5.2.2.1.1_1.4-1 and 5.2.2.1.1_1.4-2. This dual reference introduces potential ambiguity in determining the correct criteria for test case verdict.

# Discussion

## Requirement references in test cases

In every test case, the Test Procedure provides detailed steps for executing the test, while the Test Requirements define the criteria for the final pass/fail verdict—specifically, whether the measured performance meets the thresholds outlined in the requirement tables.

Importantly, Annex G.1.4 is not referenced anywhere in the test case as a source for pass/fail criteria.

Observation 1. Annex G.1.4, which includes statistical limits based on sample counts, is not cited in the test case as a source for pass/fail criteria.

In Step 3 of the test procedure, the following is stated:

“Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL during each subtest and decide pass or fail according to Table G.1.5-1 in Annex G clause G.1.5.”

This wording does not imply that Table G.1.5-1 defines the pass/fail threshold. Rather, it indicates that the decision should be made in accordance with the table, which primarily defines minimum test time, not performance thresholds.

The actual criteria for pass/fail are specified in the Test Requirements.

Observation 2. The possible discrepancy in interpreting test verdict criteria arises from conflating the roles of the Test Procedure, Test Requirements, and Annex G. While the Test Procedure outlines how the test should be executed, the Test Requirements—imported from TS 38.101-4 [2]—clearly define the performance thresholds that determine pass/fail outcomes.

## Minimum test time vs Throughput limit

As stated in Annex G.1.5:

Simulation method to derive minimum test time:

With a level, corresponding a throughput at the test limit (here 30 % or 35 % or 50 % or 70 % of the max. throughput) the preliminary throughput versus time converges towards the final throughput.

The minimum test time defined in Annex G.1.5 has been calculated for a particular propagation conditions (SNR, fading profile, doppler…) considering those test limits. If a test limit other than 30%, 35%, 50%, or 70% is being considered (like the 64.13% in the Example for the interpretation 1), the minimum test time must also be adjusted accordingly.

Observation 3. Using a test limit different than 30%, 35%, 50% or 70% would imply to adjust also de minimum test time.

## Test Tolerances applied to test requirements

The requirements are derived from TS 38.101-4 [2], and within RAN5, they have already been relaxed based on the outcome of measurement uncertainties analysis. Annex F reflects the tolerances applied for each test case, as shown in the image below for the test case of the example.

As stated in the picture above, T-put limit remains unchanged but RAN5 defines tolerance in SNR requirements based on measurement uncertainties. Therefore, if Annex G.1.4 should define the threshold (interpretation 1), in the example test limit is 64.13%, which is 5.87% below the throughput limit defined by TS 38.101-4 [2], then would effectively double-relax the requirements, one in throughput requirements via Annex G.1.4. and other SNR requirements via test tolerance.

Observation 4. Using Annex G.1.4. as verdict criteria would effectively double-relax the requirements, one in throughput requirements via Annex G.1.4. and other SNR requirements via test tolerance.

Proposal 1. Use the fraction of maximum throughput defined in the Test Requirements tables as the sole criteria for pass/fail decisions.

Proposal 2. RAN5 to evaluate which test cases should apply Annex G.1.4 (if any).

Proposal 3. Include a reference in the Test Procedure to specify in each test case where to find the verdict threshold (Example in Annex A).

Observation 5. Same principles exposed in this discussion paper applies to demodulation for NTN test cases included in TS 36.521-4 and TS 38.521-5.

Proposal 4. To extend the same understanding of using the fraction of maximum throughput defined in the Test Requirements as pass/fail decision criteria to the demodulation test cases defined in TS 36.521-4 and TS 38.521-5.

# Conclusion

The following observations and proposals were made in this contribution.

Observation 1. Annex G.1.4, which includes statistical limits based on sample counts, is not cited in the test case as a source for pass/fail criteria.

Observation 2. The possible discrepancy in interpreting test verdict criteria arises from conflating the roles of the Test Procedure, Test Requirements, and Annex G. While the Test Procedure outlines how the test should be executed, the Test Requirements—imported from TS 38.101-4 [2]—clearly define the performance thresholds that determine pass/fail outcomes.

Observation 3. Using a test limit different than 30%, 35%, 50% or 70% would imply to adjust also de minimum test time.

Observation 4. Using Annex G.1.4. as verdict criteria would effectively double-relax the requirements, one in throughput requirements via Annex G.1.4. and other SNR requirements via test tolerance.

Observation 5. Same principles exposed in this discussion paper applies to demodulation for NTN test cases included in TS 36.521-4 and TS 38.521-5.

Proposal 1. Use the fraction of maximum throughput defined in the Test Requirements tables as the sole criteria for pass/fail decisions.

Proposal 2. RAN5 to evaluate which test cases should apply Annex G.1.4 (if any).

Proposal 3. Include a reference in the Test Procedure to specify in each test case where to find the verdict threshold (Example in Annex A).

Proposal 4. To extend the same understanding of using the fraction of maximum throughput defined in the Test Requirements as pass/fail decision criteria to the demodulation test cases defined in TS 36.521-4 and TS 38.521-5.

# References

- TS 38.521-4, “NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 4: Performance”, v18.8.0 (2025-10)

- TS 38.101-4, “NR; User Equipment (UE) radio transmission and reception; Part 4: Performance requirements”, v19.0.0 (2025-10)

# Annex A	Proposal of change in Test Procedure

Using the same example than the one used in this discussion paper, currently the test procedure looks like this:

5.2.2.1.1_1.3.2	Test procedure

1.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Tables 5.2.2.1.1_1.4-1 and 5.2.2.1.1_1.4-2. The SS sends downlink MAC padding bits on the DL RMC.

2.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix and the SNR according to Tables 5.2.2.1.1_1.4-1 and 5.2.2.1.1_1.4-2 as appropriate.

3.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL during each subtest and decide pass or fail according to Table G.1.5-1 in Annex G clause G.1.5.

4.	Repeat steps from 1 to 3 for each subtest in Tables 5.2.2.1.1_1.4-1 and 5.2.2.1.1_1.4-2 as appropriate.

The test procedure could avoid misunderstanding by modifying this sentence:

5.2.2.1.1_1.3.2	Test procedure

1.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Tables 5.2.2.1.1_1.4-1 and 5.2.2.1.1_1.4-2. The SS sends downlink MAC padding bits on the DL RMC.

2.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix and the SNR according to Tables 5.2.2.1.1_1.4-1 and 5.2.2.1.1_1.4-2 as appropriate.

3.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL during each subtest and decide pass or fail according to Table G.1.5-1 in Annex G clause G.1.5 based on Fraction of maximum throughput (%) in Table 5.2.2.1.1_1.4-1 and Table 5.2.2.1.1_1.4-2.

4.	Repeat steps from 1 to 3 for each subtest in Tables 5.2.2.1.1_1.4-1 and 5.2.2.1.1_1.4-2 as appropriate.