3GPP TSG-RAN5 Meeting #108			R5-25xxxx

Bengaluru, India 25 - 29 August 2025

Title:	[DRAFT] LS on E-UTRA uplink configurations for intra-band ENDC MSD

Response to:	LS (R5-253716) on E-UTRA uplink configurations for intra-band ENDC MSD from WG4

Release:	Rel-19

Work Item:	NR_CADC_NR_LTE_DC_R19-UEConTest

Source:	TSG RAN WG5

To:	TSG RAN WG4

Cc:

Contact Person:

#### Name:	Chunying Gu

E-mail Address:	guchunying@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN5 would like to thank RAN4 for informing their agreements about the intra-band contiguous EN-DC MSD requirements when the E-UTRA uplink configurations are specified as “N/A”.

Currently RAN5 interprets the “N/A” in the uplink configuration of REFSENS MSD requirements as there is no scheduling of PUSCH during the REFSENS measurement period. The E-UTRA carrier will be still configured and activated to ensure UE is working with the corresponding EN-DC configuration. This also applies to inter-band EN-DC MSD requirements.

RAN5 would like to further share the specific status related to RAN4 agreements as below:

RAN4 agreement 1:

E-UTRA uplink carrier frequency “Fc (UL)”: RAN4 recommends configuring the E-UTRA Fc (UL) according to the E-UTRA band’s default Tx-Rx frequency separation specified in TS 36.101 Table 5.7.4-1;

RAN5 status:

In RAN5 test procedures, the E-UTRA uplink carrier frequency is set according to the E-UTRA band’s default Tx-Rx frequency separation.

RAN4 agreement 2:

E-UTRA uplink RB allocation “Lcrb”: RAN4 recommends configuring the E-UTRA UL RB allocation according to the UL configuration specified in TS 36.101 Table 7.3.1-2 of the corresponding E-UTRA DL CBW;

RAN5 status:

In RAN5 test procedures, the E-UTRA UL RB allocation is set to 0 during the REFSENS measurement period, i.e.  no scheduling of PUSCH. Before the REFSENS measurement, the E-UTRA carrier is configured and activated following the RAN5 generic procedure so that UE could work with the expected EN-DC configuration.

RAN4 agreement 3:

E-UTRA transmission power: RAN4 recommends configuring the E-UTRA transmit power as anchor agnostic, or preferably at minimum output power.

RAN5 status:

In RAN5 test procedure, only PUCCH is transmitted on E-UTRA carrier. RAN5 will continue to figure out methods to avoid the interference from PUCCH on the REFSENS measurement, including the option of configuring the PUCCH transmit power to minimum output power.

2. Actions:

To RAN4 group.

ACTION: 	RAN5 asks RAN4 to take into consideration of RAN5’s current handling of the “N/A” in uplink configurations of REFSENS MSD requirements and provide feedback if this is not aligned with RAN4’s assumption of the requirements.

3. Date of Next TSG-RAN WG5 Meetings:

TSG-RAN5 Meeting#109	 17th – 21st November 2025	Dallas, US

TSG-RAN5 Meeting#110	 9th – 13th February 2026	Gothenburg, Sweden