3GPP TSG-RAN5 Meeting #110	R5w260011r1

# Gothenburg Metropolitan Area, Sweden, 9th Feb 2026 - 13th Feb 2026

Agenda Item:	TTCN sidebar

Source:	MCC TF160

Title:	Rel-17 NR: Discussion on SSSG Switch TC 7.1.1.12.5

Document for:	Endorsement

# 1	Introduction

The TC 7.1.1.12.5 is testing Search Space Set Group (SSSG) switching with DL DCI format 1_1. It was delivered as verifiable in TTCN-3 delivery iwd-25wk50. The aim of this document is to explain how it is implemented in TTCN using the current ASP and test model.

At RAN5#110, a new SSSG TC (TC 7.1.1.12.6) is proposed to be added to TS 38.523-1. This test is addressing the SSSG switching with a UL DCI format 0_1 and it is planned to implement it with the same method.

In revision r1 clause 5 is added proposing an ASP enhancement to help SS implementation.

# 2	General

In TC 7.1.1.12.5, UE is configured with 3 ue-specific Search Spaces each one associated to a different SSSG.

At initial configuration (RRCReconfiguration), UE monitors SSSG#0. Upon receipt of the Monitoring Adaptation Field in DCI 1_1 indicating a different group, UE switches SSSG, hence switches SearchSpace.

At the end of the test, UE is configured with 3 Search Spaces configured without any group. UE monitors the 3 Search Spaces together.

Note: according to TS 38.212 and TS 38.213 an SSSG may include multiple search spaces. When this occurs, the UE monitors all search spaces within the applicable group.

In the TTCN implementation, SS is configured with the Search Space actually used by UE. This Search Space is configured as a standard ‘normal’ search space, equivalent to a Search Space without SSSG. This approach allows test case implementation without any ASP change. This is also allowing a deterministic SS behaviour when multiple Search Spaces are monitored by UE also aligned with the prose requirements.

# 3	Test model clarifications

In the DL BWP configuration, the SS is provided with a list of Search Spaces, each search space is defined with:

- Search Space configuration also provided to the UE via the RRC reconfiguration message (timing, coreset, candidate, etc…)

- additional specific parameters for SS: aggregation, priority and a unique SearchSpaceType.

In DCCH/DTCH configuration, the DL/UL DCI are configured with the associated BWP and associated unique SearchSpaceType, hence making the link to the Search Space defined in the DL BWP.

Here are the types in the ASPs definitions

NR_CellConfigInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_CellConfigInfo_Type |  |  |
| Comment | common information for initial cell configuration or reconfiguration; in case of reconfiguration omit means 'keep configuration as it is' |  |  |
| StaticResourceConfig | NR_SS_StaticCellResourceConfig_Type | opt | mandatory for the initial configuration; to be omitted afterwards |
| CellConfigCommon | NR_CellConfigCommon_Type | opt | common configuration parameters which are not specific to physical layer (or any other layer) |
| PhysicalLayer | NR_CellConfigPhysicalLayer_Type | opt | Physical layer configuration: physical channels, signals and BWPs for UL and DL; DCI |
| BcchConfig | NR_BcchConfig_Type | opt | configuration of BCCH/BCH; SS is triggered to configure RLC/MAC accordingly; BCCH data on the PDSCH is distinguished by the SI-RNTI PBCH: MIB; PDSCH: scheduling and resource allocation; SIBs |
| PcchConfig | NR_PcchConfig_Type | opt | configuration of PCCH/PCH; SS is triggered to configure RLC/MAC accordingly; PCCH data on the PDSCH is distinguished by the P-RNTI (needed even to modify SI => shall be configured for CELL_BROADCASTING) |
| RachProcedureConfig | NR_RachProcedureConfig_Type | opt | to configure the SS's behaviour for the RACH procedure; may be omitted at initial configuration e.g. in case that the cell shall not have an uplink; NOTE: there is no way to explicitly remove the RACH procedure configuration after it has been configured for a cell |
| DcchDtchConfig | NR_DcchDtchConfig_Type | opt | Parameters related to DCCH/DTCH in UL and DL |
| ServingCellConfig | NR_ServingCellConfig_Type | opt | To be configured at initial configuration of a cell: for non-CA scenarios it shall be either 'SpCell' or 'None' ('None' applies for pure neighbouring cells) |
| SidelinkScheduled | NR_SidelinkScheduled_Type | opt | To configure SS for sidelink resource allocation mode 1 |
| NtnConfig | NR_NTN_Config_Type | opt | to configure parameters specifically needed for NTN operation |
| AtgConfig | NR_ATG_Config_Type | opt | to configure parameters specifically needed for ATG operation |

NR_CellConfigPhysicalLayer_Type -> Downlink -> BWPs -> Pdcch leads to NR_BWP_PDCCH_Configuration_Type

NR_BWP_PDCCH_Configuration_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_BWP_PDCCH_Configuration_Type |  |  |
| Comment | PDCCH configuration at the SS for specific BWP; NOTE: There are no fields for PDCCH-Config's "downlinkPreemption", "slotFormatIndicator", "tpc-PUSCH" and "tpc-PUCCH": This information is related to triggering DCI formats 2_X and shall be configured there (NR_DCI_Trigger_Type) according to test case requirements |  |  |
| SearchSpaceArray | NR_BWP_SearchSpaceList_Type | opt |  |
| CoresetArray | NR_BWP_CoresetList_Type | opt |  |
| RelativeTxPower | NR_PDCCH_EPREs_Type | opt |  |
| Coreset0_OffsetRBs | integer | opt | Number of Offset RBs as specified in 38.213 clause 13. Applied to CORESET#0 only |
| Coreset0_PuncturingSize | integer | opt | Set to 4 or 9 RBs when CORESET#0 puncturing is applied according to TS 38.211 clause 7.3.2.2 else omitted. Applied to CORESET#0 only. |

NR_BWP_SearchSpaceList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_BWP_SearchSpaceList_Type |
| Comment | list of search spaces defined for a single BWP; according to TS 38.213 clause 10.1 "The Type0-PDCCH common search space has search space index 0" NOTE: even though in general (array) index and searchSpaceId are the same it is not clear what is meant by TS 38.213 |
| record of NR_BWP_SearchSpaceConfig_Type |  |

NR_BWP_SearchSpaceConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_BWP_SearchSpaceConfig_Type |  |  |
| Comment | configuration of a single search space at the SS: The position of a particular search space candidate in frequency and time domain depends on configuration of the search space and its associated CORESET (see TS 38.213 clause 10.1): - Common or UE-specific search space - aggregation level L - number of candidates per aggregation level - PDCCH monitoring periodicity and offset - frequency domain resources - number of symbols (time domain) in addition in case of UE-specific search space: - C-RNTI (as configured for the active cell) - carrier indicator field value (in case of cross carrier scheduling) all fields are mandatory as modification of a single field may cause inconsistencies |  |  |
| TypeAndPriorityList | NR_SearchSpaceTypeAndPriorityList_Type |  | list of search space types (according to TS 38.213 clause 10.1) to be mapped to the given search space configuration. NOTE 1: In general the lists of search space types for different search spaces shall be mutual exclusive; NOTE 2: TS 38.213 clause 10.1 could be read as if the different types of search spaces use distinct instances of search spaces; nevertheless RRC type definitions allow use of different types in one and the same search space |
| AggregationLevel | NR_PDCCH_CCE_AggregationLevel_Type |  | aggregation level to be applied for an actual PDCCH of the given search space |
| SearchSpaceConfigAtUE | NR_ASN1_SearchSpace_Type |  | search space configuration as sent to the UE; contains searchSpaceId and controlResourceSetId referring to the associated CORESET; furthermore the SS may need e.g. the number of candidates to detect error situations in context of candidate selection |

NR_SearchSpaceTypeAndPriority_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SearchSpaceTypeAndPriority_Type |  |  |
| Comment |  |  |  |
| Type | NR_SearchSpaceType_Type |  |  |
| CandidatePriority | NR_SearchSpaceCandidatePriority_Type |  | priority to be considered when candidates of a different or the same search space overlap the same CCEs |

NR_SearchSpaceType_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | NR_SearchSpaceType_Type |
| Comment | types of search spaces according to TS 38.213 clause 10.1; NOTE 1: In principle there could be more than one instance for the same kind of search space; if this is ever needed, this enumerated shall be enhanced by introduction of a second entry for a particular type. NOTE 2: For the USS UL and DL are distinguished as they may need different priority in terms of scheduling of DCIs |
| cssType0 | Type0-PDCCH common search space for scheduling of SIB1 (SI-RNTI); the SS shall scramble the DCI format's CRC - with SI-RNTI for a DCI format 1_0 - with MCCH-RNTI or G-RNTI for a DCI format 4_0 for MBS broadcast; aggregation level and number of candidates correspond to TS 38.213 Table 10.1-1; For stand-alone options parameters of search space and associated CORESET are according MIB.pdcch-ConfigSIB1 (TS 38.213 clause 13) |
| cssType0A | Type0A-PDCCH common search space for scheduling of other system information (SI-RNTI); the SS shall scramble the DCI format's CRC with SI-RNTI |
| cssType0B | Type0B-PDCCH common search space for a DCI format 4_0 with CRC scrambled by MCCH-RNTI or G-RNTI |
| cssType1 | Type1-PDCCH common search space for scheduling of Msg2 or Msg4 of RACH procedure; the SS shall scramble the DCI format's CRC with RA-RNTI for Msg2 or with TC-RNTI for Msg4 (temporary C-RNTI) as configured for Msg4 (NR_TempC_RNTI_Type in NR_RAR_Payload_Type) |
| cssType1A | Type1A-PDCCH common search space for scheduling DCCH/DTCH and configured for sdt-SearchSpace for a DCI format with CRC scrambled by a C-RNTI or a CS-RNTI |
| cssType2 | Type2-PDCCH common search space for scheduling of Paging messages or short messages (P-RNTI); the SS shall scramble the DCI format's CRC with P-RNTI |
| cssType2A | Type2A-PDCCH common search space for scheduling of Paging Early Indication messages the SS shall scramble the DCI format's CRC with PEI-RNTI |
| cssType3 | Type3-PDCCH common search space for other purpose DCIs (INT-RNTI, SFI-RNTI, TPC-PUSCH-RNTI, TPC-PUCCH-RNTI, TPC-SRS-RNTI, C-RNTI, CS-RNTI(s), cellDTRX-RNTI); the SS shall scramble the DCI format's CRC with the RNTI-value as according to the ASP triggering the DCI to be sent |
| ussDL | UE-specific search space (C-RNTI, CS-RNTI(s)); the SS shall scramble the DCI format's CRC with the C-RNTI value as stored for the cell (NR_CellConfigCommon_Type) |
| ussUL | UE-specific search space: as USS_DL but in general with lower priority to give DL assignments precedence over periodic UL grants |
| ussSL | UE-specific sidelink search space (SL-RNTI, SL-CS-RNTI); SS configured with NR_CellConfigInfo_Type.AddOrReconfigure.PhysicalLayer.Downlink.BWPs.BwpArray.SL_Pdcch. The SS shall scramble the DCI format's CRC with the SL-RNTI/SL-CS-RNTI value provided in NR_CellConfigInfo_Type.AddOrReconfigure.SidelinkScheduled.SL_DCI_Config.SearchSpaceAndDci.DciInfo.Format |
| ussDL_DCI_MC | UE-specific search space (C-RNTI) with DCI format 1_3; the SS shall scramble the DCI format's CRC with the C-RNTI value as stored for the cell (NR_CellConfigCommon_Type) |
| ussUL_DCI_MC | UE-specific search space (C-RNTI) with DCI format 0_3: as USS_DL but in general with lower priority to give DL assignments precedence over periodic UL grants |

On the DCCH/DTCH configuration side, we have

NR_DcchDtchConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_DcchDtchConfig_Type |  |  |
| Comment |  |  |  |
| DL | NR_DcchDtchConfigDL_Type | opt | Scheduling, parameters related to DCCH and DTCH in DL |
| UL | NR_DcchDtchConfigUL_Type | opt | Scheduling, parameters related to DCCH and DTCH in UL |
| DrxCtrl | NR_DrxCtrl_Type | opt | DRX configuration as sent to the UE (or 'None' when the UE does not support connected mode DRX) |
| MeasGapCtrl | NR_MeasGapCtrl_Type | opt | to tell the SS when no assignments/grants shall be assigned to the UE |
| DL2 | NR_DcchDtchConfigDL_Type | opt | Present if multiple CORESETPoolIndex are configured. Scheduling, parameters related to DCCH and DTCH in DL. If present, then both DL2 and DL the explicit mode is used in Downlink Scheduling |
| UL2 | NR_DcchDtchConfigUL_Type | opt | Present if multiple CORESETPoolIndex are configured. Scheduling, parameters related to DCCH and DTCH in UL |

NR_DcchDtchConfigDL_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_DcchDtchConfigDL_Type |  |  |
| Comment | configuration for DCCH/DTCH mapped to DL-SCH mapped to PDSCH; NOTE: in contrast to EUTRA the configuration of HARQ processes to be used is done as part of DCI configuration |  |  |
| SearchSpaceAndDci | NR_SearchSpaceDlDciAssignment_Type | opt | in general DCCH/DTCH transmissions happen at the BWP currently being the active BWP at the SS and the UE specific search space is used for scheduling; DCI configuration for Msg2 of the RACH procedure is done as part of the RACH procedure configuration (NR_RandomAccessResponseConfig_Type) |
| CoresetPoolIndex | NR_ASN1_CoresetPoolIndex_Type | opt | Present if multiple CORESETPoolIndex are configured. If the field is absent, the value 0 applies. |

NR_SearchSpaceDlDciAssignment_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SearchSpaceDlDciAssignment_Type |  |  |
| Comment | configuration of DCI for a specific search space; in general the configuration belongs to a transport channel configuration (e.g. BCH, PCH, DL-SCH): the DCI is applied for DL transmission on the respective channel and can be explicitly initiated by TTCN (e.g. PCH, DL-SCH) or automatically by the SS (e.g. BCH or Msg2/Msg4 scheduling of RACH procedure); all fields are mandatory for the first configuration of an instance for modifications "omit" means "keep as it is" |  |  |
| AssignedBWPs | NR_AssignedBWPs_Type | opt | BWP where given DCI shall be scheduled in given search space; NOTE 1: When there is no BWP according to configuration of AssignedBWPs, there is no DL assignment; this can be used e.g. to prevent automatic scheduling of system information; NOTE 2: In principle a DL assignment can happen simultaneously in more than one BWP e.g. if system information shall be scheduled in active and initial BWP; but as long as there is no use case for simultaneous DL assignments, configuration of AssignedBWPs shall be restricted to a maximum of one BWP |
| SearchSpaceType | NR_SearchSpaceType_Type | opt | search space to be used for sending of given DCI; when at the scheduled point in time of a DL transmission there is no such search space configured at the given BWP, the SS may raise an error |
| DciInfo | NR_DciDlInfo_Type | opt | DCI to be used |

# Test case implementation

Here is the list of the Search Spaces configured in SS:

| Entry | CORESET | TypeAndPriorityList | Comment |
| --- | --- | --- | --- |
| 0 | CORESET#0 | Type := cssType0 | SIB1 |
|  |  | Type := cssType2 | Paging |
| 1 | CORESET#0 | Type := cssType1 | Msg2 or Msg4 of RACH procedure |
|  |  | Type := cssType3 | C-RNTI on common searchSpace |
| 2 | CORESET#1 | Type := ussDL | USS DL |
|  |  | Type := ussUL | USS UL |
| 3 | CORESET#0 | Type := cssType0A | SIs |

During the test execution, entry 2 is reconfigured to update the Search Space parameters as according to the one applied to UE. i.e the field SearchSpaceConfigAtUE is reconfigured in NR_BWP_SearchSpaceConfig_Type.

The initial configuration of DCCH/DTCH during the preamble is as follows:

DcchDtchConfig := {

DL := {

SearchSpaceAndDci := {

AssignedBWPs := {

ActiveBWP := true,

…

},

SearchSpaceType := ussDL,

DciInfo := {

ResoureAssignment := {…..},

VrbPrbMapping := {Index := '0'B},

Format := {Format_1_1 :=

{

CarrierIndicator := {None := true},

…

PdcchMonitoringAdaptationIndication := {None := true},

PucchCellIndicator := {None := true}

}}}

}

},

UL := {

SearchSpaceAndDci := {

AssignedBWPs :=

{

ActiveBWP := true,

…

},

SearchSpaceType := ussUL,

DciInfo :=

{

ResoureAssignment := {…..},

PuschHoppingCtrl := {None := true},

TpcCommandPusch := {Value := '01'B},

UL_SUL_Indicator := {None := true},

Format := {Format_0_1 :=

{

CarrierIndicator := {None := true},

DfiFlag := {None := true},

BWPIndicator := {Index := ''B},

PdcchMonitoringAdaptationIndication := {None := true}

….

}}

}

},

….

}

During the test execution, the SearchSpaceType remains fixed on ussDL/ussUL during the whole execution of the test. The DCI is reconfigured to update the Monitoring Adaptation field content to be transmitted to UE.

# MCC160 proposal to help SS implementation

Comments were received from R&S, to help SS implementation the following is proposed :

- In TTCN, configure the UE-Specific SearchSpace list with the searchSpacesToAddModListExt_v1700 set as according to UE. So that the SS knows all configured SearchSpace and SS knows if the SSSG feature is configured.

- Add ussDL2, ussDL3, ussUL2, ussUL3 in the NR_SearchSpaceType_Type. Each UE-Specific SearchSpace will be mapped to a different type: ussDL/UL, ussDL2/UL2, ussDL3/ussUL3. The DCI will be mapped to the type to be used, hence SS can do the mapping to the SearchSpace Id.

NR_SearchSpaceType_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | NR_SearchSpaceType_Type |
| Comment | types of search spaces according to TS 38.213 clause 10.1; NOTE 1: In principle there could be more than one instance for the same kind of search space; if this is ever needed, this enumerated shall be enhanced by introduction of a second entry for a particular type. NOTE 2: For the USS UL and DL are distinguished as they may need different priority in terms of scheduling of DCIs |
| cssType0 | Type0-PDCCH common search space for scheduling of SIB1 (SI-RNTI); the SS shall scramble the DCI format's CRC - with SI-RNTI for a DCI format 1_0 - with MCCH-RNTI or G-RNTI for a DCI format 4_0 for MBS broadcast; aggregation level and number of candidates correspond to TS 38.213 Table 10.1-1; For stand-alone options parameters of search space and associated CORESET are according MIB.pdcch-ConfigSIB1 (TS 38.213 clause 13) |
| cssType0A | Type0A-PDCCH common search space for scheduling of other system information (SI-RNTI); the SS shall scramble the DCI format's CRC with SI-RNTI |
| cssType0B | Type0B-PDCCH common search space for a DCI format 4_0 with CRC scrambled by MCCH-RNTI or G-RNTI |
| cssType1 | Type1-PDCCH common search space for scheduling of Msg2 or Msg4 of RACH procedure; the SS shall scramble the DCI format's CRC with RA-RNTI for Msg2 or with TC-RNTI for Msg4 (temporary C-RNTI) as configured for Msg4 (NR_TempC_RNTI_Type in NR_RAR_Payload_Type) |
| cssType1A | Type1A-PDCCH common search space for scheduling DCCH/DTCH and configured for sdt-SearchSpace for a DCI format with CRC scrambled by a C-RNTI or a CS-RNTI |
| cssType2 | Type2-PDCCH common search space for scheduling of Paging messages or short messages (P-RNTI); the SS shall scramble the DCI format's CRC with P-RNTI |
| cssType2A | Type2A-PDCCH common search space for scheduling of Paging Early Indication messages the SS shall scramble the DCI format's CRC with PEI-RNTI |
| cssType3 | Type3-PDCCH common search space for other purpose DCIs (INT-RNTI, SFI-RNTI, TPC-PUSCH-RNTI, TPC-PUCCH-RNTI, TPC-SRS-RNTI, C-RNTI, CS-RNTI(s), cellDTRX-RNTI); the SS shall scramble the DCI format's CRC with the RNTI-value as according to the ASP triggering the DCI to be sent |
| ussDL | UE-specific search space (C-RNTI, CS-RNTI(s)); the SS shall scramble the DCI format's CRC with the C-RNTI value as stored for the cell (NR_CellConfigCommon_Type) |
| ussUL | UE-specific search space: as USS_DL but in general with lower priority to give DL assignments precedence over periodic UL grants |
| ussSL | UE-specific sidelink search space (SL-RNTI, SL-CS-RNTI); SS configured with NR_CellConfigInfo_Type.AddOrReconfigure.PhysicalLayer.Downlink.BWPs.BwpArray.SL_Pdcch. The SS shall scramble the DCI format's CRC with the SL-RNTI/SL-CS-RNTI value provided in NR_CellConfigInfo_Type.AddOrReconfigure.SidelinkScheduled.SL_DCI_Config.SearchSpaceAndDci.DciInfo.Format |
| ussDL_DCI_MC | UE-specific search space (C-RNTI) with DCI format 1_3; the SS shall scramble the DCI format's CRC with the C-RNTI value as stored for the cell (NR_CellConfigCommon_Type) |
| ussUL_DCI_MC | UE-specific search space (C-RNTI) with DCI format 0_3: as USS_DL but in general with lower priority to give DL assignments precedence over periodic UL grants |
| ussDL2 | UE-specific search space (C-RNTI, CS-RNTI(s)); the SS shall scramble the DCI format's CRC with the C-RNTI value as stored for the cell (NR_CellConfigCommon_Type). To be used when UE is configured with multiple UE-specific search spaces. In that case the DCI is mapped to the one SS is expected to use to transmit the DCI. |
| ussUL2 | UE-specific search space: as USS_DL but in general with lower priority to give DL assignments precedence over periodic UL grants. To be used when UE is configured with multiple UE-specific search spaces. In that case the DCI is mapped to the one SS is expected to use to transmit the DCI. |
| ussDL3 | UE-specific search space (C-RNTI, CS-RNTI(s)); the SS shall scramble the DCI format's CRC with the C-RNTI value as stored for the cell (NR_CellConfigCommon_Type). To be used when UE is configured with multiple UE-specific search spaces. In that case the DCI is mapped to the one SS is expected to use to transmit the DCI. |
| ussUL3 | UE-specific search space: as USS_DL but in general with lower priority to give DL assignments precedence over periodic UL grants. To be used when UE is configured with multiple UE-specific search spaces. In that case the DCI is mapped to the one SS is expected to use to transmit the DCI. |

In the TTCN implementation, SS is reconfigured with the 3 UE-specific Search Spaces. This Search Space is configured with searchSpacesToAddModListExt_v1700 set as done for UE.

Here is the list of Search spaces configured in the DL BWP:

| Entry | CORESET | SearchSpaceId | TypeAndPriorityList | Comment |
| --- | --- | --- | --- | --- |
| 0 | CORESET#0 | 0 | Type := cssType0 | SIB1 |
|  |  |  | Type := cssType2 | Paging |
| 1 | CORESET#0 | 1 | Type := cssType1 | Msg2 or Msg4 of RACH procedure |
|  |  |  | Type := cssType3 | C-RNTI on common searchSpace |
| 2 | CORESET#1 | 2 | Type := ussDL | USS DL |
|  |  |  | Type := ussUL | USS UL |
| 3 | CORESET#0 | 3 | Type := cssType0A | SIs |
| 4 | CORESET#1 | 4 | Type := ussDL2 | USS2 DL |
|  |  |  | Type := ussUL2 | USS2 UL |
| 5 | CORESET#1 | 5 | Type := ussDL3 | USS3 DL |
|  |  |  | Type := ussUL3 | USS3 UL |
| Note: X is the SearchSpace currently used |  |  |  |  |

Before a SSSG switching:

- SS DL BWP configuration remains unchanged

- DCCH/DTCH shall be reconfigured to update the SearchSpaceType (i.e. ussDL, ussDL2, ussDL3, ussUL, ussUL2, ussUL3) that needs to be used to transmit the DL/UL DCI

This ASP change has impact only on TC 7.1.1.12.5.