3GPP TSG-RAN5 Meeting #110	R5w260012

# Gothenburg, Sweden, Feb.09th – Feb.13st 2026

Agenda Item:	TTCN sidebar

Source:	MCC TF160

Title:	Rel-18 NR POS enh. Phase2: Correction to ASPs

Document for:	Endorsement

This document proposes some correction to the ASPs endorsed in R5w260005r1 at the last TTCN workshop #72.

- The field EmbededSCI_Payload in NR_SL_SciFormat2D_Type is currently defined as bitstring, but it makes more sense to define it as union of two other existing types. The typo in the field name is also corrected.

type record NR_SL_SciFormat2D_Type {   /* Acc to TS 38.212 cl 8.4.1.4 SCI format 2-D

For all fields: 'omit' means that the information shall not be 
                                            contained in the SCI */

NR_SL_PRS_ResourceID_Type    SL_PRS_ResourceID optional,

NR_SL_PRS_RequestSci2D_Type  SL_PRS_Request optional,

B2_Type                      EmbeddedSCI_Format optional,  /* According to TS 38.212, Table 8.4.1.4-1 */

NR_SL_PRS_Sci2D_EmbeddedSCI_Payload_Type   EmbeddedSCI_Payload optional  /* number of bits determined 
                                                                                       according to TS 38.212, 
                                                                                       Table 8.4.1.4-1 */

};

type union NR_SL_PRS_Sci2D_EmbeddedSCI_Payload_Type {

NR_SL_SciFormat2A_Type    SciFormat2A_Payload,  // Acc to TS 38.212 cl 8.4.1.1 SCI format 2-A

NR_SL_SciFormat2B_Type    SciFormat2B_Payload   // Acc to TS 38.212 cl 8.4.1.2 SCI format 2-B

};

- The field SL_PRS_Resources in NR_SL_PRS_DedicatedResourcePool_Type is defined as:

SL_PRS_ResourcePool_r18.sl_PRS_ResourcesDedicatedSL_PRS_RP_r18   NR_SL_PRS_ResourcesDedicated_Type

which is a list, as defined in TS 38.331:

As we plan to configure only one SL-PRS resource in the NR-SS-UE, it is proposed to remove the list and make the field SL_PRS_Resources directly of type SL_PRS_ResourceDedicatedSL_PRS_RP_r18 and to change the name of the field to SL_PRS_Resource.

type SL_PRS_ResourcePool_r18.sl_PRS_PSCCH_Config_r18    NR_SL_PRS_PSCCH_Config_Type;

type SL_PRS_ResourcePool_r18.sl_StartRB_SubchannelDedicatedSL_PRS_RP_r18  NR_SL_PRS_StartRB_Subchannel_Type;

type SL_PRS_ResourcePool_r18.sl_RB_Number_r18  NR_SL_PRS_RB_Number_Type;

type SL_PRS_ResourcePool_r18.sl_TimeResource_r18   NR_SL_TimeResource_r18_Type;

type SL_PRS_ResourcePool_r18.sl_PRS_ResourceReservePeriodList_r18   NR_SL_PRS_ResourceReservePeriodList_Type;

type SL_PRS_ResourcePool_r18.sl_NumSubchannelDedicatedSL_PRS_RP_r18  NR_SL_PRS_NumSubchannel_Type;

type SL_PRS_ResourcePool_r18.sl_SubchannelSizeDedicatedSL_PRS_RP_r18  NR_SL_PRS_SubchannelSize_Type;

type SL_PRS_ResourcePool_r18.sl_NumReservedBitsSCI1B_DedicatedSL_PRS_RP_r18  NR_SL_PRS_NumBitsSCI1B_Type;

type record NR_SL_PRS_DedicatedResourcePool_Type {

NR_SL_PRS_PSCCH_Config_Type               SL_PRS_PSCCH_Config  optional,

NR_SL_PRS_StartRB_Subchannel_Type         SL_PRS_StartRB_Subchannel  optional,

NR_SL_PRS_RB_Number_Type                  SL_RB_Number  optional,

NR_SL_TimeResource_r18_Type               SL_TimeResource  optional,

NR_SL_PRS_ResourceReservePeriodList_Type  SL_PRS_ResourceReservePeriodList  optional,

SL_PRS_ResourceDedicatedSL_PRS_RP_r18         SL_PRS_Resource  optional,

NR_SL_PRS_NumSubchannel_Type              SL_PRS_NumSubchannel  optional,

NR_SL_PRS_SubchannelSize_Type             SL_PRS_SubchannelSize optional,

NR_SL_PRS_NumBitsSCI1B_Type               SL_PRS_NumBitsSCI1B  optional

};