3GPP TSG-RAN5 Meeting #108	R5-253751r1

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508 | CR | 1538 | rev | - | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to default SIB24 message for NR channel BW less than 5MHz for FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc., Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR1_lessthan_5MHz_BW-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To apply for NR channel BW less than 5MHz for FR1, SIB24 message content needs to be updated based on TS 36.331. |  |  |  |  |  |  |  |  |
| Summary of change: |  | SIB24 was updated for the impacted Idle mode TC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work Plan will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | r1: specified the values of FreqBandIndicatorNR-r15 and carrierFreq-r15 for condition LessThan5MHz. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

-	SystemInformationBlockType24

The IE SystemInformationBlockType24 contains information relevant only for inter-RAT cell re-selection i.e. information about NR frequencies and NR neighbouring cells relevant for cell re-selection. The IE includes cell re-selection parameters common for a frequency.

Table 4.4.3.3-20: SystemInformationBlockType24

| Derivation Path: TS 36.331 [17], clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType24-r15 ::= SEQUENCE { |  |  |  |
| carrierFreqListNR-r15 SEQUENCE (SIZE (1..maxFreq)) OF CarrierFreqNR-r15 { | The same number of entries as the configured NR carriers For Signalling test cases, see Table 6.3.1.8-1. | n denotes the index of the entry |  |
| carrierFreq-r15[n] | Downlink NR ARFCN of SSB under test | For signalling test cases see TS 38.508-1 [67] subclause 6.2.3. Otherwise, see TS 38.508-1 [67] subclause 4.3.1. |  |
|  | 250 |  | LessThan5MHz |
| multiBandInfoList-r15 [n] SEQUENCE (SIZE (1..maxMultiBandsNR-r15)) OF FreqBandIndicatorNR-r15 { | 1 entry |  |  |
| FreqBandIndicatorNR-r15[1] | NR Operating band under test | entry 1 |  |
|  | 200 | entry 1 | LessThan5MHz |
| } |  |  |  |
| multiBandInfoListSUL-r15[n] SEQUENCE (SIZE (1..maxMultiBandsNR-r15)) OF FreqBandIndicatorNR-r15 {} | Not present |  |  |
| measTimingConfig-r15[n] SEQUENCE { |  |  |  |
| periodicityAndOffset-r15 CHOICE { |  |  |  |
| sf20 | 0 |  | FR1 |
| sf160 | 0 |  | FR2 |
| } |  |  |  |
| ssb-Duration-r15 | sf2 |  | FR1 |
|  | sf3 |  | FR2 |
| } |  |  |  |
| subcarrierSpacingSSB-r15[n] | kHz15 |  | SCS_15kHz |
|  | kHz30 |  | SCS_30kHz |
|  | kHz120 |  | SCS_120kHz |
| ss-RSSI-Measurement-r15[n] | Not present |  |  |
| cellReselectionPriority-r15[n] | 5 |  |  |
| cellReselectionSubPriority-r15[n] | Not present |  |  |
| threshX-High-r15[n] | 2 (4 dB) |  |  |
| threshX-Low-r15[n] | 1 (2 dB) |  |  |
| threshX-Q-r15[n] | Not present |  |  |
| threshX-Q-r15[n] SEQUENCE { |  |  | QBASED |
| threshX-HighQ-r15 | 5 (5 dB) |  |  |
| threshX-LowQ-r15 | 5 (5 dB) |  |  |
| } |  |  |  |
| q-RxLevMin-r15[n] | -70 (-140 dBm) | For RF/RRM test cases |  |
|  | -55 (-110 dBm) | For signalling test cases |  |
| q-RxLevMinSUL-r15[n] | -70 (-140 dBm) | For RF/RRM test cases | SUL |
|  | -55(-110 dBm) | For signalling test cases |  |
| p-MaxNR-r15[n] | 23 |  | FR1 |
|  | 26 |  | FR2 FR1_RF_PC2 |
| ns-PmaxListNR-r15[n] | Not present |  |  |
| q-QualMin-r15[n] | Not present |  |  |
|  | -20 (-20dB) |  | QBASED |
| deriveSSB-IndexFromCell-r15[n] | False |  |  |
| maxRS-IndexCellQual-r15[n] | Not present |  |  |
| threshRS-Index-r15[n] | Not present |  |  |
| } |  |  |  |
| t-ReselectionNR-r15 | 0 |  |  |
| t-ReselectionNR-SF-r15 | Not present | Not required unless speed-dependent cell re-selection is tested. |  |
| lateNonCriticalExtension | Not present |  |  |
| carrierFreqListNR-v1810 SEQUENCE (SIZE (1..maxFreq)) OF CarrierFreqNR-v1810 { | The same number of entries as the configured NR carriers For Signalling test cases, see Table 6.3.1.8-1. | n denotes the index of the entry | LessThan5MHz |
| CarrierFreqNR-v1810[n] SEQUENCE { |  | entry n |  |
| carrierFreq-r18 | Downlink NR ARFCN of SSB under test | For signalling test cases see TS 38.508-1 [67] subclause 6.2.3. Otherwise, see TS 38.508-1 [67] subclause 4.3.1. |  |
| multiBandInfoList-r18 SEQUENCE (SIZE (1.. maxMultiBandsNR-r15)) OF FreqBandIndicatorNR-r15 { | 1 entry |  |  |
| FreqBandIndicatorNR-r15[1] | NR Operating band under test | entry 1 |  |
| } |  |  |  |
| multiBandInfoListAerial-r18 | Not present |  |  |
| ns-PmaxListNR-Aerial-r18 | Not present |  |  |
| multiBandNsPmaxListNR-Aerial-r18 | Not present |  |  |
| mobileIAB-CellList-r18 | Not present |  |  |
| mobileIAB-Freq-r18 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SCS_15kHz | SCS=15kHz for frequency of the cell according to TS 38.508-1 [67] clause 6.2.3 for signalling test cases and clause 4.3.1 otherwise |
| SCS_30kHz | SCS=30kHz for frequency of the cell according to TS 38.508-1 [67] clause 6.2.3 for signalling test cases and clause 4.3.1 otherwise |
| SCS_120kHz | SCS=120kHz for frequency of the cell according to TS 38.508-1 [67] clause 6.2.3 for signalling test cases and clause 4.3.1 otherwise |
| QBASED | This condition applies to Quality based cell (re)selection signalling test cases. |
| FR1_RF_PC2 | FR1 RF testing with Power Class 2 |
| LessThan5MHz | For test cases using a NR neighbouring celll with channel BW less than 5 MHz for FR1 |

[End of change]