3GPP TSG-RAN WG5 Meeting #108	R5-253848r1

Bengaluru, India, 25th – 29th Aug, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-5 | CR | 0237 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of sidelink positioning related information and assistance data |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Support of sidelink positioning in NR systems is one of the objectibve of the work item Expanded and improved NR positioning. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Corrected the specification scope NR positioning methods description. - Added TS 38.355 as reference file and added SLPP related abbreviations - Added Anchor UE Location Information for SL-TDOA, SL-RTT, SL-AoA and SL-TOA |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The release 18 positioning work item will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1, 2, 3, 13(new), 14(new), 15(new), 16(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Corrected the city name on the cover sheet. |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

1	Scope

The present document specifies the test scenarios and assistance data required for the conformance and minimum performance tests for FDD or TDD mode of UTRA, E-UTRA and NR for the User Equipment (UE) that supports one or more of the defined positioning methods. For UTRA these are Assisted Global Positioning System (A-GPS) and Assisted Global Navigation Satellite System (A-GNSS). For E-UTRA these are A-GNSS, Observed Time Difference of Arrival (OTDOA), Enhanced Cell ID (ECID), Wireless Local Area Network (WLAN), Metropolitan Beacon Systems (MBS) and Bluetooth. For NR these are A-GNSS, Observed Time Difference of Arrival (LTE) (OTDOA (LTE)), Enhanced Cell ID (LTE) (ECID (LTE)), Wireless Local Area Network (WLAN), Metropolitan Beacon Systems (MBS), Bluetooth, NR Enhanced Cell-ID (NR-ECID), Multiple-Round Trip Time (Multi-RTT), Downlink Angle-of-Departure (DL-AoD), Downlink Time Difference Of Arrival (DL-TDOA), Sidelink Round Trip Time Positioning (SL-RTT), Sidelink Angle-of-Arrival (SL-AoA), Sidelink Time Difference of Arrival (SL-TDOA) and Sidelink Time of Arrival (SL-TOA).

The present document also specifies the GNSS scenario files for the test scenarios defined in TS 36.508 for V2X and aerial testing.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

	For a specific reference, subsequent revisions do not apply.

	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document unless the context in which the reference is made suggests a different Release is relevant (information on the applicable release in a particular context can be found in e.g. test case title, description or applicability, message description or content).

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception".

[3]	Void

[4]	Void

[5]	Void

[6]	3GPP TS 37.571-1: “User Equipment (UE) conformance specification for UE positioning; Part 1: Terminal conformance”.

[7]	3GPP TS 37.571-2: “User Equipment (UE) conformance specification for UE positioning; Part 2: Protocol conformance”.

[8]	3GPP TS 37.355: " LTE Positioning Protocol (LPP)".

[9]	IS-GPS-200, Revision D, Navstar GPS Space Segment/Navigation User Interfaces, March 7th, 2006.

[10]	IS-GPS-705, Navstar GPS Space Segment/User Segment L5 Interfaces, September 22, 2005.

[11]	IS-GPS-800, Navstar GPS Space Segment/User Segment L1C Interfaces, September 4, 2008.

[12]	IS-QZSS, Quasi Zenith Satellite System Navigation Service Interface Specifications for QZSS, Ver.1.1, July 31, 2009.

[13]	Galileo OS Signal in Space ICD (OS SIS ICD), Issue 1.2, February 2014, European Union.

[14]	Global Navigation Satellite System GLONASS Interface Control Document, Version 5.1, 2008.

[15]	Specification for the Wide Area Augmentation System (WAAS), US Department of Transportation, Federal Aviation Administration, DTFA01-96-C-00025, 2001.

[16]	3GPP TS 25.331: “Radio Resource Control (RRC); Protocol specification”

[17]	STANAG 4294: NATO STANAG 4294. Navstar Global Positioning System (GPS) System Characteristics.

[18]	3GPP TS 36.104: "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception".

[19]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[20]	3GPP TS 36.508: "Common test environments for User Equipment (UE) conformance testing".

[21]	ATIS-0500027: "Recommendations for Establishing Wide Scale Indoor Location Performance", May 2015.

[22]	IEEE 802.11, Part 11: "Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications".

[23]	Bluetooth Special Interest Group (SIG): “Bluetooth Core Specification v4.2”, December 2014.

[24]	3GPP TS 38.508-1: "User Equipment (UE) conformance specification; Part 1: Common test environment".

[25]	BDS-SIS-ICD-B1I-3.0: "BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B1I (Version 3.0)", China Satellite Navigation Office, December 2019.

[26]	BDS-SIS-ICD-B1C-1.0: "BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B1C (Version 1.0)", December 2017.

[27]	BDS-SIS-ICD-B2a-1.0: "BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B2a (Version 1.0)", December, 2017.

[28]	BDS-SIS-ICD-B3I-1.0: "BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B3I (Version 1.0)", February, 2018.

[29]	3GPP TS 38.355: "NR; Sidelink Positioning Protocol (SLPP); Protocol specification".

3	Definitions, symbols and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1], TS 36.101 [2], TS 36.104 [18] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Horizontal Dilution Of Precision (HDOP): measure of position determination accuracy that is a function of the geometrical layout of the satellites used for the fix, relative to the receiver antenna

3.2	Symbols

For the purposes of the present document, the following symbols apply:

B1I	BeiDou B1I navigation signal with carrier frequency of 1561.098 MHz.

B1C	BeiDou B1C navigation signal with carrier frequency of 1575.420 MHz.

B2a	BeiDou B2a navigation signal with carrier frequency of 1176.450 MHz.

B3I	BeiDou B3I navigation signal with carrier frequency of 1268.520 MHz.

E1	Galileo E1 navigation signal with carrier frequency of 1575.420 MHz.

E5	Galileo E5 navigation signal with carrier frequency of 1191.795 MHz.

E6	Galileo E6 navigation signal with carrier frequency of 1278.750 MHz.

G1	GLONASS navigation signal in the L1 sub-bands with carrier frequencies 1602 MHz ± k  562.5 kHz.

G2	GLONASS navigation signal in the L2 sub-bands with carrier frequencies 1246 MHz ± k  437.5 kHz.

k		GLONASS channel number, k = -7…13.

L1 C/A	GPS or QZSS L1 navigation signal carrying the Coarse/Acquisition code with carrier frequency of 1575.420 MHz.

L1C	GPS or QZSS L1 Civil navigation signal with carrier frequency of 1575.420 MHz.

L2C	GPS or QZSS L2 Civil navigation signal with carrier frequency of 1227.600 MHz.

L5	GPS or QZSS L5 navigation signal with carrier frequency of 1176.450 MHz.

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

A-Galileo	Assisted-Galileo

A-GANSS	Assisted- Galileo and Additional Navigation Satellite Systems

A-GLONASS	Assisted- GLObal'naya NAvigatsionnaya Sputnikovaya Sistema (English: Global Navigation Satellite System)

A-GNSS	Assisted Global Navigation Satellite System

A-GPS	Assisted - Global Positioning System

AP	Access Point

AWGN	Additive White Gaussian Noise

BDS	BeiDou Navigation Satellite System

C/A	Coarse/Acquisition

DL-AoD	Downlink Angle-of-Departure

DL-PRS	Downlink Positioning Reference Signal

DL-TDOA	Downlink Time Difference Of Arrival

DUT	Device Under Test

ECEF	Earth Centred, Earth Fixed

ENB	Evolved Node B

EN-DC	E-UTRA-NR Dual Connectivity

E-UTRA 	Evolved UMTS Terrestrial Radio Access

E-UTRAN	Evolved UMTS Terrestrial Radio Access Network

FDD	Frequency Division Duplex

FFS	For further study

GANSS	Galileo and Additional Navigation Satellite Systems

GEO	Geostationary Earth Orbit

GLONASS	GLObal'naya NAvigatsionnaya Sputnikovaya Sistema (English: Global Navigation Satellite System)

GNSS	Global Navigation Satellite System

GPS	Global Positioning System

GSS	GNSS System Simulator

HDOP	Horizontal Dilution Of Precision

ICD	Interface Control Document

ICS	Implementation Conformance Statement

IS	Interface Specification

LOS	Line Of Sight

LPP	LTE Positioning Protocol

MBS	Metropolitan Beacon System

Multi-RTT	Multi-Round Trip Time

NE-DC	NR-E-UTRA Dual Connectivity

NGEN-DC	NG-RAN E-UTRA-NR Dual Connectivity

NR E-CID	NR Enhanced Cell ID (positioning method)

NG-RAN	NextGen Radio Access Network

NR	New Radio

NR-DC	NR-NR Dual Connectivity

PPM	Parts per million

PRS	Positioning Reference Signal

QZSS	Quasi-Zenith Satellite System

RRC	Radio Resource Control

SBAS	Space Based Augmentation System

SCC	Secondary Component Carrier

SFN	System Frame Number

SL-AoA	Sidelink Angle-of-Arrival

SL-PRS	Sidelink Positioning Reference Signal

SL-PRS-RSRP	Sidelink PRS Reference Signal Received Power

SL-PRS-RSRPP	Sidelink PRS Reference Signal Received Path Power

SL-RSTD	Sidelink Reference Signal Time Difference

SL-RTOA	Sidelink Relative Time of Arrival

SL-RTT	Sidelink Round Trip Time

SL-TDOA	Sidelink Time Difference Of Arrival

SL-TOA	Sidelink Time Of Arrival

SLPP	Sidelink Positioning Protocol

SS	System simulator

SV	Space Vehicle

SV ID	Space Vehicle Identification

TDD	Time Division Duplex

TOD	Time Of Day

TOW	Time of Week TTFF	Time To First Fix

UE	User Equipment

UTRA	Universal Terrestrial Radio Access

UTRAN	Universal Terrestrial Radio Access Network

WAAS	Wide Area Augmentation System

WGS-84	World Geodetic System 1984

WLAN	Wireless Local Area Network

< Unchanged sections omitted >

12	DL-AoD information

12.1	ReferencePoint and RelativeLocation

The reference point and relative location information elements which shall be provided to the UE in the UE-based DL-AoD measurement tests defined in TS 37.571-1 [6] and signalling tests defined in TS 37.571-2 [7] are the same as those defined in clause 11.2.

13	SL-TDOA information

13.1	Anchor UE Location Information

This subclause defines the anchor UE location information information elements which shall be provided to the UE in the SL RSTD measurement tests defined in TS 37.571-1 [6] and the SL-TDOA signalling tests defined in TS 37.571-2 [7].

Table 13.1-1: AnchorLocationCoordinates for NR-SS-UE1

| Derivation Path: 38.355 clause 6.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AnchorLocationCoordinates :: CHOICE { |  |  |  |
| ellipsoidPointWithAltitudeAndUncertaintyEllipsoid SEQUENCE { |  |  |  |
| latitudeSign | north |  |  |
| degreesLatitude | 3331609 | 35.744287 |  |
| degreesLongitude | 6509568 | 139.680176 |  |
| altitudeDirection | height |  |  |
| altitude | 300 | 300 m |  |
| uncertaintySemiMajor | 1 |  |  |
| uncertaintySemiMinor | 1 |  |  |
| orientationMajorAxis | 0 |  |  |
| uncertaintyAltitude | 1 |  |  |
| confidence | 68 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 13.1-2: AnchorLocationCoordinates for NR-SS-UE2

| Derivation Path: 38.355 clause 6.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AnchorLocationCoordinates :: CHOICE { |  |  |  |
| ellipsoidPointWithAltitudeAndUncertaintyEllipsoid SEQUENCE { |  |  |  |
| latitudeSign | north |  |  |
| degreesLatitude | 3331803 | 35.746370 |  |
| degreesLongitude | 6509665 | 139.682259 |  |
| altitudeDirection | height |  |  |
| altitude | 300 | 300 m |  |
| uncertaintySemiMajor | 1 |  |  |
| uncertaintySemiMinor | 1 |  |  |
| orientationMajorAxis | 0 |  |  |
| uncertaintyAltitude | 1 |  |  |
| confidence | 68 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 13.1-3: AnchorLocationCoordinates for NR-SS-UE3

| Derivation Path: 38.355 clause 6.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AnchorLocationCoordinates :: CHOICE { |  |  |  |
| ellipsoidPointWithAltitudeAndUncertaintyEllipsoid SEQUENCE { |  |  |  |
| latitudeSign | north |  |  |
| degreesLatitude | 3331803 | 35.746370 |  |
| degreesLongitude | 6509665 | 139.682259 |  |
| altitudeDirection | height |  |  |
| altitude | 300 | 300 m |  |
| uncertaintySemiMajor | 1 |  |  |
| uncertaintySemiMinor | 1 |  |  |
| orientationMajorAxis | 0 |  |  |
| uncertaintyAltitude | 1 |  |  |
| confidence | 68 |  |  |
| } |  |  |  |
| } |  |  |  |

13.2	Relative Location

This subclause defines the relative location information elements which shall be provided to the UE in the SL RSTD measurement tests defined in TS 37.571-1 [6] and the SL-TDOA signalling tests defined in TS 37.571-2 [7].

Table 13.2-1: RelativeLocation for ARP

| Derivation Path: 38.355 clause 6.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RelativeLocation ::= SEQUENCE { |  |  |  |
| milliArcSecondUnits | mas30 |  |  |
| heightUnits | m |  |  |
| deltaLatitude SEQUENCE { |  |  |  |
| deltaLatitude | 0 |  |  |
| coarseDeltaLatitude | 0 |  |  |
| } |  |  |  |
| deltaLongitude SEQUENCE { |  |  |  |
| deltaLongitude | 0 |  |  |
| coarseDeltaLongitude | 0 |  |  |
| } |  |  |  |
| deltaHeight SEQUENCE { |  |  |  |
| deltaHeight | 0 |  |  |
| coarseDeltaHeight | 0 |  |  |
| } |  |  |  |
| locationUNC SEQUENCE { |  |  |  |
| horizontalUncertainty | 1 |  |  |
| horizontalConfidence | 68 |  |  |
| verticalUncertainty | 1 |  |  |
| verticalConfidence | 68 |  |  |
| } |  |  |  |
| } |  |  |  |

14	SL-RTT information

14.1	Anchor UE Location Information and Relative Location

The anchor UE location information and ARP relative location information elements which shall be provided to the UE in the SL Rx-Tx time difference measurement tests defined in TS 37.571-1 [6] and SL-RTT signalling tests defined in TS 37.571-2 [7] are the same as those defined in clause 13.

15	SL-AoA information

15.1	Anchor UE Location Information and Relative Location

The anchor UE location information and ARP relative location information elements which shall be provided to the UE in the SL-PRS-RSRP measurement tests defined in TS 37.571-1 [6] and SL-AoA signalling tests defined in TS 37.571-2 [7] are the same as those defined in clause 13.

16	SL-TOA information

16.1	Anchor UE Location Information and Relative Location

The anchor UE location information and ARP relative location information elements which shall be provided to the UE in the SL RTOA measurement tests defined in TS 37.571-1 [6] and SL-TOA signalling tests defined in TS 37.571-2 [7] are the same as those defined in clause 13.