3GPP TSG-RAN WG5 Meeting #108	R5-253905r1

# Bengaluru, IN, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3410 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to MPR UL MIMO TC for n28 PC2 40MHz |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_n28_PC2_40MHz_CBW-UEConTest |  |  |  |  |  | Date: |  |  | 2025-07-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | MPR UL MIMO TC for n28 PC2 40MHz needs to be updated as per WP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | MPR UL MIMO TC for n28 PC2 40MHz has been updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The WI can not be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2D.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1：Add Huawei, HiSilicon as co-source company to solve overlapping issue. |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

6.2D.2	UE maximum output power reduction for UL MIMO

6.2D.2.1	Test purpose

To verify that the power reduction of UE due to higher order modulations and transmit bandwidth configuration does not exceed the specified maximum power reduction.

6.2D.2.2	Test applicability

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 15 and forward that support 2-layer codebook based UL MIMO.

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 16 and forward that support UL full power transmission (ULFPTx).

NOTE:	Test execution is not necessary if TS 38.521-1 6.5D.2.4.1 is executed.

6.2D.2.3	Minimum conformance requirements

For UE with two or four transmit antenna connectors in closed-loop spatial multiplexing scheme, the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2D.1.3-1 is specified in Table 6.2.2.3-1 for PC3, Table 6.2D.2.3-1 for 2Tx PC2 when the UE does not indicate ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16 for the band and Table 6.2.2.3-2 for 2Tx PC2 when the UE indicates ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16 for the band, Table 6.2D.2.3-2 and Table 6.2D.2.3-3 for PC1.5 with 2Tx, Table 6.2D.2.3-4, 6.2D.2.3-5 for PC1.5 with 4Tx respectively. For UE power class 1.5 with 2Tx, the allowed maximum power reduction (MPR) defined in Table 6.2D.2.3-3 is in accordance with the indicated modifiedMPR-Behaviour specified in Table L.1-1 for channel bandwidths ≤ 100 MHz. The requirements shall be met with UL MIMO configurations defined in Table 6.2D.1.3-2. For UE supporting UL MIMO, the maximum output power is defined as the sum of the maximum output power from both antenna connectors.

For UE support uplink full power transmission (ULFPTx) for UL MIMO except the feature ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16, the allowed MPR for the maximum output power in Table 6.2D.1.3-1 is specified in Table 6.2.2.3-1 for PC3, Table 6.2D.2.3-1 when TxD is indicated and Table 6.2.2.3-2 when TxD is not indicated for PC2 , Table 6.2D.2.3-2 and Table 6.2D.2.3-3 for PC1.5 with 2Tx, Table 6.2D.2-4, 6.2D.2-5 for PC1.5 with 4Tx respectively, and the requirements shall be met with the PUSCH configurations specified in Table 6.2D.1.3-3, based upon UE’s support of uplink full power transmission mode. A UE with 2Tx indicating the feature ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16 for a band shall meet the maximum output power requirement with MPR according to clause 6.2.2.3. When a UE that indicates PC1.5 for a given band is limited to PC2 by the rules in clause 6.2.1.3, the MPR requirements in Table 6.2.2.3-2 apply. For UE support uplink full power transmission (ULFPTx) for UL MIMO, the maximum output power is defined as the sum of the maximum output power from all UE antenna connectors.

The same MPR requirements shall be applicable to UE with 1-layer UL MIMO transmission (either with or without ULPFTx) as with the UL MIMO configurations of using 2-layer UL MIMO transmission with codebook of .

For the UE maximum output power modified by MPR, the power limits specified in subclause 6.2D.4.3 apply.

If UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission, the corresponding requirements in clause 6.2D.1.3 apply for the power class as indicated by the ue-PowerClass field in capability signalling. A UE indicating the feature ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16 for a band shall meet the requirement in clause 6.2 with MPR according to clause 6.2.2.3 for at least one antenna connector when scheduled for single antenna-port transmission by DCI format 0_0 or by DCI format 0_1 for codebook-based transmission on a single antenna port with precoding matrix W=1 [6.3.1.5 TS 38.211].

Unless otherwise specified, ‘pi/2 BPSK’ refers to both variants of pi/2 BPSK referenced in Table 6.2.2.3-1.

Table 6.2D.2.3-1: Maximum power reduction (MPR) for power class 2 with 2Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.5 | ≤ 1 | 0 |
|  | QPSK | ≤ 3.5 | ≤ 2 | 0.5 |
|  | 16 QAM | ≤ 3.5 | ≤ 2.5 | ≤ 1.5 |
|  | 64 QAM | ≤ 3.5 | ≤ 3 |  |
|  | 256 QAM | ≤ 5.5 |  |  |
| CP-OFDM | QPSK | ≤ 4.0 | ≤ 3.5 | ≤ 2 |
|  | 16 QAM | ≤ 4.0 | ≤ 3.5 | ≤ 2.5 |
|  | 64 QAM | ≤ 4.5 |  |  |
|  | 256 QAM | ≤ 8.0 |  |  |

Table 6.2D.2.3-2: Maximum power reduction (MPR) for power class 1.5 with 2Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 6 | ≤ [2] | ≤ 0.5 |
|  | QPSK | ≤ 6.5 | ≤ [2.5] | ≤ 0.5 |
|  | 16 QAM | ≤ 6.5 | ≤ [3.5] | ≤ 1.5 |
|  | 64 QAM | ≤ 6.5 | ≤ [4] | ≤ 3.5 |
|  | 256 QAM | ≤ 6.5 | ≤ 6.5 | ≤ [6.5] |
| CP-OFDM | QPSK | ≤ 6.5 | ≤ [4.5] | ≤ 2 |
|  | 16 QAM | ≤ 6.5 | ≤ [4.5] | ≤ 2.5 |
|  | 64 QAM | ≤ 6.5 | ≤ [5] | ≤ 4.5 |
|  | 256 QAM | ≤ 8.5 | ≤ 8.5 | ≤ [8.5] |

Table 6.2D.2.3-3: Maximum power reduction (MPR) for power class 1.5 FWA with 2Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 6 | ≤ 1.5 | ≤ 0 |
|  | QPSK | ≤ 6.5 | ≤ 2 | ≤ 0 |
|  | 16 QAM | ≤ 6.5 | ≤ 3 | ≤ 1 |
|  | 64 QAM | ≤ 6.5 | ≤ 3.5 | ≤ 3 |
|  | 256 QAM | ≤ 6.5 | ≤ 5.5 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 6.5 | ≤ 4 | ≤ 1.5 |
|  | 16 QAM | ≤ 6.5 | ≤ 4 | ≤ 2 |
|  | 64 QAM | ≤ 6.5 | ≤ 4.5 | ≤ 4 |
|  | 256 QAM | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 |
| NOTE 1: This table is targeted to large FWA form factor with 20 dB or above antenna isolation. |  |  |  |  |

Table 6.2D.2.3-4: Maximum power reduction (MPR) for power class 1.5 with 4 Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 8.0 | ≤ 3.0 | ≤ 2.0 |
|  | QPSK | ≤ 8.5 | ≤ 3.5 | ≤ 2.0 |
|  | 16 QAM | ≤ 8.5 | ≤ 4.0 | ≤ 2.5 |
|  | 64 QAM | ≤ 8.5 | ≤ 4.7 | ≤ 4.5 |
|  | 256 QAM | ≤ 9.5 | ≤ 7.0 | ≤ 7.0 |
| CP-OFDM | QPSK | ≤ 9.5 | ≤ 5.0 | ≤ 3.5 |
|  | 16 QAM | ≤ 9.5 | ≤ 5.0 | ≤ 4.0 |
|  | 64 QAM | ≤ 9.5 | ≤ 7.0 | ≤ 7.0 |
|  | 256 QAM | ≤ 9.5 | ≤ 9.5 | ≤ 9.5 |
| NOTE 1: This table is targeted to vehicular UE or other industrial device form factor with 10dB antenna isolation. |  |  |  |  |

Table 6.2D.2.3-5: Maximum power reduction (MPR) for power class 1.5 with 4 Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 7.5 | ≤ 2.0 | ≤ 0.5 |
|  | QPSK | ≤ 8.0 | ≤ 2.5 | ≤ 0.5 |
|  | 16 QAM | ≤ 8.0 | ≤ 3.5 | ≤ 1.5 |
|  | 64 QAM | ≤ 8.0 | ≤ 4.0 | ≤ 3.5 |
|  | 256 QAM | ≤ 8.0 | ≤ 6.5 | ≤ 6.5 |
| CP-OFDM | QPSK | ≤ 8.0 | ≤ 4.5 | ≤ 2.0 |
|  | 16 QAM | ≤ 8.0 | ≤ 4.5 | ≤ 2.5 |
|  | 64 QAM | ≤ 8.0 | ≤ 5.0 | ≤ 5.0 |
|  | 256 QAM | ≤ 8.5 | ≤ 8.5 | ≤ 8.5 |
| NOTE 1: This table is targeted to large FWA form factor with 20 dB or above antenna isolation. |  |  |  |  |

Inner, outer and edge allocations are as defined in section 6.2.2 except for PC1.5 edge allocations which is for LCRB ≤ 4 RBs instead of LCRB ≤ 2 RBs for other power classes. The normative reference for this requirement is TS 38.101-1 [2] clause 6.2D.2.

6.2D.2.4	Test description

6.2D.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.2D.2.4.1-1, Table 6.2D.2.4.1-1a, Table 6.2D.2.4.1-2, Table 6.2D.2.4.1-2a, Table 6.2D.2.4.1-3 and Table 6.2D.2.4.1-3a. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2D.2.4.1-1: Test Configuration Table for Power Class 3 UEs supporting 2-layer codebook based UL MIMO

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest and Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A | Modulation | RB allocation (NOTE 1) |
| 1 | Default |  | CP-OFDM QPSK | Inner Full |
| 2 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 3 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 4 | Default |  | CP-OFDM QPSK | Outer Full |
| 5 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 6 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 7 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 8 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 9 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 10 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 11 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 12 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 13 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 14 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: CP-OFDM 256 QAM test applies only for UEs which supports 256QAM in FR1. |  |  |  |  |

Table 6.2D.2.4.1-1a: Test Configuration Table for power class 3 UEs supporting ULFPTx

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power Reduction (MPR) test case | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 14 | Default |  | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 24 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 34 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 44 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 5 | Default |  | DFT-s-OFDM QPSK | Inner Full |
| 6 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 7 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 8 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Inner Full |
| 10 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 13 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 14 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 15 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 16 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 17 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 18 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 19 | Default |  | CP-OFDM QPSK | Inner Full |
| 20 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 21 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 22 | Default |  | CP-OFDM QPSK | Outer Full |
| 23 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 24 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 25 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 26 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 27 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 28 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 29 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 30 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 31 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 32 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: Test ID 19 ~ 32 with CP-OFDM modulation are not needed if PDCCH DCI format 0_1 indicates ULFPTx_Mode1. NOTE 4: UE operating in FDD mode, or in TDD mode in bands other than n40, n41, n77, n78 and n79, or in TDD mode the IE powerBoostPi2BPSK is set to 0 for bands n40, n41, n77, n78 and n79. |  |  |  |  |

Table 6.2D.2.4.1-2: Test Configuration Table for Power Class 2 UEs supporting 2-layer codebook based UL MIMO

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest and Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A | Modulation | RB allocation (NOTE 1) |
| 1 | Default |  | CP-OFDM QPSK | Inner Full |
| 2 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 3 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 4 | Default |  | CP-OFDM QPSK | Outer Full |
| 5 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 6 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 7 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 8 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 9 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 10 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 11 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 12 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 13 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 14 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: CP-OFDM 256 QAM test applies only for UEs which supports 256QAM in FR1. |  |  |  |  |

Table 6.2D.2.4.1-2a: Test Configuration Table for power class 2 UEs supporting ULFPTx

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | Default | Reduction (MPR) test case | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 3 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 4 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 5 | Default |  | DFT-s-OFDM QPSK | Inner Full |
| 6 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 7 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 8 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Inner Full |
| 10 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 13 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 14 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 15 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 16 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 17 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 18 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 19 | Default |  | CP-OFDM QPSK | Inner Full |
| 20 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 21 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 22 | Default |  | CP-OFDM QPSK | Outer Full |
| 23 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 24 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 25 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 26 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 27 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 28 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 29 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 30 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 31 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 32 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: Test ID 19 ~ 32 with CP-OFDM modulation are not needed if PDCCH DCI format 0_1 indicates ULFPTx_Mode1. |  |  |  |  |

Table 6.2D.2.4.1-3: Test Configuration Table for Power Class 1.5 UEs supporting 2-layer codebook based UL MIMO

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest and Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A | Modulation | RB allocation (NOTE 1) |
| 1 | Default |  | CP-OFDM QPSK | Inner Full |
| 2 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 3 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 4 | Default |  | CP-OFDM QPSK | Outer Full |
| 5 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 6 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 7 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 8 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 9 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 10 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 11 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 12 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 13 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 14 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: CP-OFDM 256 QAM test applies only for UEs which supports 256QAM in FR1. |  |  |  |  |

Table 6.2D.2.4.1-3a: Test Configuration Table for power class 1.5 UEs supporting ULFPTx except the feature ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | Default | Reduction (MPR) test case | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 3 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 4 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 5 | Default |  | DFT-s-OFDM QPSK | Inner Full |
| 6 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 7 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 8 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Inner Full |
| 10 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 13 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 14 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 15 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 16 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 17 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 18 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: Void |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.2D.2.4.1-1, Table 6.2D.2.4.1-2 or Table 6.2D.2.4.1-3.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2D.2.4.3.

6.2D.2.4.2	Test procedure

Sub-test for 2-Layer codebook based UL-MIMO

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2D.2.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	Measure the sum of the mean power of the UE from both transmit antenna connectors in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of one active sub-frame (1ms) and in the uplink symbols. For TDD symbol with transient periods are not under test.

Sub-test for ULFPTx

4.	If UE supports ULFPTx, repeat test steps 1~3 with UL RMC according to Table 6.2D.2.4.1-1a, Table 6.2D.2.4.1-2a or Table 6.2D.2.4.1-3a. The PDCCH DCI format 0_1 is specified with the condition ULFPTx_Mode1, ULFPTx_Mode2 or ULFPTx_ModeFull in 38.508-1 [5] subclause 4.3.6.1.1.2 depending on UE reported capability. Message contents are according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED.

NOTE 1:	When switching to DFT-s-OFDM waveform, as specified in the test configuration Table 6.2D.2.4.1-1a, Table 6.2D.2.4.1-2a or Table 6.2D.2.4.1-3a, send an NR RRCReconfiguration message according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.2D.2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 ensuring Table 4.6.3-182 with the condition 2TX_UL_MIMO.

6.2D.2.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2D.2.5-1, Table 6.2D.2.5-2, Table 6.2D.2.5-2a, Table 6.2D.2.5-3, or Table 6.2D.2.5-4. The maximum output power, derived in step 4 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2D.2.5-1a, Table 6.2D.2.5-2b, Table 6.2D2.5-2c, Table 6.2D.2.5-3a, or Table 6.2D.2.5-4a.

Table 6.2D.2.5-1: UE Power Class test requirements (for Band n1, n2, n3, n5, n7, n8, n13, n24, n25, n26, n28, n30, n34, n38, n39, n40, n41, n48, n66, n70, n71, n77, n78, n79, n85, n97, n104, n105) for Power Class 3 UEs supporting 2-layer codebook based UL MIMO

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 1.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 |  | 3 (44) | 25.0 + TT | 16.5 - TT | （15.02 - TT） |
| 2 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 3 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 4 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 5 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 3 (44) | 25.0 + TT | 16.0 - TT | （14.52 - TT） |
| 6 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 7 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 8 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 9 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 (44) | 25.0 + TT | 14.5 - TT | （13.02 - TT） |
| 10 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 (44) | 25.0 + TT | 14.5 - TT | （13.02 - TT） |
| 11 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 (44) | 25.0 + TT | 14.5 - TT | （13.02 - TT） |
| 12 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 3 (44) | 25.0 + TT | 11.5 - TT | （9.02 - TT） |
| 13 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 3 (44) | 25.0 + TT | 11.5 - TT | （9.02 - TT） |
| 14 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 3 (44) | 25.0 + TT | 11.5 - TT | （9.02 - TT） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n2, n3, n7, n8, n25, n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2D.2.5-5. NOTE 4: Applicable for band n24. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2.5-1a: UE Power Class test requirements (for Band n1, n2, n3, n5, n7, n8, n13, n24, n25, n26, n28, n30, n34, n38, n41, n48, n66, n70, n71, n77, n78, n79, n85, n104, n105) for Power Class 3 supporting ULFPTx

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | (5.02) | 3 (44) | 25.0 + TT | 20.0 - TT (19.0 – TT5) | （16.52 - TT） |
| 2 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 (44) | 25.0 + TT | 17.5 - TT | （16.02 - TT） |
| 3 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 (44) | 25.0 + TT | 17.5 - TT | （16.02 - TT） |
| 4 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 (44) | 25.0 + TT | 17.5 - TT | （16.02 - TT） |
| 5 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | (5.02) | 3 (44) | 25.0 + TT | 20.0 - TT (19.0 – TT5) | （16.52 - TT） |
| 6 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 5.0 |  | 3 (44) | 25.0 + TT | 17.0 - TT | （15.52 - TT） |
| 7 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 5.0 |  | 3 (44) | 25.0 + TT | 17.0 - TT | （15.52 - TT） |
| 8 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 5.0 |  | 3 (44) | 25.0 + TT | 17.0 - TT | （15.52 - TT） |
| 9 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 5.0 |  | 3 (44) | 25.0 + TT | 17.0 - TT | （15.52 - TT） |
| 10 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 3 (44) | 25.0 + TT | 16.0 - TT | （14.52 - TT） |
| 11 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 3 (44) | 25.0 + TT | 16.0 - TT | （14.52 - TT） |
| 12 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 3 (44) | 25.0 + TT | 16.0 - TT | （14.52 - TT） |
| 13 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 5.0 |  | 3 (44) | 25.0 + TT | 15.5 - TT | （14.02 - TT） |
| 14 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 5.0 |  | 3 (44) | 25.0 + TT | 15.5 - TT | （14.02 - TT） |
| 15 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 5.0 |  | 3 (44) | 25.0 + TT | 15.5 - TT | （14.02 - TT） |
| 16 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 5.0 |  | 3 (44) | 25.0 + TT | 13.5 - TT | （12.02 - TT） |
| 17 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 5.0 |  | 3 (44) | 25.0 + TT | 13.5 - TT | （12.02 - TT） |
| 18 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 5.0 |  | 3 (44) | 25.0 + TT | 13.5 - TT | （12.02 - TT） |
| 19 | 23 | 0 | 1.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 |  | 3 (44) | 25.0 + TT | 16.5 - TT | （15.02 - TT） |
| 20 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 21 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 22 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 23 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 3 (44) | 25.0 + TT | 16.0 - TT | （14.52 - TT） |
| 24 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 25 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 26 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 27 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 (44) | 25.0 + TT | 14.5 - TT | （13.02 - TT） |
| 28 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 (44) | 25.0 + TT | 14.5 - TT | （13.02 - TT） |
| 29 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 (44) | 25.0 + TT | 14.5 - TT | （13.02 - TT） |
| 30 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | (6.02) | 3 (44) | 25.0 + TT | 11.5 - TT | （9.02 - TT） |
| 31 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | (6.02) | 3 (44) | 25.0 + TT | 11.5 - TT | （9.02 - TT） |
| 32 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | (6.02) | 3 (44) | 25.0 + TT | 11.5 - TT | （9.02 - TT） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n2, n3, n7, n8, n25 and n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2D.2.5-3. NOTE 4: Only applicable for band n24. NOTE 5: For band n24 when transmission bandwidths are not confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2.5-2: UE Power Class test requirements (for Bands n1, n3, n8, n28, n40, n41, n66, n70, n77, n78, n79, n104) for Power Class 2 UEs supporting 2-layer codebook based UL MIMO without indicating ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 2 | 0 | （1.52） | 24.0 | （22.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 21.0 - TT | （17.52 - TT） |
| 2 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 |  | 3 | 28.0 + TT | 17.0 - TT | （15.52 - TT） |
| 3 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 |  | 3 | 28.0 + TT | 17.0 - TT | （15.52 - TT） |
| 4 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 5 | 26 | 0 | 2.5 | 0 | （1.52） | 23.5 | （22.02） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.5 - TT | （17.02 - TT） |
| 6 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 |  | 3 | 28.0 + TT | 17.0 - TT | （15.52 - TT） |
| 7 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 |  | 3 | 28.0 + TT | 17.0 - TT | （15.52 - TT） |
| 8 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 9 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 |  | 3 | 28.0 + TT | 16.5 - TT | （15.02 - TT） |
| 10 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 |  | 3 | 28.0 + TT | 16.5 - TT | （15.02 - TT） |
| 11 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 |  | 3 | 28.0 + TT | 16.5 - TT | （15.02 - TT） |
| 12 | 26 | 0 | 8.0 | 0 | （1.52） | 18.0 | （16.52） | 5.0 |  | 3 | 28.0 + TT | 13.0 - TT | （11.52 - TT） |
| 13 | 26 | 0 | 8.0 | 0 | （1.52） | 18.0 | （16.52） | 5.0 |  | 3 | 28.0 + TT | 13.0 - TT | （11.52 - TT） |
| 14 | 26 | 0 | 8.0 | 0 | （1.52） | 18.0 | （16.52） | 5.0 |  | 3 | 28.0 + TT | 13.0 - TT | （11.52 - TT） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For n1, n3, and Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2D.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2.5-2a: UE Power Class test requirements (for Bands n8, n28, n40, n41, n77, n78, n79, n104) for Power Class 2 UEs supporting 2-layer codebook based UL MIMO and indicating ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 1.5 | 0 | （1.52） | 24.5 | （23.02） | 3.0 |  | 3 | 28.0 + TT | 21.5 - TT | （20.02 - TT） |
| 2 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 3 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 4 | 26 | 0 | 3 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.0 - TT | （16.52 - TT） |
| 5 | 26 | 0 | 2 | 0 | （1.52） | 24.0 | （22.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 21.0 - TT | （17.52 - TT） |
| 6 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 7 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 8 | 26 | 0 | 3 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.0 - TT | （16.52 - TT） |
| 9 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 10 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 11 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 12 | 26 | 0 | 6.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 | 28.0 + TT | 14.5 - TT | （13.02 - TT） |
| 13 | 26 | 0 | 6.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 | 28.0 + TT | 14.5 - TT | （13.02 - TT） |
| 14 | 26 | 0 | 6.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 | 28.0 + TT | 14.5 - TT | （13.02 - TT） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2D.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2.5-2b: UE Power Class test requirements (for Bands n1, n3, n8, n28, n40, n41, n66, n70, n77, n78, n79, n104) for Power Class 2 UEs supporting ULFPTx and ul-FullPwrMode1-TPMIGroup-r1 and indicating TxD

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 0 | 0 | （1.52） | 26.0 | （24.52） | 3.0 |  | 3 | 28.0 + TT | 23.0 - TT | （21.52- TT） |
| 2 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 3 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02- TT） |
| 4 | 26 | 0 | 1.0 | 0 | （1.52） | 25.0 | （23.52） | 3.0 |  | 3 | 28.0 + TT | 22.0 - TT | （20.52 - TT） |
| 5 | 26 | 0 | 0.5 | 0 | （1.52） | 25.5 | （24.02） | 3.0 |  | 3 | 28.0 + TT | 22.5 - TT | （21.02 - TT） |
| 6 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 7 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 8 | 26 | 0 | 2 | 0 | （1.52） | 24.0 | （22.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 21.0 - TT | （17.52 - TT） |
| 9 | 26 | 0 | 1.5 | 0 | （1.52） | 24.5 | （23.02） | 3.0 |  | 3 | 28.0 + TT | 21.5 - TT | （20.02 - TT） |
| 10 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 11 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 12 | 26 | 0 | 2.5 | 0 | （1.52） | 23.5 | （22.02） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.5 - TT | （17.02 - TT） |
| 13 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 14 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 15 | 26 | 0 | 3 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.0 - TT | （16.52 - TT） |
| 16 | 26 | 0 | 5.5 | 0 | （1.52） | 20.5 | （19.02） | 5.0 |  | 3 | 28.0 + TT | 14.5 - TT | （14.02 - TT） |
| 17 | 26 | 0 | 5.5 | 0 | （1.52） | 20.5 | （19.02） | 5.0 |  | 3 | 28.0 + TT | 14.5 - TT | （14.02 - TT） |
| 18 | 26 | 0 | 5.5 | 0 | （1.52） | 20.5 | （19.02） | 5.0 |  | 3 | 28.0 + TT | 14.5 - TT | （14.02 - TT） |
| 19 | 26 | 0 | 2 | 0 | （1.52） | 24.0 | （22.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 21.0 - TT | （17.52 - TT） |
| 20 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 |  | 3 | 28.0 + TT | 17.0 - TT | （15.52 - TT） |
| 21 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 |  | 3 | 28.0 + TT | 17.0 - TT | （15.52 - TT） |
| 22 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 23 | 26 | 0 | 2.5 | 0 | （1.52） | 23.5 | （22.02） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.5 - TT | （17.02 - TT） |
| 24 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 |  | 3 | 28.0 + TT | 17.0 - TT | （15.52 - TT） |
| 25 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 |  | 3 | 28.0 + TT | 17.0 - TT | （15.52 - TT） |
| 26 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 27 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 |  | 3 | 28.0 + TT | 16.5 - TT | （15.02 - TT） |
| 28 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 |  | 3 | 28.0 + TT | 16.5 - TT | （15.02 - TT） |
| 29 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 |  | 3 | 28.0 + TT | 16.5 - TT | （15.02 - TT） |
| 30 | 26 | 0 | 8.0 | 0 | （1.52） | 18.0 | （16.52） | 5.0 |  | 3 | 28.0 + TT | 13.0 - TT | （11.52 - TT） |
| 31 | 26 | 0 | 8.0 | 0 | （1.52） | 18.0 | （16.52） | 5.0 |  | 3 | 28.0 + TT | 13.0 - TT | （11.52 - TT） |
| 32 | 26 | 0 | 8.0 | 0 | （1.52） | 18.0 | （16.52） | 5.0 |  | 3 | 28.0 + TT | 13.0 - TT | （11.52 - TT） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n1, n3, and n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2.5-2c: UE Power Class test requirements (for Bands n1, n3, n8, n28, n40, n41, n66, n70, n77, n78, n79, n104) for Power Class 2 UEs supporting ULFPTx and, (supporting ul-FullPwrMode1-TPMIGroup-r1 without indicating TxD) or (supporting ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 0 | 0 | （1.52） | 26.0 | （24.52） | 3.0 |  | 3 | 28.0 + TT | 23.0 - TT | （21.52 - TT） |
| 2 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 3 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02- TT） |
| 4 | 26 | 0 | 0.5 | 0 | （1.52） | 25.5 | （24.02） | 3.0 |  | 3 | 28.0 + TT | 22.5 - TT | （21.02 - TT） |
| 5 | 26 | 0 | 0 | 0 | （1.52） | 26.0 | （24.52） | 3.0 |  | 3 | 28.0 + TT | 23.0 - TT | （21.52 - TT） |
| 6 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 7 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 8 | 26 | 0 | 1 | 0 | （1.52） | 25.0 | （23.52） | 3.0 |  | 3 | 28.0 + TT | 22.0 - TT | （20.52 - TT） |
| 9 | 26 | 0 | 1 | 0 | （1.52） | 25.0 | （23.52） | 3.0 |  | 3 | 28.0 + TT | 22.0 - TT | （20.52 - TT） |
| 10 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 11 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 12 | 26 | 0 | 2 | 0 | （1.52） | 24.0 | （22.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 21.0 - TT | （17.52 - TT） |
| 13 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 14 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02- TT） |
| 15 | 26 | 0 | 2.5 | 0 | （1.52） | 23.5 | （22.02） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.5 - TT | （17.02 - TT） |
| 16 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 |  | 3 | 28.0 + TT | 16.5 - TT | （15.02 - TT） |
| 17 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 |  | 3 | 28.0 + TT | 16.5 - TT | （15.02 - TT） |
| 18 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 |  | 3 | 28.0 + TT | 16.5 - TT | （15.02 - TT） |
| 19 | 26 | 0 | 1.5 | 0 | （1.52） | 24.5 | （23.02） | 3.0 |  | 3 | 28.0 + TT | 21.5 - TT | （20.02 - TT） |
| 20 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 21 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 22 | 26 | 0 | 3 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.0 - TT | （16.52 - TT） |
| 23 | 26 | 0 | 2 | 0 | （1.52） | 24.0 | （22.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 21.0 - TT | （17.52 - TT） |
| 24 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 25 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 26 | 26 | 0 | 3 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.0 - TT | （16.52 - TT） |
| 27 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 28 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 29 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 30 | 26 | 0 | 6.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 | 28.0 + TT | 14.5 - TT | （13.02 - TT） |
| 31 | 26 | 0 | 6.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 | 28.0 + TT | 14.5 - TT | （13.02 - TT） |
| 32 | 26 | 0 | 6.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 | 28.0 + TT | 14.5 - TT | （13.02 - TT） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2.5-3: UL MIMO MPR test requirements (for Band n41, n77, n78, n79) for Power Class 1.5 UEs

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 29 | 0 | 2 | 0 | （1.52） | 27.0 | （25.52） | 3.0 |  | 3 | 31.0 + TT | 24.0 - TT | （22.52 - TT） |
| 2 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.02 - TT） |
| 3 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.02 - TT） |
| 4 | 29 | 0 | 4.5 | 0 | （1.52） | 24.5 | （23.02） | 3.0 |  | 3 | 31.0 + TT | 21.5 - TT | （20.02 - TT） |
| 5 | 29 | 0 | 2.5 | 0 | （1.52） | 26.5 | （25.02） | 3.0 |  | 3 | 31.0 + TT | 23.5 - TT | （22.02 - TT） |
| 6 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.02 - TT） |
| 7 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.02 - TT） |
| 8 | 29 | 0 | 4.5 | 0 | （1.52） | 24.5 | （23.02） | 3.0 |  | 3 | 31.0 + TT | 21.5 - TT | （20.02 - TT） |
| 9 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.02 - TT） |
| 10 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.02 - TT） |
| 11 | 29 | 0 | 5 | 0 | （1.52） | 24.0 | （22.52） | 3.0 | （5.02） | 3 | 31.0 + TT | 21.0 - TT | （17.52 - TT） |
| 12 | 29 | 0 | 8.5 | 0 | （1.52） | 20.5 | （19.02） | 5.0 |  | 3 | 31.0 + TT | 15.5 - TT | （14.02 - TT） |
| 13 | 29 | 0 | 8.5 | 0 | （1.52） | 20.5 | （19.02） | 5.0 |  | 3 | 31.0 + TT | 15.5 - TT | （14.02 - TT） |
| 14 | 29 | 0 | 8.5 | 0 | （1.52） | 20.5 | （19.02） | 5.0 |  | 3 | 31.0 + TT | 15.5 - TT | （14.02 - TT） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n1, n3, and n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2D.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2.5-3a: UL MIMO MPR test requirements (for Band n41, n77, n78, n79) for Power Class 1.5 UEs supporting ULFPTx except the feature ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 29 | 0 | 0.5 | 0 | （1.52） | 28.5 | （27.02） | 3.0 |  | 3 | 31.0 + TT | 25.5 – TT | （24.02 - TT） |
| 2 | 29 | 0 | 6 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | (5.02) | 3 | 31.0 + TT | 20.0 – TT | （18.52 - TT） |
| 3 | 29 | 0 | 6 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | (5.02) | 3 | 31.0 + TT | 20.0 – TT | （18.52 - TT） |
| 4 | 29 | 0 | 2 | 0 | （1.52） | 27.0 | （25.52） | 3.0 |  | 3 | 31.0 + TT | 24.0 – TT | （22.52 - TT） |
| 5 | 29 | 0 | 0.5 | 0 | （1.52） | 28.5 | （27.02） | 3.0 |  | 3 | 31.0 + TT | 25.5 – TT | （24.02 - TT） |
| 6 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 – TT | （16.02 - TT） |
| 7 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 – TT | （16.02 - TT） |
| 8 | 29 | 0 | 2.5 | 0 | （1.52） | 26.5 | （25.02） | 3.0 |  | 3 | 31.0 + TT | 23.5 - TT | （22.02 - TT） |
| 9 | 29 | 0 | 1.5 | 0 | （1.52） | 27.5 | （26.02） | 3.0 |  | 3 | 31.0 + TT | 24.5 - TT | （23.02 - TT） |
| 10 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.02 - TT） |
| 11 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.02 - TT） |
| 12 | 29 | 0 | 3.5 | 0 | （1.52） | 25.5 | （24.02） | 3.0 |  | 3 | 31.0 + TT | 22.5 - TT | （21.02 - TT） |
| 13 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.02 - TT） |
| 14 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.02 - TT） |
| 15 | 29 | 0 | 4 | 0 | （1.52） | 25.0 | （23.52） | 3.0 |  | 3 | 31.0 + TT | 22.0 - TT | （20.52 - TT） |
| 16 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.02 - TT） |
| 17 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.02 - TT） |
| 18 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.02 - TT） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2D.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2.5-4: UL MIMO MPR test requirements (for Band n41, n77, n78, n79) for Power Class 1.5 FWA UEs

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 29 | 0 | 1.5 | 0 | （1.53） | 27.5 | （26.03） | 3.0 |  | 3 | 31.0 + TT | 24.5 - TT | （23.03 - TT） |
| 2 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | （21.03） | 5.0 |  | 3 | 31.0 + TT | 17.5 – TT | （16.03 - TT） |
| 3 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | （21.03） | 5.0 |  | 3 | 31.0 + TT | 17.5 – TT | （16.03 - TT） |
| 4 | 29 | 0 | 4 | 0 | （1.53） | 25.0 | （23.53） | 3.0 |  | 3 | 31.0 + TT | 22.0 – TT | （20.53 - TT） |
| 5 | 29 | 0 | 2 | 0 | （1.53） | 27.0 | （25.53） | 3.0 |  | 3 | 31.0 + TT | 24.0 – TT | （22.53 - TT） |
| 6 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | （21.03） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.03 - TT） |
| 7 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | （21.03） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.03 - TT） |
| 8 | 29 | 0 | 4 | 0 | （1.53） | 25.0 | （23.53） | 3.0 |  | 3 | 31.0 + TT | 22.0 - TT | （20.53 - TT） |
| 9 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | （21.03） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.03 - TT） |
| 10 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | （21.03） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.03 - TT） |
| 11 | 29 | 0 | 4.5 | 0 | （1.53） | 24.5 | （23.03） | 3.0 |  | 3 | 31.0 + TT | 21.5 - TT | （20.03 - TT） |
| 12 | 29 | 0 | 7.5 | 0 | （1.53） | 21.5 | （20.03） | 5.0 |  | 3 | 31.0 + TT | 16.5 - TT | （15.03 - TT） |
| 13 | 29 | 0 | 7.5 | 0 | （1.53） | 21.5 | （20.03） | 5.0 |  | 3 | 31.0 + TT | 16.5 - TT | （15.03 - TT） |
| 14 | 29 | 0 | 7.5 | 0 | （1.53） | 21.5 | （20.03） | 5.0 |  | 3 | 31.0 + TT | 16.5 - TT | （15.03 - TT） |
| NOTE 1: This table is targeted to large FWA form factor with 20 dB or above antenna isolation. NOTE 2: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 3: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 4: TT for each frequency and channel bandwidth is specified in Table 6.2D.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2.5-4a: UL MIMO MPR test requirements (for Band n41, n77, n78, n79) for Power Class 1.5 FWA UEs supporting ULFPTx except the feature ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 29 | 0 | 0 | 0 | （1.53） | 29.0 | （27.53） | 3.0 |  | 3 | 31.0 + TT | 26.0 – TT | （24.53 - TT） |
| 2 | 29 | 0 | 6 | 0 | （1.53） | 23.0 | （21.53） | 3.0 | (5.03) | 3 | 31.0 + TT | 20.0 – TT | （18.53 - TT） |
| 3 | 29 | 0 | 6 | 0 | （1.53） | 23.0 | （21.53） | 3.0 | (5.03) | 3 | 31.0 + TT | 20.0 – TT | （18.53 - TT） |
| 4 | 29 | 0 | 1.5 | 0 | （1.53） | 27.5 | （26.03） | 3.0 |  | 3 | 31.0 + TT | 24.5 – TT | （23.03 - TT） |
| 5 | 29 | 0 | 0 | 0 | （1.53） | 29.0 | （27.53） | 3.0 |  | 3 | 31.0 + TT | 26.0 – TT | （24.53 - TT） |
| 6 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | （21.03） | 5.0 |  | 3 | 31.0 + TT | 17.5 – TT | （16.03 - TT） |
| 7 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | （21.03） | 5.0 |  | 3 | 31.0 + TT | 17.5 – TT | （16.03 - TT） |
| 8 | 29 | 0 | 2 | 0 | （1.53） | 27.0 | （25.53） | 3.0 |  | 3 | 31.0 + TT | 24.0 - TT | （22.53 - TT） |
| 9 | 29 | 0 | 1 | 0 | （1.53） | 28.0 | （26.53） | 3.0 |  | 3 | 31.0 + TT | 25.0 - TT | （23.53 - TT） |
| 10 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | （21.03） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.03 - TT） |
| 11 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | （21.03） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.03 - TT） |
| 12 | 29 | 0 | 3 | 0 | （1.53） | 26.0 | （24.53） | 3.0 |  | 3 | 31.0 + TT | 23.0 - TT | （21.53 - TT） |
| 13 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | （21.03） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.03 - TT） |
| 14 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | （21.03） | 5.0 |  | 3 | 31.0 + TT | 17.5 – TT | （16.03 - TT） |
| 15 | 29 | 0 | 3.5 | 0 | （1.53） | 25.5 | （24.03） | 3.0 |  | 3 | 31.0 + TT | 22.5 - TT | （21.03 - TT） |
| 16 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | （21.03） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.03 - TT） |
| 17 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | （21.03） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.03 - TT） |
| 18 | 29 | 0 | 5.5 | 0 | （1.53） | 23.5 | （22.03） | 3.0 | (5.03) | 3 | 31.0 + TT | 20.5 - TT | （17.03 - TT） |
| NOTE 1: This table is targeted to large FWA form factor with 20 dB or above antenna isolation. NOTE 2: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 3: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 4: TT for each frequency and channel bandwidth is specified in Table 6.2D.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2.5-5: Test Tolerance (Maximum Power Reduction (MPR))

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7 | 1.0 |
| 40MHz < BW ≤ 100MHz | 1.0 | 1.0 |

<< END OF CHANGES >>