3GPP TSG-RAN5 Meeting #108	R5-254002r1

# Bangalore, India, 25th - 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1616 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of test case 17.19 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test |  |  |  |  |  | Date: |  |  | 2025-08-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - According to RAN5 working assumption red/1000 is optional in the initial SDP offer and therefore present in the subsequent SDP messages only if present in the initial offer. - Notes 1 in the specific message contents of the INVITE and the PRACK are not precise. - Editorial issues. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - SDP message contents adapted to RAN5 working assumption: As t140/1000 does not need to have format specific parameters, a=fmtp needs to be removed from the RTT media description in the INVITE. - Wording of notes 1 changed in the specific message contents of the INVITE and the PRACK to clarify that if there is no c= line in the session description there shall be a c= line in each media description - Editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent test specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 17.19 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: SDP message contents adapted to RAN5 working assumption; wording of notes 1 in the specific message contents of the INVITE and the PRACK made precise. |  |  |  |  |  |  |  |  |

## 17.19	MO Speech / add RTT and remove RTT / EVS / Default configuration

### 17.19.1	Definition

Test to verify that the UE is able to add a bidirectional text (RTT) component to an ongoing IMS Multimedia telephony voice call. This process is described in 3GPP TS 24.229 [10], TS 24.173 [65] and TS 26.114 [66].

### 17.19.2	Conformance requirement

[TS 24.229, clause 5.1.2A.1.1]

After the dialog is established the UE may change the dialog capabilities (e.g. add a media or request a supplementary service) if defined for the IMS communication service as identified by the ICSI value using the same dialog. Otherwise, the UE shall initiate a new initial request to the other user.

[TS 26.114, clause 6.2.5]

The SDP shall include bandwidth information for each media stream and also for the session in total. The bandwidth information for each media stream and for the session is defined by the Application Specific (AS) bandwidth modifier as defined in RFC 4566.

[TS 26.114, clause 6.3]:

During session renegotiation for adding or removing media components, the SDP offerer should continue to use the same media (m=) line(s) from the previously negotiated SDP for the media components that are not being added or removed.

[TS 26.114, clause 7.3.1]

The bandwidth for RTCP traffic shall be described using the "RS" and "RR" SDP bandwidth modifiers at media level, as specified by RFC 3556.

### 17.19.3	Test purpose

1)	To verify that when adding an RTT component to an ongoing IMS Multimedia Telephony voice call the UE performs correct exchange of SIP protocol signalling messages; and

2)	To verify that within SIP signalling the UE performs correct SDP offer/answer exchanges for negotiating media and indicating preconditions for resource reservation (as described by 3GPP TS 24.229 [10], clause 6.1); and

3)	To verify that when removing the RTTcomponent from the IMS Multimedia Telephony call the UE performs correct exchange of SIP and SDP protocol messages.

### 17.19.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC. UE has discovered P-CSCF, registered to IMS services and set up the MO call, by executing annex C.52.

SS is configured with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) configured on the UICC card equipped into the UE. SS has performed AKAv1-MD5 authentication with the UE and accepted the registration and MO call.

Test procedure

1)	Adding RTT to the voice call is initiated on the UE.

2-10)	UE executes the procedure described in TS 36.508 [94] table 4.5A.nn.3-1, steps 2 to 7. In detail the following steps are done in IMS.

2)	UE to sends a re-INVITE request to the SS.

3)	SS responds to the re-INVITE request with a 100 Trying response.

4)	SS responds to the re-INVITE request with a 183 Session Progress response.

5)	SS waits for the UE to send a PRACK request possibly containing the second SDP offer for update of precondition state.

6)	SS responds to the PRACK request with a valid 200 OK response.

7)	SS waits for the UE to optionally send an UPDATE request containing the final SDP offer. UE will not send the UPDATE request if the PRACK in step 4 already contained the final offer with preconditions met.

8)	SS responds to the UPDATE request (if UE sent one) with a valid 200 OK response.

9)	SS responds to the re-INVITE request with a valid 200 OK response.

10)	SS waits for the UE to send an ACK to acknowledge receipt of the 200 OK for INVITE.

11)	The UE is triggered to remove the RTT stream from the multimedia call

12)	SS waits the UE to send a re-INVITE request with a SDP offer indicating the removal of the RTT stream.

13)	SS responds to the re-INVITE request with a 100 Trying response.

14)	SS deactivates the EPS bearer corresponding to the RTT stream and releases the associated radio resources by applying the procedure described in TS 36.508 [94] clause 4.5A.15

15)	SS responds to the re-INVITE request with a valid 200 OK response.

16)	SS waits for the UE to send an ACK to acknowledge receipt of the 200 OK for re-INVITE.

17-21)	MO Call release according to procedure C.32.

Expected sequence

NOTE:	Only the IMS procedure relevant to the test purpose is described below.

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | Make UE add RTT to the voice call. |  |
| 2 |  |  | INVITE | UE sends re-INVITE with an SDP offer containing media lines for both voice and RTT |
| 3 |  |  | 100 Trying | The SS responds with a 100 Trying provisional response |
| 4 |  |  | 183 Session Progress | SS responds with an SDP answer indicating that SS has not reserved its resources for RTT. |
| 5 |  |  | PRACK | UE acknowledges the receipt of 183 response with PRACK and optionally offers second SDP to indicate the changed precondition status. |
| 6 |  |  | 200 OK | The SS responds PRACK with 200 OK and answers the second SDP (if any) with mirroring its contents. |
| 7 |  |  | UPDATE | UE sends an UPDATE after having reserved the resources for RTT if meeting the preconditions was not already indicated in step 2 or 5. |
| 8 |  |  | 200 OK | The SS responds UPDATE, if received, with 200 OK and indicates having reserved the resources |
| 9 |  |  | 200 OK | The SS responds re-INVITE with 200 OK |
| 10 |  |  | ACK | The UE acknowledges the receipt of 200 OK for re-INVITE |
| 11 |  |  | Make UE release RTT from the media call |  |
| 12 |  |  | INVITE | UE sends re-INVITE with a SDP offer indicating that the RTT component is removed from the call |
| 13 |  |  | 100 Trying | The SS responds with a 100 Trying provisional response |
| 14 |  |  | SS deactivates the EPS bearer for RTT |  |
| 15 |  |  | 200 OK | The SS responds re-INVITE with 200 OK |
| 16 |  |  | ACK | The UE acknowledges the receipt of 200 OK for re-INVITE |
| 17-21 |  |  | Steps defined in annex C.32 | MO Call release |

Specific Message Contents

INVITE (Step 2)

Use the default message “INVITE for MO Call” in annex A.2.1 with condition A5 (re-INVITE within a dialog) and the following exceptions:

| Header/param | Value/Remark |
| --- | --- |
| Supported |  |
| option-tag | precondition |
| Message-body | The following SDP types and values. Session description: - v=0 - o=(username) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) [Note 3] - s=(session name) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) Time description: - t= (start-time) (stop-time) Media description: - m=audio (transport port) RTP/AVP (fmt) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) - b=RS: (bandwidth-value) - b=RR: (bandwidth-value) Attributes for media: - a=rtpmap: (payload type) EVS/16000 [Note 2] - a=fmtp: (format) Attributes for preconditions: - a=curr:qos local sendrecv - a=curr:qos remote sendrecv - a=des:qos mandatory local sendrecv - a=des:qos optional remote sendrecv or a=des:qos mandatory remote sendrecv Media description: - m=text (transport port) RTP/AVP (fmt) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) - b=RS: (bandwidth-value) - b=RR: (bandwidth-value) Attributes for media: - a=rtpmap: (payload type) t140/1000 Attributes for preconditions: - a=curr:qos local none - a=curr:qos remote none - a=des:qos mandatory local sendrecv - a=des:qos optional remote sendrecv or a=des:qos mandatory remote sendrecv Note 1: If there is no c= line in the session description there shall be a c= line in each media description. Note 2: The AMR channel number shall be “/1” or omitted. Note 3: "o=" line identical to previous SDP sent by UE except that sess-version is incremented by one. |

183 Session Progress (Step 4)

Use the default message "183 Session Progress" in annex A.2.3 with condition A15 (183 sent for re-INVITE within an established dialog) and the following exceptions:

| Header/param | Value/Remark |
| --- | --- |
| Require |  |
| option-tag | precondition |
| Message-body | The following SDP types and values. Session description: - v=0 - "o=" line identical to previous SDP sent by SS except that sess-version is incremented by one - s=- - c=IN (addrtype) (connection-address for SS) - b=AS: (bandwidth-value) [Note 1] Time description: - t=0 0 Media description: - m=audio (transport port) RTP/AVP (fmt) [Note 1] - b=AS: (bandwidth-value) [Note 1] - b=RS: (bandwidth-value) [Note 1] - b=RR: (bandwidth-value) [Note 1] Attributes for media: - a=rtpmap: (payload type) EVS/16000/1 [Note 1] - a=fmtp: (format) [Note 1] - a=ptime:20 - a=maxptime:240 Attributes for preconditions: - a=curr:qos local sendrecv - a=curr:qos remote sendrecv - a=des:qos mandatory local sendrecv - a=des:qos mandatory remote sendrecv Media description: - m=text (transport port) RTP/AVP (fmt) [Note 1] - b=AS: (bandwidth-value) [Note 1] - b=RS: (bandwidth-value) [Note 1] - b=RR: (bandwidth-value) [Note 1] Attributes for media: - a=rtpmap: (payload type) t140/1000 [Note 1] - a=rtpmap: (payload type) red/1000 [Note 1] - a=fmtp: (format) (format specific parameters) [Note 1] Attributes for preconditions: - a=curr:qos local none - a=curr:qos remote none - a=des:qos mandatory local sendrecv - a=des:qos mandatory remote sendrecv - a=conf:qos remote sendrecv Note 1: The value for fmt, bandwidth, payload type, format and format specific parameters copied from step 2. Note 2: If red/1000 is present in step 2 the values for fmt, payload type, format specific parameters and format are copied from step 2, otherwise the red/1000 codec is not present. |

PRACK (Step 5)

Use the default message “PRACK” in annex A.2.4 with the exceptions:

| Header/param | Value/Remark |
| --- | --- |
| Supported option-tag | precondition [Note 4] |
| Message-body | Header optional Contents if present: The following SDP types and values shall be present. Session description: - v=0 - o=(username) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) [Note 2] - s=(session name) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) Time description: - t=0 0 Media description: - m=audio (transport port) RTP/AVP (fmt) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) - b=RS: (bandwidth-value) - b=RR: (bandwidth-value) Attributes for media: - a=rtpmap: (payload type) EVS/16000 [Note 3] - a=fmtp: (format) Attributes for preconditions: - a=curr:qos local sendrecv - a=curr:qos remote sendrecv - a=des:qos mandatory local sendrecv - a=des:qos optional remote sendrecv or a=des:qos mandatory remote sendrecv Media description: - m=text (transport port) RTP/AVP (fmt) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) - b=RS: (bandwidth-value) - b=RR: (bandwidth-value) Attributes for media: - a=rtpmap: (payload type) t140/1000 - a=rtpmap: (payload type) red/1000 [Note 5] - a=fmtp: (format) [Note 5] Attributes for preconditions: - a=curr:qos local sendrecv - a=curr:qos remote none - a=des:qos mandatory local sendrecv - a=des:qos optional remote sendrecv or a=des:qos mandatory remote sendrecv Note 1: If there is no c= line in the session description there shall be a c= line in each media description. Note 2: "o=" line identical to previous SDP sent by UE except that sess-version is incremented by one. Note 3: The AMR channel number shall be “/1” or omitted. Note 4: Shall be present if message-body present. Note 5: Present only if present at step 2. |

200 OK for PRACK (Step 6)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Require option-tag | precondition (shall be present if SDP message-body present) |
| Content-Type | Header optional Contents if present: |
| media-type | application/sdp |
| Content-Length | Contents if header Content-Type is present: |
| Value | length of message-body |
| Message-body | Header present if Prack (step 5) contained SDP. Contents if present: SDP body of the 200 response copied from the received PRACK and modified as follows: - "o=" line identical to previous SDP sent by SS except that sess-version is incremented by one - IP address on "c=" line and transport port on "m=" lines changed to indicate to which IP address and port the UE should start sending the media; Attributes for preconditions (text): - a=curr:qos remote sendrecv |

UPDATE (Step 7)

Use the default message “UPDATE” in annex A.2.5 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Supported | Same contents as specified in step 5. |
| Message-body | Same contents as specified in step 5. |

200 OK for UPDATE (Step 8)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Require option-tag | precondition (shall be present if SDP message-body present) |
| Content-Type | Header optional Contents if present: |
| media-type | application/sdp |
| Content-Length | Contents if header Content-Type is present: |
| Value | length of message-body |
| Message-body | SDP body of the 200 response copied from the received UPDATE and modified as follows: - "o=" line identical to previous SDP sent by SS except that sess-version is incremented by one - IP address on "c=" line and transport port on "m=" lines changed to indicate to which IP address and port the UE should start sending the media; Attributes for preconditions (text): - a=curr:qos remote sendrecv |

200 OK for INVITE (Step 9)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1 with condition A23 (Response sent for re-INVITE within an established dialog).

ACK (Step 10)

Use the default message “ACK” in annex A.2.7 with condition A5.

INVITE (Step 12)

Use the default message “INVITE for MO Call” in annex A.2.1 with condition A5 (re-INVITE within a dialog) and the following exceptions:

| Header/param | Value/Remark |
| --- | --- |
| Supported |  |
| option-tag | precondition |
| Message-body | The following SDP types and values. Session description: - v=0 - o=(username) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) [Note 2] - s=(session name) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) Time description: - t= (start-time) (stop-time) Media description: - m=audio (transport port) RTP/AVP (fmt) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) - b=RS: (bandwidth-value) - b=RR: (bandwidth-value) Attributes for media: - a=rtpmap: (payload type) EVS/16000 [Note 3] - a=fmtp: (format) Attributes for preconditions: - a=curr:qos local sendrecv - a=curr:qos remote sendrecv - a=des:qos mandatory local sendrecv - a=des:qos optional remote sendrecv or a=des:qos mandatory remote sendrecv Media description: - m=text 0 RTP/AVP (fmt) Note 1: At least one "c=" field shall be present. Note 2: "o=" line identical to previous SDP sent by UE except that sess-version is incremented by one. Note 3: The AMR channel number shall be “/1” or omitted. |

200 OK for INVITE (Step 15)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1 with condition A23 (Response sent for re-INVITE within an established dialog) and the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Require option-tag | precondition |
| Content-Type |  |
| media-type | application/sdp |
| Content-Length |  |
| Value | length of message-body |
| Message-body | SDP body of the 200 response copied from the received INVITE and modified as follows: - "o=" line identical to previous SDP sent by SS except that sess-version is incremented by one - IP address on "c=" line and, for audio, transport port on "m=" line changed to indicate to which IP address and port the UE should start sending the media; |

ACK (Step 16)

Use the default message “ACK” in annex A.2.7 with condition A5.

### 17.19.5	Test requirements

The UE shall send requests and responses as described in clause 17.19.4