3GPP TSG-RAN5 Meeting #108 	R5-254674r1

# Bengaluru, India, 25th August 2025 – 29th August  2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-3 | CR | 3103 | rev | - | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Test Tolerance update for HD-FDD UE Timing Advance Adjustment Accuracy Test 7.2.9,13.4.2.1 and 13.4.2.2. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Korea |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Existing TT analysis for TC 7.2.9,13.4.2.1 and 13.4.2.2 has incorrect uncertainty considered. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Test tolerance is updated for test cases 7.2.9,13.4.2.1 and 13.4.2.2 with correct uncertainty consideration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Test case implementation will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.9,13.4.2.1,13.4.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TR 36.903 CR 0471 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: The measurement uncertainty for relative UE timing adjustment is incorrectly derived with no proper justification for the MU value. If we take legacy test case as an example, a much higher MU needs to be considered. More detailed analysis is needed for coming up with MU value. Reverted back the changes to the test case and added editor’s note to mark the test case incomplete. |  |  |  |  |  |  |  |  |

<< Start of Changes >>

### 7.2.9	HD-FDD UE Timing Advance Adjustment Accuracy Test for Category NB1 UE in Standalone Mode under Enhance Coverage

Editor’s note: This test case is incomplete due to the following aspects

-	The MU value derivation is incorrect and needs further analysis.

#### 7.2.9.1	Test Purpose

The purpose of the test is to verify E-UTRAN Timing Advance adjustment accuracy requirements for category NB1 UE in enhanced coverage in an AWGN model.

#### 7.2.9.2	Test applicability

This test applies to all types of NB-IoT HD-FDD UE release 13 and forward of UE Category NB1 and higher.

#### 7.2.9.3	Minimum conformance requirements

The timing advance is initiated from E-UTRAN with MAC message that implies and adjustment of the timing advance, see TS 36.321 [17] clause 5.2.

UE shall adjust the timing of its uplink transmission timing at sub-frame n+12 for a timing advance command received in sub-frame n. In case repetitions are used on the downlink, sub-frame n refers to the last subframe in the repetition period in which the message containing the MAC control information was received. The UE shall not apply a TA command during an uplink repetition period.

The UE shall adjust the timing of its transmissions with a relative accuracy better than or equal to ±13.33* TS seconds to the signalled timing advance value compared to the timing of preceding uplink transmission. The timing advance command is expressed in multiples of 16* TS and is relative to the current uplink timing.

In case of random access response, 11-bit timing advance command, TA, indicates NTA values by index values of TA = 0, 1, 2, ..., 1282, where an amount of the time alignment is given by NTA = TA × 16 according to 3GPP TS 36.213 [8] clause 16.1.2. NTA is defined in 3GPP TS 36.211 [9].

In other cases, 6-bit timing advance command, TA, indicates adjustment of the current NTA value, NTA,old, to the new NTA value, NTA,new, by index values of  TA = 0, 1, 2,..., 63,  where NTA,new = NTA,old + (TA - 31) × 16 according to 3GPP TS 36.213 [8] clause 16.1.2. Adjustment of NTA value by a positive or a negative amount indicates advancing or delaying the uplink transmission timing by a given amount respectively.

For UE timers, the UE shall comply with the timer accuracies according to 3GPP TS 36.133 [4] table 7.21.2-1. The requirements are only related to the actual timing measurements internally in the UE.

The normative reference for this requirement is 3GPP TS 36.133 [4] clause 7.22 and A.7.2.9.

#### 7.2.9.4	Test description

##### 7.2.9.4.1	Initial conditions

Test Environment: Normal, as defined in 3GPP TS 36.508 [7] clause 8.1.1.

Frequencies to be tested: According to Annex E  table Annex E-4 and 3GPP TS 36.508 [7] clauses 8.1.3 and 8.1.4.2.

Channel Bandwidth to be tested: 200 KHz.

1.	Connect the SS (node B emulator) and AWGN noise sources to the UE antenna connectors as shown in 3GPP TS 36.508 [7] Annex A, Figure A.18 using only main UE Tx/Rx antenna.

2.	The general test parameter settings are set according to Table 7.2.9.4.1-1.

3.	Propagation conditions are set according to Annex B clause B.0.

4.	There is one cell specified in the test. Ncell 1 is the cell used for connection setup with the power level set according to Annex C.0 and C.1 for this test.

Table 7.2.9.4.1-1: General Test Parameters for E-UTRAN Timing Advance Accuracy Test for Category NB1 UE in Standalone Mode under Enhanced Coverage

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NB-IoT operational mode |  |  | Standalone |  |
| CP Length |  |  | Normal |  |
| Timing Advance Command (TA) value during T1 |  |  | 31 | NTA = 0 for the purpose of establishing a reference value from which the timing advance adjustment accuracy can be measured during T2 |
| Timing Advance Command (TA) value during T2 |  |  | 39 | NTA = 128 |
| Number of repetitions | NPDCCH |  | 128 |  |
|  | NPDSCH |  | 128 |  |
|  | NPUSCH |  | 32 |  |
| DRX |  |  | OFF |  |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |

##### 7.2.9.4.2	Test procedure

The test consists of a single cell. The test consists of two successive time periods, with time duration of T1 and T2 respectively. In each time period, timing advance commands are sent to the UE and the UE is scheduled in uplink subframes to transmit NPUSCH, which is received by the test equipment. By measuring the reception of the NPUSCH, the transmit timing, and hence the timing advance adjustment accuracy, can be measured. The UE Time Alignment Timer (timeAlignmentTimer IE), defined in 3GPP TS 36.321 [11] clause 5.2, shall be configured so that it does not expire in the duration of the test.

- Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to 3GPP TS 36.508 [7] clause 8.1.5.

- Set the parameters according to T1 in Tables 7.2.9.5-1. Propagation conditions are set according to Annex B clause B.1.1.

- SS sends uplink scheduling information via NPDCCH DCI format N1 for C_RNTI to schedule uplink subframes according to 3GPP TS 36.508 [7] clause 8.1.3.6, Table 8.1.3.6.1.2-2.

- SS shall transmit one message with a timing advance command, TA. The timing advance command, TA, shall be set to 31 which indicate adjustment of the current NTA value. The timing advance adjustment during T1 shall be NTA = 0.

- The UE shall transmit NPUSCH (NPUSCH Format 2 is used to carry ACK/NACK) and adjust its uplink timing at the beginning of sub-frame n+12 for a timing advance command TA received in sub-frame n. Sub-frame n is the last subframe in the repetition period of NPDSCH in which the timing advance command is received by the UE.

- When T1 expires, the SS shall switch the timing advance command (TA) from T1 to T2 as specified in Table 7.2.9.5-1.

- SS shall transmit a sequence of messages with timing advance command, TA. The timing advance command, TA, shall be set to 39 which indicate adjustment of the current NTA value. The timing advance adjustment during T2 shall be NTA = 128.

- The UE shall transmit NPUSCH (NPUSCH Format 2 is used to carry ACK/NACK) and adjust its uplink timing at the beginning of sub-frame n+12 for a timing advance command TA received in sub-frame n. Sub-frame n is the last subframe in the repetition period of NPDSCH in which the timing advance command is received by the UE.

- The result from the NPUSCH and adjustment of the timing advance in step 8) is used to measure that the UE adjusts the timing of its transmission with a relative accuracy better than or equal to ±13.83 × TS to the signalled timing advance value compared to the timing of preceding uplink transmission.

- If the UE adjust the timing of its transmission within a relative accuracy greater than or equal to ± 13.83 × TS to the signalled timing advance value compared to the timing of preceding uplink transmission then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

- The SS shall transmit RRCConnectionRelease-NB message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

12.	After the RRC connection release, the SS:

- transmits in Ncell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE is in State 2A-NB with CP CIoT Optimisation according to 3GPP TS 36.508 [7] clause 8.1.5 (if the paging fails, switches off and on the UE and ensures the UE is in State 2A-NB with CP CIoT Optimisation according to 3GPP TS 36.508 [7] clause  8.1.5),

or

- switches off and on the UE and ensures the UE is in State 2A-NB with CP CIoT Optimisation according to 3GPP TS 36.508 [7] clause 8.1.5.

13.	Repeat step 2-12 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

##### 7.2.9.4.3	Message contents

Message contents are according to 3GPP TS 36.508 [7] clause 8.1.6 with the following exceptions.

Table 7.2.9.4.3-1: NPDCCH-ConfigDedicated-NB-DEFAULT

| Derivation Path: 3GPP TS 36.331 clause 6.7.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPDCCH-ConfigDedicated-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| npdcch-NumRepetitions-r13 | r128 |  |  |
| npdcch-StartSF-USS-r13 | V2 |  |  |
| npdcch-Offset-USS-r13 | zero |  |  |
| } |  |  |  |

Table 7.2.9.4.3-2: NB-IoT Physical layer parameters for DCI format N1

| Parameter | Value |  | Value in binary |  |  |
| --- | --- | --- | --- | --- | --- |
| Repetition number |  | NRep = 128 |  | “0111” |
| DCI subframe repetition number |  | R = 128 |  | “11” |

Table 7.2.9.4.3-3: NPUSCH-ConfigDedicated-NB-DEFAULT

| Derivation Path: 3GPP TS 36.331 clause 6.7.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPUSCH-ConfigDedicated-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| ack-NACK-NumRepetitions-r13 | r32 |  |  |
| } |  |  |  |

#### 7.2.9.5	Test requirement

Tables 7.2.9.4.1-1, 7.2.9.5-1 and 7.2.9.5-2 define the primary level settings for HD-FDD UE timing advance adjustment accuracy test.

Table 7.2.9.5-1: Cell specific Test Parameters for E-UTRAN Timing Advance Accuracy Test for Category NB1 UE in Standalone Mode under Enhanced Coverage

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |
| BWchannel | kHz | 200 |  |
| NPDSCH parameters: DL Reference Measurement Channel defined in A.10.2 |  | R.18 HD-FDD |  |
| NPDCCH parameters: DL Reference Measurement Channel defined in A.10.1 |  | R.30 HD-FDD |  |
| NOCNG Patterns defined in D.3 |  | NOP.3 FDD |  |
| NPBCH_RA | dB | 0 |  |
| NPBCH_RB | dB |  |  |
| NPSS_RA | dB |  |  |
| NSSS_RA | dB |  |  |
| NPDCCH_RA | dB |  |  |
| NPDCCH_RB | dB |  |  |
| NPDSCH_RA | dB |  |  |
| NPDSCH_RB | dB |  |  |
| NOCNG_RANote1 | dB |  |  |
| NOCNG_RBNote1 | dB |  |  |
| Timing Advance Command (TA) |  | 31 | 39 |
|  | dB | -12 |  |
|  | dBm/15 KHz | -88 |  |
|  | dB | -12 |  |
| IoNote2 | dBm/ 180 KHz | -76.9 |  |
| Antenna Configuration |  | 1x1 |  |
| Propagation Condition |  | AWGN |  |
| NOTE 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Io level has been derived from other parameters for information purpose. It is not a settable parameter. |  |  |  |

The UE shall apply the signalled Timing Advance value to the transmission timing at subframe n+12, where subframe n is the last subframe in the repetition period of NPDSCH in which the timing advance command is received by the UE.

The UE shall adjust the timing of its transmission with a relative accuracy better than or equal to ±13.83 × TS seconds to the signalled timing advance value compared to the timing of preceding uplink transmission. This requirement includes test tolerances.

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

<< Skipped sections >>

#### 13.4.2.1	HD-FDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Normal Coverage for Satellite Access

Editor’s note: This test case is incomplete due to the following aspects

##### -	The MU value derivation is incorrect and needs further analysis.

##### 13.4.2.1.1	Test purpose

The purpose of the test is to verify E-UTRAN Timing Advance adjustment accuracy requirements for UE category NB1 in normal coverage, defined in TS 36.133 [4] clause 7.22A.2.2, in an AWGN model.

##### 13.4.2.1.2	Test applicability

This test applies to all types of NB-IoT HD-FDD UE release 17 and forward of UE Category NB1 that supports NTN.

##### 13.4.2.1.3	Minimum conformance requirements

The timing advance is initiated from E-UTRAN with MAC message that implies and adjustment of the timing advance, see TS 36.321 [11] clause 5.2.

The UE shall adjust the timing of its transmissions with a relative accuracy better than or equal to ±13.33* TS seconds to the signalled timing advance value compared to the timing of preceding uplink transmission. The timing advance command is expressed in multiples of 16* TS and is relative to the current uplink timing.

In case of random access response, an 11-bit timing advance command, TA, indicates NTA values by index values of TA = 0, 1, 2, ..., 1536, where an amount of the time alignment is given by NTA = TA 16 according to 3GPP TS 36.213 [8] clause 16.1.2. NTA is defined in 3GPP TS 36.211 [9].

In other cases, a 6-bit timing advance command, TA, indicates adjustment of the current NTA value, NTA,old, to the new NTA value, NTA,new, by index values of TA = 0, 1, 2,..., 63, where NTA,new = NTA,old + (TA 31)16 according to 3GPP TS 36.213 [8] clause 16.1.2. Here, adjustment of NTA value by a positive or a negative amount indicates advancing or delaying the uplink transmission timing by a given amount respectively.

For UE timers, the UE shall comply with the timer accuracies according to 3GPP TS 36.133 [4] table 7.21A.2-1. The requirements are only related to the actual timing measurements internally in the UE.

The normative reference for this requirement is 3GPP TS 36.133 [4] clause 7.22A and A.13.4.2.1.

##### 13.4.2.1.4	Test description

There is one NB-IoT SAN cell (Ncell 1), which is the active cell in the test. The test consists of two successive time periods, with time duration of T1 and T2 respectively. In each time duration, timing advance commands are sent to the UE and the UE is scheduled in every uplink subframe to transmit NPUSCH, which is received by the test equipment. By measuring the reception of the NPUSCH, the transmit timing, and hence the timing advance adjustment accuracy, can be measured.

During time duration T1, the test equipment shall send one message with a Timing Advance Command MAC Control Element, as specified in Clause 6.1.3.5 in TS 36.321 [11]. The Timing Advance Command value shall be set to 31, which according to Clause 16.1.2 in TS 36.213 [8] results in zero adjustment of the Timing Advance. In this way, a reference value for the timing advance used by the UE is established.

During time duration T2, the test equipment shall send a sequence of messages with Timing Advance Command MAC Control Elements, with Timing Advance Command value specified in table 13.4.2.1.4.1-2. This value shall result in changes of the timing advance used by the UE, and the accuracy of the change shall then be measured, using the NPUSCH sent from the UE.

As specified in TS 36.133 [4] clause 7.22A.2.1, the UE adjusts its uplink timing at sub-frame n+12+k-Offset-r17+1 for a timing advance command received in sub-frame n, where sub-frame n refers to the last subframe in the repetition period in which the MAC control element containing timing advance command was received and k-Offset-r17 is specified in [5]. In addition, the UE shall not apply a TA command during an uplink repetition period. The timing advance adjustment accuracy is verified via the uplink transmission of NPUSCH carrying ACK/NACK response to the NPDSCH carrying TA command. k0 in ACK/NACK resource filed in DCI is set as 13.

Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

The UE shall be provided with the valid information about the SAN serving cells before the test.

13.4.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 13.4.2.1.4.1-1.

Table 13.4.2.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, HD-FDD duplex mode |
| 2 | NGSO, HD-FDD duplex mode |
| Note: If UE supports both NGSO and GSO, the test case Config 1 can be skipped if the UE passes test case Config 2. |  |

Test Environment: Normal, as defined in 3GPP TS 36.508 [7] clause 8.1.1.

Frequencies to be tested: According to Annex E table Annex E-4 and 3GPP TS 36.508 [7] clauses 8.1.3 and 8.1.4.2.

Channel Bandwidth to be tested: 200 KHz as defined in 3GPP TS 36.508 [7] clause 8.1.3.1.

1.	Connect the SS (node B emulator) and AWGN noise sources to the UE antenna connectors as shown in 3GPP TS 36.508 [7] Annex A, Figure A.18 using only main UE Tx/Rx antenna.

2.	The general test parameter settings are set according to Table 13.4.2.1.4.1-2.

3.	Propagation conditions are set according to Annex B clause B.0.

4.	Message contents are defined in clause 13.4.2.1.4.3.

5.	There is one cell specified in this test. Ncell 1 is the cell used for connection setup with the power level set according to Annex C.0 and C.1 for this test.

6.	UE location according to TS 36.508 [12] clause 8.4.6.1 is provided to the UE through any preconfigured means.

7.	Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 36.508 [12] clause 8.4.6.2.1 depending on the type of satellite under test. Test system shall send same SIB31-NB information during the duration of the test as defined in TS 36.508 [12] clause 8.4.6.3.1.

8.	Deactivate UE prediction of satellite trajectory through any preconfigured means.

Table 13.4.2.1.4.1-2: General Test Parameters for HD-FDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Normal Coverage for Satellite Access

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NB-IoT operational mode |  |  | Standalone |  |
| CP Length |  |  | Normal |  |
| Satellite information | Config 1 |  | SSC.1 |  |
|  | Config 2 |  | SSC.2 |  |
| Timing Advance Command (TA) value during T1 |  |  | 31 | NTA = 0 for the purpose of establishing a reference value from which the timing advance adjustment accuracy can be measured during T2 |
| Timing Advance Command (TA) value during T2 |  |  | 39 | NTA = 128 |
| Number of repetitons | NPDCCH |  | 128 |  |
|  | NPDSCH |  | 128 |  |
|  | NPUSCH |  | 32 |  |
| DRX |  |  | OFF |  |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |

13.4.2.1.4.2	Test procedure

Prior to the start of the time duration T1, the UE shall be fully synchronized to Ncell 1. The UE is scheduled in every possible uplink subframe to transmit NPUSCH, which is received by the test equipment.

- Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to 3GPP TS 36.508 [7] clause 8.1.5.

- Set the parameters according to T1 in Tables 13.4.2.1.4.1-2. Propagation conditions are set according to Annex B clause B.1. T1 starts.

- SS sends uplink scheduling information via NPDCCH DCI format N1 for C_RNTI to schedule uplink subframes according to 3GPP TS 36.508 [7] clause 8.1.3.6, Table 8.1.3.6.1.2-2.

- SS shall transmit one message with a timing advance command, TA. The timing advance command, TA, shall be set to 31 which indicate adjustment of the current NTA value. The timing advance adjustment during T1 shall be NTA = 0.

- The UE shall transmit NPUSCH (NPUSCH Format 2 is used to carry ACK/NACK) and adjust its uplink timing at the beginning of sub-frame n+12+k-Offset-r17+1 for a timing advance command TA received in sub-frame n. Sub-frame n is the last subframe in the repetition period of NPDSCH in which the timing advance command is received by the UE.

- When T1 expires, the SS shall switch the timing advance command (TA) from T1 to T2 as specified in Table 13.4.2.1.5-1. T2 starts.

- SS shall transmit a sequence of messages with timing advance command, TA. The timing advance command, TA, shall be set to 39 which indicate adjustment of the current NTA value. The timing advance adjustment during T2 shall be NTA = 128.

- The UE shall transmit NPUSCH (NPUSCH Format 2 is used to carry ACK/NACK) and adjust its uplink timing at the beginning of sub-frame n+12+k-Offset-r17+1 for a timing advance command TA received in sub-frame n. Sub-frame n is the last subframe in the repetition period of NPDSCH in which the timing advance command is received by the UE.

- The result from the NPUSCH and adjustment of the timing advance in step 8) is used to measure that the UE adjusts the timing of its transmission with a relative accuracy better than or equal to ±13.83 × TS to the signalled timing advance value compared to the timing of preceding uplink transmission.

- If the UE adjust the timing of its transmission within a relative accuracy greater than or equal to ±13.83 × TS to the signalled timing advance value compared to the timing of preceding uplink transmission, then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

- The SS shall transmit RRCConnectionRelease-NB message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

12.	After the RRC connection release, the SS:

- transmits in Ncell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE is in State 2A-NB with CP CIoT Optimisation according to 3GPP TS 36.508 [7] clause 8.1.5 (if the paging fails, switches off and on the UE and ensures the UE is in State 2A-NB with CP CIoT Optimisation according to 3GPP TS 36.508 [7] clause 8.1.5),

or

- switches off and on the UE and ensures the UE is in State 2A-NB with CP CIoT Optimisation according to 3GPP TS 36.508 [7] clause 8.1.5.

13.	Repeat step 2-12 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

13.4.2.1.4.3	Message contents

Message contents are according to 3GPP TS 36.508 [7] clause 8.1.6 and clause 8.1.4.3 using condition "Standalone" and “NTN” with the following exceptions:

Table 13.4.2.1.4.3-1: NPDCCH-ConfigDedicated-NB-DEFAULT

| Derivation Path: 3GPP TS 36.508 [7] clause 8.1.6.3, Table 8.1.6.3-3: NPDCCH-ConfigDedicated-NB-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPDCCH-ConfigDedicated-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| npdcch-NumRepetitions-r13 | r128 |  |  |
| npdcch-StartSF-USS-r13 | V2 |  |  |
| npdcch-Offset-USS-r13 | zero |  |  |
| } |  |  |  |

Table 13.4.2.1.4.3-2: NB-IoT Physical layer parameters for DCI format N1

| Parameter | Value | Value in binary |
| --- | --- | --- |
| Repetition number | NRep = 128 | “0111” |
| DCI subframe repetition number | R = 128 | “11” |

Table 13.4.2.1.4.3-3: NPUSCH-ConfigDedicated-NB-DEFAULT

| Derivation Path: 3GPP TS 36.331 clause 6.7.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPUSCH-ConfigDedicated-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| ack-NACK-NumRepetitions-r13 | r32 |  |  |
| } |  |  |  |

##### 13.4.2.1.5	Test requirement

Table 13.4.2.1.5-1: Cell specific Test Parameters for HD-FDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Normal Coverage for Satellite Access

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |
| BWchannel | KHz | 200 |  |
| NPDSCH parameters: DL Reference Measurement Channel defined in A.10.2.2-1 |  | R.18 HD-FDD |  |
| NPDCCH parameters: DL Reference Measurement Channel defined in A.10.1.2 |  | R.30 HD-FDD |  |
| NOCNG Patterns defined in D.3.3-1 |  | NOP.3 FDD |  |
| NPBCH_RA | dB | 0 |  |
| NPBCH_RB | dB |  |  |
| NPSS_RA | dB |  |  |
| NSSS_RA | dB |  |  |
| NPDCCH_RA | dB |  |  |
| NPDCCH_RB | dB |  |  |
| NPDSCH_RA | dB |  |  |
| NPDSCH_RB | dB |  |  |
| NOCNG_RANote1 | dB |  |  |
| NOCNG_RBNote1 | dB |  |  |
| Timing Advance Command (TA) |  | 31 | 39 |
|  | dB | 4 |  |
|  | dBm/15 KHz | -88 |  |
|  | dB | 4 |  |
| IoNote2 | dBm/ 180 KHz | -71.7 |  |
| Antenna Configuration |  | 1x1 |  |
| Propagation Condition |  | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Io level has been derived from other parameters for information purpose. It is not a settable parameter. |  |  |  |

The UE shall apply the signalled Timing Advance value to the transmission timing at subframe n+12+k-Offset-r17+1, where subframe n is the last subframe in the repetition period of NPDSCH in which the timing advance command is received by the UE.

The UE shall adjust the timing of its transmission with a relative accuracy better than or equal to ±13.83 × TS seconds to the signalled timing advance value compared to the timing of preceding uplink transmission. This requirement includes test tolerances.

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

#### 13.4.2.2	HD-FDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhance Coverage for Satellite Access

Editor’s note: This test case is incomplete due to the following aspects

##### -	The MU value derivation is incorrect and needs further analysis.

##### 13.4.2.2.1	Test purpose

The purpose of the test is to verify E-UTRAN Timing Advance adjustment accuracy requirements for UE category NB1 in enhanced coverage, defined in TS 36.133 [4] clause 7.22A.2.2, in an AWGN model.

##### 13.4.2.2.2	Test applicability

This test applies to all types of NB-IoT HD-FDD UE release 17 and forward of UE Category NB1 that supports NTN.

##### 13.4.2.2.3	Minimum conformance requirements

The timing advance is initiated from E-UTRAN with MAC message that implies and adjustment of the timing advance, see TS 36.321 [11] clause 5.2.

The UE shall adjust the timing of its transmissions with a relative accuracy better than or equal to ±13.33* TS seconds to the signalled timing advance value compared to the timing of preceding uplink transmission. The timing advance command is expressed in multiples of 16* TS and is relative to the current uplink timing.

In case of random access response, an 11-bit timing advance command, TA, indicates NTA values by index values of TA = 0, 1, 2, ..., 1536, where an amount of the time alignment is given by NTA = TA 16 according to 3GPP TS 36.213 [8] clause 16.1.2. NTA is defined in 3GPP TS 36.211 [9].

In other cases, a 6-bit timing advance command, TA, indicates adjustment of the current NTA value, NTA,old, to the new NTA value, NTA,new, by index values of TA = 0, 1, 2,..., 63, where NTA,new = NTA,old + (TA 31)16 according to 3GPP TS 36.213 [8] clause 16.1.2. Here, adjustment of NTA value by a positive or a negative amount indicates advancing or delaying the uplink transmission timing by a given amount respectively.

For UE timers, the UE shall comply with the timer accuracies according to 3GPP TS 36.133 [4] table 7.21A.2-1. The requirements are only related to the actual timing measurements internally in the UE.

The normative reference for this requirement is 3GPP TS 36.133 [4] clause 7.22A and A.13.4.2.2.

##### 13.4.2.2.4	Test description

There is one NB-IoT SAN cell (Ncell 1), which is the active cell in the test. The test consists of two successive time periods, with time duration of T1 and T2 respectively. In each time duration, timing advance commands are sent to the UE and the UE is scheduled in every uplink subframe to transmit NPUSCH, which is received by the test equipment. By measuring the reception of the NPUSCH, the transmit timing, and hence the timing advance adjustment accuracy, can be measured.

During time duration T1, the test equipment shall send one message with a Timing Advance Command MAC Control Element, as specified in Clause 6.1.3.5 in TS 36.321 [11]. The Timing Advance Command value shall be set to 31, which according to Clause 16.1.2 in TS 36.213 [8] results in zero adjustment of the Timing Advance. In this way, a reference value for the timing advance used by the UE is established.

During time duration T2, the test equipment shall send a sequence of messages with Timing Advance Command MAC Control Elements, with Timing Advance Command value specified in table 13.4.2.1.4.1-2. This value shall result in changes of the timing advance used by the UE, and the accuracy of the change shall then be measured, using the NPUSCH sent from the UE.

As specified in TS 36.133 [4] clause 7.22A.2.1, the UE adjusts its uplink timing at sub-frame n+12+k-Offset-r17+1 for a timing advance command received in sub-frame n, where sub-frame n refers to the last subframe in the repetition period in which the MAC control element containing timing advance command was received and k-Offset-r17 is specified in [5]. In addition, the UE shall not apply a TA command during an uplink repetition period. The timing advance adjustment accuracy is verified via the uplink transmission of NPUSCH carrying ACK/NACK response to the NPDSCH carrying TA command. k0 in ACK/NACK resource filed in DCI is set as 13.

Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

The UE shall be provided with the valid information about the SAN serving cells before the test.

13.4.2.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 13.4.2.2.4.1-1.

Table 13.4.2.2.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, HD-FDD duplex mode |
| 2 | NGSO, HD-FDD duplex mode |
| Note: If UE supports both NGSO and GSO, the test case Config 1 can be skipped if the UE passes test case Config 2. |  |

Test Environment: Normal, as defined in 3GPP TS 36.508 [7] clause 8.1.1.

Frequencies to be tested: According to Annex E table Annex E-4 and 3GPP TS 36.508 [7] clauses 8.1.3 and 8.1.4.2.

Channel Bandwidth to be tested: 200 KHz as defined in 3GPP TS 36.508 [7] clause 8.1.3.1.

1.	Connect the SS (node B emulator) and AWGN noise sources to the UE antenna connectors as shown in 3GPP TS 36.508 [7] Annex A, Figure A.18 using only main UE Tx/Rx antenna.

2.	The general test parameter settings are set according to Table 13.4.2.2.4.1-2.

3.	Propagation conditions are set according to Annex B clause B.0.

4.	Message contents are defined in clause 13.4.2.2.4.3.

5.	There is one cell specified in this test. Ncell 1 is the cell used for connection setup with the power level set according to Annex C.0 and C.1 for this test.

6.	UE location according to TS 36.508 [12] clause 8.4.6.1 is provided to the UE through any preconfigured means.

7.	Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 36.508 [12] clause 8.4.6.2.1 depending on the type of satellite under test. Test system shall send same SIB31-NB information during the duration of the test as defined in TS 36.508 [12] clause 8.4.6.3.1.

8.	Deactivate UE prediction of satellite trajectory through any preconfigured means.

Table 13.4.2.2.4.1-2: General Test Parameters for HD-FDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhance Coverage for Satellite Access

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NB-IoT operational mode |  |  | Standalone |  |
| CP Length |  |  | Normal |  |
| Satellite information | Config 1 |  | SSC.1 | GSO |
|  | Config 2 |  | SSC.2 | NGSO |
| Timing Advance Command (TA) value during T1 |  |  | 31 | NTA = 0 for the purpose of establishing a reference value from which the timing advance adjustment accuracy can be measured during T2 |
| Timing Advance Command (TA) value during T2 |  |  | 39 | NTA = 128 |
| Number of repetitons | NPDCCH |  | 128 |  |
|  | NPDSCH |  | 128 |  |
|  | NPUSCH |  | 32 |  |
| DRX |  |  | OFF |  |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |

13.4.2.2.4.2	Test procedure

Prior to the start of the time duration T1, the UE shall be fully synchronized to Ncell 1. The UE is scheduled in every possible uplink subframe to transmit NPUSCH, which is received by the test equipment.

1.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to 3GPP TS 36.508 [7] clause 8.1.5.

2.	Set the parameters according to T1 in Tables 13.4.2.2.4.1-2. Propagation conditions are set according to Annex B clause B.1. T1 starts.

3.	SS sends uplink scheduling information via NPDCCH DCI format N1 for C_RNTI to schedule uplink subframes according to 3GPP TS 36.508 [7] clause 8.1.3.6, Table 8.1.3.6.1.2-2.

4.	SS shall transmit one message with a timing advance command, TA. The timing advance command, TA, shall be set to 31 which indicate adjustment of the current NTA value. The timing advance adjustment during T1 shall be NTA = 0.

5.	The UE shall transmit NPUSCH (NPUSCH Format 2 is used to carry ACK/NACK) and adjust its uplink timing at the beginning of sub-frame n+12+k-Offset-r17+1 for a timing advance command TA received in sub-frame n. Sub-frame n is the last subframe in the repetition period of NPDSCH in which the timing advance command is received by the UE.

6.	When T1 expires, the SS shall switch the timing advance command (TA) from T1 to T2 as specified in Table 13.4.2.2.5-1. T2 starts.

7.	SS shall transmit a sequence of messages with timing advance command, TA. The timing advance command, TA, shall be set to 39 which indicate adjustment of the current NTA value. The timing advance adjustment during T2 shall be NTA = 128.

8.	The UE shall transmit NPUSCH (NPUSCH Format 2 is used to carry ACK/NACK) and adjust its uplink timing at the beginning of sub-frame n+12+k-Offset-r17+1 for a timing advance command TA received in sub-frame n. Sub-frame n is the last subframe in the repetition period of NPDSCH in which the timing advance command is received by the UE.

9.	The result from the NPUSCH and adjustment of the timing advance in step 8) is used to measure that the UE adjusts the timing of its transmission with a relative accuracy better than or equal to ±13.83 × TS to the signalled timing advance value compared to the timing of preceding uplink transmission.

10.	If the UE adjust the timing of its transmission within a relative accuracy greater than or equal to ±13.83  × TS to the signalled timing advance value compared to the timing of preceding uplink transmission, then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

11.	The SS shall transmit RRCConnectionRelease-NB message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

12.	After the RRC connection release, the SS:

- transmits in Ncell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE is in State 2A-NB with CP CIoT Optimisation according to 3GPP TS 36.508 [7] clause 8.1.5 (if the paging fails, switches off and on the UE and ensures the UE is in State 2A-NB with CP CIoT Optimisation according to 3GPP TS 36.508 [7] clause 8.1.5),

or

- switches off and on the UE and ensures the UE is in State 2A-NB with CP CIoT Optimisation according to 3GPP TS 36.508 [7] clause 8.1.5.

13.	Repeat step 2-12 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

13.4.2.2.4.3	Message contents

Message contents are according to 3GPP TS 36.508 [7] clause 8.1.6 and clause 8.1.4.3 using condition "Standalone", "RRM_NTN_Enh" and "NTN" with the following exceptions:

Table 13.4.2.2.4.3-1: NPDCCH-ConfigDedicated-NB-DEFAULT

| Derivation Path: 3GPP TS 36.508 [7] clause 8.1.6.3, Table 8.1.6.3-3: NPDCCH-ConfigDedicated-NB-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPDCCH-ConfigDedicated-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| npdcch-NumRepetitions-r13 | r128 |  |  |
| npdcch-StartSF-USS-r13 | V2 |  |  |
| npdcch-Offset-USS-r13 | zero |  |  |
| } |  |  |  |

Table 13.4.2.2.4.3-2: NB-IoT Physical layer parameters for DCI format N1

| Parameter | Value | Value in binary |
| --- | --- | --- |
| Repetition number | NRep = 128 | “0111” |
| DCI subframe repetition number | R = 128 | “11” |

Table 13.4.2.2.4.3-3: NPUSCH-ConfigDedicated-NB-DEFAULT

| Derivation Path: 3GPP TS 36.331 clause 6.7.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPUSCH-ConfigDedicated-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| ack-NACK-NumRepetitions-r13 | r32 |  |  |
| } |  |  |  |

##### 13.4.2.2.5	Test requirement

Table 13.4.2.2.5-1: Cell specific Test Parameters for HD-FDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhance Coverage for Satellite Access

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |
| BWchannel | KHz | 200 |  |
| NPDSCH parameters: DL Reference Measurement Channel defined in A.10.2.2-1 |  | R.18 HD-FDD |  |
| NPDCCH parameters: DL Reference Measurement Channel defined in A.10.1.2 |  | R.30 HD-FDD |  |
| NOCNG Patterns defined in D.3.3-1 |  | NOP.3 FDD |  |
| NPBCH_RA | dB | 0 |  |
| NPBCH_RB | dB |  |  |
| NPSS_RA | dB |  |  |
| NSSS_RA | dB |  |  |
| NPDCCH_RA | dB |  |  |
| NPDCCH_RB | dB |  |  |
| NPDSCH_RA | dB |  |  |
| NPDSCH_RB | dB |  |  |
| NOCNG_RANote1 | dB |  |  |
| NOCNG_RBNote1 | dB |  |  |
| Timing Advance Command (TA) |  | 31 | 39 |
|  | dB | -12 |  |
|  | dBm/15 KHz | -88 |  |
|  | dB | -12 |  |
| IoNote2 | dBm/ 180 KHz | -76.9 |  |
| Antenna Configuration |  | 1x1 |  |
| Propagation Condition |  | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Io level has been derived from other parameters for information purpose. It is not a settable parameter. |  |  |  |

The UE shall apply the signalled Timing Advance value to the transmission timing at subframe n+12+k-Offset-r17+1, where subframe n is the last subframe in the repetition period of NPDSCH in which the timing advance command is received by the UE.

The UE shall adjust the timing of its transmission with a relative accuracy better than or equal to ±13.83 × TS seconds to the signalled timing advance value compared to the timing of preceding uplink transmission. This requirement includes test tolerances.

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

<< End of Changes >>