3GPP TSG RAN5 Meeting #109	 	R5-255705r1

Dallas, Texas, USA, 17th - 21st November 2025

Source:		Rohde & Schwarz

Title:	On the MU for FR2 multi-Rx RRM test cases

Agenda Item:		5.3.26.10

Document for:	Discussion and Endorsement

# Introduction

In the last meeting the discussion of the MU for FR2 multi-Rx RRM test cases started [1, 2] and the following action point has been created:

| Action ID | sWG | Action | Responsible | Relevant Tdoc | Deadline | Status |
| --- | --- | --- | --- | --- | --- | --- |
| AP#108.21 | RF | Define MU terms, ‘DUT repositioning’ ,’ influence of spherical coverage grid’ , ‘mismatch’ , ‘amplifier uncertainty’ , to define ref MU for 2AoA and 3AoA FR2 MultiRx RRM tests , as a working assumption until the availability of commercial devices supporting these RRM tests. | Keysight, R&S, Anritsu , Apple, Qualcomm, | R5-255346, R5-254045 | RAN5#110 | Open |

In this contribution, we present the results of our MU assessment for mismatch and propose to reuse the VNA uncertainty of the Rx spherical coverage test case and to specify the value of systematic error related to EIS spherical coverage.

# Discussion

## Mismatch Uncertainty

RRM multi-Rx test cases require the test system to provide simultaneous signals on two AoAs which increases the complexity of the conducted part of the test system. Therefore, the uncertainty related to the conducted part needs to be addressed.

We have assessed the mismatch uncertainty of a possible test system implementation and propose to apply a value of 1.7 dB for the mismatch uncertainty in stage 1 for enhanced IFF test systems in the frequency range from 23.45 GHz to 40.8 GHz.

Proposal 1: We propose an intermediate value of 1.7 dB in [] for the mismatch uncertainty in stage 2 for enhanced IFF test systems in the frequency range from 23.45 GHz to 40.8 GHz for multi-Rx RRM TCs.

## VNA uncertainty

For the VNA uncertainty, we propose to reuse the value of the EIS spherical coverage RF test case.

Proposal 2: Reuse the value of the VNA uncertainty in Table B.19.2-2 of TR 38.903 [3] for enhanced IFF test systems for multi-Rx RRM TCs.

## Systematic error related to EIS spherical coverage

Multi-Rx RRM test cases use a spherical coverage type of requirement for the side conditions. In the associated test procedure, an EIS search is performed with a minimum step size of 0.2 dB. This value needs to be considered as systematic uncertainty. In alignment with the uncertainty budget of EIS spherical coverage in Table B.19.2-2 of TR 38.903 [3], we propose a value of 0.2 dB for the systematic error related to EIS spherical coverage.

Proposal 3: We propose using "DL power step size, 0.2 dB" as value for the systematic error related to EIS spherical coverage for the MU of multi-Rx RRM TCs.

Accompanying CR in Ref. [4] documents the MU assessment in TR 38.903 [3].

# Conclusion

In summary, we have assessed the uncertainty of some MU contributors and make the following proposals:

Proposal 1: We propose an intermediate value of 1.7 dB  in [] for the mismatch uncertainty in stage 2 for enhanced IFF test systems in the frequency range from 23.45 GHz to 40.8 GHz for multi-Rx RRM TCs.

Proposal 2: Reuse the value of the VNA uncertainty in Table B.19.2-2 of TR 38.903 [3] for enhanced IFF test systems for multi-Rx RRM TCs.

Proposal 3: We propose using "DL power step size, 0.2 dB" as value for the systematic error related to EIS spherical coverage for the MU of multi-Rx RRM TCs.

# References

- R5-255346, Discussion on multi-Rx measurement uncertainty, QUALCOMM Europe Inc. - Spain, 3GPP TSG RAN WG 5 Meeting #108, August 2025

- R5-254044, On Re-Positioning Concept for 3 AoA FR2 RRM TCs, Keysight Technologies UK Ltd, 3GPP TSG RAN WG 5 Meeting #108, August 2025

- TR 38.903, V19.0.0, 3GPP TSG RAN WG5, September 2025

- R5-255706, Documentation of MU for FR2 multi-Rx RRM test cases, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #109, November 2025