3GPP TSG-RAN5 Meeting #107 – TTCN Sidebar	R5w250211

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

Agenda Item:	TTCN Sidebar

Source:	MCC TF160

Title:	Rel-18 LTE Band 106: Further Test Model and ASP updates

Document for:	Endorsement

# 1	Introduction

Rel-18 LTE_900MHz_US-UEConTest WI introduced new frequency bands defined with CBW 3MHz. The initial test model was presented in R5w250109, further investigation was done in R5w250205.

The intention of this document is to cover BL/CE UE and DCI Format 2A. No ASP change is needed.

EPDCCH is configured in only 2 test cases still verifiable:

| 7.1.3.14 | Correct handling of DL assignment / Dynamic case / EPDCCH | verifiable |
| --- | --- | --- |
| 7.1.3.15 | Correct handling of DL assignment / Semi-persistent case / EPDCCH | verifiable |

It is proposed to analyse the impact of 3MHz later when the test cases are verified.

# 2	Generic solution for BL/CE UE

According to TS 36.101 clause 5.5E, band 106 is applicable to UE category M1 in both full-duplex FDD and half-duplex FDD mode:

“

UE category M1 and M2 is designed to operate in the E-UTRA operating bands 1, 2, 3, 4, 5, 7, 8, 11, 12, 13, 14, 18, 19, 20, 21, 24, 25, 26, 27, 28, 31, 54, 66, 71, 72, 73, 74, 85, 87, 88, 106, 111 in both half duplex FDD mode and full-duplex FDD mode, and in bands 39, 40, 41, 42, 43 and 48 in TDD mode. The E-UTRA bands are defined in Table 5.5-1.

“

According to TS 36.211 clause 6.2.7, the 2 Narrow Bands for band 106 are defined in PRBs 1-6 and 8-13.

For BL/CE UE only DCI combination 1 is applied, the Physical resource allocation bitmap can be done as follows:

Physical resource allocation bitmap or DCI combination 1 (3 MHz)

| NPRB | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PBCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SIB1-BR |  | NB 0 |  |  |  |  |  |  | NB 1 |  |  |  |  |  |  |
| SIx-BR |  | NB 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UE-Dedicated |  | NB 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MPDCCH/P-RNTI |  | NB 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MPDCCH |  |  |  |  |  |  |  |  | NB 1 |  |  |  |  |  |  |

NOTE:	The allocation assumes that SIB1/SIB1-BR/SIx/SIx-BR are not transmitted in the same subframe.

It is unavoidable to use NB 0 for both SIB1-BR and UE-dedicated. This means that UE-dedicated transmission cannot be used in even SFN subframe#4 in even physical layer cell Id and in odd SFN subframe#4 in odd physical layer cell Id.

UE-dedicated transmission cannot be done in subframe transmitting PBCH: subframe#0 in every SFN.

UE-dedicated transmission cannot be done in subframe transmitting SIx-BR.

Narrow band allocation per cell (3 MHz)

| Physical cell ID mod 2 | SIB1-BR set | SIx-BR | UE-dedicated | MPDCCH | MPDCCH associated with the P-RNTI |
| --- | --- | --- | --- | --- | --- |
| 0 | NB 0, NB 1 | NB 0 | NB 0 | NB 1 | NB 0 |
| 1 | NB 1, NB 0 | NB 0 | NB 0 | NB 1 | NB 0 |

Timing considerations:

- PBCH is transmitted in subframe#0 in every SFN on PRBs overlapping NB0 and NB1. There is no retransmission of PBCH.

- SIB1-BR is transmitted in subframe#4 in

  - even SFN in cells with even physical layer cell ID

  - odd SFN in cells with odd physical layer cell ID

- As per TS 36.213 clause 7.1.11, the DL data will be 2 subframes after the subframe containing DL Grant on MPDCCH

- As per TS 36.213 clause 8.0, the UL data will be 4 subframes after the subframe containing UL Grant on MPDCCH

- As per TS 36.213 clause 8.0, a BL/CE UE is not expected to be configured with simultaneous PUSCH and PUCCH transmission

- SIx-BR is transmitted from the first radio frame of the SI window, i.e. in even SFN (as per TS 36.331)

- Paging is transmitted in subframe#9

It is proposed to specify the following offset for sysinfo scheduling:

SI SubframeOffset values

| Scheduling Information No. Acc to TS 36.508 [3], clause 4.4.3.1.2 | subframeOffset (FDD) | subframeOffset (FDD DL bandwidth = 3 MHz) | subframeOffset (TDD DL bandwidth > 5 MHz) | subframeOffset (TDD DL bandwidth = 5 MHz) |
| --- | --- | --- | --- | --- |
| SI1 | 1 | 0 | 0 | 10 |
| SI2 | 1 | 0 | 0 | 10 |
| SI3 | 3 | 0 | 0 | 10 |
| SI4 | 3 | 0 | 0 | 10 |
| SI1-BR | 0 | 7 | 0 | 0 |
| SI2-BR | 0 | 7 | 0 | 0 |
| SI3-BR | 0 | 7 | 0 | 0 |
| SI4-BR | 0 | 7 | 0 | 0 |
| NOTE: The subframe offsets for SIx/SIx-BR of 0 and 10 are suitable for all TDD uplink-downlink configurations. |  |  |  |  |

In TTCN the following timing will be applied for explicit scheduling (with activation time) for full-duplex FDD UE.

Default data scheduling for 3 MHz full-duplex FDD Bandwidth

| Subframe |  | NB | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL |  |  |  |  |  |  |  |  |  |  |  |  |
| Even physical layer cell IDs | Even SFN | NB0 | SIx / PBCH |  |  |  | SIB1-BR | SIB1 |  | SIx-BR | PDSCH | Paging |
|  |  | NB1 |  |  |  |  | SIB1-BR |  | MPDCCH DL grant/DL HARQ |  | MPDCCH UL grant |  |
|  | Odd SFN | NB0 | PBCH |  |  |  |  | SIB1 |  |  | PDSCH | Paging |
|  |  | NB1 |  |  |  |  |  |  | MPDCCH DL grant/DL HARQ |  | MPDCCH UL grant |  |
| Even physical layer cell IDs | Even SFN | NB0 | SIx / PBCH |  |  |  |  | SIB1 |  | SIx-BR | PDSCH | Paging |
|  |  | NB1 |  |  |  |  |  |  | MPDCCH DL grant/DL HARQ |  | MPDCCH UL grant |  |
|  | Odd SFN | NB0 | PBCH |  |  |  | SIB1-BR | SIB1 |  |  | PDSCH | Paging |
|  |  | NB1 |  |  |  |  | SIB1-BR |  | MPDCCH DL grant/DL HARQ |  | MPDCCH UL grant |  |
| UL |  |  |  |  |  |  |  |  |  |  |  |  |
| SFN+1 |  |  |  |  | PUSCH/HARQ |  |  |  |  |  |  |  |
| SFN+2 |  |  |  |  | PUSCH/HARQ |  |  |  |  |  |  |  |

Note: this scheduling to be applied in TTCN is aligned with half-duplex to make TTCN implementation easier.

For half-duplex FDD UE, the following scheduling is proposed with the assumptions:

-	SS will be configured per cell by providing UE category info and FDD half-duplex operation.

-	When explicit timing info is provided in any DL transmission, the SS shall ignore the UE capability and shall follow the explicit scheduling. It will be TTCN responsibility to schedule UL/DL transmissions based on UE capability. Paging is always explicitly scheduled and hence SS follows explicit scheduling for paging.

-	the same SI scheduling as full-duplex UE is applied.

-	BL/CE UE acquires sysinfo in RRC_IDLE. In RRC_CONNECTED BL/CE UE acquires MIB/SIB1-BR only when T311 is running.

-	For non-explicitly scheduled new DL transmissions/ new UL grant allocations are as per below:

Scheduling pattern for type B half-duplex FDD BL/CE UE (non-explicitly scheduled) DL Bandwidth = 3 MHz

| SFN\ SubFrame | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X (any) | MIB | Blank | New UL Data/ HARQ | Blank | SIB1-BR | Blank | DL HARQ/DL Grant | SI-BR | New DL Data/UL Grant | Blank |

-	If HARQ retransmissions are required:

-	DL retransmissions being asynchronous in nature shall be scheduled by the SS as per above schedule (i.e. subframe 8).

-	UL retransmissions being synchronous will not be limited to subframe 2. i.e. the first UL retransmission will be in frame X+1, subframe 0, second retransmission in frame X+1, subframe 8 and so on.

-	HARQ retransmissions shall be prioritised over new transmissions.

# 3	Generic solution for DCI format 2A

DCI format 2A is applied to DL Assignment for Open Loop MIMO.

The TBS xls sheets DCI-2A-RA0-ExplicitConfig and DCI-2A-RA1-ExplicitConfig shall be extended for 3 MHz, this is shown in the attached xls file. The changes are shown in blue.

No changes are needed in TS 36.523-3 and no ASP changes are needed.

# 4	Proposal for in 36.523-3

See R5-253974