3GPP TSG-S4 Meeting MBS ad hoc post #133-e	S4-251598

# Electronic, Online, 3rd–5th September 2025	revision of S4aI250152

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.512 | CR | 0097 | rev | 2 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [AMD_PRO-MED] Consolidated additions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BBC, Dolby, Samsung, Huawei, Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AMD_PRO-MED |  |  |  |  |  | Date: |  |  | 2025-08-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Satisfy the objectives of the cited Work Item. |  |  |  |  |  |  |  |  |
| Summary of change: |  | CR0086 [S4aI250159]: “WT2: Technology-independent feature updates to enable media delivery from multiple service locations” - Required technology-independent feature updates to enable media delivery from multiple service locations and service chaining of the 5GMS AS. - Updates to supporting procedures aligned with the above. CR0087 [S4aI250150]: “WT3: Multi-access media delivery” - Client API for configuring delivery protocol at reference point M4 and preference for multipath transport. - Corresponding Service URL parameters. - Procedures to motivate the above. CR0089 [S4aI250146]: “WT1: CMCD provisioning (M1), reporting (M3) and exposure (R5/R6)” - Procedures for provisioning CMCD-based in-band client data reporting using metrics reporting configuration APIs. - Procedures and APIs for configuring CMCD-based in-band client data reporting via Media Session Handling API. - Client APIs for configuring CMCD-based in-band client data reporting, including 3GPP Service URL. - Controlled vocabularies to support the above. - Mapping of CMCD-based in-band client data reporting into DASH. CR0090 [S4-251485]: “WT8: Support of Improved QoS for media streaming services” - Cross-referencing Policy Template provisioning procedures in TS 26.510. - Specify use of ECN marking for L4S and QoS monitoring in the media plane. - Basic capability exposure in DASH Media Player client API at reference point M7d/M11d. CR0091 [S4aI250160]: “WT2: Mapping of CMMF into 5GMS System (annexes)” - New annex specifying the use of Coded Multisource Media Format (CMMF) in the 5GMS System. - Illustrative example Media Entry Point documents. - Illustrative example provisioning and Content Hosting Configurations. CR0093 [S4-251507]: “WT4: DRM support in DASH” - Requirements for content protection signalling at reference point M4 in the presentation manifest. - Requirements for a Content Preparation Template for DRM, aligning with CPIX structures. CR0099 [S4aI250171]: “WT2: Multiple service locations in DASH” - Mapping of multiple service locations into DASH, specifically: content preparation, client-side switching, Content Steering and SAND4M. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Objectives of the Work Item not satisfied. |  |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.1, 3.3, 4.2, 4.3.1, 4.3.4.1, 4.3.5.1, 4.3.6.1, 4.3.7.1, 4.5.4.1, 4.5.5.1, 4.5.6 (new), 4.5.7 (new), 4.5.8 (new), 4.5.9 (new), 4.5.10 (new), 4.6.0 (new), 4.6.1, 4.6.2, 4.6.3 (new), 4.7.2.1, 4.7.3, 4.7.5, 4.9, 4.9.1A (new), 4.9.2, 4.9.3, 4.10A, 4.10B, 5.1, 5.2, 5.6.1 (new), 5.6.2 (new), 5.9, 5.9.1 (new), 5.9.2 (new), 6.0.2.2, 6.2.1.2, 7.4.1, 7.6.1, 7.6.4.1, 7.6.4.2, 7.6.4.3, 7.6.4.4, 7.6.4.5, 7.6.4.6 (new), 7.6.4.7 (new), 7.6.4.8 (new), 7.8.1, 8, 8.1, 8.2, 8.3, 8.4, 8.5, 8.6, 8.7, 8.8 (new), 9.4.3, 9.5.3, 10.1, 10.1A, 10.2, 10.3A (new), 10.4.1, 10.4.2, 10.4.3, 11, 11.1, 11.2.1, 11.3.3.1, 11.3.3.2, 11.4.1, 11.4.2, 11.4.3.1 (new), 11.4.3.2 (new), 11.4.3.3 (new), 12.4, 13.2.1, 13.2.4, 13.2.5, 13.2.6, 18.3.2, 18.3.3, B.1.2, B.1.3, B.2.1, B.2.2, B.3 (new), B.4 (new), C.3.0, C.3A.0, C.4.0, C.6 (new), E.2, E.2.2, E.2.3 (new), E.2.4 (new), E.2.5 (new) G (new), H (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | Y |  | Other core specifications |  |  |  | TS 26.510 CR0032 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | CR0097 [S4-251229]: Submitted for WG endorsement. CR0097r1 [S4aI250152]: Submitted for SWG agreement. CR0097r2 SA4 version of the SWG AH agreement |  |  |  |  |  |  |  |  |

# Code changes for delivery from multiple service locations

The code changes associated with this Change Request are available for review at the following URL on 3GPP Forge:

https://forge.3gpp.org/rep/sa4/amd-pro-med/-/merge_requests/5

https://forge.3gpp.org/rep/sa4/amd-pro-med/-/merge_requests/5/diffs?commit_id=a3dca77fb7b8f84055d5487b93ce8323be0998ed

The proposed changes are reproduced below for posterity.

## TS26512_Mas_Configuration_ContentHosting.yaml

---a/TS26512_Mas_Configuration_ContentHosting.yaml
+++b/TS26512_Mas_Configuration_ContentHosting.yaml

@@ -1,7 +1,7 @@

1	1		openapi: 3.0.0

2	2		info:

3	3		  title: Mas_Configuration_ContentHosting

4		-	  version: 1.0.2

4	+	  version: 1.1.0

5	5		  description: |

6	6		    5GMS AS Configuration API: Content Hosting

7	7		    © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

@@ -12,7 +12,7 @@ tags:

12	12		    description: '5G Media Streaming: Application Server Configuration (M3) APIs: Content Hosting'

13	13

14	14		externalDocs:

15		-	  description: 'TS 26.512 V18.6.0; 5G Media Streaming (5GMS); Protocols'

15	+	  description: 'TS 26.512 V19.0.0; 5G Media Streaming (5GMS); Protocols'

16	16		  url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/'

17	17

18	18		servers:

@@ -335,16 +335,10 @@ components:

335	335		      allOf:

336	336		        - $ref: 'TS26510_Maf_Provisioning_ContentHosting.yaml#/components/schemas/BaseDistributionConfiguration'

337	337		        - type: object

338		-	          required:

339		-	            - canonicalDomainName

340		-	            - baseURL

341	338		          properties:

342	339		            canonicalDomainName:

343	340		              type: string

344		-	              description: 'Default Fully-Qualified Domain Name assigned by the Media AF for use at reference point M4.'

345		-	            baseURL:

346		-	              allOf:

347		-	                - $ref: 'TS26510_CommonData.yaml#/components/schemas/AbsoluteUrl'

341	+	              description: 'Default Fully-Qualified Domain Name assigned by the Media AF for use at reference point M4 and M10.'

348	342

349	343		    # Schema for the resource itself

350	344		    ApplicationServerContentHostingConfiguration:

## TS26512_Mas_Configuration_ContentPublishing.yaml

---a/TS26512_Mas_Configuration_ContentPublishing.yaml
+++b/TS26512_Mas_Configuration_ContentPublishing.yaml

@@ -1,7 +1,7 @@

1	1		openapi: 3.0.0

2	2		info:

3	3		  title: Mas_Configuration_ContentPublishing

4		-	  version: 1.0.2

4	+	  version: 1.1.0

5	5		  description: |

6	6		    5GMS AS Configuration API: Content Publishing

7	7		    © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

@@ -12,7 +12,7 @@ tags:

12	12		    description: '5G Media Streaming: Application Server Configuration (M3) APIs: Content Publishing'

13	13

14	14		externalDocs:

15		-	  description: 'TS 26.512 V18.6.0; 5G Media Streaming (5GMS); Protocols'

15	+	  description: 'TS 26.512 V19.0.0; 5G Media Streaming (5GMS); Protocols'

16	16		  url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/'

17	17

18	18		servers:

@@ -334,17 +334,6 @@ components:

334	334		      description: 'A content contribution configuration used to configure a Media AS.'

335	335		      allOf:

336	336		        - $ref: 'TS26510_Maf_Provisioning_ContentPublishing.yaml#/components/schemas/BaseContributionConfiguration'

337		-	        - type: object

338		-	          required:

339		-	            - canonicalDomainName

340		-	            - baseURL

341		-	          properties:

342		-	            canonicalDomainName:

343		-	              type: string

344		-	              description: 'Default Fully-Qualified Domain Name assigned by the Media AF for use at reference point M4.'

345		-	            baseURL:

346		-	              allOf:

347		-	                - $ref: 'TS26510_CommonData.yaml#/components/schemas/AbsoluteUrl'

348	337

349	338		    # Schema for the resource itself

350	339		    ApplicationServerContentPublishingConfiguration:

# Code changes for CMCD-based in-band client data reporting

The code changes associated with this Change Request are available for review at the following URL on 3GPP Forge:

https://forge.3gpp.org/rep/sa4/amd-pro-med/-/merge_requests/1/diffs?commit_id=50c85a84e0741c99e377028fb99001a2e854a231

The proposed changes are reproduced below for posterity.

## TS26512_ClientData.yaml

---/dev/null
+++b/TS26512_ClientData.yaml

@@ -0,0 +1,141 @@

1	+	openapi: 3.0.0

2	+	info:

3	+	  title: '5G Media Streaming: Client data syntax for QoE metrics reporting'

4	+	  version: 1.0.0

5	+	  description: |

6	+	    5GMS client data syntax for QoE metrics reporting

7	+	    © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

8	+	    All rights reserved.

9	+

10	+	tags:

11	+	  - name: '5GMS client data syntax for QoE metrics reporting'

12	+	    description: '5G Media Streaming: Client data syntax for use in QoE metrics reporting envelope'

13	+

14	+	externalDocs:

15	+	  description: 'TS 26.512 V19.0.0; 5G Media Streaming (5GMS); Protocols'

16	+	  url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/'

17	+

18	+	paths: {}

19	+

20	+	components:

21	+	  schemas:

22	+	    MediaStreamingClientData:

23	+	      description: 'Envelope for client data pertaining to the 5G Media Streaming System.'

24	+	      type: object

25	+	      required:

26	+	        - requestUrl

27	+	      anyOf:

28	+	        - required: [cmcdSessionInfo]

29	+	        - required: [cmcdObjectInfo]

30	+	        - required: [cmcdRequestInfo]

31	+	        - required: [cmcdStatusInfo]

32	+	      properties:

33	+	        requestUrl:

34	+	          $ref: 'TS26510_CommonData.yaml#/components/schemas/AbsoluteUrl'

35	+	        cmcdSessionInfo:

36	+	          $ref: '#/components/schemas/CmcdSessionInfo'

37	+	        cmcdObjectInfo:

38	+	          $ref: '#/components/schemas/CmcdObjectInfo'

39	+	        cmcdRequestInfo:

40	+	          $ref: '#/components/schemas/CmcdRequestInfo'

41	+	        cmcdStatusInfo:

42	+	          $ref: '#/components/schemas/CmcdStatusInfo'

43	+

44	+	    ##########################################

45	+	    # Common Media Client Data per CTA-5004 V1

46	+	    ##########################################

47	+	    CmcdSessionInfo:

48	+	      description: 'An object containing session-scope CMCD keys'

49	+	      type: object

50	+	      required:

51	+	        - sid

52	+	      properties:

53	+	        v:

54	+	          description: 'CMCD version'

55	+	          type: integer

56	+	          minimum: 1

57	+	        sid:

58	+	          description: 'Session identifier'

59	+	          type: string

60	+	        cid:

61	+	          description: 'Content identifier'

62	+	          type: string

63	+	        st:

64	+	          allOf:

65	+	            - description: 'Stream type'

66	+	            - $ref: '#/components/schemas/CmcdStreamType'

67	+	        sf:

68	+	          allOf:

69	+	            - description: 'Streaming format'

70	+	            - $ref: '#/components/schemas/CmcdStreamingFormat'

71	+	        pr:

72	+	          description: 'Playback rate'

73	+	          type: number

74	+

75	+	    CmcdObjectInfo:

76	+	      description: 'An object containing object-scope CMCD keys'

77	+	      type: object

78	+	      properties:

79	+	        ot:

80	+	          allOf:

81	+	            - description: 'Object type'

82	+	            - $ref: '#/components/schemas/CmcdObjectType'

83	+	        d:

84	+	          description: 'Object duration (milliseconds)'

85	+	          type: integer

86	+	        br:

87	+	          description: 'Encoded bit rate (kilobits per second)'

88	+	          type: integer

89	+	        tb:

90	+	          description: 'Top bit rate (kilobits per second)'

91	+	          type: integer

92	+

93	+	    CmcdRequestInfo:

94	+	      description: 'An object containing request-scope CMCD keys'

95	+	      type: object

96	+	      properties:

97	+	        su:

98	+	          description: 'Start-up'

99	+	          type: boolean

100	+	        mtp:

101	+	          description: 'Measured throughput (kilobits per second)'

102	+	          type: integer

103	+	        dl:

104	+	          description: 'Deadline (milliseconds)'

105	+	          type: integer

106	+	        bl:

107	+	          description: 'Buffer length (milliseconds)'

108	+	          type: integer

109	+	        nor:

110	+	          allOf:

111	+	            - description: 'Next object request'

112	+	            - $ref: 'TS26510_CommonData.yaml#/components/schemas/RelativeUrl'

113	+	        nrr:

114	+	          description: 'Next range request'

115	+	          type: string

116	+

117	+	    CmcdStatusInfo:

118	+	      description: 'An object containing status-scope CMCD keys'

119	+	      type: object

120	+	      properties:

121	+	        rtp:

122	+	          description: 'Requested maximum throughput (kilobits per second)'

123	+	          type: integer

124	+	        bs:

125	+	          description: 'Buffer starvation'

126	+	          type: boolean

127	+

128	+	    CmcdStreamType:

129	+	      description: 'Stream type'

130	+	      type: string

131	+	      enum: [v, l]

132	+

133	+	    CmcdStreamingFormat:

134	+	      description: 'Streaming format'

135	+	      type: string

136	+	      enum: [d, h, s, o]

137	+

138	+	    CmcdObjectType:

139	+	      description: 'Media type of object'

140	+	      type: string

141	+	      enum: [m, a, v, av, i, c, tt, k, o]

## TS26512_Mas_Configuration_ContentHosting.yaml

---a/TS26512_Mas_Configuration_ContentHosting.yaml
+++b/TS26512_Mas_Configuration_ContentHosting.yaml

@@ -1,7 +1,7 @@

1	1		openapi: 3.0.0

2	2		info:

3	3		  title: Mas_Configuration_ContentHosting

4		-	  version: 1.0.2

4	+	  version: 1.1.0

5	5		  description: |

6	6		    5GMS AS Configuration API: Content Hosting

7	7		    © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

@@ -12,7 +12,7 @@ tags:

12	12		    description: '5G Media Streaming: Application Server Configuration (M3) APIs: Content Hosting'

13	13

14	14		externalDocs:

15		-	  description: 'TS 26.512 V18.6.0; 5G Media Streaming (5GMS); Protocols'

15	+	  description: 'TS 26.512 V19.0.0; 5G Media Streaming (5GMS); Protocols'

16	16		  url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/'

17	17

18	18		servers:

@@ -352,11 +352,15 @@ components:

352	352		      description: "A representation of a Content Hosting Configuration resource used to configure a Media AS."

353	353		      required:

354	354		        - name

355	+	        - externalServiceId

355	356		        - ingestConfiguration

356	357		        - distributionConfigurations

357	358		      properties:

358	359		        name:

359	360		          type: string

361	+	        externalServiceId:

362	+	          type: string

363	+	          description: 'External service identifier of the parent Provisioning Session for use in retrieving Service Access Information at reference point M3.'

360	364		        ingestConfiguration:

361	365		          $ref: 'TS26510_Maf_Provisioning_ContentHosting.yaml#/components/schemas/IngestConfiguration'

362	366		        distributionConfigurations:

## TS26512_Mas_Configuration_ContentPublishing.yaml

---a/TS26512_Mas_Configuration_ContentPublishing.yaml
+++b/TS26512_Mas_Configuration_ContentPublishing.yaml

@@ -1,7 +1,7 @@

1	1		openapi: 3.0.0

2	2		info:

3	3		  title: Mas_Configuration_ContentPublishing

4		-	  version: 1.0.2

4	+	  version: 1.1.0

5	5		  description: |

6	6		    5GMS AS Configuration API: Content Publishing

7	7		    © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

@@ -12,7 +12,7 @@ tags:

12	12		    description: '5G Media Streaming: Application Server Configuration (M3) APIs: Content Publishing'

13	13

14	14		externalDocs:

15		-	  description: 'TS 26.512 V18.6.0; 5G Media Streaming (5GMS); Protocols'

15	+	  description: 'TS 26.512 V19.0.0; 5G Media Streaming (5GMS); Protocols'

16	16		  url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/'

17	17

18	18		servers:

@@ -352,11 +352,15 @@ components:

352	352		      description: "A representation of a Content Publishing Configuration resource used to configure a Media AS."

353	353		      required:

354	354		        - name

355	+	        - externalServiceId

355	356		        - contributionConfigurations

356	357		        - egestConfiguration

357	358		      properties:

358	359		        name:

359	360		          type: string

361	+	        externalServiceId:

362	+	          type: string

363	+	          description: 'External service identifier of the parent Provisioning Session for use in retrieving Service Access Information at reference point M3.'

360	364		        contributionConfigurations:

361	365		          type: array

362	366		          items:

References

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 26.501: "5G Media Streaming (5GMS); General description and architecture".

[3]	DASH Industry Forum, "Specification of Live Media Ingest", 
https://dashif-documents.azurewebsites.net/Ingest/master/DASH-IF-Ingest.pdf

[4]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[5]	Standard ECMA-262, 5.1 Edition: "ECMAScript Language Specification", June 2011.

[6]	IETF RFC 6234: "US Secure Hash Algorithms (SHA and SHA-based HMAC and HKDF)".

[7]	3GPP TS 23.003: "Numbering, addressing and identification".

[8]	ITU-T Recommendation X.509 (2005) | ISO/IEC 9594-8:2005: "Information Technology – Open Systems Interconnection – The Directory: Public-key and attribute certificate frameworks".

[9]	Void

[10]	IETF RFC 4648: "The Base16, Base32, and Base64 Data Encodings".

[11]	IEEE Standard 1003.1™, Issue 7: "The Open Group Base Specifications", 2018.
https://pubs.opengroup.org/onlinepubs/9699919799/

[12]	3GPP TS 29.571: "Common Data Types for Service Based Interfaces; Stage 3".

[13]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[14]	3GPP TS 36.321: "Evolved Universal Terrestrial Radio Access (E-UTRA); Medium Access Control (MAC) protocol specification".

[15]	3GPP TS 27.007: "AT Command set for User Equipment (UE)".

[16]	Void[17]	IETF RFC 7468: "Textual Encodings of PKIX, PKCS, and CMS Structures", April 2015.

[18]	ISO 3166-1: "Codes for the representation of names of countries and their subdivisions — Part 1: Country codes".

[19]	ISO 3166-2: "Codes for the representation of names of countries and their subdivisions — Part 2: Country subdivision code".

[20]	IETF RFC 5280: "Internet X.509 Public Key Infrastructure Certificate and Certificate Revocation List (CRL) Profile", May 2008.

[21]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[22]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[23]	OpenAPI: "OpenAPI 3.0.0 Specification", https://github.com/OAI/OpenAPI-Specification/blob/master/versions/3.0.0.md.

[24]	IETF RFC 9112: "HTTP/1.1", June 2022.

[25]	IETF RFC 9110: "HTTP Semantics", June 2022.

[26]	Void

[27]	Void

[28]	IETF RFC 9111: "HTTP Caching", June 2022.

[29]	Void

[30]	IETF RFC 8446: "The Transport Layer Security (TLS) Protocol Version 1.3", August 2018.

[31]	IETF RFC 9113: "HTTP/2", June 2022.

[32]	ISO/IEC 23009-1: "Information technology; Dynamic adaptive streaming over HTTP (DASH) — Part 1: Media presentation description and segment formats".

[33]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[34]	3GPP TS 29.514: "5G System; Policy Authorization Service; Stage 3".

[35]	3GPP TS 26.511: "5G Media Streaming (5GMS); Profiles, codecs and formats".

[36]	Void.

[37]	3GPP TS 26.244: "Transparent end-to-end packet switched streaming service (PSS); 3GPP file format (3GP)".

[38]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format", December 2017.

[39]	ISO/IEC 14496-12: "Information technology – Coding of audio-visual objects – Part 12: ISO base media file format".

[40]	ISO/IEC 23000-19: "Information technology – Multimedia application format (MPEG-A) – Part 19: Common media application format (CMAF) for segmented media".

[41]	IETF RFC 3986: "URI Generic Syntax".

[42]	3GPP TS 26.118: "Virtual Reality (VR) profiles for streaming applications".

[43]	3GPP TS 24.558: "Enabling Edge Applications; Protocol specification".

[44]	3GPP TS 29.558: "Enabling Edge Applications; Application Programming Interface (API) specification; Stage 3".

[45]	3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2".

[46]	3GPP TS 29.517: "5G System; Application Function Event Exposure Service; Stage 3".

[47]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[48]	3GPP TS 26.531: "Data Collection and Reporting; General Description and Architecture".

[49]	3GPP TS 26.532: "Data Collection and Reporting; Protocols and Formats".

[50]	3GPP TS 29.522: "5G System. Network Exposure Function Northbound APIs; Stage 3".

[51]	3GPP TS 26.346: "Multimedia Broadcast/Multicast Service (MBMS); Protocols and codecs".

[52]	3GPP TS 26.347: "Multimedia Broadcast/Multicast Service (MBMS); Application Programming Interface and URL".

[53]	IETF draft-bhutton-json-schema-validation: "JSON Schema Validation: A Vocabulary for Structural Validation of JSON", June 2022.

[54]	IETF RFC 3339: "Date and Time on the Internet: Timestamps", July 2002.

[55]	3GPP 29.591: "Network Exposure Function Southbound Services; Stage 3".

[56]	3GPP TS 26.510: "Media delivery; interactions and APIs for provisioning and media session handling".

[57]	IETF RFC 2045: "Multipurpose Internet Mail Extensions (MIME) Part One: Format of Internet Message Bodies".

[58]	IETF RFC 9000: "QUIC: A UDP-Based Multiplexed and Secure Transport", May 2021.

[59]	IETF RFC 9001: "Using TLS to Secure QUIC", May 2021.

[60]	IETF RFC 9114: "HTTP/3", June 2022.

[61]	IETF RFC 8673: "HTTP Random Access and Live Content", November 2019.

[62]	Consumer Technology Association CTA-5005-A: "Web Application Video Ecosystem – DASH-HLS Interoperability Specification".

[63]	DASH-IF Guidelines: "Low-latency Modes for DASH", available here:
https://dash-industry-forum.github.io/docs/CR-Low-Latency-Live-r8.pdf

[64]	3GPP TS 26.517: "5G Multicast-Broadcast User Services; Protocols and Formats".

[65]	Consumer Technology Association CTA-5004: "Web Application Video Ecosystem – Common Media Client Data", September 2020,
https://cdn.cta.tech/cta/media/media/resources/standards/pdfs/cta-5004-final.pdf.

[66]	IETF RFC 4122: "A Universally Unique IDentifier (UUID) URN Namespace", July 2005.

[67]	IETF RFC 9330:"Low Latency, Low Loss, Scalable Throughput (L4S) Internet Service: Architecture", January 2023.

[68]	IETF RFC 9331: "Explicit Congestion Notification (ECN) Protocol for Very Low Queuing Delay (L4S)", January 2023.

[69]	IETF RFC 9332: "Dual-Queue Coupled Active Queue Management (AQM) for Low Latency, Low Loss, and Scalable Throughput (L4S)", January 2023.

[70]	CTA-5004: "Web Application Video Ecosystem: Common Media Client Data (CMCD)", September 2020.

[71]	IETF RFC 9293: "Transmission Control Protocol (TCP)", August 2022.

[72]	IETF RFC 8684: "TCP Extensions for Multipath Operation with Multiple Addresses", March 2020.

[73]	IETF Draft: "Multipath Extension for QUIC", draft-ietf-quic-multipath-15, July 2025.

[74]	DASH Interoperability Forum: "DASH-IF Interoperability Points; Part 6: Content protection and security".

[75]	ETSI TS 103 799: "Content Protection Information Exchange Format (CPIX)".

[76]	ETSI TS 103 998: "Content Steering for DASH".

[77]	ETSI TS 103 973: "Coded Multisource Media Format (CMMF) for Content Distribution and Delivery".

Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1], TS 26.501 [2] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1] or TS 26.501 [2].

media resource: A media object that is rendered by the 5GMSd Media Player or captured by the 5GMSu Media Streamer.

transport resource: A uniquely identifiable object constructed for the purposes of transmission from a service location at reference points M2, M4 or M10 that contains a variant or representation of a media resource.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GMS	5G Media Streaming

5GMSd	5GMS downlink

5GMSu	5GMS uplink

5GMSA	5GMS Architecture

5GMS EAS	Edge-enabled 5GMS Application Server

BMFF	(ISO) Base Media File Format

ABR	Adaptive Bit Rate

ACR	Application Context Relocation

AF	Application Function

ANBR	Access Network Bit rate Recommendation

AS	Application Server

BM-SC	Broadcast Multicast Switching Centre

CBCS	Cipher Block Chaining

CDN	Content Delivery Network / Content Distribution Network

CENC	Common Encryption

CGI	Cell Global Identifier

CMAF	Common Media Application Format

CMMF	Coded Multisource Media Format

CRUD	Create, Read, Update, Delete

CNAME	Canonical Name

CORS	Cross-Origin Resource Sharing

CPIX	Content Protection Information Exchange

CRL	Certificate Revocation List

DANE	DASH-Aware Network Element

DASH	Dynamic Adaptive Streaming over HTTP

DER	Distinguished Encoding Rule

DNN	Domain Name News

DNS	Domain Name Server

EAS	Edge Application Server

ECGI	E-UTRAN Cell Global Identifier

ECMA	European Computer Manufacturers Association

ECN	Explicit Congestion Notification

EES	Edge Enabler Server

EFDT	Extended File Delivery Table

eMBMS	evolved MBMS

FQDN	Fully Qualified Domain Name

GPSI	Generic Public Subscription Identifier

HLS	HTTP Live Streaming

ISO	International Organization for Standardization

JSON	JavaScript Object Notation

L4S	Low Latency, Low Loss and Scalable Throughput

LCID	Logical Channel IDentifier

MBMS	Multimedia Broadcast Multicast Service

MBS	Multicast-Broadcast Service

MBSTF	MBS Transport Function

MFBR	Maximum Flow Bit Rate

MIME	Multipurpose Internet Mail Extensions

MNO	Mobile Network Operator

MPD	Media Presentation Description

MSISDN	Mobile Subscriber ISDN number

NCGI	NR Cell Global Identifier

NEF	Network Exposure Function

OAM	Operations, Administration and Maintenance

PCC	Policy Control and Charging

PCF	Policy Control Function

PEM	Privacy-Enhanced Mail

PFD	Packet Flow Description

PFDF	Packet Flow Description Function

QoE	Quality of Experience

QoS	Quality of Service

SDF	Service Data Flow

SHA	Secure Hash Algorithm

TLS	Transport Layer Security

TOI	Transport Object Identifier

URI	Uniform Resource Identifier

URL	Uniform Resource Locator

UTC	Coordinated Universal Time

Procedures for Downlink Media Streaming

## 4.2	APIs relevant to downlink media streaming

Table 4.2-1 summarises the APIs used to provision and use the various downlink media streaming features specified in TS 26.501 [2].

Table 4.2-1: Summary of APIs relevant to downlink media streaming features

| 5GMSd feature | Abstract | Relevant APIs |  |  |
| --- | --- | --- | --- | --- |
|  |  | Interface | API name | Clause |
| Content protocols discovery | Used by the 5GMSd Application Provider to interrogate which content ingest protocols are supported by 5GMSd AS(s). | M1d | Content Protocols Discovery API | 7.5 |
| Content hosting | Content is ingested, hosted and distributed by the 5GMSd AS according to a Content Hosting Configuration associated with a Provisioning Session. | M1d | Provisioning Sessions API | 7.2 |
|  |  |  | Server Certificates Provisioning API | 7.3 |
|  |  |  | Content Preparation Templates Provisioning API | 7.4 |
|  |  |  | Content Hosting Provisioning API | 7.6 |
|  |  | M2d | HTTP pull-based content ingest protocol | 8.2 |
|  |  |  | DASH-IF push-based content ingest protocol | 8.3 |
|  |  |  | HTTP low-latency pull-based content ingest protocol | 8.4 |
|  |  | M3d | Server Certificates configuration API | 9.2 |
|  |  |  | Content Preparation Templates configuration API | 9.3 |
|  |  |  | Content Hosting configuration API | 9.4 |
|  |  | M4d | MPEG-DASH [4] or 3GP-DASH [37] or DASH-IF push-based content distribution | 10.2 |
|  |  |  | HTTP low-latency content distribution | 10.3 |
|  |  | M5d | Service Access Information API | 11.2 |
|  |  | M10d | HTTP pull-based content ingest protocol | 8.2 |
|  |  |  | DASH-IF push-based content ingest protocol | 8.3 |
|  |  |  | HTTP low-latency pull-based content ingest protocol | 8.4 |
| Metrics reporting | The 5GMSd Client uploads metrics reports to the 5GMSd AF according to a provisioned Metrics Reporting Configuration it obtains from the Service Access Information for its Provisioning Session. | M1d | Provisioning Sessions API | 7.2 |
|  |  |  | Metrics Reporting Provisioning API | 7.8 |
|  |  | M3d | Service Access Information API | 11.2 |
|  |  |  | Metrics Reporting API | 11.4 |
|  |  | M5d | Service Access Information API | 11.2 |
|  |  |  | Metrics Reporting API | 11.4 |
| Consumption reporting | The 5GMSd Client or 5GMSd AS provides feedback reports on currently consumed content according to a provisioned Consumption Reporting Configuration it obtains from the Service Access Information for its Provisioning Session. | M1d | Provisioning Sessions API | 7.2 |
|  |  |  | Consumption Reporting Provisioning API | 7.7 |
|  |  | M5d | Service Access Information API | 11.2 |
|  |  |  | Consumption Reporting API | 11.3 |
| Dynamic Policy invocation | The 5GMSd Client activates different traffic treatment policies selected from a set of Policy Templates configured in its Provisioning Session. | M1d | Provisioning Sessions API | 7.2 |
|  |  |  | Policy Templates Provisioning API | 7.9 |
|  |  | M5d | Service Access Information API | 11.2 |
|  |  |  | Dynamic Policies API | 11.5 |
| Network Assistance | The 5GMSd Client requests bit rate recommendations and delivery boosts from the 5GMSd AF. | M5d | Service Access Information API | 11.2 |
|  |  |  | Network Assistance API | 11.6 |
| Edge content processing | Edge resources are provisioned for processing content in 5GMS downlink media streaming sessions. | M1d | Provisioning Sessions API | 7.2 |
|  |  |  | Edge Resources Provisioning API | 7.10 |
|  |  | M5d | Service Access Information API | 11.2 |
| 5GMS via eMBMS | The 5GMSd AF provisions the delivery of content via eMBMS and MBMS User Services. | M1d | Provisioning Sessions API | 7.2 |
|  |  | M5d | Service Access Information API | 11.2 |
|  |  | M4d | MPEG-DASH [4] or 3GP-DASH [37] or HLS | 10 |
| 5GMS via MBS | The 5GMSd AF provisions the delivery of content via MBS User Services. | M1d | Provisioning Sessions API | 7.2 |
|  |  | M5d | Service Access Information API | 11.2 |
|  |  | M4d | MPEG-DASH [4] or 3GP-DASH [37] or HLS | 10 |
| 5GMS via eMBMS | The 5GMSd AF provisions the delivery of content via eMBMS. | M1d | Provisioning Sessions API | 7.2 |
|  |  | M5d | Service Access Information API | 11.2 |
|  |  | M4d | MPEG-DASH [4] or 3GP-DASH [37] or HLS content distribution | 10 |
| UE data collection, reporting and exposure | UE data related to downlink 5G Media Streaming is reported to the Data Collection AF instantiated in the 5GMSd AF for exposure to Event consumers. | M1d | Event Data Processing Provisioning API | 7.11 |
|  |  | R4 | Ndcaf_DataReporting service | 17 |
|  |  | R5, R6 | Naf_EventExposure service | 18 |

Dynamic Policy provisioning Procedures (M1)

### 4.3.1	General

A 5GMS Application Provider may use the procedures in this clause to provision the network for media streaming sessions that are operated by that 5GMS Application Provider. For downlink media streaming, these sessions may be DASH streaming sessions, progressive download sessions, or any other type of media streaming or distribution (e.g. HLS) sessions. For uplink media streaming, the content format and delivery protocol are defined by the 5GMSu Application Provider and may be either non-fully standardized or employ standardized HTTP-based streaming of ISO BMFF content fragments as profiled by CMAF [39].

Reference point M1 offers three different sets of procedures:

-	For downlink media streaming, configuration of content ingest at reference point M2d or M10d for onward distribution by the 5GMSd AS over reference point M4d or M10d, or via other distribution systems such as eMBMS or MBS. The API at this reference point is designed to offer equivalent functionality as that exposed by a public CDN. For uplink media streaming, configuration of content egest at reference point M2u or M10u for the media content received by the 5GMSu AS from the 5GMSu Client over reference point M4u or M10u. The resource types involved in content hosting configuration are provisioning session (see clause 4.3.2), content hosting procedures (see clause 4.3.3), ingest protocols (see clause 4.3.4), content preparation template (see clause 4.3.5), and server certificates (see clause 4.3.6).

-	Configuration of dynamic policies: allows the configuration of Policy Templates at M5 that can be applied to M4 downlink/uplink media streaming sessions.

-	Configuration of reporting: permits the MNO to collect, at M5, QoE metrics and consumption reports about M4 downlink sessions, as well as permits the MNO to collect, at M5, QoE metrics reports about M4 uplink sessions.

A 5GMS Application Provider may use any of these procedures, in any combination, to support its media streaming sessions.

#### 4.3.4.1	General

The 5GMS Application Provider shall use the operations specified in clause 5.2.3 of TS 26.510 [56] at reference point M1 when it wants to discover the set of downlink content ingest or uplink content egest protocols supported by the 5GMS AS at reference point M2 and M10.

#### 4.3.5.1	General

For downlink media streaming, the 5GMSd AS may be required to process content ingested at reference point M2d or M10d before serving it from reference point M4d service locations. For uplink media streaming, the 5GMSu AS may be required to process content it receives from the 5GMSu Client at reference point M4u before passing it to the 5GMSu Application Provider on the egest interface at reference point M2u or to another 5GMSu AS at reference point M10u.

The 5GMS Application Provider shall use the operations specified in clause 5.2.5 of TS 26.510 [56] at reference point M1 when it wants to create and subsequently manipulate Content Preparation Templates in the 5GMS AF.

#### 4.3.6.1	General

Each X.509 server certificate [8] presented by the 5GMSd AS at reference point M4d service locations or at reference point xMB-U is represented by a Server Certificate resource at M1d. The 5GMS Application Provider shall use the operations specified in clause 5.2.4 of TS 26.510 [56] at reference point M1 when it wants to create and subsequently manipulate Server Certificates in the 5GMS AF. These enable a Server Certificate resource to be created within the scope of a Provisioning Session, and subsequently referenced by a Content Hosting Configuration created in the scope of the same Provisioning Session.

NOTE:	As a consumer of media from the 5GMSd AS in a combined architecture using 5GMS and eMBMS, the BM-SC needs to be able to trust the content it is receiving comes from a bona fide source. This issue is left to implementation. Likewise, in the case of a combined architecture using 5GMS and MBS, the MBSTF needs to be able to trust the content it ingests.

#### 4.3.7.1	General

The 5GMS Application Provider shall use the operations specified in clause 5.2.7 of TS 26.510 [56] at reference point M1 when it wants to create and subsequently manipulate Policy Templates available for the use of downlink or uplink media streaming sessions of a particular Provisioning Session in the 5GMS AF.

As specified in clause 5.2.7.1 of TS 26.510 [56], the 5GMS AF is responsible for ensuring that 5GMS AS supports L4S according to RFC 9330 [67], RFC 9331 [68] and RFC 9332 [69] at reference point M4 before accepting a Policy Template that declares a preference for ECN marking for L4S functionality.

Procedures of the M3d interface

#### 4.5.4.1	General

The 5GMSd AF shall configure Content Hosting Configuration resources of type Application‌Server‌Content‌Hosting‌Configuration in the 5GMSd AS using the procedures defined in this clause such that they remain synchronised with the Content Hosting Configurations provisioned at reference point M1 using the procedures defined in clause 4.3.3.

The format of the Content Hosting Configuration resource representation at reference point M3d is as specified in clause 8.8.3.1 of TS 26.510 [56]. The 5GMSd AF shall populate the externalServiceIdentifier property with the external service identifier from the parent Provisioning Session to enable the 5GMSd AS to retrieve Service Access Information from the 5GMSd AF at reference point M3d (see clause 4.5.6). The 5GMSd AF shall populate the canonicalDomainName and baseURL properties of each distribution configuration with the values it has chosen.

NOTE:	The 5GMSd AS implementation converts received Content Hosting Configurations into a format suitable for configuring the HTTP server that realises the content hosting feature.

#### 4.5.5.1	General

The 5GMSu AF shall configure Content Publishing Configuration resources of type Application‌Server‌Content‌Publishing‌Configuration in the 5GMSu AS using the procedures defined in this clause such that they remain synchronised with the Content Publishing Configurations provisioned at reference point M1 using the procedures defined in clause 4.3.3.

The format of the Content Publishing Configuration resource representation at reference point M3 is as specified in clause 8.9.3.1 of TS 26.510 [56]. The 5GMSu AF shall populate the externalServiceIdentifier property with the external service identifier from the parent Provisioning Session to enable the 5GMSu AS to retrieve Service Access Information from the 5GMSu AF at reference point M3u (see clause 4.5.6). The 5GMSu AF shall populate the canonicalDomainName and baseURL properties of each contribution configuration with the values it has chosen.

NOTE:	The 5GMSu AS implementation converts received Content Publishing Configurations into a format suitable for configuring the HTTP server that realises the content publishing feature.

### 4.5.6	Service Access Information procedures

Service Access Information includes configuration information to allow the 5GMSd AS to invoke at reference point M3d procedures for dynamic policy (see clause 4.5.7), consumption reporting (clause 4.5.8), metrics reporting (clause 4.5.9) and network assistance (clause 4.5.10). For any of these purposes, the 5GMSd AS shall obtain Service Access Information from the 5GMSd AF using the operations specified in clause 5.3.2 of TS 26.510 [56] at reference point M3d, citing an external service identifier and the Service Access Information is derived by the 5GMSd AF from the Provisioning Session established at reference point M1d (see clause 4.3.2) that is tagged with the same external service identifier.

In this version of the present document, the properties of the Service Access Information resource shall be populated as follows at reference point M3 based on the properties and sub-resources of the corresponding Provisioning Session:

-	The provisioningSessionId and provisioningSessionType properties are present, as required by TS 26.510 [56].

-	The locationReporting property is present, as required by TS 26.510 [56], and shall be set to false.

-	The streamingAccess object shall be present if provisioned in the Content Hosting Configuration or Content Publishing Configuration resource.

-	If any of the metrics reporting configurations listed in table 7.8.1-1 are provisioned per clause 4.3.9, the client‌Metrics‌Reporting‌Configurations array shall be present and populated with an object corresponding to each metrics reporting configuration. Metrics Reporting Configurations for any other metrics schemes shall not be included in the client‌Metrics‌Reporting‌Configurations array.

All other properties of the Service Access Information resource shall be omitted when served at reference point M3d.

### 4.5.7	Dynamic policy invocation procedures

Dynamic policy invocation at reference point M3d is not supported by the 5GMS System in this release.

### 4.5.8	Consumption reporting procedures

Consumption reporting at reference point M3d is not supported by the 5GMS System in this release.

### 4.5.9	Metrics reporting procedures

These procedures are used by the metrics reporting function of the 5GMSd AS to submit QoE metrics reports (including CMCD reporting) to the 5GMSd AF via reference point M3d if metrics reporting is applied for a media streaming session. To determine whether and how to send metrics reports to the 5GMSd AF at reference point M3d, the 5GMSd AS shall use the procedures and operations specified in clause 5.3.5 of TS 26.510 [56].

-	If any of the CMCD metrics schemes specified in table 7.8.1-1 are indicated in a client metrics reporting configuration of the Service Access Information provided to the 5GMSd AS at reference point M3d (see clause 4.5.6), the 5GMSd AS shall report CMCD information to the 5GMSd AF at reference point M3d using the API specified in clause 11.4.1 and the report format specified in clause 11.4.3.3.

### 4.5.10	Network Assistance procedures

Network Assistance is not supported by the 5GMS System at reference point M3 in this release.

Procedures of the M4d (Media Streaming) interface

### 4.6.0	General

#### 4.6.0.1	ECN marking for L4S

When a Dynamic Policy Instance (see clause 4.7.3) enabling the ECN marking for L4S functionality is successfully activated for a downlink media streaming session, the 5GMSd AS is required to support L4S according to RFC 9330 [67], RFC 9331 [68] and RFC 9332 [69] at reference point M4d.

### 4.6.1	Procedures for DASH session

This procedure is used by a Media Player to establish a DASH session with the 5GMSd AS via reference point M4d. In order to establish such a session, the 5GMSd AS shall host an MPD as defined in ISO/IEC 23009-1 [32] or TS 26.247 [4] and the MPD URL is known to the 5GMSd Client typically using M8d.

To initiate a DASH session using downlink media streaming, the Media Player receives the URL of a Media Player Entry (MPD) resource from the 5GMSd-Aware Application through M7d by methods defined in clause 13. The Media Player shall send an HTTP GET message to the 5GMSd AS including the URL of the Media Player Entry (MPD) resource. On success, the 5GMSd AS shall respond with a 200 (OK) message that includes the requested Media Player Entry (MPD) resource.

Additional procedures for reactions to different HTTP status codes are provided in TS 26.247 [4], clause A.7 and ISO/IEC 23009-1 [32] clause A.7.

Additional procedures for handling partial file responses are provided in TS 26.247 [4], clause A.9.

This information is provided through M7d to the application for selection. In addition, the currently used service description parameters are provided as status information through M7d in order for the Media Session Handler to make use of this information, for example for Dynamic Policy and Network Assistance.

The detailed handling of service description information is documented in clause 13.2 of the present document.

For the delivery of DASH content, the delivery protocol in use to connect to a reference point M4d service location is assumed to be HTTP/1.1 [21], HTTP/2 [34] or HTTP/3 [60]. The Media Player shall support HTTP/1.1 and may support HTTP/2 and HTTP/3.

The Media Player may use a multipath transport protocol supported by the UE as specified in clause 6.2.1.2 to connect to a reference point M4d service location on the 5GMSd AS. The different multipath transport connection subflows/paths may span one or more access networks available to the UE. The different subflows/paths may be used to increase the resilience of the transport connection to packet loss or to increase data throughput.

### 4.6.2	Procedures for Progressive Download Session

This procedure is used by a Media Player to establish a Progressive Download session with the 5GMSd AS at reference point M4d. In order to establish such a session, the 5GMSd AS shall host an 3GP/MP4 file as defined in TS 26.247 [4]. The 3GP/MP4 URL is known to the Media Player (in this case a progressive download player), typically by using M8d.

To initiate progressive download using downlink media streaming, the Media Player receives the URL of a Media Player Entry (3GP/MP4) resource from the 5GMSd-Aware Application through reference point M7d by methods defined in clause 13. The Media Player shall send an HTTP GET message to the 5GMSd AS including the URL of the Media Player Entry (3GP/MP4) resource. On success, the 5GMSd AS shall respond with a 200 (OK) message that includes the requested Media Player Entry (3GP/MP4) resource.

Additional procedures for reactions to different HTTP status codes are provided in TS 26.247 [4].

For the progressive download of content, the delivery protocol in use to connect to a reference point M4d service location is assumed to be HTTP/1.1 [21], HTTP/2 [34] or HTTP/3 [60]. The Media Player shall support HTTP/1.1 and may support HTTP/2 and HTTP/3.

The Media Player may use a multipath transport protocol supported by the UE as specified in clause 6.2.1.2 to connect to a reference point M4d service location on the 5GMSd AS. The different multipath transport connection subflows/paths may span one or more access networks available to the UE. The different subflows/paths may be used to increase the resilience of the transport connection to packet loss or to increase data throughput.

### 4.6.3	Procedures for using multiple service locations

These procedures may be used to augment the procedures described in clauses 4.6.1 and 4.6.2 above to allow for media resources to be obtained from multiple service locations exposed by the 5GMSd AS at reference point M4d.

Information required by the 5GMSd Client to access media from multiple service locations exposed at M4d by the 5GMSd AS is contained within a Media Player Entry document. This information may exist, for example, within:

-	An MPD as XML elements or attributes containing the required information.

-	A Media Player Entry document containing a pointer (e.g., URL) to an MPD or 3GP/MP4 file.

-	A document pointed to by a Media Player Entry.

A 5GMSd Client may use this information to do any or all of the following:

1.	Switch between service locations exposed at reference point M4d during a downlink media streaming session, as specified in clause 10.3A.2.

2.	Obtain signalling via reference point M4d from a content steering service provided by the 5GMSd AS or 5GMSd Application Provider that can be used to influence the choice of one service location over another, as specified in clause 10.3A.4.

3.	Access media resources from multiple service locations simultaneously, for example using multi-source object coding, as specified in clause 10.3A.3.

Procedures of the M5 (Media Session Handling) interface

#### 4.7.2.1	General

Service Access Information is the set of parameters and addresses needed by the 5GMSd Client to activate reception of a downlink media streaming session or by a 5GMSu Client to activate an uplink media streaming session for contribution.  Service Access Information additionally includes configuration information to allow the Media Session Handler to invoke procedures for dynamic policy (see clause 4.7.3), consumption reporting (clause 4.7.4), metrics reporting (clause 4.7.5) and network assistance (clause 4.7.6).

The Media Session Handler may obtain Service Access Information from either the 5GMS-Aware Application (via reference point M6) or from the 5GMS AF (via reference point M5). In the former case, the Service Access Information is initially acquired by the 5GMS-Aware Application from the 5GMS Application Provider via reference point M8. In the latter case, the Media Session Handler shall use the operations specified in clause 5.3.2 of TS 26.510 [56] at reference point M5 to acquire Service Access Information from the 5GMS AF, citing an external service identifier and the Service Access Information is derived by the 5GMS AF from the Provisioning Session established at reference point M1 (see clause 4.3.2) that is tagged with the same external service identifier.

Typically, the Service Access Information for media streaming includes a set of Media Entry Points. Examples include:

-	A URL to a DASH MPD,

-	A URL to a document that provides additional details for different streaming session configurations and/or that references or includes equivalent media presentations such as a DASH MPD or

-	A URL to a progressive download file that can be consumed by the Media Stream Handler (Media Player or Media Streamer).

Based on the MIME media type or protocol, as well as the conformance profiles declared in the Service Access Information, one of these Media Entry Points is selected by the Media Session Handler or by the 5GMS-Aware Application and is handed to the Media Player via reference point M11 or M7 respectively.

NOTE:	The Media Session Handler and 5GMS-Aware Application are assumed to have prior knowledge of the types of Media Entry Point supported by the Media Player.

For downlink media streaming exclusively via eMBMS and for hybrid 5GMSd/eMBMS services as defined in clauses 5.10.2 and 5.10.5 respectively of TS 26.501 [2], the Service Access Information indicates that the 5GMSd Client acts as an MBMS-Aware Application.

For dynamically provisioned downlink media streaming via eMBMS as defined in clause 5.10.6 of TS 26.501 [2], the 5GMSd AS creates a presentation manifest that is regularly polled by the Media Player for a potential update. When an eMBMS User Service carrying the 5GMSd content is dynamically provisioned or removed by the 5GMSd AF, the 5GMSd AS shall update the presentation manifest with the locations where the updated manifest and the media segments are now available, for example to add or change to the media server in the MBMS Client.

For downlink media streaming exclusively via MBS and for hybrid 5GMSd/MBS services as defined in clauses 5.12.2 and 5.12.4 respectively of TS 26.501 [2], the Service Access Information indicates that the 5GMSd Client acts as an MBS-Aware Application.

For dynamically provisioned downlink media streaming via MBS as defined in clause 5.12.4 of TS 26.501 [2], the 5GMSd AS creates or hosts a presentation manifest that is regularly polled by the Media Player for a potential update. When an MBS User Service carrying the 5GMSd content is dynamically provisioned or removed by the 5GMSd AF, the 5GMSd AS shall update the presentation manifest with the resource locations where the updated manifest and the media segments are now available, for example to additionally or alternatively point to the Media Server in the MBSTF Client.

If one or more Metrics Reporting Configurations are provisioned per clause 4.3.9, a subset of these is included in the corresponding Service Access Information provided to the Media Session Handler at reference point M5d as follows:

-	If the metrics scheme urn:‌3GPP:‌ns:‌PSS:‌DASH:‌QM10 is indicated in the metrics reporting configuration of a Provisioning Session for downlink media streaming, a corresponding client metrics reporting configuration shall be included in the Service Access Information provided to the Media Session Handler at reference point M5d to enable QoE metrics reporting for DASH-based downlink media streaming by the 5GMSd Client at reference point M5d per clause 11.4.3.2.

-	If the metrics scheme specified in clause 9.3 of TS 26.118 [42] is indicated in the metrics reporting configuration of a Provisioning Session for downlink media streaming, a corresponding client metrics reporting configuration shall be included in the Service Access Information provided to the Media Session Handler at reference point M5d to enable QoE metrics reporting for Virtual Reality media by the 5GMSd Client at reference point M5d per clause 11.4.3.2.

-	If any of the CMCD metrics schemes listed in table 7.8.1-1 are indicated in the metrics reporting configuration of a Provisioning Session for downlink media streaming, a corresponding client metrics reporting configuration shall be included in the Service Access Information provided to the Media Session Handler at reference point M5d to enable metrics reporting of CMCD information by the 5GMSd Client to the 5GMSd AS at reference point M4d per [70].

If an Edge Resources Configuration with client-driven management is provisioned per clause 4.3.10, a Client Edge Resources Configuration is included in the corresponding Service Access Information.

### 4.7.3	Procedures for dynamic policy invocation

These procedures are used by a Media Session Handler to manage Dynamic Policy Instance resources in the 5GMS AF. To do this, the Media Session Handler shall use the operations specified in clause 5.3.3 of TS 26.510 [56] at reference point M5 to instantiate Policy Templates in the 5GMS AF that are described in the Dynamic Policy Configuration provided in the Service Access Information (see clause 4.7.2).

The choice of Policy Template by the Media Session Handler shall be influenced by the following considerations:

-	When instantiating a Policy Template whose Policy Template Binding indicates l4S‌Enablement‌Preference set to true, the Media Session Handler shall set the l4SRequired flag to true in the Dynamic Policy request only if an L4S-capable media transport stack is present and in use. This is the case if the Media Session Handler detects that an L4S-capable media transport stack is present and in use by the Media Player or if the Media Player configuration and settings API (see clause 13.2.4) declares CAPABILITY_L4S.

-	When instantiating a Policy Template whose Policy Template Binding indicates qoSMonitoring‌Enablement‌Preference set to true, the Media Session Handler shall set the qoS‌Monitoring‌Required flag to true in the Dynamic Policy request only if the Media Player is capable of consuming QoS monitoring results.

The Media Session Handler shall interrogate the capabilities of the Media Player in order to discover whether it is capable of consuming QoS monitoring results and shall select a Policy Template that indicates the preference for QoS monitoring functionality only if the Media Player has this capability.

### 4.7.5	Procedures for metrics reporting

These procedures are used by the Media Session Handler and the Metrics Reporting function of the 5GMS Client to submit QoE metrics reports to the 5GMS AF via reference point M5 if metrics reporting is applied for a media streaming session. To determine whether and how to send metrics reports the 5GMS AF at reference point M5, the Media Session Handler shall use the procedures and operations specified in clause 5.3.5 of TS 26.510 [56].

When the metrics collection and reporting feature is provisioned for a media streaming Provisioning Session, one or more Client Metrics Reporting Configurations, each associated with a metrics scheme, are provided to the 5GMS Client in the Service Access Information (see clause 4.7.2).

-	For progressive download and DASH streaming services, the listed metrics in a given Client Metrics Reporting Configuration are associated with the 3GPP metrics scheme and shall correspond to one or more of the metrics as specified in clauses 10.3 and 10.4, respectively, of TS 26.247 [4]. When individual metrics or groups of metrics are to be reported, these are identified using controlled terms from the vocabulary specified in clause E.2.1.

-	For downlink DASH streaming services, CMCD information within the scope of one of the CMCD metrics schemes listed in table 7.8.1-1 may be listed in a Metrics Reporting Configuration. When individual metrics or groups of metrics are to be reported as in-band client data, these are identified using controlled terms from the vocabulary specified in clause E.2.5. As a result, the Media Player is configured by the Media Session Handler at reference point M11d to report the corresponding CMCD information to the 5GMSd AS at reference point M4d using either HTTP request headers or URL query parameters, as specified in [70].

-	Metrics relating to Virtual Reality media, as specified in clause 9.3 of TS 26.118 [42], may be listed in a Client Metrics Reporting Configuration. When individual metrics or groups of metrics are to be reported, these are identified using controlled terms from the vocabulary specified in clause E.2.2.

-	Metrics relating to eMBMS delivery, as specified in clause 9.4.6 of TS 26.346 [51], may be listed in a Client Metrics Reporting Configuration.

NOTE:	Metrics reporting for MBS is not specified by TS 26.517 [64] in this version of the present document.

Procedures of the M7d (UE Media Player) interface

## 4.9	Procedures of the M7d/M11d (UE Media Player) interface

(Snip)

### 4.9.1A	Procedures for configuring downlink content delivery

The 5GMSd-Aware Application and the Media Session Handler may configure content delivery for downlink media streaming between the 5GMSd AS and the Media Player using the client API at reference points M7d and M11d respectively by populating the desired‌Content‌Delivery‌Configuration object in the Media Player configuration as specified in clause 13.2.4.

-	A list of content delivery protocols (see table 13.2.4-3) to be used at reference point M4d may be provided in order of preference.

-	A preference for using a multipath transport protocol at reference point M4d may be expressed.

The Media Player should attempt to satisfy the desired content delivery configuration within the limits of its capabilities and those of the underlying UE.

In the absence of a desired content delivery configuration, the Media Player may select any content delivery protocol that meets the requirement of the provided Media Player Entry, and may select a multipath transport protocol compatible with its choice of content delivery protocol.

The Media Player should inform the 5GMSd-Aware Application and the Media Session Handler using reference point M7d and M11d notifications respectively about changes to the status of reference point M4d transport connections.

### 4.9.2	Metrics reporting procedures

These procedures shall be used by the Media Session Handler function to control metrics reporting when such reporting is configured via metadata sent in-band via the media manifest.

When a streaming session is started, the Media Session Handler shall check if the manifest contains any client metrics configuration, as specified in TS 26.247 [4] clauses 10.4 and 10.5, or TS 26.118 [42] clause 9.3. If such a configuration is found, the Media Session Handler shall use it for the current streaming session.

The Media Session Handler shall first determine whether metrics from this session shall be reported. The determination shall be based on the samplePercentage attribute specified in the metrics configuration, according to TS 26.247 [4] clause 10.5.

If metrics are reported for the session, the Media Session Handler shall request the Media Player to create a metrics collection job. The Media Player shall return a reference to the created job, which the Media Session Handler shall use in all subsequent actions related to this job.

The Media Session Handler shall configure the metrics collector job with the set of metrics which shall be collected during the session. The format of the configuration shall be according to TS 26.247 clause L.2, but note that only the metrics attribute in the configuration shall be used for this purpose.

If one of the CMCD metrics schemes listed in table 7.8.1-1 is indicated in the client metrics configuration, the Media Session Handler shall set a transmission mode in the metrics collector job to enable and configure in-band reporting of CMCD information [70] at reference point M4d. The default transmission mode in this version of the present document is to report using HTTP request headers. The 5GMS-Aware Application may override this configuration.

The Media Session Handler shall regularly request the collected metrics from the Media Player according to the reportingInterval specified in the metrics configuration. The metrics returned by the Media Player shall use the format as described in TS 26.247 clause 10.6, and (for virtual reality media) in TS 26.118 [42] clause 9.4 and the Media Session Handler shall forward these to the server address(es) specified in the metrics configuration using the specified DNN according to the procedures described in TS 26.247 clause 10.6.

When the session is finished the Media Session Handler shall delete the metrics collection job.

### 4.9.3	Dynamic Policy procedures

These procedures are used between a Media Player and a Media Session Handler at reference point M11.

If the Media Player is capable of consuming QoS monitoring results, it shall subscribe to receive QoS monitoring results notifications from the Media Session Handler at reference point M11 as specified in clause 5.4.3 of TS 26.510 [56].

New Procedures for Downlink Media Streaming

## 4.10A	Procedures of the M10d interface

The procedures at reference point M10d are used by a 5GMSd AS to ingest content from a 5GMSd AS in another 5GMS System. The procedures at this reference point are referred to as service chaining.

The following 5GMS AS service chaining protocols specified by the present document may be used at reference point M10d to support downlink media streaming:

-	An HTTP pull-based content ingest protocol is specified in clause 8.2, including specific handling for HTTP redirects issued to the 5GMS AS by the 5GMS Application Provider's origin server.

-	A DASH-IF push-based content ingest protocol is specified in clause 8.3.

-	A HTTP low-latency pull-based content ingest protocol is specified in clause 8.4.

## 4.10B	Procedures of the M13d interface

No specific procedures are defined at reference point M13d, but it is expected that the Media Player follows similar procedures when interacting with the 5GMSd Application Provider as those defined between the Media Player and the 5GMSd AS at reference point M4d, as outlined in clause 4.6.

Procedures for Uplink Media streaming

## 5.1	General

Uplink media streaming functional entities in the 5GMS System include the 5GMSu Application Provider, 5GMSu AF, 5GMSu AS and the UE. To make use of these other entities, the UE includes a 5GMSu-Aware Application that is provided by the 5GMSu Application Provider and a 5GMSu Client comprising the Media Session Handler and the Media Streamer.

The M1 Provisioning API enables the 5GMSu Application Provider to establish and manage the uplink media session handling and streaming options of the 5GMSu System.

The content egest interface enables uplink media streaming content that has been sent by the 5GMSu Client to the 5GMSu AS at reference point M4u to be subsequently delivered to the 5GMSu Application Provider via reference point M2u or to another 5GMSu AS via reference point M10u. Uplink media streaming media transfer from the 5GMSu AS to the 5GMSu Application Provider at reference point M2u or from one 5GMSu AS to another at reference point M10u may be either pull-based and initiated by the 5GMSu Application Provider using the HTTP GET method, or push-based and initiated by the 5GMSu AS using the HTTP PUT method. The 5GMSu Application Provider's target endpoint for push-based streaming content delivery at reference point M2u is provided to the 5GMSu AF as part of the M1 Provisioning Session and this is passed to the 5GMSu AS as part of the M3u configuration procedures.

The 5GMSu AF, having been successfully provisioned at reference point M1u, sets up corresponding resources at a reference point M5u endpoint from which Service Access Information for uplink media streaming session management, metrics reporting, network assistance and request for policy and/or charging treatment may be retrieved using its provisioned external application identifier. Certain types of configuration and policy information accessed over reference point M5u by the Media Session Handler, such as uplink metrics reporting, QoS policy, or support for AF-based network assistance are further passed to the Media Streamer via the M7u API.

The 5GMSu-Aware Application initiates a new uplink media streaming session by launching the Media Stream Handler at reference point M6u using a 3GPP Service URL for 5GMS (see clause 4.8.3). The 3GPP Service URL indicates the external application identifier. This may be used to retrieve Service Access Information from the 5GMSu AF at reference point M5. Alternatively, if the 5GMSu-Aware Application has already acquired all necessary Service Access Information via private means at reference point M8, this may be supplied directly to the Media Session Handler at reference point M6u as additional parameters.

Based on a request from the 5GMSu-Aware Application or from the Media Streamer received over the M6u API, and based on the Service Access Information acquired from the 5GMSu AF via reference point M5u, the Media Session Handler sets up an uplink media streaming session with a unique media delivery session identifier. Upon successful session establishment, the Media Session Handler triggers the Media Streamer to begin uplink media streaming of media content to the 5GMSu AS over reference point M4u.

Subscription to status and other event notification services is offered by the Media Session Handler to the 5GMSu-Aware Application and to the Media Streamer via the UE media session handling APIs exposed by the Media Session Handler at reference point M6u.

Subscription to status and other event notification services is also offered by the Media Streamer to the 5GMSu-Aware Application and to the Media Session Handler via the UE media stream handling APIs exposed by the Media Player at reference point M11u.

## 5.2	APIs relevant to Uplink Media Streaming

Table 5.2-1 summarises the APIs used to provision and use the various uplink media streaming features specified in TS 26.501 [2].

Table 5.2-1: Summary of APIs relevant to uplink media streaming features

| 5GMSu feature | Abstract | Relevant APIs |  |  |
| --- | --- | --- | --- | --- |
|  |  | Interface | API name | Clause |
| Content protocols discovery | Used by the 5GMSu Application Provider to query which content egest protocols are supported by 5GMSu AS(s). | M1u | Content Protocols Discovery API | 7.5 |
| Content publishing | Content is contributed to the 5GMSu AS and published to 5GMSu Application Providers according to a Content Publishing Configuration associated with a Provisioning Session. | M1u | Provisioning Sessions API | 7.2 |
|  |  |  | Server Certificates Provisioning API | 7.3 |
|  |  |  | Content Preparation Templates Provisioning API | 7.4 |
|  |  |  | Content Publication Provisioning API | 7.6A |
|  |  | M2u | HTTP pull-based content egest protocol | 8.5 |
|  |  |  | DASH-IF push-based content egest protocol | 8.6 |
|  |  |  | HTTP low-latency pull-based content egest protocol | 8.7 |
|  |  | M3u | Server Certificates configuration API | 9.2 |
|  |  |  | Content Preparation Templates configuration API | 9.3 |
|  |  |  | Content Publication configuration API | 9.5 |
|  |  | M4u | DASH-IF push-based contribution protocol | 10.4.2 |
|  |  | M5u | Service Access Information API | 11.2 |
|  |  | M10u | HTTP pull-based content egest protocol | 8.5 |
|  |  |  | DASH-IF push-based content egest protocol | 8.6 |
|  |  |  | HTTP low-latency pull-based content egest protocol | 8.7 |
| Metrics reporting | The 5GMSu Client uploads metrics reports to the 5GMSu AF according to a provisioned Metrics Reporting Configuration it obtains from the Service Access Information for its Provisioning Session. | M1u | Provisioning Sessions API | 7.2 |
|  |  |  | Metrics Reporting Provisioning API | 7.8 |
|  |  | M5u | Service Access Information API | 11.2 |
|  |  |  | Metrics Reporting API | 11.4 |
| Dynamic Policy invocation | The 5GMSu Client activates different traffic treatment policies selected from a set of Policy Templates configured in its Provisioning Session. | M1u | Provisioning Sessions API | 7.2 |
|  |  |  | Policy Templates Provisioning API | 7.9 |
|  |  | M5u | Service Access Information API | 11.2 |
|  |  |  | Dynamic Policies API | 11.5 |
| Network Assistance | The 5GMSu Client requests bit rate recommendations and delivery boosts from the 5GMSu AF. | M5u | Service Access Information API | 11.2 |
|  |  |  | Network Assistance API | 11.6 |
| Edge content processing | Edge resources are provisioned for processing content in 5GMS uplink media streaming sessions. | M1u | Provisioning Sessions API | 7.2 |
|  |  |  | Edge Resources Provisioning API | 7.10 |
|  |  | M5u | Service Access Information API | 11.2 |
| UE data collection, reporting and exposure | UE data related to uplink 5G Media Streaming is reported to the Data Collection AF instantiated in the 5GMSu AF for exposure to Event consumers. | M1u | Event Data processing Provisioning API | 7.11 |
|  |  | R4 | Ndcaf_DataReporting service | 17 |
|  |  | R5, R6 | Naf_EventExposure service | 18 |

Procedures of the M4u (Media Streaming) interface
(implemented on top of Rel-18 26512-CR0096)

### 5.6.1	General

This procedure is used by a Media Streamer to establish an uplink media streaming session with the 5GMSu AS at reference point M4u.

To initiate uplink media streaming, the Media Streamer receives the URL of a Media Streamer Entry from the 5GMSu-Aware Application through reference point M7u. The Media Streamer shall send an HTTP GET message to the 5GMSu AS including the URL of the Media Streamer Entry resource. On success, the 5GMSu AS shall respond with a 200 (OK) message that includes the requested Media Streamer Entry resource.

The Media Streamer may use a multipath transport protocol supported by the UE as specified in clause 6.2.1.2 to connect to a reference point M4u service location on the 5GMSu AS. The different multipath transport connection subflows/paths may span one or more access networks available to the UE. According to Media Streamer configuration (see clause 5.9.2) the different subflows/paths may be used to increase the resilience of the transport connection to packet loss or to increase data throughput.

Additional details of procedures at this reference point are for further study.

### 5.6.2	ECN marking for L4S

When a Dynamic Policy Instance (see clause 4.7.3) enabling the ECN marking for L4S functionality is successfully activated for an uplink media streaming session, the 5GMSu AS is required to support L4S according to RFC 9330 [67], RFC 9331 [68] and RFC 9332 [69] at reference point M4u.

## 5.9	Procedures of the M7u/M11u (UE Media Streamer) interface

### 5.9.1	General

This clause specifies procedures for the interaction between the 5GMSu-Aware Application or the Media Session Handler and the Media Streamer using the client API at reference points M7u and M11u respectively.

### 5.9.2	Procedures for configuring uplink content delivery

In the absence of a desired content delivery configuration, the Media Streamer may select any content delivery protocol that meets the requirements of the provided Media Streamer Entry, and may select a multipath transport protocol compatible with its choice of content delivery protocol.

The Media Streamer should inform the 5GMSu-Aware Application and the Media Session Handler using reference point M7u and M11u notifications respectively about changes to the status of reference point M4u transport connections.

Additional details of procedures at this reference point are for further study.

General aspects of APIs for 5G Media Streaming

#### 6.0.2.2	Canonical 5GMS AS authority at reference point M4

Media Entry Points provisioned in distribution configurations of a Content Hosting Configuration or in contribution configurations of Content Publishing Configuration shall be exposed by the 5GMS AS at reference point M4 from service locations with the following canonical domain name respectively:

{modifiedDistributionId}.{modifiedExternalServiceId}.ms.as.3gppservices.org

or

{modifiedContributionId}.{modifiedExternalServiceId}.ms.as.3gppservices.org

where:

-	{modifiedDistributionId} is a modified form of the DistributionConfiguration.distributionId property assigned by the 5GMS Application Provider upon provisioning of a Content Hosting Configuration (see clause 5.2.8.2 of TS 26.510 [56]).

-	{modifiedContributionId} is a modified form of the ContributionConfiguration.contributionId property assigned by the 5GMS Application Provider upon provisioning of a Content Publishing Configuration (see clause 5.2.9.2 of TS 26.510 [56]).

-	{modifiedExternalServiceId} is a modified form of the external service identifier indicated by the 5GMS Application Provider in the parent Provisioning Session resource at reference point M1 (see clause 5.2.2.1 of TS 26.510 [56]).

In all of the above identifiers, every period character ('.') is replaced with a single hyphen character ('-').

For example, the canonical 5GMS AS domain name for a Content Hosting Configuration with a distribution configuration assigned a distributionId property value of distributionA.service or Content Publishing Configuration with a contribution configuration assigned a contributionId property value of contributionA.service created under the Provisioning Session with external service identifier com.provider.service is respectively:

distributionA-service.com-provider-service.ms.as.3gppservices.org

or

contributionA-service.com-provider-service.ms.as.3gppservices.org

The DNS service provided by the 5G System shall resolve each such canonical domain name to the IP address(es) of deployed 5GMS AS service location(s) providing content hosting or content publishing endpoint(s) at reference point M4 on behalf of the parent Provisioning Session in question.

NOTE:	Access to the 5GMS AS using domain name aliases at this reference point is not precluded.

The 5GMS AS shall expose all service locations at reference point M4 via the default listening port number(s) for the version(s) of HTTP specified in clause 6.2.1.2 for use at this reference point.

#### 6.2.1.2	5GMS AS

Implementations of the 5GMS AS shall expose HTTP/1.1 [24] endpoints at reference points M2, M4 and M10; and implementations may additionally expose HTTP/2 [31] or HTTP/3 [60] endpoints at these reference points. For each protocol version, TLS [16] shall be supported and HTTPS interactions should be used in preference to cleartext HTTP. Multipath TCP per RFC 8684 [72] may be used as the underlying transport protocol at these two reference points.

For pull-based content ingest into the 5GMSd AS:

-	The 5GMSd Application Provider shall expose an HTTP/1.1-based origin endpoint to the 5GMSd AS at reference point M2d and may additionally expose HTTP/2- and/or HTTP/3-based origin endpoints at this reference point.

-	The 5GMSd AS shall expose an HTTP/1.1-based origin endpoint at reference point M10d and may additionally expose HTTP/2- and/or HTTP/3-based origin endpoints at this reference point.

For push-based content ingest into the 5GMSd AS:

-	The 5GMSd Application Provider may use any supported HTTP protocol version to push content at reference point M2d.

-	The 5GMSd AS may use any supported HTTP protocol version to push content at reference point M10d.

For pull-based content egest from the 5GMSu AS:

-	The 5GMSu AS shall expose an HTTP/1.1-based origin endpoint to the 5GMSu Application Provider at reference point M2u and may additionally expose HTTP/2- and/or HTTP/3-based origin endpoints at this reference point.

-	The 5GMSu AS shall expose an HTTP/1.1-based origin endpoint at reference point M10u and may additionally expose HTTP/2- and/or HTTP/3-based origin endpoints at this reference point.

For push-based content egest from the 5GMSu AS:

-	The 5GMSu AS may use any supported HTTP protocol version to push content to the 5GMSu Application Provider at reference point M2u.

-	The 5GMSu AS may use any supported HTTP protocol version to push content at reference point M10u.

Implementations of the 5GMS AS should expose HTTP/3 [60] endpoints at reference point M4. In HTTP/3, the QUIC protocol [58] or its multipath variant [73] may be used for transport, and in both cases TLS [59] is used for the initial handshake and key exchange.

The 5GMS AF may use any supported HTTP protocol version at reference point M3.

The Media Stream Handler (Media Player or Media Streamer) may use any supported HTTP protocol version at reference point M4. The Media Stream Handler may use multipath TCP per RFC 8684 [72] as the underlying transport protocol for HTTP/1.1 [24] or HTTP/2 [31] at this reference point. The Media Stream Handler may use multipath QUIC [73] to support HTTP/3 [60] connections at this reference point.

Content Preparation Templates Provisioning API

### 7.4.1	Overview

The API used by the 5GMS Application Provider at reference point M1 to instantiate and manipulate Content Preparation Templates associated with a particular downlink or uplink media streaming Provisioning Session in the 5GMS AF is specified in clause 8.5 of TS 26.510 [56]. Content Preparation Templates are used to specify manipulations applied by a 5GMS AS to downlink media resources ingested at reference point M2d or M10d for distribution at reference point M4d, or to uplink media resources contributed at reference point M4u or M10u for egest at reference point M2u. The Content Preparation Templates Provisioning API is used to provision a Content Preparation Template within the scope of a Provisioning Session that can subsequently be referenced from a Content Hosting Configuration.

Content Hosting Provisioning API (M1)

### 7.6.1	Overview

The API used by the 5GMSd Application Provider at reference point M1d to create and manipulate the 5GMSd AS Content Hosting Configuration associated with a particular downlink media streaming Provisioning Session in the 5GMSd AF is specified in clause 8.8 of TS 26.510 [56].

Within a Content Hosting Configuration, one or more distribution configurations may be defined where each may specify different caching, purging and content preparation behaviours for media resources ingested at reference point M2d or M10d. The Content Hosting Configuration may further specify, through the declaration of affinity groups how reference point M4d service locations associated with each distribution configuration are deployed in the 5GMS System.

#### 7.6.4.1	Overview

This clause defines the behaviour that is expected from the 5GMSd AS when the Content Hosting Configuration has been successfully provisioned as specified in clause 5.2.8 of TS 26.510 [56]. The main operations that are performed affect content caching and purging of cached content, as well as media processing for content preparation prior to distribution from one or more service locations.

#### 7.6.4.2	Content caching

A distribution configuration defined within the Content Hosting Configuration may specify caching rules to be applied to transport resources when they are distributed by the 5GMSd AS from reference point M4d service locations. The 5GMSd AS shall use the DistributionConfiguration.‌CachingConfigurations[ ]‌.urlPatternFilter property of the Content Hosting Configuration resource specified in clause 8.8.3.1 of TS 26.510 [56] to determine which caching directives apply to that transport resource. In the case where a distribution configuration has multiple caching configurations and a transport resource’s URL matches the pattern filter of more than one, the first match shall apply. In case no caching configuration is identified as a match, the 5GMSd AS shall apply the caching directives that were received from the upstream ingest source at reference point M2d or M10d. In the absence of these, the 5GMSd AS shall apply default caching directives as specified in clause 8.8.3.1 of TS 26.510 [56] based on the media resource type.

A caching directive shall indicate that a matching transport resource is:

-	Not to be cached by the 5GMSd AS, nor by downstream M4d clients, when noCache is set to true, or

-	To be cached for maxAge seconds by the 5GMSd AS, and potentially by downstream M4d clients, when noCache is set to false.

The maxAge value applies relative to the time when a transport resource was ingested by the 5GMSd AS, defined here as t_ingest regardless of whether or not it is further modified by a Content Preparation Template. For an HTTP-based ingest, this corresponds to the Date header field in the HTTP request/response that carries the transport resource at M2d or M10d. At the time t_ingest + maxAge, the transport resource is considered stale and should not be served from the 5GMSd AS cache. The 5GMSd AS shall compensate for any synchronization skew between the origin and its own clock. For instance, this can be done by including the max-stale HTTP cache directive in HTTP responses sent from reference point M4d service locations.

The maxAge value may be signalled by the 5GMSd AS at reference point M4d service locations using the Expires HTTP response header or the HTTP Cache-Control directives max-age or s-maxage.

When distributing a transport resource using HTTP, a no-cache request may be translated into a no-cache and no-store HTTP Cache-Control directive and/or a max-age=0 HTTP Cache-Control directive.

By default, all origin HTTP header fields shall be assumed as not forwarded by the 5GMSd AS, unless specified otherwise by setting the flag originCacheHeaders to true.

#### 7.6.4.3	Cache purging

The 5GMSd Application Provider shall use the procedures and operations specified in clause 5.2.8.6 of TS 26.510 [56] to invalidate some or all cached transport resources of a particular Content Hosting Configuration. As a consequence, the 5GMSd AF shall invoke an operation on the 5GMSd AS at reference point M3d to remove those transport resources (e.g., in the case the underlying media resource has been modified by a Content Preparation Template – see clause 7.6.4.4) from the 5GMSd AS cache across all distribution configurations associated with that Content Hosting Configuration, as specified in clause 9.

#### 7.6.4.4	Content processing

The 5GMSd AS may be required to perform various content processing tasks (such as repackaging, encryption, ABR transcoding, multi-source object coding, etc.) on media resources ingested at reference point M2d or M10d prior to distributing them from reference point M4d service locations. These processing tasks shall be specified in a Content Preparation Template resource referenced from a distribution configuration within the Content Hosting Configuration.

#### 7.6.4.5	URL signing

The URL signing procedure allows the 5GMSd Application Provider to prevent deep linking and unauthorized access to M4d transport resources. It works by cryptographically signing some elements of the M4d request URL and then appending this authentication token to the URL as an additional query parameter. The token is generated by the 5GMSd Application Provider and supplied to the player, for example as part of an initial URL. When it receives a request that requires URL signing, the 5GMSd AS verifies the presence and validity of the token in the M4d request URL before allowing access to the requested transport resource. The 5GMSd AS instance(s) and the origin share a secret that is encoded as part of the query parameter hash, but not shared with the 5GMSd Media Player.

The validity of the authentication token can also be limited to a single UE. If useIPAddress is set to True, then the public IP address of the UE as viewed by the 5GMSd AS, ue_public_ip_address, shall be incorporated into the token calculation. The parameter name shall be indicated by ipAddressName.

The shared secret shall be provided in the urlSignature.passphrase property of the Content Hosting Configuration resource. The parameter name for the passphrase to be used in the authentication token shall be provided by passphraseName.

The expiry time of the signed URL, tokenExpiry, shall be included as an additional query parameter in the URL exposed at M4d with the name indicated in tokenExpiryName. The expiry time shall be the string representation of the number of seconds from 1970-01-01T00:00:00Z UTC until the desired expiry UTC date/time, ignoring leap seconds, as defined in section 4.16 of POSIX.1 [11].

Given the above, the authentication token shall be calculated as:

token := SHA512(url&UrlSignature.tokenExpiryName=token_expiry&UrlSignature.ipAddressName=‌ue_public_ip_address&‌UrlSignature.passphraseName=passphrase)

where the SHA512 function shall be the SHA-512 hash [6] of the enclosed string. The url parameter shall be the original M4d transport resource request URL, including the scheme, authority and path components but excluding any query and fragment components.

The resulting token value shall be “base64url” encoded, as specified in section 5 of RFC 4648 [10], prior to inclusion in the M4d URL.

The query part of the signed URL presented by the 5GMSd Media Player at M4d as proof of authenticity shall be composed as follows:

query := urlSignature.tokenExpiryName=token_expiry&urlSignature.tokenName=base64url(token)

For all transport resources requested at reference point M4d that match the regular expression specified in urlSignature.‌urlPattern, whether modified by the 5GMSd AS or not, the 5GMSd AS shall validate the query presented in the request URL according to the following steps:

1)	If the parameter indicated by urlSignature.tokenName is absent from query, or if the supplied token value is malformed, the 5GMSd AS shall respond with a 403 (Forbidden) error response message and terminate further processing of the M4d request.

2)	If the parameter indicated by urlSignature.tokenExpiryName is absent from query, or if the supplied token_expiry value has expired, or if the supplied token_expiry is malformed, the 5GMSd AS shall respond with a 403 (Forbidden) error response message and terminate further processing of the M4d request.

3)	The 5GMSd AS shall compute the authentication token according to the token production specified above using the requesting UE’s public IP address as the value of ue_public_ip_address if required by urlSignature.useIPAddress being set to true. After applying “base64url” encoding, the 5GMSd AS shall compare this with the value supplied in the URL query parameter whose name is urlSignature.tokenName. If the two values differ, the 5GMSd AS shall respond with a 403 (Forbidden) error response message and terminate further processing of the M4d request.

4)	Otherwise, the presented authentication token is valid. The 5GMSd AS shall either return the requested transport resource in a 200 (OK) response message (if it is able to serve that transport resource), or else return an appropriate error response, such as 404 (Not Found) or 503 (Service Unavailable).

#### 7.6.4.6	Geofencing

The 5GMSd Application Provider may wish to limit access to the media content it makes available at reference point M2d to UEs located in certain geographical zones. Geofencing is used to configure the zone from which content is accessible.

The geoFencing.locatorType shall be set to one of the controlled term identifiers in the first column of table B.1-1 of TS 26.510 [56] and each member of the geoFencing.locators array in the distribution configuration shall then be set as specified in the third column of that table.

#### 7.6.4.7	Service chaining

The 5GMSd Application Provider may chain content hosting services by provisioning two or more Content Hosting Configurations as described in clause 5.2.8.2 of TS 26.510 [56] where:

-	At least one Content Hosting Configuration ingests media content from the 5GMSd Application Provider at reference point M2d.

-	Additional Content Hosting Configurations ingest media content from an upstream 5GMSd AS at reference point M10d where the IngestConfiguration.baseURL property of each Content Hosting Configuration corresponds to the DistributionConfiguration.baseURL property of an already provisioned Content Hosting Configuration in the upstream 5GMSd AS.

#### 7.6.4.8	Service location deployment

Reference point M4d service locations associated with distribution configurations within the Content Hosting Configuration are deployed within the 5GMS System at the discretion of the 5GMSd AF, taking into account any deployment affinity requirements as specified below.

The DistributionConfiguration.affinityGroup property within the Content Hosting Configuration (see clause 5.2.8.2 of TS 26.510 [56]) may be defined by the 5GMSd Application Provider to guide deployment of service locations within the 5GMSd AS according to the following:

-	The scope of the affinityGroup property applies only to those distribution configurations defined within a single Content Hosting Configuration. Service locations associated with distribution configurations with the same affinityGroup value but belonging to different Content Hosting Configurations may or may not be deployed together within the 5GMSd AS at the discretion of the 5GMSd AF.

-	When any two distribution configurations have the same affinityGroup value or when this property is not defined anywhere within a Content Hosting Configuration, the deployment of service locations within the 5GMSd AS is at the discretion of the 5GMSd AF.

-	When any two distribution configurations defined within a single Content Hosting Configuration have different affinityGroup values (including those that are not defined), a service location associated with one distribution configuration should not be deployed together with (e.g., at the same physical location) the service location associated with the other distribution configuration.

Content Publishing Provisioning API

### 7.6A.1	Overview

The API used by the 5GMSu Application Provider at reference point M1u to create and manipulate the 5GMSu AS Content Publishing Configuration associated with a particular uplink media streaming Provisioning Session in the 5GMSu AF is specified in clause 8.9 of TS 26.510 [56].

Within a Content Publishing Configuration, one or more contribution configurations may be defined where each may specify different caching, purging and content preparation behaviours for media resources contributed at reference point M4u or M10u. The Content Publishing Configuration may further specify, through the declaration of affinity groups, how reference point M4u or M10u service locations associated with each contribution configuration are deployed in the 5GMS System.

Metrics reporting PRovisioning API (M1)

### 7.8.1	Overview

The API used by the 5GMS Application Provider at reference point M1 to instantiate and manipulate Metrics Reporting Configurations associated with a particular downlink or uplink media streaming Provisioning Session in the 5GMS AF is specified in clause 8.11 of TS 25.510 [56].

In the case of downlink media streaming, the metrics scheme indicated in the scheme property of the Metrics Reporting Configuration shall be urn:‌3GPP:‌ns:‌PSS:‌DASH:‌QM10 and the QoE metrics (if any) listed in the metrics property shall be term identifiers from the vocabulary specified in clause E.2.1.

In the case of downlink DASH media streaming, any of the CMCD metrics schemes listed in table 7.8.1-1 below may be provisioned, each one in the scheme property of a separate Metrics Reporting Configuration. The metrics (if any) listed in the metrics property shall be term identifiers from the vocabulary specified in clause E.2.5.

Table 7.8.1-1: CMCD metrics scheme identifiers

| CMCD information class | Applicability | Metrics scheme identifier |
| --- | --- | --- |
| CMCD per-session information | Downlink | urn:3gpp:5gms:metrics:common-media-client-data:session |
| CMCD per-object information | Downlink | urn:3gpp:5gms:metrics:common-media-client-data:object |
| CMCD per-request information | Downlink | urn:3gpp:5gms:metrics:common-media-client-data:request |
| CMCD status information | Downlink | urn:3gpp:5gms:metrics:common-media-client-data:status |

Metrics related to Virtual Reality media, as specified in TS 26.118 [42] clause 9.3, may be listed in the metrics property of a metrics configuration when the scheme property indicates that metrics scheme. These shall be term identifiers from the vocabulary specified in clause E.2.2.

No standardised metrics schemes are defined in the present document for uplink media streaming.

Media ingest and publish (M2) protocols

# 8	Media ingest and publish (M2 and M10) protocols

## 8.1	General

The set of content protocols supported by the 5GMS AS is listed in table 8.1-1 below:

Table 8.1-1: Supported content protocols

| Description | Term identifier | Clause |
| --- | --- | --- |
| Content ingest protocols at reference point M2d or M10d |  |  |
| HTTP pull-based content ingest protocol | urn:3gpp:5gms:content-protocol:http-pull or urn:3gpp:5gms:content-protocol:http-pull-ingest (see NOTE) | 8.2 |
| DASH-IF push-based content ingest protocol | http://dashif.org/ingest/v1.2/interface-1 or http://dashif.org/ingest/v1.2/interface-2 or urn:3gpp:5gms:content-protocol:dash-if-ingest (see NOTE) | 8.3 |
| HTTP low-latency pull-based content ingest protocol | urn:3gpp:5gms:content-protocol:http-ll-pull | 8.4 |
| Content egest protocols at reference point M2u or M10u |  |  |
| HTTP pull-based content egest protocol | urn:3gpp:5gms:content-protocol:http-pull | 8.5 |
| DASH-IF push-based content egest protocol | http://dashif.org/ingest/v1.2/interface-1 or http://dashif.org/ingest/v1.2/interface-2 | 8.6 |
| HTTP low-latency pull-based content egest protocol | urn:3gpp:5gms:content-protocol:http-ll-pull | 8.7 |
| NOTE: Term identifier deprecated in this version of the present document. |  |  |

CMCD information conveyed to the 5GMSd AS at reference point M4d (see clause 10.5) shall not be included in requests to the 5GMSd Application Provider at reference point M2d.

## 8.2	HTTP pull-based content ingest protocol

The following provisions shall apply if IngestConfiguration.protocol is set to urn:3gpp:‌5gms:‌content-protocol:‌http-pull or to the deprecated value urn:‌3gpp:‌5gms:‌content-protocol:‌http-pull in the Content Hosting Configuration:

-	Media resources shall be ingested by the 5GMSd AS from the 5GMSd Application Provider or from another 5GMSd AS using HTTP [25].

NOTE 0:	Any supported HTTP protocol version may be used for HTTP pull-based content ingest at reference point M2d or M10d.

-	The IngestConfiguration.mode property shall be set to PULL, indicating that a pull-based protocol is used.

-	The IngestConfiguration.baseURL property shall point at the 5GMSd Application Provider's origin server or the DistributionConfiguration.‌baseURL property of another Content Hosting Configuration, as specified in table 8.8.3.1-1 of TS 26.510 [56], and may indicate the use of HTTPS [30].

When the 5GMSd AS receives a request for a media resource at reference point M4d service location that cannot be satisfied from its content cache, the request shall be transformed into a corresponding HTTP GET request directed to the 5GMSd Application Provider's origin server via interface M2d or to another 5GMSd AS via reference point M10d as follows:

1.	The prefix of the request URL indicated in the Distribution‌Configuration.‌baseURL of the applicable Content Hosting Configuration is replaced with that of the corresponding Ingest‌Configuration‌.baseURL.

NOTE 1:	It is the responsibility of the 5GMSd AF to assign unique M4d and M10d base URLs to each provisioned Content Hosting Configuration so as to ensure that this substitution is unambiguous.

2.	The path rewrite rules (if provisioned in DistributionConfiguration.pathRewriteRules) are applied in strict order to the remainder of the request URL (i.e., the path segments following Distribution‌Configuration.‌baseURL). The requestPathPattern of the first matching path rewrite rule is replaced with the corresponding mappedPath.

In the case where the 5GMSd Application Provider's origin server issues an HTTP 3xx redirect at reference point M2d pointing to another location, or an upstream 5GMSd AS issues such a redirect at reference point M10d, the 5GMSd AS shall issue an equivalent HTTP redirect to the Media Player via reference point M4d whose location is a dynamically generated M4d endpoint. Requests to this location shall be rewritten by the 5GMSd AS to the target location of the M2d or M10d redirection, as appropriate.

NOTE 2:	This explicit handling of HTTP redirects received by the 5GMSd AS at reference point M2d or M10d ensures that it is not bypassed by the Media Player. The general concept underlying this is commonly referred to as a "reverse mapping rule" by HTTP reverse proxies.

## 8.3	DASH-IF push-based content ingest protocol

The following provisions shall apply if IngestConfiguration.protocol is set to http://dashif.org/‌ingest/‌v1.2‌/interface-1 or http://dashif.org/‌ingest/‌v1.2/‌interface-2 or to the deprecated value urn:‌3gpp:‌5gms:‌content-protocol:‌dash-if-ingest in the Content Hosting Configuration:

-	Media resources shall be published by the 5GMSd Application Provider to the 5GMSd AS as specified by the DASH-IF Live Media Ingest specification [3].

NOTE:	The protocol in [3] is specified for use with HTTP/1.1 [24] only.

-	The IngestConfiguration.mode property shall be set to PUSH, indicating that a push-based protocol is used.

-	The IngestConfiguration.baseURL property shall be set by the 5GMSd AF to the base URL that is to be used by the 5GMSd Application Provider or by an upstream 5GMSd AS to upload the DASH segments and MPD(s) to the 5GMSd AS at reference point M2d or M10d respectively.

## 8.4	HTTP low-latency pull-based content ingest protocol

The provisions specified in clause 8.2 shall apply if IngestConfiguration.protocol is set to urn:3gpp:‌5gms:‌content-protocol:‌http-ll-pull.

In addition, if HTTP/1.1 [24] is used by at reference point M2d or M10d:

-	The requesting 5GMSd AS shall make partially received media segments available immediately for retrieval by 5GMS Clients at reference point M4d or M10d instead of waiting until the full segment is received.

-	The 5GMSd Application Provider should use HTTP chunked transfer coding as defined in section 7.1 of [24]. In this case, the requesting 5GMSd AS shall accept chunked HTTP/1.1 response messages and shall make partially received media segments (i.e., HTTP Chunks) available immediately for retrieval by 5GMS Clients at reference point M4d or M10d instead of waiting until the full segment is received.

-	If the DASH-IF Low Latency mode as defined in [63] is used, then the content is packaged as a series of CMAF Segments [40]. Further, each CMAF Segment is typically subdivided into one or more multiple CMAF Chunks to support low-latency content generation. According to the DASH-IF Live Media Ingest specification [3], each HTTP Chunk should contain at most one CMAF Chunk in order to minimise the latency.

NOTE:	Usage of HTTP/2.0 and HTTP/3 at reference points M2d and M10d is for future study.

## 8.5	HTTP pull-based content egest protocol

If EgestConfiguration.‌protocol is set to urn:3gpp:‌5gms:‌content-protocol:‌http-pull-egest in the Content Publishing Configuration, media resources shall be retrieved by the 5GMSu Application Provider from the 5GMSu AS at reference point M2u or by an upstream 5GMSu AS at reference point M10u using HTTP [25]. Media segments contributed to the 5GMSu AS by the 5GMSu Client shall be processed according to the Content Preparation Template(s) specified in the corresponding Content Publishing Configuration (if any) prior to making them available at reference point M2u or M10u.

In this case:

-	The EgestConfiguration.‌mode property shall be set to PULL, indicating that a pull-based protocol is used.

-	The EgestConfiguration.‌baseURL property shall be set by the 5GMSu AF to the base URL on the 5GMSu AS where it will publish media segments, presentation manifests and metadata for retrieval by the 5GMSu Application Provider at reference point M2u or by an upstream 5GMSu AS at reference point M10u.

-	The EgestConfiguration.‌entryPoint.‌relativePath property shall point at a Media Entry Point document below this base URL, as specified in table 8.9.3.1-1 of TS 26.510 [56], and may indicate the use of HTTPS [30]. This document describes the location of media content and associated metadata exposed by the 5GMSu AS at reference point M2u or M10u which are expected to be pulled by the 5GMSu Application Provider or by an upstream 5GMSu AS respectively.

In the absence of content preparation, the 5GMSu AS shall publish media resources by replacing the prefix Contribution‌Configuration.‌baseURL of its URL at M4u with that of the corresponding EgestConfiguration.‌baseURL.

## 8.6	DASH-IF push-based content egest protocol

If EgestConfiguration.‌protocol is set to http://dashif.org/‌ingest/‌v1.2/‌interface-1 or http://dashif.org/‌ingest/‌v1.2/‌interface-2 in the Content Publishing Configuration, media resources shall be published by the 5GMSu AS to the 5GMSu Application Provider at reference point M2u or to an upstream 5GMSu AS at reference point M10u as specified in the DASH-IF Live Media Ingest specification [3]. Media segments contributed to the 5GMSu AS by the 5GMSu Client shall be processed according to the Content Preparation Template(s) specified in the corresponding Content Publishing Configuration (if any) prior to publishing them at reference point M2u or M10u.

NOTE 1:	The protocol in [3] is specified for use with HTTP/1.1 [24] only.

NOTE 2:	A 5GMSu AS implementation that advertises support for the egest of content at reference point M2u or M10u using interface 2 of the DASH-IF Live Media Ingest specification [3] is required to produce a suitable DASH presentation manifest.

In this case:

-	The EgestConfiguration.‌mode property shall be set to PUSH, indicating that a push-based protocol is used.

-	The EgestConfiguration.‌baseURL property shall be set by the 5GMSu Application Provider to the base URL that is to be used by the 5GMSu AS to upload media segments and presentation manifests to the 5GMSu Application Provider at reference point M2u or to an upstream 5GMSu AS at reference point M10u.

If the 5GMSu Application Provider has provisioned an egest Media Entry Point, and if such document has been contributed to or produced by 5GMSu AS, the 5GMSu AS shall publish this document to the URL formed by the concatenation of EgestConfiguration.‌baseURL with EgestConfiguration.‌entryPoint.‌relativePath, as specified in table 8.9.3.1-1 of TS 26.510 [56]. This URL may indicate the use of HTTPS [30].

In the absence of any content preparation, each media resource uploaded at reference point M4u shall be published to the 5GMSu Application Provider at the URL formed by replacing the prefix Contribution‌Configuration.‌baseURL of its URL at M4u with that of the corresponding EgestConfiguration.‌baseURL.

## 8.7	HTTP low-latency pull-based content egest protocol

The following provisions shall apply if EgestConfiguration.protocol is set to urn:3gpp:5gms:content-protocol:http-ll-pull the following provisions shall apply.

The content shall be packaged as a series of CMAF Segments [40]. Each CMAF Segment shall be subdivided into multiple one or more CMAF Chunks.

In addition:

-	If HTTP/1.1 [24] is used at reference point M2u or M10u, partially available media segments may be accessed by the 5GMSu Application Provider using an HTTP byte range request, as specified in section 14 of RFC 9110 [25]. If the 5GMS Application Provider makes a byte-range request for a partially available media segment (the first media segment it retrieves) and the first-pos of that range is non-zero and the 5GMS Application Provider is expecting an aggregating response, then the 5GMS Application Provider should signal that expectation following the convention of IETF RFC 8673 [61]. Specifically, it should use a last-pos value of 9007199254740991. In this case, the 5GMSu AS is required to respond with a 206 (Partial Content) HTTP response without a Content-length response header instead of waiting for the end of the segment and responding with a 200 (OK) HTTP response code.

## 8.8	Key exchange for encrypted content

If the 5GMSd AS supports content encoding and packaging, but the DRM server (including License and Key Servers) is deployed externally in the 5GMSd Application Provider domain, then the 5GMSd AS needs to communicate with the License Server for content encoding and packaging.

In order to support exchange of public keys between the Encryptor/Packager on the 5GMSd AS and the external License Server and Authorization Server via reference point M2d, the 5GMSd AS shall support the Content Protection Information Exchange Format (CPIX) as specified in ETSI TS 103 799 [75] at reference point M2d.

Application Server configuration (M3) APIs

### 9.4.3	Data model

The representation of the Content Hosting Configuration resource used by the 5GMSd AF to configure the 5GMSd AS at reference point M3d shall be the same as that specified in clause 8.8.3.1 of TS 26.510 [56] except that:

-	externalServiceId is an additional read/write property of Application‌Server‌ContentHosting‌Configuration populated by the 5GMSd AF from the parent Provisioning Session.

-	canonicalDomainName and baseURL are read/write properties of ApplicationServer‌Distribution‌Configuration set by the 5GMS AF.

### 9.5.3	Data model

The representation of the Content Publishing Configuration resource used by the 5GMSu AF to configure the 5GMSu AS at reference point M3u shall be the same as that specified in clause 8.9.3.1 of TS 26.510 [56] except that:

-	externalServiceId is an additional read/write property of Application‌Server‌ContentPublishing‌Configuration populated by the 5GMSu AF from the parent Provisioning Session.

-	canonicalDomainName and baseURL are read/write properties of ApplicationServer‌Contribution‌Configurationset by the 5GMS AF.

Media Streaming (M4) interface

## 10.1	General

This clause specifies the interface for downlink and uplink media streaming at reference point M4 using different distribution formats and protocols. TS 26.511 [35] defines several media codecs and distribution formats for 5G Media Streaming. It also provides requirements and recommendations for the support of these media codecs and formats in profiles specific to 5G Media Streaming. However, 5GMS is not restricted to the media profiles and distribution formats defined in TS 26.511 [35]. For example, any CMAF media profile that integrates with the APIs specified in the present document may be used for media streaming at reference point M4.

A media resource shall be made available at reference point M4 as one or more transport resources where each transport resource shall be uniquely addressable.

## 10.1A	Media delivery session identification

All media requests addressed by the Media Stream Handler (Media Player or Media Streamer) to the 5GMS AS at reference point M4 shall cite a media delivery session identifier using the HTTP header specified in clause 6.2.3.6. The value of this identifier shall be different for every media streaming session. When a media delivery session uses multiple service locations, the value of this identifier shall be the same regardless of the service location used.

## 10.2	DASH distribution

In the case of DASH distribution, M4d is relevant for the distribution as shown in figure 10.2-1.

Figure 10.2-1: M4d usage for DASH distribution

For DASH-based distribution according to TS 26.247 [4] and ISO/IEC 23009-1 [32], two main formats are of relevance:

1)	The Media Presentation Description (MPD) that is processed in the DASH Access Client.

2)	The Segment formats that are passed through the DASH Access Client and processed in the Media Playback and Content Decryption Platform. Note that the DASH Access Client may parse Segments to extract, for example, In-band Events or producer reference times.

Other resources may be referenced in the MPD. Examples include:

-	Service locations in the form of BaseURL elements from which Segments can be downloaded.

-	Content steering instructions provided by a content steering service.

-	DRM related information.

The Segment formats for DASH Streaming in the context of 5G Media Streaming are defined in TS 26.511 [35] based on the CMAF encapsulation. The DASH Access Client downloads the Segments from the 5GMSd AS based on the instructions in the MPD and the instructions from the 5GMSd-Aware Application through M7d (see clause 13 for details).

The interface between the DASH Access Client and the Media Playback and Content Decryption Platform as well as the 5GMSd Client requirements for media codecs are documented in TS 26.511 [35].

The following requirements apply at reference point M4d:

1)	The Media Presentation Description (MPD) and Segments shall conform to an MPD according to ISO/IEC 23009-1 [32] or TS 26.247 [4].

2)	The Segment formats should conform to CMAF addressable resources as well as to the requirements in TS 26.511 [35].

3)	The Media Presentation should conform to the 5G Media Streaming DASH Interoperability Point as defined in clause 7.3.11 of TS 26.247 [4].

A 5GMSd Client shall support the 5G Media Streaming DASH Interoperability Point as defined in TS 26.247 [4], clause 7.3.11. A 5GMSd Client may support additional DASH profiles and interoperability points.

The MPD may contain a one or several ServiceDescription elements that include operational parameters. The MPD may also include multiple configurations for the media (different codecs, different content protection, different resolutions, etc.), for example for playback under different operating policies. For more details refer to annex G.

The handling of this information within the 5GMSd Client is documented in clause 13.2.

If the media segment formats conform to CMAF addressable resources as defined ISO/IEC 23000-19 [27], the same CMAF content may then be provided for DASH and HLS. In order to support common deployment, the media segment content should conform to CTA-5005-A [62].

## 10.3A	Content distribution from multiple service locations

### 10.3A.1	General

This clause extends clauses 10.2 and 10.3 to provide Media Access Client capabilities that allow for content distribution using multiple service locations exposed by the 5GMSd AS at reference point M4d. These capabilities may be used independently or in combination.

Necessary information required by the Media Access Client to use multiple service locations exposed by the 5GMSd AS at reference point M4d during a media streaming session may be provided within a Media Player Entry (or a document pointed to by a Media Player Entry) and/or by a content steering service. Service locations provided in the Media Player Entry and/or by a content steering service may be distinguishable and identifiable via their base URLs.

A Media Access Client that supports content distributed from multiple service locations should support the following:

-	Identification of the base URL(s) of the service location(s) to be used and the construction of the absolute URL(s) for the transport resources that apply to the media resource(s) to be acquired.

-	Acquiring transport resources associated with a media resource from one or more service locations.

-	Recovering or reconstituting media resources from the acquired transport resources and making them available to the Media Playback and Content Decryption Platform (as specified in TS 26.511 [35]) for immediate or delayed consumption.

Additional Media Access Client capabilities, depending on the method(s) used to acquire media resources from one or more service locations, are provided in the following clauses.

A Media Access Client accessing multiple service locations should take into account that the throughput estimation for one service location may not accurately predict the throughput of another service location.

### 10.3A.2	Media Access Client capabilities to support switching between multiple service locations during content distribution

The Media Access Client of the 5GMSd Client may have the capability to switch between service locations during a media streaming session.

When switching between multiple service locations, the Media Access Client of the 5GMSd Client should have the ability to select one or more service locations to use when acquiring a transport resource. For example, the service location(s) selected have the highest priority based on the contents of the Media Player Entry or signalling from a content steering service; or the Media Access Client selects the service location(s) using internal logic.

In case a previous request using a particular service location fails or the Media Access Client decides to switch to a new service location, another service location may be selected to acquire media resources.

### 10.3A.3	Media Access Client capabilities to support concurrent use of multiple service locations for content distribution

The Media Access Client of the 5GMSd Client may have the ability to acquire media resources from more than one service location in parallel (i.e., through simultaneous use of multiple service locations). When using multiple service locations concurrently, the Media Access Client of the 5GMSd Client should have the additional capabilities:

-	The ability to select more than one service location to be used to acquire transport resources. For example, the service locations selected have the highest priority for usage based on the contents of the Media Player Entry or signalling from a content steering service; the Media Access Client selects the service locations using internal logic; or all the identified service locations are used. In another example, the video transport resources and the audio transport resources may be offered from different service locations, and the Media Player needs to access both service locations in parallel to provide a full media experience.

-	The ability to acquire more than one transport resource simultaneously from the identified service locations, where each may be the original or a transformed (e.g., encoded) representation of the media resource. This may include downloading (either partially or in full) transport resources from more than one reference point M4d service location simultaneously.

### 10.3A.4	Media Access Client capabilities to support content steering for content distribution

In the case when content steering is used, the Media Access Client of the 5GMSd Client may communicate via reference point M4d with a content steering service provisioned within the 5GMSd AS to obtain information about which service location(s) to use to acquire transport resources at reference point M4d. Content steering information may be provided within the Media Player Entry (e.g., a DASH MPD) or in a document pointed to by the Media Player Entry.

When obtaining content steering information from the content steering service, the Media Access Client shall use a protocol appropriate to that service.

### 10.4.1	General

The contribution protocols supported by the 5GMSu AS at reference point M4u and M10u are listed in table 10.4.1-1 below:

Table 10.4.1-1: Supported contribution protocols at reference point M4u and M10u

| Description | Term identifier | Clause |
| --- | --- | --- |
| DASH-IF push-based content ingest protocol | http://dashif.org/ingest/v1.2/interface-1 | 10.4.2 |

### 10.4.2	DASH-IF push-based contribution protocol

If streamingAccess.‌entryPoints.‌protocol is set to http://dashif.org/ingest/v1.2/interface-1 in the Service Access Information, media resources shall be streamed to the 5GMSu AS as specified by the DASH-IF Live Media Ingest specification Interface-1 [3]. The content shall conform to at least one of the conformance profiles listed in streamingAccess.profiles, if any.

The content uploaded to the 5GMSu AS using this protocol is processed according to the Content Preparation Template(s) specified in the corresponding Content Publishing Configuration (if any), and the result is made available to the 5GMSu Application Service Provider at reference point M2u or to another 5GMSu AS at reference point M10u using the egest protocol indicated in EgestConfiguration as specified in clause 8.

### 10.4.3	Concurrent use of service locations for content contribution

The Media Streamer Entry (or a document pointed to by the Media Streamer Entry) shall be used to identify multiple service locations exposed by the 5GMSu AS at reference point M4u and the method used to contribute transport resources to the 5GMSu through simultaneous use of those multiple service locations. The following applies:

-	Service locations provided in the Media Streamer Entry shall be distinguishable and identifiable via their base URLs.

-	Any necessary configuration information required by the Access Client of the 5GMSu Client to contribute transport resources to multiple service locations concurrently shall be provided within the Media Streamer Entry.

When using multiple service locations concurrently, it is the responsibility of the Media Access Client of the 5GMSu Client to:

-	Identify the different base URLs in the Media Streamer Entry that apply to the media resource to be contributed.

-	Select one or more service locations to be used to upload the media resource. For example, the service locations selected have the highest priority for usage based on the contents of the Media Streamer Entry, the Access Client selects the service locations using internal logic, or all the identified service locations are used.

-	If applicable, identify the absolute URLs or combine the base URLs of the selected service locations with the relative path of the transport resource identified in the Media Streamer Entry. In the case where uniquely identifiable transport resources are used to contribute partial representations of the media resource to each of the selected service locations, the Media Access Client shall either identify or construct the absolute URLs of the transport resource used to transmit the original media resource to each service location.

-	Prepare and upload the transport resources containing complete or partial representations of the media resource to the identified service locations. This may include uploading (either partially or in full) the media resource and/or the transport resources to one or more reference point M4u service locations concurrently.

## 10.5	In-band client data reporting

### 10.5.1	CMCD-based client data reporting

If one or more of the CMCD metrics schemes specified in table 7.8.1-1 is indicated in the client metrics reporting configurations provided to the Media Session Handler, the corresponding class(es) of CMCD information shall be collected by the Media Player and reported to the 5GMSd AS at reference point M4d per [70]. In addition:

-	The currently effective Media Player Entry may restrict the subset of Service Locations and/or media object types for which CMCD information is reported. These are honoured by the Media Player.

-	The Media Player shall include in every CMCD report the current media delivery session identifier in the sessionId property of the MetricsSession (see clause 9.5.3.2 of TS 26.510 [56]). If the currently effective Media Player Entry declares a session identifier of its own for the purpose of client data reporting, this is typically used to populate the CMCD sid key (e.g., as required by clauses K.3.7 and K.4.2.7 of ISO/IEC 23009-1 [32]); otherwise, this key shall be populated with the current media delivery session identifier.

-	If available in its configuration (i.e., if provided via the configuration and settings API specified in clause 13.2.4 or via the relevant query parameter of the 3GPP Service URL specified in clause 12.4, as applicable), the Media Player shall include in every CMCD report the content identifier of the media streaming asset being consumed in the contentId property of the MetricsSession (see clause 9.5.3.2 of TS 26.510 [56]). If the currently effective Media Player Entry declares a content identifier of its own for the purpose of client data reporting, this is typically used to populate the CMCD cid key (e.g., as required by clauses K.3.7 and K.4.2.7 of ISO/IEC 23009-1 [32]); otherwise, this key shall be populated with the current content identifier in the Media Player’s configuration, if available.

-	The Media Player Entry may explicitly specify a set of CMCD keys to be reported (e.g., as required by clauses K.3.7 and K.4.2.7 of ISO/IEC 23009-1 [32]). These are typically reported by the Media Player in addition to those configured in the currently effective client metrics reporting configuration.

-	A Media Player supporting CMCD metrics may implement the collection and reporting of all metrics defined in CMCD or only a subset. In the context of the present document, a Media Player shall support at least the sid key.

-	If both the metricsConfiguration[ ] and the cmcdReporting are present in the configuration and settings API specified in clause 13.2.4 and they configure different metrics, the Media Player shall report the union of the two sets.

-	The use by the Media Player of HTTP request headers or URL query parameters to report CMCD information is governed by the configuration and settings API specified in clause 13.2.4 or by the relevant query parameter of the 3GPP Service URL specified in clause 12.4 (as applicable). This value shall take precedence over any client data reporting mode indicated in the currently effective Media Player Entry.

NOTE:	Section 3.1 of the CMCD specification [70] recommends that HTTP request headers and URL query parameters are sent in alphabetical order of key name when transmitted. This reduces the fingerprinting surface exposed by the Media Player.

Clause G.5 specifies how to instantiate CMCD-based client data reporting in DASH.

Media Session Handling API (M5/M3)

# 11	Media Session Handling (M5/M3) APIs

## 11.1	General

This clause defines the Media Session Handling APIs used by the Media Session Handler at reference point M5d and by the 5GMS AS at reference point M3d to access resources exposed by the 5GMS AF.

NOTE:	While the entirety of the Media Session Handling APIs apply to downlink media streaming, only a subset is applicable to uplink media streaming. Specifically, the Consumption Reporting API is not applicable to uplink media streaming.

### 11.2.1	General

The API used by the Media Session Handler at reference point M5d and by the 5GMS AS at reference point M3d to acquire Service Access Information from the 5GMS AF is specified in clause 9.2 of TS 26.510 [56]. The Service Access Information enables the Media Session Handler and/or 5GMS AS (as applicable) to go on to use the other Media Session Handling APIs specified in clause 11.3 et seq.

#### 11.3.3.1	ConsumptionReport data type

The ConsumptionReport data type is specified in clause 9.6.3.1 of TS 26.510 [56].

In the case of downlink media streaming with DASH [32]:

-	The mediaPlayerEntry property shall be populated with the URL of the Media Player Entry (e.g., a MPD resource or a document pointing to the MPD resource) that was retrieved at reference point M4d after following any HTTP redirects.

-	A separate Consumption Reporting Unit shall be reported in the consumptionReportingUnits array for each DASH Adaptation Set currently selected for presentation by the Media Player.

For other types of media streaming, the content of these properties is undefined.

#### 11.3.3.2	ConsumptionReportingUnit type

The ConsumptionReport data type is specified in clause 9.6.3.2 of TS 26.510 [56].

In the case of downlink media streaming with DASH [32]:

-	The mediaConsumed property shall indicate the content identifier currently being consumed (if known to the 5GMS Client) and which DASH Representation is currently selected by the Media Player for the DASH Adaptation Set in question by citing the value of its Representation@id from the DASH MPD. If both the content identifier and the DASH Representation identifier are available, they shall be provided in that order with a single pipe character "|" delimiting them, and this character shall not be present in the value of either identifier.

For other types of media streaming, the content of this property is undefined.

### 11.4.1	General

The API used by the Media Session Handler at reference point M5 and by the 5GMS AF at reference point M3 to submit metrics reports to the 5GMS AF is specified in clause 9.5 of TS 26.510 [56]. Metrics reports shall be submitted according to the metrics scheme indicated in each client metrics reporting configuration described by the Service Access Information resource (see clause 9.2.3.1 of TS 26.510 [56]) at the applicable reference point.

NOTE:	Multiple metrics configurations may be active at the same time in the Service Access Information.

-	If the metrics scheme urn:‌3GPP:‌ns:‌PSS:‌DASH:‌QM10 is indicated in the client metrics reporting configuration, QoE metrics for DASH-based downlink media streaming shall be reported by the Media Session Handler to the 5GMSd AF at reference point M5d per clause 11.4.3.2.

-	If the metrics scheme specified in clause 9.3 of TS 26.118 [42] is indicated in the client metrics reporting configuration, QoE metrics related to Virtual Reality media shall be reported by the Media Session Handler to the 5GMSd AF at reference point M5d per clause 11.4.3.2.

-	If one or more of the metrics schemes specified in table 7.8.1-1 is indicated in the client metrics reporting configurations provided to the 5GMSd AS, QoE metrics reports containing the corresponding class(es) of CMCD information shall be reported by the 5GMSd AS to the 5GMSd AF at reference point M3d per clause 11.4.3.3.

### 11.4.3	Report format

#### 11.4.3.1	General

Metrics reports shall be submitted to the 5GMS AF in a format specified by the metrics reporting scheme in question. The Content-Type HTTP request header shall be set in accordance with the relevant metrics reporting scheme specification.

#### 11.4.3.2	QoE metrics reporting for DASH-based downlink media streaming

For DASH-based downlink media streaming sessions, the following applies when QoE metrics reporting at reference point M5d has been provisioned:

-	When the metrics scheme 3GPP urn:‌3GPP:‌ns:‌PSS:‌DASH:‌QM10 is indicated in the client metrics reporting configuration of the Service Access Information at reference point M5d, metrics shall be reported by the Media Session Handler according to the quality reporting scheme specified in clause 10.5 of TS 26.247 [7]. Clauses 10.6.1 and 10.6.2 respectively of TS 26.247 [7] specify the required MIME media type and document format for submitting metrics reports to the 5GMSd AF at reference point M5d.

-	When the metrics scheme for Virtual Reality media is indicated in the client metrics reporting configuration of the Service Access Information at reference point M5d, the format of metrics reports submitted to the 5GMSd AF at reference point M5d is further extended as defined in clause 9.4 of TS 26.118 [42].

In both cases, the ReceptionReport@clientID attribute should be populated with a GPSI value as defined by TS 23.003 [7], if present and available to the Media Session Handler. Otherwise, this attribute should be represented by a stable and globally unique string.

In both cases, the QoEReport@recordingSessionId attribute shall be populated with the media delivery session identifier.

#### 11.4.3.3	Client data reporting

##### 11.4.3.3.1	General

The syntax and MIME media type of metrics reports conveying client data to the 5GMSd AF shall comply with the JSON-based metrics reporting envelope specified in clause 9.5.3.2 of TS 26.510 [56]. The mediaStreaming‌ClientData property (of type MediaStreamingClientData, as specified in table 11.4.3.3.1-1 below) shall be present in every MetricsSample object contained in this metrics reporting envelope.

Table 11.4.3.3.1-1: Definition of MediaStreamingClientData data type

| Property name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| requestUrl | AbsoluteUrl | 0..1 | The client request URL to which this client data sample pertains (see NOTE). |
| cmcdSessionInfo | CmcdSessionInfo | 0..1 | CMCD per-session information (see table 11.4.3.3-2). |
| cmcdObjectInfo | CmcdObjectInfo | 0..1 | CMCD per-request information (see table 11.4.3.3-3). |
| cmcdRequestInfo | CmcdRequestInfo | 0..1 | CMCD per-object information (see table 11.4.3.3-4). |
| cmcdStatusInfo | CmcdStatusInfo | 0..1 | CMCD status information (see table 11.4.3.3-5). |
| NOTE: Data type AbsoluteUrl is specified in TS 26.510 [56]. |  |  |  |

The OpenAPI syntax of the above data type is specified annex C.6.

##### 11.4.3.3.2	Client data reporting for DASH-based downlink media streaming

For DASH-based downlink media streaming sessions, the following applies for the reporting of client data by the 5GMSd AS to the 5GMSd AF at reference point M3d:

-	When one or more of the CMCD metrics schemes listed in table 7.8.1-1 is indicated in client metrics reporting configurations of the Service Access Information provided to the 5GMSd AS at reference point M3d, the syntax and MIME media type of metrics reports conveying CMCD information [70] to the 5GMSd AF at reference point M3d shall comply with clause 11.4.3.3.1 above, and at least one of the following properties shall be populated in this ClientData object: cmcdSessionInfo, cmcdObjectInfo, cmcdRequestInfo, cmcdStatusInfo.

NOTE:	The syntax of these objects aligns with the payload definition for JSON transmission specified in section 3.1 of the CMCD specification [70].

Aligned with section 3.1 of the CMCD specification [70], key–value pairs conveyed by these objects should appear in alphabetical order of key name when transmitted. (This reduces the fingerprinting surface exposed by different sources of client data.)

The OpenAPI syntax of the following data types is specified annex C.6.

Table 11.4.3.3.2-1: Definition of CmcdSessionInfo data type

| Property name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| v | integer | 0..1 | CMCD specification version. If omitted, version 1 is implied. |
| sid | string | 1..1 | Media delivery session identifier. |
| cid | string | 0..1 | Content identifier. |
| st | CmcdStreamType | 0..1 | Stream type (see table 11.4.3.3.2-5). |
| sf | CmcdStramingFormat | 0..1 | Streaming format (see table 11.4.3.3.2-6). |
| pr | number | 0..1 | Playback rate. |

Table 11.4.3.3.2-2: Definition of CmcdObjectInfo data type

| Property name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| ot | CmcdObjectType | 0..1 | Object type (see table 11.4.3.3.2-7). |
| d | integer | 0..1 | Object duration expressed in milliseconds. |
| br | integer | 0..1 | Encoded bit rate expressed in kilobits per second. |
| tb | integer | 0..1 | Top bit rate expressed in kilobits per second. |

Table 11.4.3.3.2-3: Definition of CmcdRequestInfo data type

| Property name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| su | boolean | 0..1 | Start-up. |
| mtp | integer | 0..1 | Measured throughput expressed in kilobits per second. |
| dl | integer | 0..1 | Deadline expressed in milliseconds. |
| bl | integer | 0..1 | Buffer length expressed in milliseconds. |
| nor | RelativeUri | 0..1 | Next object request (see NOTE). |
| nrr | string | 0..1 | Next range request. |
| NOTE: Data type RelativeUri is specified in TS 26.510 [56]. |  |  |  |

Table 11.4.3.3.2-4: Definition of CmcdStatusInfo data type

| Property name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| rtp | integer | 0..1 | Requested throughput expressed in kilobits per second. |
| bs | boolean | 0..1 | Buffer starvation. |

Table 11.4.3.3.2-5: Definition of CmcdStreamType enumeration

| Enumeration value | Description |
| --- | --- |
| v | All media segments are available, e.g. Video-on-Demand. |
| l | Media segments become available over time, e.g. live. |

Table 11.4.3.3.2-6: Definition of CmcdStreamingFormat enumeration

| Enumeration value | Description |
| --- | --- |
| d | MPEG-DASH [32]. |
| h | HTTP Live Streaming (HLS). |
| s | Smooth Streaming. |
| o | Other. |

Table 11.4.3.3.2-7: Definition of CmcdObjectType enumeration

| Enumeration value | Description |
| --- | --- |
| m | Text, e.g. manifest or playlist. |
| a | Audio only. |
| v | Video only. |
| av | Multiplexed audio and video. |
| i | Initialisation segment. |
| c | Caption or subtitle. |
| tt | ISO BMFF Timed Text track. |
| k | Cryptographic key, licence or certificate. |
| o | Other. |

UE Media Session Handling (M6/M11) APIs
For uplink and downlink

## 12.4	3GPP Service URL for 5G Media Streaming

The 3GPP Service URL for 5G Media Streaming is based on the generic 3GPP Service URL defined in clause 6 of TS 26.510 [56].

If the service type discriminator service in the URL indicates ms, then the target service is a 5G Media Streaming service.

The parameters of the 3GPP Service URL for 5G Media Streaming are defined in table 12.4-1.

Table 12.4-1: 3GPP Service URL parameters for 5G Media Streaming

| Path element | Cardinality | Description |  |
| --- | --- | --- | --- |
| service_id | 1 | An External Service Identifier that resolves to a Provisioning Session in the 5GMS System. |  |
| Query parameter | Cardinality | Description |  |
| af-host-address | 0..* | The Fully Qualified Domain Name and optional port number of a 5GMS AF endpoint to be used by the Media Session Handler at reference point M5 with the format hostname[:port]. More than one occurrence of this parameter may be present in the Service URL to indicate alternative host endpoint addresses. Any of these may be used by the Media Session Handler at reference point M5. Supplied by the invoking 5GMS-Aware Application when the 5GMS AF is deployed in an External DN. The endpoint address(es) may, for example, have been passed to the 5GMS-Aware Application via reference point M8. If omitted, the Media Session Handler assumes the default 5GMS AF host endpoint address ms.af.3gppservices.org:443 is to be used at reference point M5. |  |
| access-token | 0..1 | A token that is presented by the Media Session Handler to the 5GMS AF at reference point M5 that asserts its right to invoke the media session handling operations exposed by the 5GMS AF. |  |
| service-operation-point | 0..1 | A reference to the target Service Operation Point of the 5G Media Streaming session. Overrides any default Service Operation Point signalled in the resource pointed to by media-entry-point. |  |
| content-id | 0..1 | Identifies the media consumed. If provided, this value shall be used by the Media Session Handler to populate the mediaConsumed property of ConsumptionReportingUnit (see clause 11.3.3.2). If provided, this value shall be used to populate the content identifier field for in-band client data reporting, such as the cid key defined by CMCD [70]. |
| estimated-volume | 0..1 | An estimate of the volume of media data to be transferred between the 5GMS Client and the 5GMS AS at reference point M4 during the 5G Media Streaming session. Required if service-operation-point or media-entry-point refers to a Policy Template with an associated Background Data Transfer policy. |  |
| media-entry-point | 0..1 | A Media Entry Point reference expressed as a fully qualified URL per RFC 3986 [41], suitable for presentation to a 5GMS AS at reference point M4. If supplied, used by the Media Session Handler to launch the Media Stream Handler (Media Player or Media Streamer) after successfully initiating media session handling. |  |
| delivery-protocol | 0..* | A content delivery protocol suggested to be used by the Media Stream Handler (Media Player or Media Streamer) for media delivery at reference point M4. - The value HTTP_1_1 indicates the use of the HTTP/1.1 protocol specified in RFC 9112 [24]. - The value HTTP_2 indicates the use of the HTTP/2 protocol specified in RFC 9113 [31]. - The value HTTP_3 indicates the use of the HTTP/3 protocol specified in RFC 9114 [60]. More than one occurrence of this parameter may be present in the Service URL indicating multiple choices in decreasing order of preference. |
| multipath-mode | 0..1 | Indicates a preference for using multipath connections at reference point M4. When present, the Media Stream Handler (Media Player or the Media Streamer) may establish multipath connections over one or more access networks active in the UE (see clause 4.9.1A). If omitted, the Media Stream Handler may independently choose whether or not to enable multipath delivery at reference point M4 depending on the selected content delivery protocol (see delivery-protocol parameter above) and its capability to use multipath protocols (see table 13.2.4-2). |
| client-data-reporting-mode | 0..1 | A string from the following enumeration that controls the mode of in-band client data reporting by the Media Stream Handler (Media Player or Media Streamer) to the 5GMS AS at reference point M4: - cmcd-headers indicates that CMCD key–value pairs are required to be reported in HTTP request headers per sections 2.1 and 3.2 of [70]. - cmcd-query indicates that CMCD key–value pairs are required to be reported as query parameters of the request URL per sections 2.2 and 3.2 of [70]. It is an error to request more than one of the above CMCD reporting modes in the same 3GPP Service URL. This value shall take precedence over any client data reporting mode indicated in the selected Media Entry Point. If omitted, no CMCD-based in-band client data reporting is required by the invoker, but a directive in the selected Media Entry Point (e.g., the mode parameter specified in clauses K.3.7 and K.4.2.7.2 of ISO/IEC 23009-1 [32]) is honoured by the Media Stream Handler as required. |
| content-type | 0..* | A MIME content type string conforming to section 5 of RFC 2045 [57] identifying a type of Media Entry Point that is acceptable to the Media Stream Handler (Media Player or Media Streamer). More than one occurrence of this parameter may be present in the Service URL to indicate that more than one type of Media Entry Point is acceptable. Used by the Media Session Handler to eliminate unacceptable Media Entry Points from those listed in the Service Access Information. It is an error to supply this parameter if an explicit Media Entry Point is specified using media-entry-point. |  |
| profile | 0..* | A fully-qualified term identifier from a controlled vocabulary specified outside the scope of the present document identifying a profile of Media Entry Point that is acceptable to the Media Stream Handler (Media Player or Media Streamer). The term identifier shall be formatted as a URI according to RFC 3986 [41]. More than one occurrence of this parameter may be present in the Service URL to indicate that more than one type of Media Entry Point is acceptable. Used by the Media Session Handler to eliminate unacceptable Media Entry Points from those listed in the Service Access Information. It is an error to supply this parameter if an explicit Media Entry Point is specified using media-entry-point. |  |

The service_id path element, and the af-host-address and access-token query parameters correspond to the baseline Service Access Information for downlink media streaming specified in clause 4.2.3 of TS 26.501 [2] and the baseline parameters of the 3GPP Service URL for 5G Media Streaming defined in clause 4.10.2 of [2]. Together, they enable a full set of Service Access Information to be retrieved by the Media Session Handler from the 5GMS AF using the Service Access Information API at reference point M5 specified in clause 11.2 of the present document.

The content-id parameter is used to populate consumption reports (see clauses 4.7.4 and 11.3.3) and/or metrics reports conveying client data (see clauses 4.7.5, 10.5 and 11.4.3).

The service-operation-point parameter is used to support the procedure where the desired Service Operation Point is known a priori to the invoker and/or is not encoded in the Media Entry Point.

The estimated-volume parameter is used to support the procedure where the invoker intends the launched 5G Media Streaming session to be used for the purpose of Background Data Transfer.

The media-entry-point query parameter is used to support the procedure where the Media Session Handler launches media playback in the Media Stream Handler (Media Player or Media Streamer) after successfully retrieving a full set of Service Access Information via reference point M5 (if needed) and after successfully initiating media session handling.

The delivery-protocol query parameter is used to suggest to the Media Stream Handler (Media Player or Media Streamer) the use of a specific content delivery protocol for media delivery at reference point M4. If multiple instances of this parameter are present in the URL, they indicate a decreasing order of preference, and the Media Stream Handler (Media Player or Media Streamer) should select the most preferred content delivery protocol that it supports, taking into account any preference to use a multipath transport protocol (see multipath-mode parameter below).

The multipath-mode parameter is used to indicate a preference to use multipath transport connections for media delivery at reference point M4. When this parameter is included, the Media Stream Handler (Media Player or Media Streamer) may establish multipath transport connections with the 5GMS AS at reference point M4 as specified in clauses 4.6.1, 4.6.2, and 4.9.1A of the present document. In the absence of this parameter, the Media Stream Handler (Media Player or Media Streamer) is not precluded from establishing multipath connections for media delivery at reference point M4 according to its capabilities.

The client-data-reporting-mode query parameter is used to select between different reporting modes for in-band client data provided by the 5GMS Client at reference point M4.

The remaining query parameters are used for client-side filtering of Media Entry Point information provided in the Service Access Information and selection of one Media Entry Point by the Media Session Handler. (They are mutually exclusive with the media-entry-point parameter.) In this case, media playback by the Media Stream Handler (Media Player or Media Streamer) is launched by the Media Session Handler with its chosen Media Entry Point.

If the 5GMS-Aware Application prefers to launch media streaming itself (rather than have the Media Session Handler launch media streaming on its behalf), the media-entry-point query parameter and all client-side filtering parameters shall be omitted from the 3GPP Service URL. In this case, the Media Session Handler initiates only media session handling for the 5GMS Provisioning Session identified by the External Service Identifier.

DASH Media Player APIs and functions (M7d/M11d)

### 13.2.1	Overview

In the following, it is assumed that the Media Player (in this case a DASH client) adheres to a basic set of functionalities as shown in figure 13.2-1. The DASH client downloads, processes and presents a DASH Media Presentation under the control of a 5GMSd-Aware Application via reference point M7d or of the Media Session Handler via reference point M11d.

The 5GMSd-Aware Application may, in addition, configure the presentation of the media, receive notifications on events, or query the internal status of the DASH Player, also supported through reference point M7d. Different functions of the DASH Access Client that are typically necessary to process a DASH Media Presentation, are shown in figure 13.2-1. Additional functions may be available as well.

Figure 13.2.1-1: Architecture of DASH-based 5GMSd Client

The key functionalities of each of the functions as shown in figure 13.2-1 are summarized in the following:

-	5GMSd-Aware Application: Application that makes use of the DASH-based Media Player to play back a DASH Media Presentation using the APIs defined in this clause.

-	Media Player: A complete player for the playback of a Media Presentation, including the Media Playback and Content Decryption Platform as defined in TS 26.511 [35].

-	Access Client: A part of the DASH Player that accesses and downloads of the resources and provides the downloaded resources to the Media Playback Platform and Content Decryption for the playback of DASH content.

-	Management: Controls all internal processes and the communication with the 5GMSd-aware application. In particular this includes the handling of service descriptions and operation points.

-	MPD Processing: parses and processes the MPD and extracts the relevant information.

-	Adaptation Set Selection: selects the Adaptation Set based on user, application and/or device capability information. Information provided through M7d may be used.

-	ABR Controller and Dynamic Switching: runs adaptive bit rate logic and triggers adaptive switching of Representations. Information provided to the Media Player via reference point M7d may be used.

-	Throughput Estimation: estimates the network status, i.e. throughput, congestion information on the transmission link between the Media Player and the 5GMSd Application Server. Information provided to the Media Player via reference point M11d (i.e., QoS monitoring results) may be considered as input to the throughput estimation.

When ECN marking for L4S according to RFC 9330 [67], RFC 9331 [68] and RFC 9332 [69] is activated (as notified by the Media Session Handler using the L4S_ENABLED notification at reference point M11d) ECN marking of congestion in downlink packets may additionally be considered as input to throughput estimation, subject to the availability of a suitable low-level API.

-	Metrics Collection: logs relevant low-level metrics and provides those to the metrics aggregation and reporting functions in the Media Session Handler or to the Downloader for in-band client data reporting.

-	Media Playback Management and Protection Controller: manages the media playback by moving downloaded information into the media playback platform and also addresses handling of protection and DRM-related information.

-	Media Playback and Content Decryption Platform: plays back CMAF-based media content according to the playback requirements in TS 26.511 [35]. It also provides status information as well as events that may be provided through M7d.

-	Event Processing: Processes DASH events and provides information to the 5GMSd-Aware Application as defined in TS 26.247 [4].

-	Downloader: Retrieves resources from one or more reference point M4d service locations. It may optionally support switching between service locations as described in clause 10.3A.2, communication with a content steering service as described in clause 10.3A.4, and the download and decoding of coded objects as described in clause 10.3A.3.

This clause focuses on interactions with the Media Player through reference point M7d. In particular, the following aspects of the API are defined:

1)	Methods to interact with the Media Player at this reference point are defined in clause 13.2.3.

2)	Notification and Error Events raised by the Media Player at this reference point are defined in clause 13.2.4.

3)	Configuration and Settings of the Media Player at this reference point are defined in clause 13.2.5.

4)	Status Information exposed by the Media Player at this reference point is defined in clause 13.2.6.

Communication between the Access Client and the media playback platform of the Media Player is defined in TS 26.511 [35].

A 5GMSd Client for DASH distribution shall support the APIs defined in this clause 13.

NOTE:	The initial APIs have largely been designed based on the dash.js APIs documented here: http://cdn.dashjs.org/latest/jsdoc.

### 13.2.4	Configurations and settings API

DASH streaming for a particular downlink media delivery session may be configured by the 5GMSd-Aware Application at reference point M7d or by the Media Session Handler at reference point M11d with the parameters provided in table 13.2.4-1. Note that these parameters may be set and they may also be observed.

Table 13.2.4-1: Media Player Configuration API

| Status |  |  | Type | Definition |
| --- | --- | --- | --- | --- |
| sessionId |  |  | string | A media delivery session identifier for the downlink media streaming session that has been initialised using the method specified in clause 13.2.3.2. |
| capabilities |  |  | array(Enumeration) | A read-only list of Media Player capabilities. See table 13.2.4-2. |
| source |  |  | Object | Provides the MPD and all contained information. |
| consumptionMode |  |  | Enum | Defines two modes: live: in this case the target latency is maintained, if specified in the service description, according to the parameters vod: in this case the latency is set by the application and the latency settings are ignored. |
| maxBufferTime |  |  | Integer | Maximum buffer time in milliseconds for the service. |
| serviceDescriptionId |  |  | id | Selects a service description by selecting an identifier. |
| serviceDescriptions[] |  |  | Service description parameters | Configures a service description as defined in annex K of ISO/IEC 23009-1 [32]. This allows the application to define additional service descriptions beyond those defined in the MPD, including a content identifier. |
|  | id |  | id | Sets a service description identifier different from the ones available in the service descriptions in the MPD or modifies existing service descriptions. |
|  | serviceLatency |  | Object | Sets service description parameters for the service latency, as defined in table K.3.1-1 of ISO/IEC 23009-1 [32]. |
|  | playBackRate |  | Object | Sets service description parameters for the playback rate, as defined in table K.3.2-1 of ISO/IEC 23009-1 [32] when the service is consumed in live mode. |
|  | operatingQuality |  | Object | Sets service description parameters for the operating quality, as defined in table K.3.4-1 of ISO/IEC 23009-1 [32]. |
|  | operatingBandwidth |  | Object | Sets service description parameters for the operating bandwidth, as defined in table K.3.5-1 of ISO/IEC 23009-1 [32]. |
|  | clientDataReporting |  | Object | Sets service description parameters for client data reporting as defined in table K.3.7-1 of ISO/IEC 23009-1 [32]. These parameter values shall take precedence over the equivalent value specified in the selected Media Entry Point. |
|  |  | cmcdReporting | Object | Sets service description parameters for the CMCD reporting scheme as defined in table K.3.7-2 of ISO/IEC 23009-1 [32]. If the sid CMCD key is present in the service description parameters, it shall have the same value as sessionId above. |
| mediaSettings[] |  |  | Media type audio, video, subtitle | Sets the selected Adaptation Set based on the available Adaptation Sets for each media type. |
| metricsConfiguration[ ] |  |  | Object | Zero or more sets of settings for collecting and/or reporting metrics in relation to the downlink media streaming session. See table 13.2.4-3. The key of the array is a metrics reporting scheme identifier URI. |
| desired‌Content‌Delivery‌Configuration |  |  | Object | Desired configuration of content delivery at reference point M4d. |
|  | delivery‌Protocols |  | array(Enumeration) | Enumerated values from table 13.2.4-4, listed in decreasing order of preference, to be used by the Media Player for media delivery at reference point M4d. |
|  | multipathMode |  | Enumeration | An enumerated value from table 13.2.4-5 indicating a preference on the use of multipath transport connections at reference point M4. When this is preferred, a multipath-capable Media Player should negotiate the establishment of multipath transport connections according to RFC 8484 [72] or [73] as appropriate for the selected delivery protocol. |

Table 13.2.4-2: Media Player capabilities enumeration

| Value | Definition |
| --- | --- |
| CAPABILITY_L4S | The Media Player has a protocol stack capable of handling ECN marking for L4S according to RFC 9330 [67], RFC 9331 [68] and RFC 9332 [69] (see NOTE). |
| CAPABILITY_QOS | The Media Player is capable of reacting to QoS monitoring results (see NOTE). |
| NOTE: Monitoring of progress on industry support for exposure of ECN marking for L4S and QoS monitoring is expected. |  |

Table 13.2.4-3: Metrics configuration object

| Status | Type | Definition |
| --- | --- | --- |
| reporting‌Start‌Offset | DurationSec | The time offset (expressed in seconds) from the start of a media delivery session when the Media Client is required to begin submitting metrics reports. The value shall not be negative. If omitted, the value of this parameter is assumed to be zero, i.e., directing the Media Player to start collecting and reporting metrics from the start of the media delivery session. |
| reporting‌Duration | DurationSec | The period of time (expressed in seconds) measured relative to the reporting start point, after which the Media Client is required to stop reporting metrics. The value shall not be negative. If set to zero, a single report shall be sent at reporting‌Start‌Offset. If omitted, reporting is required to continue until the end of the media delivery session. |
| reporting‌Interval | DurationSec | The time interval between successive metrics reports to be sent by the Media Session Handler. The value shall be greater than zero. If not specified, a single final report shall be sent after the media delivery session has ended. |
| sample‌Percentage | Percentage | The proportion of media delivery sessions for which QoE metrics shall be reported, expressed as a floating-point value between 0.0 and 100.0. If not specified, reports shall be sent for all media delivery sessions. |
| urlFilters | array(string) | A list of Media Entry Point URL patterns for which QoE metrics shall be reported. If not specified, reporting shall be done for all media delivery sessions initiated within the scope of the parent Provisioning Session. |
| samplingPeriod | DurationSec | The time interval the Media Player should wait between sampling the QoE metrics specified by this metrics configuration object. The value shall be greater than zero. |
| positive‌Crossing‌Thresholds | map(Uri -> array(Float) | A map of QoE metrics to their respective threshold values. - The index of the associative array shall be an unqualified term identifier from the controlled vocabulary described by this metrics configuration object. - The value of each associative array member shall be an array of floating-point threshold values. A metric in this associative array shall be reported once when its value exceeds one of the associated threshold values, and shall not be reported again until it falls below that threshold and subsequently exceeds it. |
| negative‌Crossing‌Thresholds | map(Uri -> array(Float) | A map of QoE metrics and their respective threshold values. - The index of the associative array shall be an unqualified term identifier from the controlled vocabulary described by this metrics configuration object. - The value of each associative array member shall be an array of floating-point threshold values. A metric in this associative array shall be reported once when its value falls below one of the associated threshold values, and shall not be reported again until it exceeds that threshold and subsequently falls below it. |
| location‌Filter | array(Location‌Area‌5G) | A list of one or more locations where QoE metrics collection is required. When present, a Media Player shall collect metrics only when it is located in these locations, and shall report them according to the other properties of the metrics configuration object. If omitted, QoE metrics are to be collected and reported regardless of the UE location. |
| metrics | array(Uri) | A list of QoE metrics, each indicated using an unqualified term identifier from the controlled vocabulary described by this metrics configuration object, which are required to be collected and reported by the Media Player. If omitted, the complete (or default, as applicable) set of metrics associated with the specified metrics scheme shall be collected and reported. |

Table 13.2.4-4: Content Delivery Protocols enumeration

| Value | Definition |
| --- | --- |
| DELIVERY_PROTOCOL_HTTP_1_1 | HTTP/1.1 protocol as specified in RFC 9112 [24]. |
| DELIVERY_PROTOCOL_HTTP_2 | HTTP/2 protocol as specified in RFC 9113 [31] |
| DELIVERY_PROTOCOL_HTTP_3 | HTTP/3 protocol as specified in RFC 9114 [60] |

Table 13.2.4-5: Multipath connection preference enumeration

| Value | Definition |
| --- | --- |
| MULTIPATH_PREFERRED | Preference for using multipath transport connections. |
| MULTIPATH_NOT_PREFERRED | Preference for not using multipath transport connections. |

### 13.2.5	Notifications and error events

Table 13.2.5-1 provides a list of notification events that are provided by the Media Player to 5GMSd-Aware Applications at reference point M7d and to the Media Session Handler at reference point M11d. Every notification and error event is disambiguated by a media delivery session identifier.

Table 13.2.5-1: Media Player Notification events

| Status | Definition | Payload |
| --- | --- | --- |
| AST_IN_FUTURE | Triggered when playback will not start yet as the MPD's availabilityStartTime is in the future. | Media delivery session identifier, Time before playback will start. |
| AVAILABLE_MEDIA_CHANGED | The list of available media has changed. | Media delivery session identifier, Media type: - video - audio - subtitle - all |
| BUFFER_EMPTY | Triggered when the media playback platform's buffer state changes to stalled. | Media delivery session identifier, Media Type |
| BUFFER_LOADED | Triggered when the media playback platform's buffer state changes to loaded. | Media delivery session identifier, Media Type |
| CAN_PLAY | Sent when enough data is available that the media can be played. | Media delivery session identifier |
| CMCD_DATA_GENERATED | Triggered when CMCD data was generated for a HTTP request | Media delivery session identifier |
| MANIFEST_LOADED | Triggered when the manifest load is complete | Media delivery session identifier |
| METRIC_ADDED | Triggered every time a new metric is added. | Media delivery session identifier |
| METRIC_CHANGED | Triggered every time a metric value changes. | Media delivery session identifier, Metric identifier |
| METRIC_UPDATED | Triggered when the configuration of a metric is updated. | Media delivery session identifier, Metric identifier |
| METRICS_CHANGED | Triggered whenever there is a change to the overall metrics. | Media delivery session identifier |
| OPERATION_POINT_CHANGED | Triggered whenever there is a change of a Service Operation Point parameter. | Media delivery session identifier, External reference identifier of currently selected Service Operation Point. |
| PLAYBACK_ENDED | Sent when media playback completes normally. | Media delivery session identifier |
| PLAYBACK_ERROR | Sent when an error occurs during media playback. The element's error attribute contains more information. | Media delivery session identifier, Error reason (see table 13.2.5-2). |
| PLAYBACK_PAUSED | Sent when media playback is paused. | Media delivery session identifier |
| PLAYBACK_PLAYING | Sent when the media begins to play (either for the first time, after having been paused, or after ending and then restarting). | Media delivery session identifier |
| PLAYBACK_SEEKED | Sent when a media playback seek operation completes. | Media delivery session identifier |
| PLAYBACK_SEEKING | Sent when a media playback seek operation begins. | Media delivery session identifier |
| PLAYBACK_STALLED | Sent when the media playback platform reports stalled. | Media delivery session identifier |
| PLAYBACK_STARTED | Sent when playback of the media starts after having been paused; that is, when playback is resumed after a prior pause event. | Media delivery session identifier |
| PLAYBACK_WAITING | Sent when the media playback has stopped because of a temporary lack of data. | Media delivery session identifier |
| SERVICE_DESCRIPTION_SELECTED | sent when the DASH client has selected a service description. | Media delivery session identifier |
| SERVICE_DESCRIPTION_CHANGED | Sent when the DASH client has changed a service description. | Media delivery session identifier |
| SERVICE_DESCRIPTION_VIOLATED | Provides notification that the service description parameters are currently not met. | Media delivery session identifier, Parameters of service description that are not met |
| SOURCE_INITIALIZED | Triggered when the source is set up and ready. | Media delivery session identifier |
| DOWNLOAD_STARTED | Sent when a non-real-time content download begins. | Media delivery session identifier |
| DOWNLOAD_COMPLETED | Sent when a non-real-time content download is complete. | Media delivery session identifier |
| DOWNLOAD_ERROR | Send when an error occurs during non-real-time content download | Media delivery session identifier, Error reason (see table 13.2.5-2). |
| TRANSPORT_CONNECTION_STATUS_CHANGED | Triggered when the status of a media transport connection changes, including its initial establishment. | Media delivery session identifier, Transport connection status information (see table 13.2.6-1) 13.2.6-3). |

Table 13.2.5-2 provides a list of error reasons that are indicated for notifications of type PLAYBACK_ERROR and DOWNLOAD_ERROR.

Table 13.2.5-2: Media Player Error reasons

| Error reason | Definition |
| --- | --- |
| ERROR_MEDIA_ENTRY_NOT_FOUND | The Media Entry Point resource requested by the Media Player could not be located. |
| ERROR_CONTENT_NOT_FOUND | Other content requested by the Media Player could not be located. |
| ERROR_MEDIA_PLAYBACK | There is an error from the media playback platform buffer. |
| ERROR_INVALID_MEDIA_ENTRY | The Media Entry Point resource supplied is not syntactically valid. |
| ERROR_INACCESSIBLE_MEDIA_TIME | The media time requested in a seek operation is not accessible in the current media presentation. |
| ERROR_UNSUPPORTED_PROFILE | The profile of the media presentation described by the Media Entry Point resource is not supported by the media playback platform. |
| ERROR_DOWNLOAD_DEADLINE_MISSED | The download of content was not completed before the requested deadline and the incomplete download has been discarded. |
| ERROR_UNSUPPORTED_‌METRICS_SCHEME | One of the configured metrics schemes (see table 13.4.2-1) is not supported by the Media Player. |

### 13.2.6	Dynamic Status Information

Table 13.2.6-1 provides a list of dynamically changing status information that can be obtained from the Media Player via reference point M7d or M11d. A separate set of Dynamic Status Information is provided for each active downlink media streaming session, indexed by its media delivery session identifier initialised per clause 13.2.3.2.

Table 13.2.6-1: Media Player Dynamic Status information

| Status | Type | Parameter | Definition |
| --- | --- | --- | --- |
| state | Enumeration |  | An enumerated value from table 13.2.2-1 indicating the current state of the Media Player. |
| averageThroughput | float | none | Current average throughput computed in the ABR logic in bit/s. |
| bufferLength | float | MediaType "video", "audio" and "subtitle" | Current length of the buffer for a given media type, in seconds. If no type is passed in, then the minimum of video, audio and subtitle buffer length is returned. NaN is returned if an invalid type is requested, the presentation does not contain that type, or if no arguments are passed and the presentation does not include any adaption sets of valid media type. |
| liveLatency | float | none | Current live stream latency in seconds based on the latency measurement. |
| mediaSetting[ ] | MPDAdaptationSet | MediaType "video", "audio" and "subtitle" | Current media settings for each media type based on the CMAF Header and the MPD information based on the selected Adaptation Set for this media type. |
| mediaTime | float | None | Current media playback time from media playback platform. The media time is in seconds and is relative to the start of the playback and provides the media that is actually rendered. |
| playbackRate | float | None | The current rate of playback. For a video that is playing twice as fast as the default playback, the playbackRate value should be 2.00. |
| availableServiceDescriptions[ ] | Array of service descriptions |  | Provides the list of available selectable service descriptions with an id to select from. Those are either configured ones or the ones in the MPD. |
| availableMediaOptions[ ] | List of Adaptation Set or Preselection ids | MediaType "video", "audio" "subtitle" "all" | Provides the list of available media options that can be selected by the application based on the capability discovery and the subset information. |
| service‌Operation‌Points | array(Service‌Operation‌Point) |  | The set of Service Operation Points declared in the presentation manifest (e.g. DASH MPD) of the current media presentation. See table 13.2.6-2. |
| operative‌Service‌Operation‌Point | integer |  | A zero-based index into the service‌Operation‌Points array indicating the Service Operation Point currently operative in the playback session. Set to -1 if the array is empty. |
| pathmetrics[ ][ ] | Metrics |  | A data blob of metrics for each configured metrics collecting scheme. |
| transportConnectionStatuses[ ] | array(Transport‌Connection‌Status) |  | Status information about the current set of reference point M4d transport connection(s) for media delivery. See table 13.2.6-3. |

Table 13.2.6-2 provides a list of configured operation point information that can be obtained from the client. Any change to a parameter below shall be announced with a notification OPERATION_POINT_CHANGED as specified in table 13.2.5-1.

Table 13.2.6-2: Media Player Service Operation Point Information

| Parameter |  |  | Type | Definition |
| --- | --- | --- | --- | --- |
| ServiceOperationPoint |  |  | Object | The currently configured Service Operation Point parameters according to which the DASH client is operating. |
|  | externalIdentifier |  | String | The external identifier uniquely identifying this Service Operation Point in the presentation manifest (e.g. DASH MPD). |
|  | mode |  | Enum | The following operation modes are defined: live: The DASH client operates to maintain configured target latencies using playback rate adjustments and possibly resynchronisation. vod: The DASH client operates without latency requirements and rebuffering may result in additional latencies |
|  | maxBufferTime |  | Integer | maximum buffer time in milliseconds for the service. |
|  | switchBufferTime |  | Integer | buffer time threshold below which the DASH clients attempt to switch Representations. |
|  | latency |  | Object | Defines the latency parameters used by the DASH client when operating in live mode. |
|  |  | target | Integer | The target latency for the service in milliseconds. |
|  |  | max | Integer | The maximum latency for the service in milliseconds. |
|  |  | min | Integer | The maximum latency for the service in milliseconds. |
|  | playbackRate |  | MediaType audio, video, all | Defines the playback rate parameters used by the DASH client for catchup mode and deceleration to avoid buffer underruns and maintaining target latencies. |
|  |  | max | Real | The maximum playback rate for the purposes of automatically adjusting playback latency and buffer occupancy during normal playback, where 1.0 is normal playback speed. |
|  |  | min | Real | The minimum playback rate for the purposes of automatically adjusting playback latency and buffer occupancy during normal playback, where 1.0 is normal playback speed. |
|  | bitRate |  |  | Defines the operating bit rate parameters used by the DASH client used for a specific media type or aggregated. The values are on IP level. |
|  |  | target | Integer | The target bit rate for the service in bit/s that the client is configured to consume. |
|  |  | max | Integer | The maximum bit rate for the service in bit/s that the client is configured to consume. |
|  |  | min | Integer | The minimum bit rate for the service in bit/s that the client is configured to consume. |
|  | playerSpecificParameters |  |  | Player-specific parameters may be provided, for example about the used algorithm, etc. |

Table 13.2.6-3 specifies the transport connection status information object. Any change to a parameter below shall be announced with the TRANSPORT_CONNECTION_STATUS_CHANGED notification as specified in table 13.2.5-1.

Table 13.2.6-3: Transport connection status information

| Parameter |  | Type | Definition |
| --- | --- | --- | --- |
| TransportConnectionStatus |  | Object | Status information of a transport connection used for media delivery at reference point M4d. |
|  | componentIds | array(String) | Identifies which media presentation components are conveyed by this transport connection. Depending on implementation this could, for example, be a set of MPEG-DASH Representation identifiers or a set of Service Location labels. |
|  | transportProtocol | Enumeration | An enumerated value from table 13.2.6-4 indicating the transport protocol used by this transport connection. |
|  | numberOfActivePaths | Integer | The current number of active subflows/paths in this transport connection. |

Table 13.2.6-4: Transport Protocols enumeration

| Value | Definition |
| --- | --- |
| TRANSPORT_PROTOCOL_TCP | TCP as specified in RFC 9293 [71]. |
| TRANSPORT_PROTOCOL_QUIC | QUIC as specified in RFC 9000 [58]. |
| TRANSPORT_PROTOCOL_MPTCP | Multipath TCP as specified in RFC 8684 [72]. |
| TRANSPORT_PROTOCOL_MPQUIC | Multipath QUIC as specified in [73]. |

Event Exposure API (R5/R6)

### 18.3.2	QoEMetricsEvent data type

QoEMetricsEvent is a concrete data type describing a set of, or summaries of, QoE metric samples of the same type.

Table 18.3.2-1: QoEMetricsEvent data type

| Property name |  |  | Data Type | Cardinality | Description |
| --- | --- | --- | --- | --- | --- |
| recordType |  |  | Event‌Record‌Type | 1..1 | One of the following: - INDIVIDUAL_SAMPLE - SUMMARY_MEAN - SUMMARY_MINIMMUM - SUMMARY_MAXIMUM - SUMMARY_SUM |
| recordTimestamp |  |  | DateTime | 1..1 | For individual records, the date–time at which the parent QoE metrics report was generated by the Media Session Handler for submission at reference point M5 or generated by the 5GMS AS for submission at reference point M3. Otherwise, the date–time at which the summary record was generated by the Data Collection AF instantiated in the 5GMS AF. |
| appId |  |  | ApplicationId | 1..1 | Identifying the application (see table 5.4.2-1 of TS 29.571 [12]) to which the UE data carried in this record pertains. |
| provisioningSessionId |  |  | Resource‌Id | 0..1 | The identifier of the Provisioning Session to which this record pertains. Present only for individual data sample recordType. |
| session‌Id |  |  | Media‌Delivery‌SessionId | 0..1 | A value that uniquely identifies the media streaming session to which this record pertains. Present only for individual data sample recordType. |
| ueIdentification |  |  | string | 0..1 | GPSI of the requesting UE or a stable globally unique string identifying the requesting Media Session Handler. Present only for individual data sample recordType and only when exposure is permitted by the data exposure restrictions in force. |
| dataNetworkName |  |  | Dnn | 0..1 | Identifying the Data Network of the M4 media streaming session. Present only for individual data sample recordType. |
| sliceId |  |  | Snssai | 0..1 | The S-NSSAI identifying the Network Slice of the M4 media streaming session. Present only for individual data sample recordType. |
| ueLocations |  |  | array(Location‌Area5G) | 0..1 | The location(s) of the UE when the data described by this record was sampled. Present only for individual data sample recordType and only when exposure is permitted by the data exposure restrictions in force. |
| metric‌Type |  |  | Uri | 1..1 | A fully-qualified term identifier that uniquely identifies the QoE metrics reporting scheme and the type of QoE metric included this record, as specified in clause E.2, up to but excluding the first hierarchical separator. For example, urn:‌3GPP:‌ns:‌PSS:‌DASH:‌QM10#AvgThroughput or urn:‌3gpp:‌5gms:‌event-exposure:‌common-media-client-data#session. |
| samples |  |  | array(object) | 1..1 | An ordered list of one or more samples of type metricType derived from a single QoE metrics report. |
|  | sampleTimestamp |  | DateTime | 0..1 | Where applicable for the metric indicated by metricType, the moment in time at which this QoE metric was sampled. |
|  | sampleDuration |  | Duration | 0..1 | Where applicable for the metric indicated by metricType, the time duration over which this QoE metric was sampled. |
|  | mediaTimestamp |  | Duration | 0..1 | Where applicable for the metric indicated by metricType, the time point (expressed relative to the start of the media streaming presentation) at which this QoE metric was sampled. |
|  | metrics |  | array(object) | 1..1 | A set of key–value pairs for the sampled metrics associated with this QoE metric sample. |
|  |  | key | string | 1..1 | A token that uniquely identifies metric subtype within the scope of the QoE metric type. For example: numbytes. There shall be at most one instance of this property's value in the parent array. |
|  |  | value | {} | 0..1 | A value (of any type) associated with the metric indicated by key. |

### 18.3.3	Exposure of client data

#### 18.3.3.1	Exposure of CMCD version 1 information

CMCD version 1 information [70] submitted to the 5GMSd AF and passed to the Data Collection AF instantiated in the 5GMS AF shall be packed into QoEMetricsEvent records (see clause 18.3.2) as follows:

-	recordTimestamp shall be populated with the timestamp of an HTTP request received by the 5GMS AS at reference point M4d.

-	appId shall be populated with the external service identifier of the Provisioning Session.

-	sessionId shall be populated with the CMCD session identifier, which is typically the same value as the media delivery session identifier (see clause 6.2.3.6).

-	metricType shall be populated with one of the fully-qualified term identifier URIs listed in table E.2.5-1, indicating the class of CMCD information conveyed in this record.

In the case of individual records (recordType set to INDIVIDUAL_SAMPLE) the samples array shall contain a single entry, corresponding to CMCD information of the class indicated by metricType from a single HTTP request at reference point M4d.

-	sampleTimestamp, sampleDuration and mediaTimestamp shall be omitted; the timestamp of all metrics in the record is indicated by recordTimestamp, as specified above.

-	The metrics array shall be populated with the set of CMCD keys and values from a single HTTP request at reference point M4d belonging to the class of CMCD information indicated in metricType.

-	key shall be one the key names reserved in section 3.3 of the CMCD specification [70] for the class of CMCD information indicated in metricType.

Aligned with section 3.1 of the CMCD specification [70], key–value pairs should appear in alphabetical order of key name when transmitted. (This reduces the fingerprinting surface exposed by different sources of client data.)

-	value shall be the value of the key indicated by key. In the case of Boolean keys, value may be omitted, implying the value true.

Content Hosting Configuration examples

## B.1.2	Desired URL mapping

In the example shown in table B.1.2-1 below, media resources are exposed by a reference point M4d service location with a default canonical domain name dist-loc.com-provider-service.‌ms.‌as.‌3gppservices.‌org. This domain name is assigned by the 5GMSd AF based on a Provisioning Session with external identifier com.provider.service and a Content Hosting Configuration distribution configuration with distribution identifier dist.loc. The 5GMSd Application Provider has also assigned a custom domain name alias 5gms.provider.com by defining DistributionConfiguration.domainNameAlias in the same distribution configuration. The base URL of the 5GMSd Application Provider’s origin server is https://origin.provider.com/media.

Table B.1.2-1: Example URL mapping for pull-based ingest

| M4d request from 5GMSd Client | Mapped M2d request to origin server on 5GMSd AS cache miss |
| --- | --- |
| https://dist-loc.com-provider-service.ms.as.3gppservices.org/‌asset123456/video1/segment1000.mp4 | https://origin.provider.com/‌media/‌asset123456/video1/segment1000.mp4 |
| https://5gms.provider.com/‌asset123456/video1/segment1000.mp4 |  |
| https://dist-loc.com-provider-service.ms.as.3gppservices.org/‌asset123456/video2/segment1000.mp4 | https://origin.provider.com/‌media/‌asset123456/video2/segment1000.mp4 |
| https://5gms.provider.com/‌asset123456/video2/segment1000.mp4 |  |
| https://dist-loc.com-provider-service.ms.as.3gppservices.org/‌asset123456/audio1/segment1000.mp4 | https://origin.provider.com/‌media/‌asset123456/audio1/segment1000.mp4 |
| https://5gms.provider.com/‌asset123456/audio1/segment1000.mp4 |  |

## B.1.3	Content Hosting Configuration

Table B.1.3-1 below shows the relevant Content Hosting Configuration parameters needed to achieve the example mapping described in table B.1.2-1 above.

Table B.1.3-1: Content Hosting Configuration properties relevant to pull-based ingest

| Property | Example value |  | Set by |
| --- | --- | --- | --- |
| ProvisioningSession |  |  |  |
| externalServiceId | com.provider.service | 5GMSd Application Provider (M1d request) |  |
| IngestConfiguration |  |  |  |
| protocol | urn:3gpp:5gms:content-protocol:http-pull |  | 5GMSd Application Provider (M1d request) |
| mode | PULL |  |  |
| baseURL | https://origin.provider.com/media |  |  |
| DistributionConfiguration |  |  |  |
| distributionId | dist.loc |  | 5GMSd Application Provider (M1d request) |
| canonicalDomainName | dist.loc.com-provider-service.ms.as.3gppservices.org |  | 5GMSd AF (M1d response) |
| domainNameAlias | 5gms.provider.com |  | 5GMSd Application Provider (M1d request) |
| baseURL | https://5gms.provider.com/ |  | 5GMSd AF (M1d response) |

## B.2.1	Desired URL mapping

In the example shown in table B.2.1-1, media resources are exposed by a reference point M4d service location with a default canonical domain name dist-loc.com-provider-service.‌ms.‌as.‌3gppservices.‌org. This domain name is assigned by the 5GMSd AF based on a for the Provisioning Session with external identifier com.provider.service and a Content Hosting Configuration distribution configuration with distribution identifier dist-loc. The 5GMSd Application Provider also assigns a custom domain name alias 5gms.provider.com by defining DistributionConfiguration.domainNameAlias in the same distribution configuration. Media resources are pushed into the 5GMSd AS at M2d by the 5GMSd Application Provider using the ingest base URL https://5gmsd-as.mno.net/com-provider-service chosen by the 5GMSd AF.

Table B.2.1-1: Example URL mapping for push-based ingest

| M2d ingest URL pushed to 5GMSd AS | M4d URL exposed to 5GMSd Client |
| --- | --- |
| https://5gmsd-as.mno.net/com-provider-service/‌asset123456/video1/segment1000.mp4 | https://dist-loc.com-provider-service.ms.as.3gppservices.org/‌asset123456/video1/segment1000.mp4 |
|  | https://5gms.provider.com/‌asset123456/video1/segment1000.mp4 |
| https://5gmsd-as.mno.net/com-provider-service/‌asset123456/video2/segment1000.mp4 | https://dist-loc.com-provider-service.ms.as.3gppservices.org/‌asset123456/video2/segment1000.mp4 |
|  | https://5gms.provider.com/‌asset123456/video2/segment1000.mp4 |
| https://5gmsd-as.mno.net/com-provider-service/‌asset123456/audio1/segment1000.mp4 | https://dist-loc.com-provider-service.ms.as.3gppservices.org/‌asset123456/audio1/segment1000.mp4 |
|  | https://5gms.provider.com/‌asset123456/audio1/segment1000.mp4 |

## B.2.2	Content Hosting Configuration

Table B.2.2-1 below shows the relevant Content Hosting Configuration parameters needed to achieve the example mapping described in table B.2.1-1 above.

Table B.2.2-1: Content Hosting Configuration properties relevant to push-based ingest

| Property | Example value |  | Set by |
| --- | --- | --- | --- |
| ProvisioningSession |  |  |  |
| externalServiceId | com.provider.service | 5GMSd Application Provider (M1d request) |  |
| IngestConfiguration |  |  |  |
| protocol | http://dashif.org/‌ingest/‌v1.2‌/interface-1 |  | 5GMSd Application Provider (M1d request) |
| mode | PUSH |  |  |
| baseURL | https://5gmsd-as.mno.net/‌com-provider-service |  | 5GMSd AF (M1d response) |
| DistributionConfiguration |  |  |  |
| distributionId | dist.loc |  | 5GMSd Application Provider (M1d request) |
| canonicalDomainName | dist-loc.com-provider-service.ms.as.3gppservices.org |  | 5GMSd AF (M1d response) |
| domainNameAlias | 5gms.provider.com |  | 5GMSd Application Provider (M1d request) |
| baseURL | https://5gms.provider.com/ |  | 5GMSd AF (M1d response) |

# B.3	Pull-based content ingest with 5GMSd AS service chaining via M10d

### B.3.1	Overview

This example shows how to provision multiple Content Hosting Configurations allowing for content hosting service chaining via reference point M10d (see clause 5.2.8.2 of TS 26.510 [56]). In this example, one 5GMSd AS (edge.5gms.provider.com) is acting as an edge proxy while its upstream 5GMSd AS (shield.5gms.provider.com) is providing an origin shield function.

1.	The 5GMSd Client on the UE requests a media resource via reference point M4d.

2.	The client-facing 5GMSd AS determines that it does not have a cached copy of the requested media resource.

3.	The client-facing 5GMSd AS transforms the M4d request URL into a request to the origin server-facing 5GMSd AS via reference point M10d.

4.	The origin server-facing 5GMSd AS transforms the M10d request URL into a request to the 5GMSd Application Provider’s origin server via M2d.

### B.3.2	Desired URL mapping

In the example shown in table B.3.2-1 below, the following apply:

1.	Media resources for the Provisioning Session with external identifier com.provider.service.edge are exposed at M4d from a default canonical domain dist-loc.com-provider-service-edge.ms.‌as.‌3gppservices.‌org determined by the 5GMSd Application Provider, and a custom domain name alias edge.5gms.provider.com has also been configured by the 5GMSd Application Provider.

2.	Media resources for the Provisioning Session with external identifier com.provider.service.shield are exposed at M10d (and potentially M4d) from a default canonical domain dist-loc.com-provider-service-shield.ms.‌as.‌3gppservices.‌org determined by the 5GMSd Application Provider, and a custom domain name alias shield.‌5gms.‌provider.‌com has also been configured by the 5GMSd Application Provider. The base URL of the 5GMSd Application Provider’s origin server is https://origin.provider.com/media.

Table B.3.2-1: Example URL mapping for pull-based ingest

| M4d request from 5GMSd Client to client-facing 5GMSd AS | Mapped M10d request to origin server-facing 5GMSd AS | Mapped M2d request to origin server on 5GMSd AS cache miss |
| --- | --- | --- |
| https://dist-loc.com-provider-service-edge.ms.as .3gppservices.org/asset123456 /video1/segment1000.mp4 | https://dist-loc.com-provider-service-shield.ms.as .3gppservices.org/asset123456 /video1/segment1000.mp4 | https://origin.provider.com/‌media/‌asset123456/video1/segment1000.mp4 |
| https://edge.5gms.provider.com/‌asset123456/video1/segment1000.mp4 | https://shield.5gms.provider.com/‌asset123456/video1/segment1000.mp4 |  |
| https://dist-loc.com-provider-service-edge.ms.as .3gppservices.org/asset123456 /video2/segment1000.mp4 | https://dist-loc.com-provider-service-shield.ms.as .3gppservices.org/asset123456 /video2/segment1000.mp4 | https://origin.provider.com/‌media/‌asset123456/video2/segment1000.mp4 |
| https://edge.5gms.provider.com/‌asset123456/video2/segment1000.mp4 | https://shield.5gms.provider.com/‌asset123456/video2/segment1000.mp4 |  |
| https://dist-loc.com-provider-service-edge.ms.as .3gppservices.org/asset123456 /audio1/segment1000.mp4 | https://dist-loc.com-provider-service-shield.ms.as .3gppservices.org/asset123456 /audio1/segment1000.mp4 | https://origin.provider.com/‌media/‌asset123456/audio1/segment1000.mp4 |
| https://edge.5gms.provider.com/‌asset123456/audio1/segment1000.mp4 | https://shield.5gms.provider.com/‌asset123456/audio1/segment1000.mp4 |  |

## B.3.3	Content Hosting Configurations

Tables B.3.3-1 and B.3.3-2 below show the relevant parameters for both Content Hosting Configurations needed to achieve the example mapping described in table B.3.2-1 above.

Table B.3.3-1: Origin server-facing 5GMSd AS Content Hosting Configuration properties
relevant to pull-based ingest

| Property | Example value | Set by |
| --- | --- | --- |
| ProvisioningSession |  |  |
| externalServiceId | com.provider.service.shield | 5GMSd Application Provider (M1d request) |
| IngestConfiguration |  |  |
| protocol | urn:3gpp:5gms:content-protocol:http-pull | 5GMSd Application Provider (M1d request) |
| mode | PULL |  |
| baseURL | https://origin.provider.com/‌media |  |
| DistributionConfiguration |  |  |
| distributionId | dist.loc | 5GMSd Application Provider (M1d request) |
| canonicalDomainName | dist-loc.com-provider-service-shield.ms.as.3gppservices.org | 5GMSd AF (M1d response) |
| domainNameAlias | d1.5gms.provider.com | 5GMSd Application Provider (M1d request) |
| baseURL | https://shield.5gms.provider.com/ | 5GMSd AF (M1d response) |

Table B.3.3-2: Client-facing 5GMSd AS Content Hosting Configuration properties
relevant to pull-based ingest

| Property | Example value | Set by |
| --- | --- | --- |
| ProvisioningSession |  |  |
| externalServiceId | com.provider.service.edge | 5GMSd Application Provider (M1d request) |
| IngestConfiguration |  |  |
| protocol | urn:3gpp:5gms:content-protocol:http-pull | 5GMSd Application Provider (M1d request) |
| mode | PULL |  |
| baseURL | https://dist-loc.com-provider-service-shield.ms.as.3gppservices.org |  |
|  | https://shield.5gms.provider.com/ |  |
| DistributionConfiguration |  |  |
| distributionId | dist.loc | 5GMSd Application Provider (M1d request) |
| canonicalDomainName | dist-loc.com-provider-service-edge.ms.as.3gppservices.org | 5GMSd AF (M1d response) |
| domainNameAlias | edge.5gms.provider.com | 5GMSd Application Provider (M1d request) |
| baseURL | https://edge.5gms.provider.com/ | 5GMSd AF (M1d response) |

# B.4	Push-based content ingest with 5GMSd AS service chaining via M10d

### B.4.1	Overview

This example shows how to provision multiple Content Hosting Configurations allowing for content hosting service chaining via reference point M10d (see clause 5.2.8.2 of TS 26.510 [56]).

1.	The 5GMSd Application Provider uploads content to the origin server-facing 5GMSd AS using push-based ingest via reference point M2d.

2.	The origin server-facing 5GMSd AS rewrites the M2d upload URL to a M10d downlink URL that is exposed to a client-facing 5GMSd AS.

3.	The client-facing 5GMSd AS rewrites the M10d upload URL to a M4d downlink URL that is exposed to the 5GMSd Client on the UE.

## B.4.1	Desired URL mapping

In the example shown in table B.4.2-1 below, the following apply:

1.	Media resources for the Provisioning Session with external identifier com.provider.service.edge are pushed into the client-facing 5GMSd AS at M10d by the origin server-facing 5GMSd AS. These media resources are exposed to the 5GMSd Client at M4d using the canonical name of the client-facing 5GMSd AS dist-loc.com-provider-service-edge.‌ms.‌as.‌3gppservices.org  and an additional domain name alias edge.5gms.provider.com configured by the 5GMSd Application Provider. The ingest base URL populated by the 5GMSd AF is https://5gmsd-as-edge.mno.net/com-provider-service-edge.

2.	Media resources for the Provisioning Session with external identifier com.provider.service.shield are pushed into the origin server-facing 5GMSd AS at M2d by the 5GMSd Application Provider and exposed to the client-facing 5GMSd AS at M10d. The canonical domain name and base URL of the distribution pushing media resources to the client-facing 5GMSd AS are determined when provisioning the client-facing 5GMSd AS. The ingest base URL populated by the 5GMSd AF is https://5gmsd-as-shield.mno.net/com-provider-service-shield.

Table B.4.1-1: Example URL mapping for push-based ingest

| M2d ingest URL pushed to origin server-facing 5GMSd AS | M10d URL pushed to client-facing 5GMSd AS | M4d URL exposed to 5GMSd Client |
| --- | --- | --- |
| https://5gmsd-as-shield.mno.net/com-provider-service-shield/asset123456/ video1/segment1000.mp4 | https://5gmsd-as-edge.mno.net/‌com-provider-service-edge/asset123456 /video1/segment1000.mp4 | https://dist-loc.com-provider-service-edge.ms.as .3gppservices.org/asset123456 /video1/segment1000.mp4 |
|  |  | https://edge.5gms.provider.com/‌asset123456/video1/segment1000.mp4 |
| https://5gmsd-as-shield.mno.net/com-provider-service-shield/asset123456/ video2/segment1000.mp4 | https://5gmsd-as-edge.mno.net/‌com-provider-service-edge/asset123456 /video2/segment1000.mp4 | https://dist-loc.com-provider-service-edge.ms.as. 3gppservices.org/asset123456 /video2/segment1000.mp4 |
|  |  | https://edge.5gms.provider.com/‌asset123456/video2/segment1000.mp4 |
| https://5gmsd-as-shield.mno.net/com-provider-service-shield/asset123456/ audio1/segment1000.mp4 | https://5gmsd-as-edge.mno.net/‌com-provider-service-edge/asset123456 /audio1/segment1000.mp4 | https://dist-loc.com-provider-service-edge.ms.as .3gppservices.org/asset123456 /audio1/segment1000.mp4 |
|  |  | https://edge.5gms.provider.com/‌asset123456/audio1/segment1000.mp4 |

## B.4.2	Content Hosting Configuration

Tables B.4.2-1 and B.4.2-2 below show the relevant parameters for both Content Hosting Configurations needed to achieve the example mapping described in table B.4.1-1 above.

Table B.4.2-1: Client-facing Content Hosting Configuration properties
relevant to push-based ingest

| Property | Example value | Set by |
| --- | --- | --- |
| ProvisioningSession |  |  |
| externalServiceId | com.provider.service.edge | 5GMSd Application Provider (M1d request) |
| IngestConfiguration |  |  |
| protocol | http://dashif.org/‌ingest/‌v1.2‌/interface-1 | 5GMSd Application Provider (M1d request) |
| mode | PUSH |  |
| baseURL | https://5gmsd-as-edge.mno.net/‌com-provider-service-edge/ | 5GMSd AF (M1d response) |
| DistributionConfiguration |  |  |
| distributionId | dist.loc | 5GMSd Application Provider (M1d request) |
| canonicalDomainName | dist-loc.com-provider-service-edge.ms.as.3gppservices.org | 5GMSd AF (M1d response) |
| domainNameAlias | edge.5gms.provider.com | 5GMSd Application Provider (M1d request) |
| baseURL | https://edge.5gms.provider.com/ | 5GMSd AF (M1d response) |

Table B.4.2-2: Origin server-facing Content Hosting Configuration properties
relevant to push-based ingest

| Property | Example value |  |  | Set by |
| --- | --- | --- | --- | --- |
| ProvisioningSession |  |  |  |  |
| externalServiceId |  | com.provider.service.shield | 5GMSd Application Provider (M1d request) |  |
| IngestConfiguration |  |  |  |  |
| protocol | http://dashif.org/‌ingest/‌v1.2‌/interface-1 |  |  | 5GMSd Application Provider (M1d request) |
| mode | PUSH |  |  |  |
| baseURL | https://5gmsd-as-shield.mno.net/‌com-provider-service-shield/ |  |  | 5GMSd AF (M1d response) |
| DistributionConfiguration |  |  |  |  |
| mode | PUSH |  |  | 5GMSd Application Provider (M1d request) |
| distributionId | dist.loc |  |  |  |
| baseURL | https://5gmsd-as-edge.mno.net/‌com-provider-service-edge/ |  |  |  |

OpenAPI Syntax

## C.3.0	Maf_Provisioning API

The normative code specifying the APIs defined in this clause, including JSON Schema representations of HTTP message bodies to be used with these APIs, is published on 3GPP Forge according to the OpenAPI 3.0.0 specification [23]. The YAML files corresponding to this version of the present document shall be published to the following location:

https://forge.3gpp.org/rep/all/5G_APIs/-/tags/TSG109-Rel19

Informative copies of these YAML files shall be distributed with the present document for convenience only. Where any discrepancy exists, the version on 3GPP Forge shall be considered definitive.

For the purpose of referencing entities specified in this clause, it shall be assumed that the OpenAPI definitions are contained in a physical file named "TS26512_Maf_Provisioning.yaml".

## C.3A.0	Mas_Configuration API

The normative code specifying the APIs defined in this clause, including JSON Schema representations of HTTP message bodies to be used with these APIs, is published on 3GPP Forge according to the OpenAPI 3.0.0 specification [23]. The YAML files corresponding to this version of the present document shall be published to the following location:

https://forge.3gpp.org/rep/all/5G_APIs/-/tags/TSG109-Rel19

Informative copies of these YAML files shall be distributed with the present document for the convenience only. Where any discrepancy exists, the version on 3GPP Forge shall be considered definitive.

For the purpose of referencing entities specified in this clause, it shall be assumed that the OpenAPI definitions are contained in a physical file named "TS26512_Mas_Configuration.yaml".

## C.4.0	Maf_SessionHandling API

The normative code specifying the APIs defined in this clause, including JSON Schema representations of HTTP message bodies to be used with these APIs, is published on 3GPP Forge according to the OpenAPI 3.0.0 specification [23]. The YAML files corresponding to this version of the present document shall be published to the following location:

https://forge.3gpp.org/rep/all/5G_APIs/-/tags/TSG109-Rel19

Informative copies of these YAML files shall be distributed with the present document for convenience only. Where any discrepancy exists, the version on 3GPP Forge shall be considered definitive.

For the purpose of referencing entities specified in this clause, it shall be assumed that the OpenAPI definitions are contained in a physical file named "TS26512_Maf_SessionHandling.yaml".

# C.6	OpenAPI representation of client data

The normative code specifying the APIs defined in this clause, including JSON Schema representations of HTTP message bodies to be used with these APIs, is published on 3GPP Forge according to the OpenAPI 3.0.0 specification [23]. The YAML files corresponding to this version of the present document shall be published to the following location:

https://forge.3gpp.org/rep/all/5G_APIs/-/tags/TSG109-Rel19

Informative copies of these YAML files shall be distributed with the present document for convenience only. Where any discrepancy exists, the version on 3GPP Forge shall be considered definitive.

For the purpose of referencing entities specified in this clause, it shall be assumed that the OpenAPI definitions are contained in a physical file named "TS26512_ClientData.yaml".

Controlled Vocabularies for metrics schemes

# E.2	Controlled vocabularies of 5GMS QoE metrics reporting parameters

(Snip)

## E.2.2	Reporting parameters for Virtual Reality (VR) DASH metrics

The name space identifier for the controlled vocabulary of VR DASH metrics is:

urn:‌3gpp:‌metadata:‌2020:‌VR:‌metrics

The term identifiers in this controlled vocabulary shall be the set of key names defined in clause 9.3 of TS 26.118 [42], using a single forward slash character ('/') as the hierarchical separator in the resulting path specifier. In the case of metrics lists, the Entry object shall be omitted from the term identifier path.

EXAMPLE 1:	urn:‌3gpp:‌metadata:‌2020:‌VR:‌metrics#CompQualLatency/Latency

To select all reportable metrics below a common branch of the metrics hierarchy the relevant terminal path element(s) are pruned from the term identifier.

EXAMPLE 2:	urn:‌3gpp:‌metadata:‌2020:‌VR:‌metrics#CompQualLatency

## E.2.3	Reporting parameters for 5GMS delivery over eMBMS

The controlled vocabularies specified in clauses E.2.1 and/or E.2.2 (as applicable) shall be used to identify metrics or groups of metrics in relation to reporting the QoE of 5GMS delivery over eMBMS.

## E.2.4	Reporting parameters for 5GMS delivery over MBS

Metrics reporting for MBS User Services is not specified by TS 26.517 [64] in this release.

## E.2.5	Reporting parameters for CMCD-based client data

The name space identifier for the controlled vocabulary of CMCD-based client data is:

urn:3gpp:5gms:event-exposure:common-media-client-data

The term identifiers in this controlled vocabulary are arranged in a two-level hierarchy, the two levels being delimited by a single forward slash character ('/') as the hierarchical separator in the resulting path specifier.

The first path element of the term identifier shall indicate the class of CMCD information as specified in table E.2.3-1 below.

Table E.2.3-1: Term identifiers for CMCD information exposure

| CMCD information class | Term identifier | Fully-qualified term identifier |
| --- | --- | --- |
| CMCD per-session information | session | urn:3gpp:5gms:event-exposure:common-media-client-data#session |
| CMCD per-object information | object | urn:3gpp:5gms:event-exposure:common-media-client-data#object |
| CMCD per-request information | request | urn:3gpp:5gms:event-exposure:common-media-client-data#request |
| CMCD status information | status | urn:3gpp:5gms:event-exposure:common-media-client-data#status |

The second path element shall indicate one the reserved keys defined in section 3.3 of the CMCD specification [70] for the class of CMCD information indicated by the first path element.

EXAMPLE 1:	urn:3gpp:5gms:event-exposure:common-media-client-data#session/pr (current playback rate)

EXAMPLE 2:	urn:3gpp:5gms:event-exposure:common-media-client-data#request/mtp (measured throughput)

DASH in 5g Media Streaming

Annex G (normative):
DASH in 5G Media Streaming

# G.1	Introduction

This annex provides a binding between DASH as defined in ISO/IEC 23009-1 [32] and 3GPP TS 26.247 [4] as a media streaming protocol in the 5G Media Streaming System. In this context, it provides a mapping of DASH functionalities to generic 5G Media Streaming features. It also specifies requirements and makes recommendations on how certain features defined in 5G Media Streaming are to be used and supported when DASH is used as the media streaming protocol at reference point M4.

The annex also specifies how CMAF content as profiled in TS 26.511 [35] is required to be delivered through 5G Media Streaming using DASH.

The clause expands based on the requirements specified in clause 10.2.

# G.2	General mapping of DASH to 5G Media Streaming

Based on the basic mapping of DASH in clause 10.2, the following generic aspects are documented:

-	For any requirement that applies to the Media Player Entry in downlink streaming as part of the present document, for DASH this requirement applies to the Media Presentation Description (MPD) with content type "application/dash+xml".

-	Media resources are primarily DASH Segments, or other data referenced in the MPD. According to clause 10.2, DASH Segments may be CMAF-addressable resources per ISO/IEC 23000-19 [38], in particular CMAF media resources as profiled in TS 26.511 [35].

-	The mapping of media resources into DASH MPDs, including codec parameters, is specified in TS 26.511 [35].

# G.3	Content Protection and DRM

## G.3.1	Overview

This clause defines the requirements and recommendations to support Content Protection and Digital Rights Management in DASH-based 5G Media Streaming according to clause 5.14 in TS 26.501 [2].

## G.3.2	DASH content encoding requirements and recommendations

A DASH Media Presentation with encrypted content shall follow the requirements and recommendations in DASH-IF Interoperability Points Part 6 [74], in particular:

-	DASH Segments and the corresponding CMAF-addressable resources shall follow the requirements and recommendations concerning Segments in clause 6 of [74].

-	DASH MPDs shall follow the requirements and recommendations concerning the MPD in clause 7 of [74].

## G.3.3	Media Player requirements and recommendations

A Media Player supporting DRM-based content protection shall implement the requirements and recommendations for DASH clients defined in clauses 6 and 7 of DASH-IF Interoperability Points Part 6 [74].

## G.3.4	Content Preparation Template requirements

If content preparation is employed for DRM encryption by the 5GMS AS, the following parameters need to be supported in the Content Preparation Template:

-	An indication of one of the encryption methods profiled in TS 26.511 [35], namely CENC or CBCS.

-	The DRM systems that are to be supported, identified by their respective unique system identifiers.

-	The URL of the license server from which the Media Player fetches decryption keys. The Packager subfunction of the 5GMS AS is required to embed the license acquisition URL in the Media Presentation Description (MPD).

-	The CPIX configuration used to communicate encryption keys and DRM policies between the Packager and the Key Server.

## G.3.5	Content Preparation Template syntax

NOTE:	The format of an interoperable Content Preparation Template to support DRM encryption is for future study.

## G.3.6	Examples (informative)

For examples of MPDs including Content Protection, see clause 7.3 of DASH-IF Interoperability Points Part 6 [74].

# G.4	DASH content distribution using multiple service locations

## G.4.1	Overview

This clause defines the requirements and recommendations to support content distribution from multiple service locations at reference point M4d in DASH-based downlink media streaming according to clause 5.2.6 of TS 26.501 [2]. This clause G.4 focuses on:

-	Client-side switching

-	Content Steering for DASH

-	SAND for Multi-network access mode (SAND4M)

The focus in this clause is on media resources and transport resources that conform to DASH Segments as specified in ISO/IEC 23009-1 [32].

In particular, the base URL concept and service locations as defined in [32] are leveraged to support multiple service locations. A service location in MPEG-DASH is defined as "a collection of network resources that share commonalities and can be referred to by a common label". The BaseURL element may be used to specify one or more common locations for DASH Segments and other resources. Service locations are labels for base URLs, indicated in the MPD by an attribute of the BaseURL element.

Clause 5.6 of [32] specifies how to apply base URLs to each relative URL in the MPD to construct an absolute URL suitable for retrieving transport objects containing media resources, as well as specifying rules for handling multiple alternative base URLs, one of the key aspects to support multiple service locations in the 5GMS System.

For content hosting:

-	To provision DASH downlink media streaming in the 5GMSd AF, the 5GMSd Application Provider shall use the Content Hosting Provisioning (M1) API specified in clause 7.6, following the Content Hosting provisioning (M1) procedures specified in clause 4.3.3.

-	To configure downlink media streaming in the 5GMSd AS, the 5GMSd AF shall use the Content Hosting Configuration (M3) API specified in clause 9.4, following the Content Hosting configuration (M3) procedures specified in clause 4.5.4.

## G.4.2	General Content Preparation requirements to support multiple service locations

### G.4.2.1	Introduction

This clause specifies general requirements for preparing DASH content to make it suitable for distribution via different service locations in the 5GMS System, in particular the configuration information required by the 5GMSd AS in order to perform any necessary content preparation.

To provision Content Preparation of DASH transport resources in the 5GMSd AF, the 5GMSd Application Provider shall use the Content Preparation Templates Provisioning (M1) API specified in clause 7.4, following the Content Preparation Template provisioning (M1) procedures specified in clause 4.3.5.

To configure Content Preparation of DASH transport resources in the 5GMSd AS, the 5GMSd AF shall use the Content Preparation Templates Configuration (M3) API specified in clause 9.3, following the Content Preparation Template configuration (M3) procedures specified in clause 4.5.3.

Two different variants are supported:

1.	DASH content ingested by the 5GMSd AS through reference point M2d, including a provided DASH MPD, already includes different base URLs with service location tags. The content preparation shall provide a mapping of the service locations in the MPD to the provisioned service locations on the 5GMSd AS. For details refer to clause G.4.2.2.

2.	DASH content ingested through reference point M2d conforms to TS 26.511 [35] or at least CMAF per ISO/IEC 23000-19 [40], and the 5GMSd AS creates an MPD that includes multiple service locations based on the provisioning information and the content preparation information. For details refer to clause G.4.2.3.

The following assumptions are common to both scenarios:

1.	Multiple service locations have been provisioned at reference point M1d as distinct distribution configurations using the Content Hosting Provisioning (M1) API specified in clause 7.6 and following the Content Hosting provisioning (M1) procedures specified in clause 4.3.3.

### G.4.2.2	Pre-configured service locations

In this case, content ingested by the 5GMSd AS through reference point M2d, including a provided DASH MPD, already includes different base URLs with service location tags. The content preparation shall the base URLs of each service location in the provided MPD with a provisioned service location on the 5GMSd AS, shall make the modified MPD available to the 5GMS Client via reference point M4d.

The ingested Media Presentation is described by an MPD that shall include a BaseURL element including a @serviceLocation attribute that fulfils the requirements for a service location as specified in ISO/IEC 23009-1 [32]. If this service location is to be exposed by the 5GMSd AS at reference point M4d, it shall be associated with a Distribution Configuration provisioned in the 5GMSd AF Content Hosting Configuration by matching its @serviceLocaton attribute value with the value of a DistributionConfiguration.distributionId property (see clause 8.8.3.1 of TS 26.510 [56]).

In addition, the BaseURL may include the following optional parameters

1.	An @availabilityTimeOffset attribute adjusting the Segment availability time for this service location.

2. 	An @availabilityComplete attribute indicating whether all Segments of all associated Representations are complete at the adjusted availability start time.

3.	A @timeShiftBufferDepth attribute indicating, for a DASH Media Presentation of type dynamic, the duration of the smallest timeshift buffer for which Segments of any Representation are guaranteed to remain available.

4.	A @rangeAccess attribute indicating whether or not partially available Segments may be accessed with an HTTP byte range request.

-	In the case where the attribute value indicates that partially available Segments may be accessed with an HTTP byte range request, if a Media Player makes a byte-range request against a partially available Segment, and the first-byte position of that range request is non-zero, and the Media Player desires a continuously aggregating/live response, then it should signal this using the convention of RFC 8673 [61]. Specifically, it should use a last-byte value of 9007199254740991. When responding to Segment requests through this service location, the 5GMSd AS shall return an HTTP 206 (Partial Content) aggregating response instead of responding with a 200 (OK) response and a Content-Length response header after waiting for the end of the Segment to become available.

-	Otherwise, the Media Player should not expect a response corresponding to the requested byte range.

Based on this information, the 5GMSd AS offers DASH content associated with its provisioned service locations accordingly to the 5GMS Client via reference point M4d, taking into account the optional parameters provided above, if provided.

### G.4.2.3	Configuration of service locations in the 5GMSd AS

In this case, content ingested by the 5GMSd AS through reference point M2d conforms to TS 26.511 [35] or at least CMAF per ISO/IEC 23000-19 [40], and the 5GMSd AS creates an MPD that includes multiple service locations based on the provisioning information and the content preparation information. The ingested content may be described by an MPD conforming to a DASH profile for CMAF content as specified in ISO/IEC 23009-1 [32].

In order to create an MPD for M4d distribution using the provisioning information as well as the CMAF content annotation, the content preparation information shall include a mapping of included CMAF tracks to one or multiple service locations. If this service location is to be exposed by the 5GMSd AS at reference point M4d, it shall be associated with a Distribution Configuration provisioned in the 5GMSd AF Content Hosting Configuration by matching its @serviceLocaton attribute value with the value of a DistributionConfiguration.distributionId property (see clause 8.8.3.1 of TS 26.510 [56]).

Based on this information, the 5GMSd AS offers DASH content associated with its provisioned service locations accordingly to the 5GMS Client via reference point M4d.

## G.4.3	Client-side switching between service locations

### G.4.3.1	Introduction

For client-side switching, DASH content is offered from multiple service locations, but switching between them, for example based on throughput estimation, error behaviour, etc. is a decision made on the client side.

### G.4.3.2	DASH content offering requirements and recommendations

A 5GMSd AS offering a DASH Media Presentation with multiple service locations shall provide a service offering that conforms to a DASH Media Profile for CMAF content as defined in ISO/IEC 23009-1 [32], with the following additional requirements:

1.	For each Representation that represents a CMAF track, all BaseURL elements shall be associated with this Representation whereby the value of the element is the base URL provided in the template and the value of the @serviceLocation attribute shall be set to the distributionBaseURL of the corresponding service location provisioned in a Distribution Configuration of the Content Hosting Configuration.

2.	For each base URL, the associated parameters specified in clause G.4.2.2 may be set as well.

3.	The resources shall be made available by the 5GMSd AS at reference point M4d according to the configuration for each base URL.

### G.4.3.3	Media Player requirements and recommendations

A Media Player supporting multiple service locations with DASH and client-side switching shall support the general requirements for the media access client as defined in clause 10.3A.1 and the Media Access Client capabilities to support switching between multiple service locations during content distribution as defined in clause 10.3A.2.

In order to support service offerings, for which different Representations and/or Adaptation Sets are offered on different service locations, and the Representations are expected to the be played together, the Media Player should support the Media Access Client capabilities to support concurrent use of multiple service locations for content distribution as defined in clause 10.3A.2.

### G.4.3.4	Examples (informative)

Listing G.4.3.4-1 provides an example for which two service locations are specified in the MPD.

Listing G.4.3.4-1 MPD with multiple service locations

<?xml version="1.0" encoding="UTF-8"?> <MPD xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xmlns="urn:mpeg:dash:schema:mpd:2011" xsi:schemaLocation="urn:mpeg:dash:schema:mpd:2011 DASH-MPD.xsd" type="dynamic" minimumUpdatePeriod="PT2S" timeShiftBufferDepth="PT30M" availabilityStartTime="2014-10-17T17:17:05Z" minBufferTime="PT4S" profiles="urn:mpeg:dash:profile:isoff-live:2011" publishTime="2014-10-17T17:17:05Z"> <BaseURL serviceLocation="dist1">http://distribution-1.com-provider-service.ms.as.3gppservices.org</BaseURL> <BaseURL serviceLocation="dist2">http://distribution-2.com-provider-service.ms.as.3gppservices.org</BaseURL> <Period id="1"> <!-- Video --> <AdaptationSet mimeType="video/mp4" codecs="avc1.4D401F" frameRate="30000/1001" segmentAlignment="true" startWithSAP="1"> <BaseURL>video/</BaseURL> <SegmentTemplate timescale="90000" initialization="$Bandwidth$/init.mp4v" media="$Bandwidth$/$Time$.mp4v"> <SegmentTimeline> <S t="0" d="180180" r="432"/> </SegmentTimeline> </SegmentTemplate> <Representation id="v0" width="320" height="240" bandwidth="250000"/> <Representation id="v1" width="640" height="480" bandwidth="500000"/> <Representation id="v2" width="960" height="720" bandwidth="1000000"/> </AdaptationSet> <!-- English Audio --> <AdaptationSet mimeType="audio/mp4" codecs="mp4a.40" lang="en" segmentAlignment="0" startWithSAP="1"> <SegmentTemplate timescale="48000" initialization="audio/en/init.mp4a" media="audio/en/$Time$.mp4a"> <SegmentTimeline> <S t="0" d="96000" r="432"/> </SegmentTimeline> </SegmentTemplate> <Representation id="a0" bandwidth="64000"/> </AdaptationSet> <!-- French Audio --> <AdaptationSet mimeType="audio/mp4" codecs="mp4a.40" lang="fr" segmentAlignment="0" startWithSAP="1"> <SegmentTemplate timescale="48000" initialization="audio/fr/init.mp4a" media="audio/fr/$Time$.mp4a"> <SegmentTimeline> <S t="0" d="96000" r="432"/> </SegmentTimeline> </SegmentTemplate> <Representation id="b0" bandwidth="64000"/> </AdaptationSet> </Period> </MPD>

## G.4.4	Switching between service locations using Content Steering for DASH

### G.4.4.1	Introduction

Content steering provides a deterministic capability for a content distributor to switch the content source that a media player uses, either at presentation start-up or at any point during the presentation, by means of a remote steering service. Content Steering for DASH is specified in ETSI TS 103 998 [76]. This clause defines the requirements and recommendations to support content distribution at reference point M4d in DASH-based downlink media streaming in combination with Content Steering as specified in [76].

### G.4.4.2	DASH content offering requirements and recommendations

A 5GMSd AS offering a DASH Media Presentation to be used with Content Steering shall offer a Media Presentation according to the requirements in clause 4.3.2.

In addition, the 5GMSd AS shall:

-	Add Content Steering signalling in the DASH MPD as specified in clause 5 of ETSI TS 103 998 [76], and

-	Support the functionalities of a DASH Steering Server as specified in clause 6 of ETSI TS 103 998 [76].

### G.4.4.3	Media Player requirements and recommendations

A Media Player supporting Content Steering for DASH shall support:

-	The normative DASH client steering behaviour specified in clause 7 of ETSI TS 103 998 [76].

-	The media player requirements and recommendations in clause G.4.3.3.

-	The Media Access Client capabilities to support content steering for content distribution as defined in clause 10.3A.4 where the content steering service is the Content Steering server as provided in the MPD according to clause G.4.4.2.

### G.4.4.4	Examples (informative)

Listing G.4.4.4-1 provides an example in which three service locations are specified in the MPD. In this example, some content representations are available from service location dist1, a subset is available from service location dist2, and complementary content is available from service location dist3.

Listing G.4.4.4-1 MPD with multiple service locations and Content Steering

<MPD xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xmlns="urn:mpeg:dash:schema:mpd:2011" type="dynamic" minimumUpdatePeriod="PT10s" timeShiftBufferDepth="PT600S" minBufferTime="PT2S" profiles="urn:3GPP:PSS:profile:DASH10" publishTime="2014-10-17T17:17:05Z" availabilityStartTime="2014-10-17T17:17:05Z"> <Location>distribution-1.com-provider-service.ms.as.3gppservices.org/MPD2.mpd</Location> <Location>distribution-2.com-provider-service.ms.as.3gppservices.org/MPD2.mpd</Location> <Location>distribution-3.com-provider-service.ms.as.3gppservices.org/MPD2.mpd</Location> <ServiceDescription id="420"> <ContentSteering defaultServiceLocation="dist1" queryBeforeStart="true"> https://com-example.ms.as.3gppservices.org/steering</ContentSteering> </ServiceDescription> <Period id="1" start="PT0S"> <SegmentTemplate media="./$RepresentationID$/$Number$.m4s" initialization="$RepresentationID$-init.mp4"/> <!–- Video 720p --> <AdaptationSet mimeType="video/mp4" codecs="hvc1.1.2.L93.B0" startWithSAP="1" maxWidth="1280" maxHeight="720" frameRate="30" profile="urn:3GPP:video:op:h265-720p-HD"> <BaseURL serviceLocation="dist2">http://distribution-2.com-provider-service.ms.as.3gppservices.org</BaseURL> <SegmentTemplate timescale="30" duration="60"/> <Representation id="v2048" bandwidth="2048000"> <BaseURL serviceLocation="dist1"> http://distribution-1.com-provider-service.ms.as.3gppservices.org</BaseURL> </Representation> <Representation id="v1024" bandwidth="1024000"/> <Representation id="v512" bandwidth="512000"/> <Representation id="v128" bandwidth="128000"/> </AdaptationSet> <!–- Video HDR --> <AdaptationSet mimeType="video/mp4" codecs="hvc1.2.4.L113.B0" startWithSAP="1" maxWidth="1920" maxHeight="1080" frameRate="30" profile="urn:3GPP:video:op:h265-Full-HD-HDR"> <BaseURL serviceLocation="dist2">http://distribution-2.com-provider-service.ms.as.3gppservices.org</BaseURL> <EssentialDescriptor schemeIdUri="urn:mpeg:mpegB:cicp:MatrixCoefficients" value="9"/> <EssentialDescriptor schemeIdUri="urn:mpeg:mpegB:cicp:TransferCharacteristics" value="16"/> <EssentialDescriptor schemeIdUri="urn:mpeg:mpegB:cicp:ColourPrimaries" value="9"/> <SegmentTemplate timescale="30" duration="60"/> <Representation id="8M" bandwidth="8192000"> <Representation id="6M" bandwidth="6144000"/> <Representation id="4M" bandwidth="4096000"/> <Representation id="2M" bandwidth="2048000"/> </AdaptationSet> <!–- Audio English --> <AdaptationSet mimeType="audio/mp4" codecs="mp4a.40.2" segmentAlignment="true" startWithSAP="1" language="en"> <BaseURL serviceLocation="dist3">http://distribution-3.com-provider-service.ms.as.3gppservices.org</BaseURL> <SegmentTemplate timescale="20" duration="40"/> <Representation id="a128" bandwidth="128000"> <BaseURL serviceLocation="dist3">http://distribution-3.com-provider-service.ms.as.3gppservices.org</BaseURL> </Representation> <Representation id="a64" bandwidth="64000"> </AdaptationSet> <!–- Audio Spanish --> <AdaptationSet mimeType="audio/mp4" codecs="mp4a.40.2" segmentAlignment="true" startWithSAP="1" language="es"> <BaseURL serviceLocation="dist2">http://distribution-2.com-provider-service.ms.as.3gppservices.org</BaseURL> <SegmentTemplate timescale="20" duration="40"/> <Representation id="a128" bandwidth="128000"> <Representation id="a64" bandwidth="64000"> </AdaptationSet> </Period> </MPD>

The Content Steering message is provided to the Media Player by the DASH Steering Server instantiated in the 5GMSd. The example in listing G.4.4.4-1, it indicates a single service location that hosts one Representation is available; the HDR video and the Spanish language are available as well from service location dist3.

Listing G.4.4.4-1: Example DASH Content Steering Message indicating availability of content from 5GMSd AS service locations

{ "VERSION": 1, "TTL": 5, "RELOAD-URI": "https://com-example.ms.as.3gppservices.org/steering" "PATHWAY-PRIORITY": ["dist2","dist3","dist1"] }

In case the 5GMSd AS fails to pre-populate content for service location dist2, it recommends switching to dist1 or dist3.

Listing 5.12.2.2.2-3: Example DASH Content Steering Message indicating non-availability of content from 5GMSd AS service locations

{ "VERSION": 1, "TTL": 5, "RELOAD-URI": "https://com-example.ms.as.3gppservices.org/steering" "PATHWAY-PRIORITY": ["dist1","dist3"] }

## G.4.5	Switching between service locations using SAND4M

### G.4.5.1	Introduction

3GPP DASH as specified in clause 13.10 of 3GPP TS 26.247 [26247], defines the Server-Assisted Network Delivery (SAND) functionality that enables SAND for Multi-network support (SAND4M), and specifies required and recommended functionalities for both a DASH-Aware Network Element (DANE) and a DASH client. In the context of the 5GMS System, the DANE is mapped to the 5GMSd AS and the DASH client is realised by the 5GMSd Client.

Specifically, the following cases are relevant in the context of multiple service locations:

-	Not all resources announced in the presentation manifest (e.g. MPEG-DASH MPD) are always accessible on all access networks, e.g. certain resources are unavailable when the UE is for example in a roaming scenario.

-	Even on a particular access network, resources may not be available all the time.

-	Networks may have different availability times.

-	Networks go down dynamically and may re-appear.

-	The DANE may issue preferences for one network.

-	Information about the dynamic availability of resources from different service locations may be established via in-band and/or out-of-band channels.

### G.4.5.2	DASH content offering requirements and recommendations

A 5GMSd AS offering a DASH Media Presentation to be used with SAND4M shall offer a Media Presentation according to the requirements in clause 4.3.2.

In addition, the 5GMSd AS shall support the functionalities of a DANE for SAND4M specified in clause 13.10.2 of TS 26.247 [4].

### G.4.5.3	Media Player requirements and recommendations

A Media Player supporting SAND4M with multiple service locations shall support:

-	The DASH Client functionalities of SAND4M specified in clause 13.10.3 of TS 26.247 [4].

-	The Media Player requirements and recommendations in clause G.4.3.3.

-	The Media Access Client capabilities to support content steering for content distribution as defined in clause 10.3A.4 where the content steering service is the DANE according to clause G.4.5.2.

### G.4.5.4	Examples (informative)

Listing G.4.5.4-1 provides an example for which three service locations are specified in the MPD. In this example, some content representations are available on service location dist1, a subset is available from service location dist2, and complementary content is available from service location dist3.

Listing G.4.5.4-1 MPD with multiple service locations

<MPD xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xmlns="urn:mpeg:dash:schema:mpd:2011" type="dynamic" minimumUpdatePeriod="PT10s" timeShiftBufferDepth="PT600S" minBufferTime="PT2S" profiles="urn:3GPP:PSS:profile:DASH10" publishTime="2014-10-17T17:17:05Z" availabilityStartTime="2014-10-17T17:17:05Z"> <Location>distribution-1.com-provider-service.ms.as.3gppservices.org/MPD2.mpd</Location> <Location>distribution-2.com-provider-service.ms.as.3gppservices.org/MPD2.mpd</Location> <Location>distribution-3.com-provider-service.ms.as.3gppservices.org/MPD2.mpd</Location> <Period id="1" start="PT0S"> <SegmentTemplate media="./$RepresentationID$/$Number$.m4s" initialization="$RepresentationID$-init.mp4"/> <!–- Video 720p --> <AdaptationSet mimeType="video/mp4" codecs="hvc1.1.2.L93.B0" startWithSAP="1" maxWidth="1280" maxHeight="720" frameRate="30" profile="urn:3GPP:video:op:h265-720p-HD"> <BaseURL serviceLocation="dist2">http://distribution-2.com-provider-service.ms.as.3gppservices.org</BaseURL> <SegmentTemplate timescale="30" duration="60"/> <Representation id="v2048" bandwidth="2048000"> <BaseURL serviceLocation="dist1"> http://distribution-1.com-provider-service.ms.as.3gppservices.org</BaseURL> </Representation> <Representation id="v1024" bandwidth="1024000"/> <Representation id="v512" bandwidth="512000"/> <Representation id="v128" bandwidth="128000"/> </AdaptationSet> <!–- Video HDR --> <AdaptationSet mimeType="video/mp4" codecs="hvc1.2.4.L113.B0" startWithSAP="1" maxWidth="1920" maxHeight="1080" frameRate="30" profile="urn:3GPP:video:op:h265-Full-HD-HDR"> <BaseURL serviceLocation="dist2">http://distribution-2.com-provider-service.ms.as.3gppservices.org</BaseURL> <EssentialDescriptor schemeIdUri="urn:mpeg:mpegB:cicp:MatrixCoefficients" value="9"/> <EssentialDescriptor schemeIdUri="urn:mpeg:mpegB:cicp:TransferCharacteristics" value="16"/> <EssentialDescriptor schemeIdUri="urn:mpeg:mpegB:cicp:ColourPrimaries" value="9"/> <SegmentTemplate timescale="30" duration="60"/> <Representation id="8M" bandwidth="8192000"> <Representation id="6M" bandwidth="6144000"/> <Representation id="4M" bandwidth="4096000"/> <Representation id="2M" bandwidth="2048000"/> </AdaptationSet> <!–- Audio English --> <AdaptationSet mimeType="audio/mp4" codecs="mp4a.40.2" segmentAlignment="true" startWithSAP="1" language="en"> <BaseURL serviceLocation="dist3">http://distribution-3.com-provider-service.ms.as.3gppservices.org</BaseURL> <SegmentTemplate timescale="20" duration="40"/> <Representation id="a128" bandwidth="128000"> <BaseURL serviceLocation="dist3">http://distribution-3.com-provider-service.ms.as.3gppservices.org</BaseURL> </Representation> <Representation id="a64" bandwidth="64000"> </AdaptationSet> <!–- Audio Spanish --> <AdaptationSet mimeType="audio/mp4" codecs="mp4a.40.2" segmentAlignment="true" startWithSAP="1" language="es"> <BaseURL serviceLocation="dist2">http://distribution-2.com-provider-service.ms.as.3gppservices.org</BaseURL> <SegmentTemplate timescale="20" duration="40"/> <Representation id="a128" bandwidth="128000"> <Representation id="a64" bandwidth="64000"> </AdaptationSet> </Period> </MPD>

The SAND message is provided to the Media Player by the DANE instantiated in the 5GMSd AS. The example in listing G.4.5.4-1, indicates that only a service location that hosts one Representation is available; the HDR video and the Spanish language are available as well from service location dist3. The SAND message indicates that service location dist2 is available in the 5GMSd AS.

Listing G.4.5.4-1: Example SAND message indicating availability of content in 5GMSd AS

<SAND> <Status baseURL="http://distribution-2.com-provider-service.ms.as.3gppservices.org"> <ResourceStatus status="cached"/> </Status> <Status baseURL="http://distribution-1.com-provider-service.ms.as.3gppservices.org"> <ResourceStatus status="unavailable"/> </Status> <Status baseURL="http://distribution-3.com-provider-service.ms.as.3gppservices.org"> <ResourceStatus status="available"/> </Status> </SAND>

In case the 5GMSd AS fails to pre-populate content for service location dist2, it recommends switching to service location dist1 or dist3.

Listing G.4.5.4-2: Example SAND message indicating non-availability of content in 5GMSd AS

<SAND> <Status baseURL="http://distribution-2.com-provider-service.ms.as.3gppservices.org"> <ResourceStatus status="unavailable"/> </Status> <Status baseURL="http://distribution-1.com-provider-service.ms.as.3gppservices.org"> <ResourceStatus status="available"/> </Status> <Status baseURL="http://distribution-3.com-provider-service.ms.as.3gppservices.org"> <ResourceStatus status="available"/> </Status> </SAND>

# G.5	In-band client data reporting in DASH

## G.5.1	Overview

This clause provides details on how to instantiate in-band client data reporting as specified in clause 10.5.1 in DASH. In particular, this clause provides requirements and recommendations for the 5GMSd AS and 5GMSd Client to support the following features:

- CMCD-based client data reporting

## G.5.2	CMCD-based client data reporting

### G.5.2.1	Introduction

CMCD-based client data reporting is specified in clause 10.5.1 based on CTA-5004 [70].

### G.5.2.2	DASH content offering requirements and recommendations

A 5GMSd AS offering CMCD-based client reporting shall support:

-	The requirements and recommendations for the 5GMS AS for CMCD-based client data reporting as specified in clause 10.5.1.

-	The configuration of this feature by means of the client data reporting configuration for CMCD defined in clause K.3.7 of ISO/IEC 23009-1 [32], using the in-band Service Description defined in clause K.4.3.7.2 of ISO/IEC 23009-1 [32]. This includes for example:

-	The ability to restrict the subset of Service Locations, Adaptations Sets and/or media object types for which CMCD information is reported.

-	The ability to declare a content identifier for the purpose of client data reporting by populating the CMCD cid key based on the details in clauses K.3.7 and K.4.2.7 of ISO/IEC 23009-1 [32].

-	The ability to explicitly specify a set of CMCD keys to be reported based on the details in clauses K.3.7 and K.4.2.7 of ISO/IEC 23009-1 [32].

### G.5.2.3	Media Player requirements and recommendations

A Media Player supporting CMCD-based client data reporting shall support:

-	The requirements and recommendations for the Media Player for CMCD-based client data reporting specified in clause 10.5.1.

-	The derivation and reporting of all metrics defined in CTA-5004 [70].

-	The response to the content offering for CMCD-based client reporting as defined in clause G.5.2.2. If values are configured via the Media Player configuration and settings API at reference point M7d/M11d (see clause 13.2.4) and in the MPD, then the value provided on the M7d/M11d API shall take precedence as specified in clause 10.5.1.

### G.5.2.4	Examples (informative)

Listing G.5.2.4-1 provides an example for CMCD in-band configuration with the following properties:

-	Reporting is restricted to a single service location for either dist1 or dist2.

-	Reporting is restricted to video Adaptations Sets only.

-	Reporting is restricted to segments.

-	Content identifier reporting is requested because the CMCD cid key is included and the respective content identifier is specified.

-	A set of CMCD keys to be reported is specified as keys="br,bl,cid,dl,mtp,nor,ot,sf,v".

Listing G.5.2.4-1 MPD with CMCD inband configuration

<?xml version="1.0" encoding="UTF-8"?> <MPD xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xmlns="urn:mpeg:dash:schema:mpd:2011" xsi:schemaLocation="urn:mpeg:dash:schema:mpd:2011 DASH-MPD.xsd" type="dynamic" minimumUpdatePeriod="PT2S" timeShiftBufferDepth="PT30M" availabilityStartTime="2014-10-17T17:17:05Z" minBufferTime="PT4S" profiles="urn:mpeg:dash:profile:isoff-live:2011" publishTime="2014-10-17T17:17:05Z"> <BaseURL serviceLocation="dist1">http://distribution-1.com-provider-service.ms.as.3gppservices.org/</BaseURL> <BaseURL serviceLocation="dist2">http://distribution-2.com-provider-service.ms.as.3gppservices.org/</BaseURL> <ServiceDescription id="1250"> <Scope schemeIdUri="urn:mpeg:dash:event:service-description:2024"/> <Latency min="750" max="4200" target="1250" referenceId="7"/> <PlaybackRate min="0.96" max="1.04"/> <ClientDataReporting scheme="urn:mpeg:dash:cta-5004:2023" serviceLocations="dist1" adaptationSets="video"> <CMCDParameters mode="header" contentID="md:cid:EIDR:10.5240%2f0EFB-02CD-126E-8092-1E49-W" includeInRequests="segment" keys="br,bl,cid,dl,mtp,nor,ot,sf,v"/> </ClientDataReporting> </ServiceDescription> <ServiceDescription id="2500"> <Scope schemeIdUri="urn:mpeg:dash:event:service-description:2024"/> <Latency min="750" max="4200" target="2500" referenceId="7"/> <PlaybackRate min="0.96" max="1.04"/> <ClientDataReporting scheme="urn:mpeg:dash:cta-5004:2023" serviceLocations="dist2" adaptationSets="video"> <CMCDParameters mode="header" contentID="md:cid:EIDR:10.5240%2f0EFB-02CD-126E-8092-1E49-W" includeInRequests="segment" keys="br,bl,cid,dl,mtp,nor"/> </ClientDataReporting> </ServiceDescription> <Period id="1"> <!-- Video --> <AdaptationSet mimeType="video/mp4" codecs="avc1.4D401F" frameRate="30000/1001" segmentAlignment="true" startWithSAP="1"> <BaseURL>video/</BaseURL> <SegmentTemplate timescale="90000" initialization="$Bandwidth$/init.mp4v" media="$Bandwidth$/$Time$.mp4v"> <SegmentTimeline> <S t="0" d="180180" r="432"/> </SegmentTimeline> </SegmentTemplate> <Representation id="v0" width="320" height="240" bandwidth="250000"/> <Representation id="v1" width="640" height="480" bandwidth="500000"/> <Representation id="v2" width="960" height="720" bandwidth="1000000"/> </AdaptationSet> <!-- English Audio --> <AdaptationSet mimeType="audio/mp4" codecs="mp4a.40" lang="en" segmentAlignment="0" startWithSAP="1"> <SegmentTemplate timescale="48000" initialization="audio/en/init.mp4a" media="audio/en/$Time$.mp4a"> <SegmentTimeline> <S t="0" d="96000" r="432"/> </SegmentTimeline> </SegmentTemplate> <Representation id="a0" bandwidth="64000"/> </AdaptationSet> <!-- French Audio --> <AdaptationSet mimeType="audio/mp4" codecs="mp4a.40" lang="fr" segmentAlignment="0" startWithSAP="1"> <SegmentTemplate timescale="48000" initialization="audio/fr/init.mp4a" media="audio/fr/$Time$.mp4a"> <SegmentTimeline> <S t="0" d="96000" r="432"/> </SegmentTimeline> </SegmentTemplate> <Representation id="b0" bandwidth="64000"/> </AdaptationSet> </Period> </MPD>

CMMF in the 5GMS System

Annex H (normative):
Media streaming in the 5GMS System using CMMF

# H.1	Introduction

Coded Multisource Media Format (CMMF) as specified in ETSI TS 103 973 [77] is an extensible container format designed to facilitate the management and interchange of audio−visual media and metadata in one or more coded representations (e.g., encoded with Application Layer Forward Error Correction, linear, network, or channel codes). The coded media representations supported by CMMF enable the efficient use of multi-source, multipath, and multi-access connectivity for network-delivered applications.

This annex specifies the use of multi-source object coding using CMMF within the 5GMS System.

# H.2	Downlink media streaming from multiple service locations using CMMF

## H.2.1	General

CMMF as specified in ETSI TS 103 973 [77] may be employed to enable efficient downlink media streaming when concurrently using multiple service locations exposed by the 5GMSd AS at reference point M4d and/or by the 5GMSd Application Provider at reference point M2d or M13d. Media resources (e.g., CMAF segments) shall first be encoded into one or more CMMF transport resources using the profile specified in clause H.2.3.1, either by the 5GMSd Application Provider or by using Content Preparation in the 5GMSd AS. These CMMF transport resources shall be made available to 5GMSd Clients from one or more reference point M4d and/or M13d service locations. A CMMF Media Access Client of the Media Player in the 5GMSd Client shall then request and download these CMMF transport resources from the service locations in parallel. The CMMF Media Access Client decodes the received CMMF transport resources, recovering the media resource required for playback and presents this to its Media Playback and Content Decryption Platform.

The remainder of this annex specifies requirements necessary to enable downlink media streaming from multiple reference point M4d service locations using CMMF within the 5GMS System. Further discussion of reference points M8d and M13d for use in streaming media with CMMF are omitted since they are out of scope.

## H.2.2	Procedures for downlink media streaming from multiple service locations using CMMF

### H.2.2.1	Provisioning (M1d) and configuration (M3d) interfaces for downlink media delivery using CMMF

#### H.2.2.1.1	Overview

This clause defines all procedures and APIs used to provision and configure the 5GMS System to use CMMF for the purposes of downlink media delivery from multiple service locations.

#### H.2.2.1.2	Content Hosting

To provision CMMF downlink media streaming in the 5GMSd AF, the 5GMSd Application Provider shall use the Content Hosting Provisioning (M1) API specified in clause 7.6, following the Content Hosting provisioning (M1) procedures specified in clause 4.3.3.

To configure CMMF downlink media streaming in the 5GMSd AS, the 5GMSd AF shall use the Content Hosting Configuration (M3) API specified in clause 9.4, following the Content Hosting configuration (M3) procedures specified in clause 4.5.4.

When Content Hosting is provisioned, a service location exposed by the 5GMSd AS at reference point M4d shall host CMMF transport resources, each of which should comprise a unique CMMF representation or variant of the media resources to be streamed. Each CMMF transport resource shall be uniquely addressable as specified in clause 10.1.

#### H.2.2.1.3	Content Preparation

To provision Content Preparation of CMMF transport resources in the 5GMSd AF, the 5GMSd Application Provider shall use the Content Preparation Templates Provisioning (M1) API specified in clause 7.4, following the Content Preparation Template provisioning (M1) procedures specified in clause 4.3.5.

To configure Content Preparation of CMMF transport resources in the 5GMSd AS, the 5GMSd AF shall use the Content Preparation Templates Configuration (M3) API specified in clause 9.3, following the Content Preparation Template configuration (M3) procedures specified in clause 4.5.3.

When Content Preparation is provisioned to encode media resources into CMMF transport resources, a CMMF Encoder (as specified in clauses 4.2.1 and 4.3.3 of ETSI TS 103 973 [77]) is instantiated and configured in the 5GMSd AS by means of a Content Preparation Template. The following Content Preparation Template requirements apply:

-	A Content Preparation Template used to encode media resources (e.g., audio segments, video segments, etc.) shall create CMMF transport resources whose bitstreams conform to the profile specified in clause H.2.3.1.

-	Depending on the Content Hosting Configuration, the CMMF Encoder may be a single input, single output or a single input, multiple output process. A media resource (e.g., audio segment, video segment, etc.) made available to the 5GMSd AS at reference point M2d or M10d shall be ingested into the CMMF Encoder where it is encoded and packaged as one or more CMMF transport resources (each a unique CMMF representation or variant of the media resource), and those transport resources shall be made available at service locations exposed by the 5GMSd AS at reference point M4d.

-	CMMF transport resources shall be accessible via URLs that allow for the appropriate mapping between CMMF resource URLs used at service locations exposed by the 5GMSd AS at reference point M4d and their corresponding media resource URLs used for media ingest at reference points M2d or M10d. This mapping shall be determined using a member of the DistributionConfiguration.pathRewriteRules array specified in table 8.8.3.1-1 of TS 26.510 [56].

NOTE:	Differences between the URLs of media resources and their associated CMMF transport resources are restricted to differences in the path part of the URL between and including the leading "/" and the final "/" owing to restrictions imposed by the definition of the PathRewriteRule data type.

The Content Preparation Template for CMMF-enabled downlink media delivery is specified in clause H.2.3.2.

### H.2.2.2	Media Streaming (M4d) interface for downlink media delivery using CMMF

To serve CMMF transport resources to a CMMF Media Access Client at reference point M4d, the 5GMSd AS shall use the media streaming (M4) interface specified in clause 10.3A.3, following the media streaming (M4d) procedures specified in clause 4.6.3.

A media resource required by the Media Playback and Decryption Platform for immediate or delayed consumption shall be obtained by a CMMF Access Client downloading one or more CMMF transport resources (either partially or in full) where each CMMF transport resource is typically downloaded from a different service location exposed at reference point M4d.

If recovery of a media resource from a set of downloaded CMMF transport resources fails, the CMMF Media Access Client may attempt to acquire the media resource using an alternate method (for example, by retrieving the original media resource directly from a single service location exposed by the 5GMSd AS at reference point M4d).

### H.2.2.3	Media Session Handling (M5d) interface for downlink media streaming using CMMF

To perform media session handling for a media streaming session using CMMF, the Media Session Handler shall use the APIs specified in clause 11, following the Media Session Handling (M5) procedures specified in clause 4.7.

In cases where the Service Access Information for media streaming includes one or more Media Player Entries, at least one of these Media Player Entries shall be a URL to a CMMF Media Entry Point as specified in clause H.2.3.3.

### H.2.2.4	UE Media Player interface (M7d/M11d) for downlink media delivery using CMMF

To control a Media Player during a media streaming session using CMMF, the 5GMSd-Aware Application or Media Session Handler shall use the Media Stream Handler (M7/M11) APIs specified in clause 13, following the UE Media Player (M7d) procedures specified in clause 4.9.

### H.2.2.5	Media Player requirements and recommendations

A Media Player shall configure and use a CMMF Media Access Client upon receiving a CMMF Media Entry Point as specified in clause H.2.3.3. At a minimum, the CMMF Media Access Client shall support the following:

-	Translation of media resource (e.g., CMAF segment) URLs and the corresponding URLs pointing to CMMF transport resources containing encoded representations or variants of those media resources that are available from service locations exposed by the 5GMSd AS at reference point M4d. This mapping is provided by the CMMF Media Entry Point.

-	Decoding a received CMMF transport resource or jointly decoding multiple partially received CMMF transport resources that conform to the profile specified in clause H.2.3.1.

-	Recovery of media resources (e.g., CMAF segments) and delivery of these media resources to the Media Playback and Encryption Platform.

## H.2.3	Protocols and formats for downlink media streaming using CMMF

### H.2.3.1	HTTP-based CMMF delivery conformance profile

CMMF transport resources communicated within the 5GMS System shall conform to ETSI TS 103 973 [77].

NOTE:	A normative reference to a HTTP-based CMMF delivery conformance profile providing a minimum set of requirements that must be supported and based on ETSI TS 103 973 [77] will be added when available.

### H.2.3.2	Content Preparation Template for downlink media streaming using CMMF

This aspect is for future study.

NOTE:	A normative reference to a compliant CMMF Content Preparation Template and HTTP-based CMMF delivery conformance profile based on ETSI TS 103 973 [77] will be added when available.

### H.2.3.3	CMMF Media Player Entry

#### H.2.3.3.1	General

A CMMF Media Player Entry shall be a CMMF Extended File Delivery Table (EFDT) as specified in clause D.2.3 of ETSI TS 103 973 [77] and as further elaborated in clause H.2.3.3.2 below.

NOTE:	A normative reference to a CMMF Media Entry Point specifically for HTTP media streaming protocols (e.g., MPEG-DASH, HLS, etc.) based on ETSI TS 103 973 [77] will be added when available.

The CMMF Media Player Entry shall comprise:

-	A media presentation or a pointer to a media presentation (e.g., URL to a MPD for DASH, URL to a video file, etc.) and

-	Any necessary CMMF configuration information required by the CMMF Media Access Client to download CMMF transport resources, decode them, and recover the associated media resource(s) (e.g., CMAF segments).

When Content Preparation is provisioned within the 5GMS System to encode media resources (e.g., CMAF segments) into CMMF transport resources, URLs of the CMMF transport resources provided in the CMMF Media Player Entry shall be formatted in such a way that they can be mapped back to the URLs of their corresponding media resources (e.g., CMAF segment) by the 5GMSd AS so that the media resource may be ingested at reference point M2d. As specified in clause H.2.2.1.3, the URL format of these CMMF transport resources is determined by the provisioned Content Hosting Configuration (see the definition of DistributionConfiguration.PathRewriteRules specified in table 8.8.3.1-1 of TS 26.510 [56]).

Example CMMF Media Player Entries referencing a single video file and a DASH MPD can be found in clause H.3.2.

#### H.2.3.3.2	CMMF Media Player Entry based on Extended FDT

In the case where the media presentation (e.g., MPD for DASH, HLS playlist, etc.) referenced by a CMMF Media Entry Point is a document containing pointers to media resources (e.g., URLs to CMAF segments), the reference (e.g., URL) to the media presentation description shall be listed within the EFDT as TOI=”0”.

The EFDT shall provide a mapping between each of the URLs contained within the media presentation description and the URLs of the CMMF transport resources containing encoded representations or variants of those media resources that are available from service locations exposed by the 5GMSd AS at reference point M4d. For the EFDT, these mappings are provided using TOI values greater than 0.

The EFDT may also include media resource and CMMF transport resource availability information, including availability information relating to media resources and CMMF transport resources that may be available after the EFDT is made available to the CMMF Media Access Client. The CMMF Media Access Client may use this information to download relevant CMMF transport resources at appropriate times. Details are left for implementation.

# H.3	Examples of downlink media streaming using CMMF (informative)

## H.3.1	General

This annex provides examples of CMMF Media Player Entries and Content Hosting Configurations that support the delivery of CMMF-encoded media within the 5GMS System.

## H.3.2	CMMF Media Player Entry examples

### H.3.2.1	General

This clause provides examples showing how CMMF configuration information required by a 5GMS Client may be communicated within a CMMF Media Player Entry as specified in clause H.2.3.3. When applicable, a general example of a media asset is used, described by the DASH MPD shown in listing H.3.2.1-1, and notionally hosted by a hypothetical 5GMSd Application Provider at the URL https://example.com/manifest.mpd.

Listing H.3.2.1-1: Example MPD

<?xml version="1.0"?> <MPD profiles="urn:3GPP:PSS:profile:DASH10" type="static" minBufferTime="PT10S" mediaPresentationDuration="PT2H" availabilityStartTime="2010-04-01T09:30:47Z" availabilityEndTime="2010-04-07T09:30:47Z" xsi:schemaLocation="urn:mpeg:dash:schema:mpd:2011 3GPP-Rel10-MPD.xsd" xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xmlns="urn:mpeg:dash:schema:mpd:2011"> <ProgramInformation moreInformationURL="http://www.example.com"> <Title>Example</Title> </ProgramInformation> <Period start="PT0S"> <AdaptationSet mimeType="video/3gpp"> <ContentComponent contentType="video"/> <ContentComponent contentType="audio" lang="en"/> <Representation codecs="s263, samr" bandwidth="256000" id="256"> <BaseURL>"rep1"</BaseURL> <SegmentList duration="1000" timescale="100"> <Initialization sourceURL="seg-init.3gp"/> <SegmentURL media="seg-1.3gp"/> <SegmentURL media="seg-2.3gp"/> <SegmentURL media="seg-3.3gp"/> </SegmentList> </Representation> <Representation codecs="mp4v.20.9, mp4a.E1" bandwidth="128000" id="128"> <BaseURL>"rep2"</BaseURL> <SegmentList duration="10"> <Initialization sourceURL="seg-init.3gp"/> <SegmentURL media="seg-1.3gp"/> <SegmentURL media="seg-2.3gp"/> <SegmentURL media="seg-3.3gp"/> </SegmentList> </Representation> </AdaptationSet> </Period> <Period start="PT30S"> <SegmentTemplate duration="10" initialization="seg-init-$RepresentationId$.3gp" media="http://example.com/$RepresentationId$/$Number$.3gp"/> <AdaptationSet mimeType="video/3gpp" codecs="mp4v.20.9, mp4a.E1"> <ContentComponent contentType="video"/> <ContentComponent contentType="audio" lang="en"/> <Representation bandwidth="256000" id="1"/> <Representation bandwidth="128000" id="2"/> </AdaptationSet> </Period> </MPD>

### H.3.2.2	Single file EFDT example

The following example shows an EFDT where a single MP4 file is delivered using CMMF from three service locations exposed by the 5GMSd AS at reference point M4d.

Listing H.3.2.2-1: Single MP4 CMMF EFDT example

<?xml version="1.0" encoding="UTF-8"?> <FDTInstance xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xmlns="urn:ETSI:CMMF:2023:FDT" xsi:schemaLocation="urn:ETSI:CMMF:2023:FDT extendedFDT.xsd" Expires="2010-04-07T09:30:47Z" Complete="true" ContentType="video/mp4 codecs="mp4v.20.9, mp4a.E1" FEC-OTI-FEC-Encoding-ID="1"> <File TOI="0" Content-Location="https://example.com/video.mp4" Content-Length="64000"> <EncodedObjects type="source" complete="true"> https://example.com/video.mp4 </EncodedObjects> <EncodedObjects type="cmmf" complete="true"> https://distribution-a.com-provider-service.ms.as.3gppservices.org/cmmf-a/video.mp4 </EncodedObjects> <EncodedObjects type="cmmf" complete="true"> https://distribution-b.com-provider-service.ms.as.3gppservices.org/cmmf-b/video.mp4 </EncodedObjects> <EncodedObjects type="cmmf" complete="true"> https://distribution-c.com-provider-service.ms.as.3gppservices.org/cmmf-c/video.mp4 </EncodedObjects> </File> </FDTInstance>

### H.3.2.3	EFDT referencing a DASH MPD

The following example shows an EFDT where the contents of the presentation described by the MPD shown in listing H.3.2.1-1 are delivered using CMMF. It is assumed that CMMF transport resources for the media resources referenced within the MPD are accessible from three service locations exposed by the 5GMSd AS at reference point M4d.

In this example, a CMMF Media Access Client uses the EFDT shown in listing H.3.2.3-1 combined with the MPD shown in listing H.3.2.1-1 as the Media Player Entry. The EFDT includes a reference to the MPD (File element with TOI 0) and references to all segments referenced within the MPD are mapped to CMMF transport resources assigned TOI values 1 through 16.

Listing H.3.2.3-1: CMMF EFDT example referencing MPD

<?xml version="1.0" encoding="UTF-8"?> <FDTInstance xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xmlns="urn:ETSI:CMMF:2023:FDT" xsi:schemaLocation="urn:ETSI:CMMF:2023:FDT extendedFDT.xsd" Expires="2010-04-07T09:30:47Z" Complete="true" FEC-OTI-FEC-Encoding-ID="1"> <File TOI="0" Content-Location="https://example.com/manifest.mpd"> <EncodedObjects type="source" complete="true"> https://distribution-a.com-provider-service.ms.as.3gppservices.org/manifest.mpd </EncodedObjects> <EncodedObjects type="source" complete="true"> https://distribution-b.com-provider-service.ms.as.3gppservices.org/manifest.mpd </EncodedObjects> <EncodedObjects type="source" complete="true"> https://distribution-c.com-provider-service.ms.as.3gppservices.org/manifest.mpd </EncodedObjects> </File> <File TOI="1" Content-Location="https://example.com/rep1/seg-init.3gp"> <EncodedObjects type="source" complete="true"> https://example.com/rep1/seg-init.3gp </EncodedObjects> <EncodedObjects type="cmmf" complete="true"> https://distribution-a.com-provider-service.ms.as.3gppservices.org/rep1/cmmf-a /seg-init.3gp </EncodedObjects> <EncodedObjects type="cmmf" complete="true"> https://distribution-b.com-provider-service.ms.as.3gppservices.org/rep1/cmmf-b /seg-init.3gp </EncodedObjects> <EncodedObjects type="cmmf" complete="true"> https://distribution-c.com-provider-service.ms.as.3gppservices.org/rep1/cmmf-c /seg-init.3gp </EncodedObjects> </File> <File TOI="2" Content-Location="https://example.com/rep1/seg-1.3gp"> <EncodedObjects type="source" complete="true"> https://example.com/rep1/seg-1.3gp </EncodedObjects> <EncodedObjects type="cmmf" complete="true"> https://distribution-a.com-provider-service.ms.as.3gppservices.org/rep1/cmmf-a /seg-1.3gp </EncodedObjects> <EncodedObjects type="cmmf" complete="true"> https://distribution-b.com-provider-service.ms.as.3gppservices.org/rep1/cmmf-b /seg-1.3gp </EncodedObjects> <EncodedObjects type="cmmf" complete="true"> https://distribution-c.com-provider-service.ms.as.3gppservices.org/rep1/cmmf-c /seg-1.3gp </EncodedObjects> </File> ... <File TOI="8" Content-Location="https://example.com/rep2/seg-3.3gp"> <EncodedObjects type="source" complete="true"> https://example.com/rep2/seg-3.3gp </EncodedObjects> <EncodedObjects type="cmmf" complete="true"> https://distribution-a.com-provider-service.ms.as.3gppservices.org/rep2/cmmf-a /seg-3.3gp </EncodedObjects> <EncodedObjects type="cmmf" complete="true"> https://distribution-b.com-provider-service.ms.as.3gppservices.org/rep2/cmmf-b /seg-3.3gp </EncodedObjects> <EncodedObjects type="cmmf" complete="true"> https://distribution-c.com-provider-service.ms.as.3gppservices.org/rep2/cmmf-c /seg-3.3gp </EncodedObjects> </File> <File TOI="9" Content-Location="https://example.com/seg-init-1.3gp"> <EncodedObjects type="source" complete="true"> https://example.com/seg-init-1.3gp </EncodedObjects> <EncodedObjects type="cmmf" complete="true"> https://distribution-a.com-provider-service.ms.as.3gppservices.org/cmmf-a /seg-init-1.3gp </EncodedObjects> <EncodedObjects type="cmmf" complete="true"> https://distribution-b.com-provider-service.ms.as.3gppservices.org/cmmf-b /seg-init-1.3gp </EncodedObjects> <EncodedObjects type="cmmf" complete="true"> https://distribution-c.com-provider-service.ms.as.3gppservices.org/cmmf-c /seg-init-1.3gp </EncodedObjects> </File> <File TOI="10" Content-Location="https://example.com/1/1.3gp"> <EncodedObjects type="source" complete="true"> https://example.com/1/1.3gp </EncodedObjects> <EncodedObjects type="cmmf" complete="true"> https://distribution-a.com-provider-service.ms.as.3gppservices.org/1/cmmf-a/1.3gp </EncodedObjects> <EncodedObjects type="cmmf" complete="true"> https://distribution-b.com-provider-service.ms.as.3gppservices.org/1/cmmf-b/1.3gp </EncodedObjects> <EncodedObjects type="cmmf" complete="true"> https://distribution-c.com-provider-service.ms.as.3gppservices.org/1/cmmf-c/1.3gp </EncodedObjects> </File> ... <File TOI="16" Content-Location="https://example.com/2/3.3gp"> <EncodedObjects type="source" complete="true"> https://example.com/2/3.3gp </EncodedObjects> <EncodedObjects type="cmmf" complete="true"> https://distribution-a.com-provider-service.ms.as.3gppservices.org/2/cmmf-a/3.3gp </EncodedObjects> <EncodedObjects type="cmmf" complete="true"> https://distribution-b.com-provider-service.ms.as.3gppservices.org/2/cmmf-b/3.3gp </EncodedObjects> <EncodedObjects type="cmmf" complete="true"> https://distribution-c.com-provider-service.ms.as.3gppservices.org/2/cmmf-c/3.3gp </EncodedObjects> </File> </FDTInstance>

## H.3.3	Provisioning Session and Content Hosting Configuration examples

### H.3.3.1	General

This clause provides examples of provisioning the 5GMS System to deliver media resources using CMMF from multiple service locations exposed by the 5GMSd AS at reference point M4d. It is assumed within these examples that the CMMF Media Access Client extends functionality of the DASH Access Client (in particular the Downloader sub-function) specified in clause 13.2.1 to enable download and decoding of media resources encoded within CMMF transport resources.

### H.3.3.2	Example of media delivery from multiple service locations using CMMF

#### H.3.3.2.1	Overview

This example shows how CMMF can be used in a basic deployment where a single Provisioning Session is configured by the 5GMSd Application Provider to expose multiple service locations. This example assumes the 5GMSd Application Provider provisions the system in the following manner:

1.	A 5GMSd AS is provisioned to ingest media resources at reference point M2d.

2.	Media resources ingested at reference point M2d are encoded into CMMF transport resources by the 5GMSd AS using an appropriate CMMF Content Preparation Template as specified in clause H.2.3.2.

3.	As part of the Service Access Information, a CMMF Media Player Entry is provided as specified in clause H.2.3.3.

This implementation example is illustrated in figure H.3.3.2.1-1.

Figure H.3.3.2.1-1: Basic deployment example for 5GMSd AS ingest, content preparation, and delivery using CMMF

The following clauses describe how the 5GMS APIs and protocols are used to realize this example.

#### H.3.3.2.2	Provisioning Session provisioning

A Provisioning Session is created by the 5GMSd Application Provider using the operation specified in clause 5.2.2.3 of TS 26.510 [56] and the API specified in clause 8.2 of [56]. Example values of the provisioning session API parameters used are shown in table H.3.3.2.2-1.

Table H.3.3.2.2-1: Example ProvisioningSession resource parameters

| Property name | Property value | Assigned by |
| --- | --- | --- |
| provisioningSessionId | provisioning.session.a | 5GMSd AF |
| provisioningSessionType | MS_DOWNLINK | Media Application Provider |
| externalServiceId | com.‌provider.‌service |  |
| appId | dash.downlink.streaming |  |
| NOTE: Values for provisioningSessionId, externalServiceId, and appId are notional and may not reflect actual values. |  |  |

#### H.3.3.2.3	Content Preparation Templates provisioning

The 5GMSd Application Provider provisions one CMMF encoder Content Preparation Template within the provisioning.session.a Provisioning Session using the operation specified in clause 5.2.5.2 of TS 26.510 [56] and the API defined in clause 8.5 of [56]. The following is assumed:

-	A sub-path URL added by the 5GMSd Client in the M4d request URL is used to determine which CMMF transport resource representation of the media resource is generated. For the purposes of this example, the Content Preparation Template generates representation "CMMF-A" if a M4d request URL containing the sub-path cmmf-a is received; it generates representation "CMMF-B" if the URL contains the sub-path cmmf-b is received; and it generates representation "CMMF-C" if the URL contains the sub-path cmmf-c is received.

-	Content Preparation is triggered upon the receipt of a pull-based request to the 5GMSd AS at a service location exposed at reference point M4d.

-	The path of the requested resource is available to the Content Preparation Template associated with the service location that received the request.

-	Media resources (e.g., video segments, audio segments, etc.) are ingested by the 5GMSd AS at reference point M2d and encoded into CMMF transport resources according to the Content Preparation Template.

-	Upon completion of the CMMF encoding operation, the resulting CMMF transport resource is made available for caching by the 5GMSd AS and/or delivery to the requesting 5GMSd Client at the service location where the request was received.

Upon successful provisioning of the Content Preparation Template, the 5GMSd AF returns the Content Preparation Template ID cmmf.‌content.‌preparation.‌template to the Media Application Provider.

#### H.3.3.2.4	Server Certificates provisioning

The 5GMSd Application Provider provisions the Server Certificates for the Provisioning Session using the operation specified in clause 5.2.4.2 of TS 26.510 [56] and the API defined in clause 8.4 of [56]. The Server Certificates assume a canonical domain name as specified in clause 6.0.2.2. Example values for the certificates generated by the 5GMSd AF are provided in table H.3.3.2.4-1.

Table H.3.3.2.4-1: Example Server Certificate parameters

| CN | subjectAltName | certificateId |
| --- | --- | --- |
| *.com-provider-service.ms.as.3gppservices.org | *.com-provider-service.ms.as.3gppservices.org | server.certificate.a |

#### H.3.3.2.5	Content Hosting provisioning and configuration

The 5GMSd Application Provider provisions the Content Hosting Configuration for each Provisioning Session using the operation specified in clause 5.2.8.2 of TS 26.510 [56] and the API defined in clause 8.8 of [56].

Configuring the Content Hosting Configuration in the 5GMSd AS is performed according to:

-	The example base URL of the 5GMSd Application Provider’s origin server is https://origin.media-application-provider.com.

The 5GMSd Application Provider provides the Media Player Entry URL to the 5GMSd-Aware Application via reference point M8d. Based on this, the 5GMSd Client can download the Media Player Entry from a service location exposed by the 5GMSd AS at reference point M4d. Example Media Player Entry documents are provided in clause H.3.2.2.

Table H.3.3.2.5-1 provides example values for the Content Hosting Configuration parameters.

Table H.3.3.2.5-1: Content Hosting Configuration resource parameters

| Property name |  |  | Property value | Assigned by |
| --- | --- | --- | --- | --- |
| name |  |  | content-hosting-configuration-a | 5GMSd Application Provider |
| ingestConfiguration |  |  |  |  |
|  | mode |  | PULL | 5GMSd Application Provider |
|  | protocol |  | urn:3gpp:‌5gms:‌content-protocol:‌http-pull |  |
|  | baseURL |  | https://origin.media-application-provider.com |  |
| distributionConfiguration |  |  |  |  |
|  | affinityGroup |  | affinity.group.a | 5GMSd Application Provider |
|  | contentPreparation‌TemplateId |  | cmmf.content.preparation.template |  |
|  | certificateId |  | server.certificate.a |  |
|  | canonical‌DomainName |  | distribution-a.com-provider-service.ms.as.3gppservices.org | 5GMSd AF |
|  | baseURL |  | https://distribution-a.com-provider-service.ms.as.3gppservices.org |  |
|  | pathRewriteRule |  |  |  |
|  |  | requestPathPattern | cmmf-a/ | 5GMSd Application Provider |
|  |  | mappedPath |  |  |
| distributionConfiguration |  |  |  |  |
|  | affinityGroup |  | affinity.group.b | 5GMSd Application Provider |
|  | contentPreparation‌TemplateId |  | cmmf.content.preparation.template |  |
|  | certificateId |  | server.certificate.a |  |
|  | canonical‌DomainName |  | distribution-b.com-provider-service.ms.as.3gppservices.org | 5GMSd AF |
|  | baseURL |  | https://distribution-b.com-provider-service.ms.as.3gppservices.org |  |
|  | pathRewriteRule |  |  |  |
|  |  | requestPathPattern | cmmf-b/ | 5GMSd Application Provider |
|  |  | mappedPath |  |  |
| distributionConfiguration |  |  |  |  |
|  | affinityGroup |  | affinity.group.c | 5GMSd Application Provider |
|  | contentPreparation‌TemplateId |  | cmmf.content.preparation.template |  |
|  | certificateId |  | server.certificate.a |  |
|  | canonical‌DomainName |  | distribution-c.com-provider-service.ms.as.3gppservices.org | 5GMSd AF |
|  | baseURL |  | https://distribution-c.com-provider-service.ms.as.3gppservices.org |  |
|  | pathRewriteRule |  |  |  |
|  |  | requestPathPattern | cmmf-c/ | 5GMSd Application Provider |
|  |  | mappedPath |  |  |

#### H.3.3.2.6	End-to-end URL mapping

Table H.3.3.2.6-1 provides an example of the end-to-end mapping for requests initiated by the Media Player for a subset of the URLs provided in the example MPD shown in listing H.3.2.1-1 where the CMMF Media Access Client uses the corresponding URL mapping provided in the EFDT shown in listing H.3.2.2.3-1.

Table H.3.3.2.6-1: End-to-End URL mapping example

| MPD URL | M4d Request URLs | M2d Request URL |
| --- | --- | --- |
| rep1/seg-1.3gp | https://distribution-a.com-provider-service.‌ms.as.3gppservices.org/rep1/cmmf-a/seg-1.3gp | https://origin.media-application-provider.com/‌rep1/seg-1.3gp |
|  | https://distribution-b.com-provider-service.‌ms.as.3gppservices.org/rep1/cmmf-b/seg-1.3gp |  |
|  | https://distribution-c.com-provider-service.‌ms.as.3gppservices.org/rep1/cmmf-c/seg-1.3gp |  |
| http://example.com/1/1.3gp | https://distribution-a.com-provider-service.‌ms.as.3gppservices.org/1/cmmf-a/1.3gp | https://origin.media-application-provider.com/‌1/1.3gp |
|  | https://distribution-b.com-provider-service.‌ms.as.3gppservices.org/1/cmmf-b/1.3gp |  |
|  | https://distribution-c.com-provider-service.‌ms.as.3gppservices.org/1/cmmf-c/1.3gp |  |

### H.3.3.3	Example of media delivery from multiple service locations using CMMF and 5GMSd AS service chaining at reference point M10d

#### H.3.3.3.1	Overview

This example shows how CMMF can be used in a deployment where 5GMSd AS service chaining at reference point M10d is provisioned. This example assumes the 5GMSd Application Provider provisions the system in the following manner:

1.	A 5GMSd AS is provisioned to serve as an origin shield intended to reduce requests for content at reference point M2d. Furthermore, the creation of CMMF transport resources from ingested content at reference point M2d using Content Preparation is performed within this 5GMSd AS.

2.	Two 5GMSd AS instances are provisioned to serve 5GMSd Clients from exposed service locations at reference point M4d. Both 5GMSd AS instances are configured to ingest CMMF transport resources (unique to their exposed service locations) from the 5GMSd AS serving as the origin shield at reference M10d.

2.	Media resources ingested at reference point M2d are encoded into CMMF transport resources using an appropriate CMMF Content Preparation Template as specified in clause H.2.3.1.

3.	A CMMF Media Player Entry as specified in clause H.2.3.3.

This implementation example is illustrated in figure H.3.3.3.1-1.

Figure H.3.3.3.1-1: Centralized 5GMSd AS content preparation and ingest deployment example

The following clauses describe how the 5GMS APIs and protocols are used to realize this example.

#### H.3.3.3.2	Provisioning Session provisioning

Three provisioning sessions are created by the 5GMSd Application Provider using the operation specified in clause 5.2.2.3 of TS 26.510 [56] and the API specified in clause 8.2 of [56]. Example values for the three Provisioning Session parameters used are shown in table H.3.3.3.2-1.

Table H.3.3.3.2-1: ProvisioningSession resource parameters

| Property name | Provisioning Session A (Trusted DN) | Provisioning Session B (Trusted DN) | Provisioning Session C (External DN) | Assigned by |
| --- | --- | --- | --- | --- |
| provisioningSessionId | provisioning. session.a | provisioning. session.b | provisioning. session.c | Media AF |
| provisioningSession‌Type | MS_DOWNLINK | MS_DOWNLINK | MS_DOWNLINK | Media Application Provider |
| externalServiceId | com.‌provider.‌service.a | com.‌provider.‌service.b | com.‌provider.‌service.c |  |
| appId | dash.downlink.‌streaming | dash.downlink.‌streaming | dash.downlink.‌streaming |  |
| NOTE: Values for provisioningSessionId, externalServiceId, and appId are notional and may not reflect actual values. |  |  |  |  |

#### H.3.3.3.3	Content Preparation Templates provisioning

The 5GMSd Application Provider provisions one CMMF encoder Content Preparation Template within the provisioning.session.a Provisioning Session using the operation specified in clause 5.2.5.2 of TS 26.510 [56] and the API defined in clause 8.5 of [56]. The following is assumed:

-	Content Preparation is triggered upon the receipt of a pull-based request at reference point M10d.

-	The path of the requested resource is available to the Content Preparation Template.

-	Media resources (e.g., video segments, audio segments, etc.) are ingested by the 5GMSd AS at reference point M2d and encoded into CMMF transport resources by the Content Preparation Template.

-	A sub-path URL added by the 5GMSd Client in the M4d request URL is used to determine which CMMF transport resource representation of the media resource is generated. For the purposes of this example, the Content Preparation Template generates representation "CMMF-B" if the URL contains the sub-path cmmf-b is received, and it generates representation "CMMF-C" if the URL contains the sub-path cmmf-c is received.

-	Upon completion of the CMMF encoding operation, the CMMF transport resource is made available for caching by the 5GMSd AS and/or delivery to the requesting entity.

Upon successful provisioning of the Content Preparation Template, the 5GMSd AF returns the Content Preparation Template ID cmmf.‌content.‌preparation.‌template to the Media Application Provider.

#### H.3.3.3.4	Server Certificates provisioning

The 5GMSd Application Provider provisions the Server Certificates for each Provisioning Session using the operation specified in clause 5.2.4.2 of TS 26.510 [56] and the API defined in clause 8.4 of [56]. The Server Certificates assume a canonical domain name as specified in clause 6.0.2.2. Example values for the certificates generated by the 5GMSd AF are provided in table H.3.3.3.4-1.

Table H.3.3.3.4-1: Server Certificates

| provisioningSessionId | CN | subjectAltName | certificateId |
| --- | --- | --- | --- |
| provisioning.session.a | *.com-provider-service-a.‌ms.as.‌3gppservices.org | *.com-provider-service-a.‌ms.as.‌3gppservices.org | server.‌certificate.a |
| provisioning.session.b | *.com-provider-service-b.‌ms.as.‌3gppservices.org | *.com-provider-service-b.‌ms.as.‌3gppservices.org | server.‌certificate.b |
| provisioning.session.c | *.com-provider-service-c.‌ms.as.‌3gppservices.org | *.com-provider-service-c.‌ms.as.‌3gppservices.org | server.‌certificate.c |

#### H.3.3.3.5	Content Hosting provisioning and configuration

The 5GMSd Application Provider provisions the Content Hosting Configuration for each Provisioning Session using the operation specified in clause 5.2.8.2 of TS 26.510 [56] and the API defined in clause 8.8 of [56].

Configuring the Content Hosting Configuration in each 5GMSd AS is performed according to:

-	The example base URL of the Media Application Provider’s origin server is https://origin.media-application-provider.com.

-	The creation of the Content Hosting Configuration for Provisioning Session provisioning.session.a is completed prior to the creation of the Content Hosting Configuration for Provisioning Sessions provisioning.session.b and provisioning.session.c.

The 5GMSd Application Provider provides the Media Player Entry URL to the 5GMSd-Aware Application via reference point M8d. Based on this, the 5GMSd Client can download the Media Player Entry from a service location exposed by the 5GMSd AS at reference point M4d. Example Media Player Entry documents are provided in clause H.3.2.2.

Table H.3.3.3.5-1 provides example values for the Content Hosting Configuration API parameters for all three Provisioning Sessions.

Table H.3.3.3.5-1: ContentHostingConfiguration resource parameters

| Property name |  |  | ProvisioningSession A |  | ProvisioningSession B |  | Provisioning Session C |  | Assigned by |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| name |  |  | content-hosting-configuration-a |  | content-hosting-configuration-b |  | content-hosting-configuration-c |  | 5GMSd Application Provider |
| ingestConfiguration |  |  |  |  |  |  |  |  |  |
|  | mode |  |  | PULL |  | PULL |  | PULL | 5GMSd Application Provider |
|  | protocol |  |  | urn:3gpp:‌5gms:‌content-protocol:‌http-pull |  | urn:3gpp:‌5gms:‌content-protocol:‌http-pull |  | urn:3gpp:‌5gms:‌content-protocol:‌http-pull |  |
|  | baseURL |  |  | https://origin.‌media-application-provider.com |  | https://‌distribution-a.‌com-provider-service-a.‌ms.as.‌3gppservices.org |  | https://‌distribution-a.‌com-provider-service-a.‌ms.as.‌3gppservices.org |  |
|  | distributionConfiguration |  |  |  |  |  |  |  |  |
|  | contentPreparation‌TemplateId |  |  | cmmf.content.‌preparation.‌template |  |  |  |  | 5GMSd Application Provider |
|  | certificateId |  |  | server.‌certificate.a |  | server.‌certificate.b |  | server.‌certificate.c |  |
|  | canonical‌DomainName |  |  | distribution-a.‌com-provider-service-a.‌ms.as.‌3gppservices.org |  | distribution-a.‌com-provider-service-b.‌ms.as.‌3gppservices.org |  | distribution-a.‌com-provider-service-c.‌ms.as.‌3gppservices.org | 5GMSd AF |
|  | baseURL |  |  | https://‌distribution-a.‌com-provider-service-a.‌ms.as.‌3gppservices.org |  | https://‌distribution-a.com-provider-service-b.ms.as.‌3gppservices.org |  | https://‌distribution-a.‌com-provider-service-c.‌ms.as.‌3gppservices.org |  |
|  | pathRewriteRule |  |  |  |  |  |  |  |  |
|  |  | requestPathPattern |  | (cmmf-[b-c]/)$ |  |  |  |  | 5GMSd Application Provider |
|  |  | mappedPath |  |  |  |  |  |  |  |

#### H.3.3.3.6	End-to-end URL mapping

Table H.3.3.3.6-1 provides an example of the end-to-end mapping for requests initiated by the Media Player for a subset of the URLs provided in the example MPD shown in listing H.3.2.1-1 where the CMMF Media Access Client uses the corresponding URL mapping provided in the EFDT shown in listing H.3.2.2.3-1.

Table H.3.3.3.6-1: End-to-End URL mapping example

| MPD URL | M4d Request URL | M10d Request URL | M2d Request URL |
| --- | --- | --- | --- |
| rep1/seg-1.3gp | https://distribution-a.com-provider-service-b.‌ms.as.3gppservices.org/ rep1/cmmf-b/seg-1.3gp | https://distribution-a.com-provider-service-b.‌ms.as.3gppservices.org/ rep1/cmmf-b/seg-1.3gp | https://‌origin.media-application-provider.com/‌rep1/seg-1.3gp |
|  | https://distribution-a.‌com-provider-service-c.‌ms.as.3gppservices.org/‌rep1/cmmf-c/seg-1.3gp | https://distribution-a.‌com-provider-service-c.‌ms.as.3gppservices.org/‌rep1/cmmf-c/seg-1.3gp |  |
| http://example.com/1/1.3gp | https://distribution-a.‌com-provider-service-b.‌ms.as.3gppservices.org/‌1/cmmf-b/1.3gp | https://distribution-a.‌com-provider-service-b.‌ms.as.3gppservices.org/‌1/cmmf-b/1.3gp | https://‌origin.media-application-provider.com/‌1/1.3gp |
|  | https://distribution-a.‌com-provider-service-c.ms.as.3gppservices.org/‌1/cmmf-c/1.3gp | https://distribution-a.‌com-provider-service-c.‌ms.as.3gppservices.org/‌1/cmmf-c/1.3gp |  |

End of changes