3GPP TSG-SA WG6 Meeting #68	S6-253478

Goteborg, Sweden 25th – 29th August 2025	(revision of S6-25xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |
|  | 23.281 | CR | 0256 | rev | - | Current version: | 20.0.0 |  |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Ambient viewing response IE corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC |  |  |  |  |  | Date: |  |  | 2025-08-18 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR align the ambient viewing call response IE with the same IE from the ambient listening in MCPTT. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changing the IE Result to mandatory |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment between the IE of ambient viewing and ambient listening. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 7.6.2.2	Ambient viewing call response

Table 7.6.2.2-1 describes the information flow ambient viewing call response from the MCVideo client to the MCVideo server and MCVideo server to the MCVideo client.

Table 7.6.2.2-1: Ambient viewing call response

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the "viewing" user |
| MCVideo ID | M | The MCVideo ID of the "viewed to" user |
| SDP answer (NOTE 1) | O | Media parameters |
| Ambient viewing type (NOTE 2) | O | The ambient viewing type indicates remotely initiated ambient viewing call or locally initiated ambient viewing call. |
| Result | M | It indicates the success or failure of the ambient viewing request |
| Reason (NOTE 2) | O | It indicates the reason of failure of ambient viewing request, e.g., due to the engagement of the "viewed to" user in MCVideo communication. |
| NOTE 1: This information element is included when the Result information element is success. NOTE 2: This information element is included when the Result information element is failure. |  |  |

# * * * * End of changes * * * *