3GPP TSG-SA WG6 Meeting #69	S6-254426

Wuhan, China 13th – 17th October 2025	(revision of S6-254206)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0705 | rev | 1 | Current version: | 20.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Preconfigured ad hoc group regroup procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson; Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | FRMCS_Ph6-MC |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The clause introduces the possibility to combine multiple ad hoc group calls via preconfigured regroup. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a new procedure for preconfigured ad hoc group regroup |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 1 requirements related to combine multiple ad hoc groups is not fulfilled |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.15.3.2.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 10.15.3.2.x	Preconfigured ad hoc group regroup formation in a single MC system

Figure 10.15.3.2.x-1 illustrates the procedure to initiate an ad hoc group regroup using a preconfigured MC service group based on a request from an authorised MC service user.

NOTE:	The upper limit of the number of ad hoc groups to be regrouped is up to implementation.

Pre-conditions:

-	An ad hoc group communication associated to ad hoc group 1 is ongoing.

-	An ad hoc group communication associated to ad hoc group 2 is ongoing.

-	MC service clients 2, …, n are implicitly affiliated to ad hoc group 1.

-	MC service clients n+1, …, m are implicitly affiliated to ad hoc group 2.

-	MC service client 1 is authorized to initiate a preconfigured ad hoc group regroup procedure.

Figure 10.15.3.2.x-1: Preconfigured ad hoc group regroup procedure in a single MC system

1.	The authorized user of MC service client 1 initiates a preconfigured ad hoc group regroup procedure, specifying the list of MC service group IDs to be regrouped (MC service groups 1 and 2), the MC service group ID of the regroup group (if available) and a potential MC service group ID of the group from which configuration information for the regroup group may be taken.

2.	MC service client 1 sends the preconfigured regroup request to the MC service server.

3.	The MC service server checks that MC service client 1 is authorized to initiate a preconfigured ad hoc group regroup procedure, and resolves the group identities of the MC service groups requested in step 1. The MC service server also checks that none of the MC service groups that are requested for regrouping are already regrouped by any mechanism. If the preconfigured regroup request is authorized, the MC service server assigns a MC service group ID for this regroup group if:

i. one is not provided in the request or

ii. the one provided in the request is not accepted.

4a.	If the MC service server determines that any of the ad hoc groups requested for regrouping, including the regroup group, have been regrouped by other group regrouping procedures, the MC service server shall then send a preconfigured regroup reject back to MC service client 1 with a reject reason indicating that one of the groups has already been regrouped, and this procedure terminates.

4b.	Otherwise, the MC service server shall send the preconfigured regroup request return message to MC service client 1 containing the below:

i.	result of whether the preconfigured regroup is authorized or not; and

ii.	the MC service group ID.

iii.	the pre-configured group ID, whose configuration is used for the preconfigured ad hoc group regroup in case the one provided in step 2 is not suitable.

5a/b.	The MC service server sends the preconfigured regroup requests to MC service clients 2, …, n and MC service clients n+1, …, m.

6a/b.	MC service clients 2, …, n and MC service clients n+1, …, m notify their users of the regrouping.

7a/b.	MC service clients 2, …, n and MC service clients n+1 may send the preconfigured regroup response to the MC service server to acknowledge the regrouping action. These acknowledgements are not sent in response to a multicast transmission of the preconfigured regroup request.

8.	The MC service server implicitly affiliates the regrouped MC service clients to the regroup group.

9.	The MC service server sends a preconfigured regroup response to MC service client 1.

10.	The media plane and floor control associated with the regroup is established.

The regrouping remains in effect until the ongoing ad hoc group communication associated with the ad hoc regroup group is released. Once the ad hoc group communication associated with the regroup ad hoc group is terminated, the regroup ad hoc group ceases to exist.

Editor's Note:	It is FFS whether there is a need for a procedure to regroup multiple ad hoc groups from different MC service servers.

# * * * * End of changes * * * *