3GPP TSG-SA WG6 Meeting #70	S6-255346

Dallas, Texas, USA 17th – 21st November 2025	(revision of S6-255062)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.379 | CR | 0489 | rev | 2 | Current version: | 20.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Call forwarding for Ad hoc Group calls single MCPTT system (procedures) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Kontron Transportation France |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | FRMCS_Ph6-MC |  |  |  |  |  | Date: |  |  | 2025-11-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Stage1 has agreed CR0083 against TS 22.179 on call forwarding for ad hoc group call (clause 6.20.1): [R-6.20.1-001] The MCPTT Service shall provide a mechanism for an authorized MCPTT User, receiving an Ad hoc Group call, based on its MCPTT user Identity or its associated functional alias to configure the forwarding of an incoming Ad hoc Group call, to another MCPTT user in the following situations: - Always - If the receiving MCPTT User is not reachable (e.g., out of coverage) - If the incoming Ad hoc Group call is a call with manual commencement mode and the receiving MCPTT User does not answer within a configured period of time - If the receiving MCPTT User rejects an incoming call (in manual commencement mode) [R-6.20.1-002] The MCPTT Service shall provide a mechanism to inform the initiator of the Ad hoc Group call that the call has been successfully forwarded to another MCPTT user by providing the MCPTT user Identity and/or associated functional alias of the forwarding target. This CR addresses the requirements from above. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the procedures to specify call forwarding for ad hoc group calls within a single MCPTT system |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 1 requirement to provide call forwarding for ad hoc group call is not fulfilled |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.19.3.1.8, (new) 10.19.3.1.8.1, (new) 10.19.3.1.8.2, (new) 10.19.3.1.8.3, (new) 10.19.3.1.8.4, (new) 10.19.3.1.8.5, (new) 10.19.3.1.8.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

##### 10.19.3.1.8	Call forwarding for ad hoc group calls

###### 10.19.3.1.8.1	General

Call forwarding of MCPTT ad hoc group calls allows users to configure forwarding of incoming ad hoc group call requests (with or without Floor control) to another MCPTT user depending on the following conditions:

-	Immediate

-	No answer within a configured period (manual commencement mode)

-	Based on manual user input (manual commencement mode)

Based on manual input of the called MCPTT user is on a per call basis where the called MCPTT user responds to an incoming MCPTT ad hoc group call request by requesting redirection towards another MCPTT user.

###### 10.19.3.1.8.2	Updating the configuration for MCPTT ad hoc group call forwarding

An authorized user can update the call forwarding related parameters stored in the user profile using the procedures as defined in 3GPP TS 23.280 [8] clause 10.1.4.5. The parameters controlling call forwarding for MCPTT ad hoc group call are the same ones as used for MCPTT private call forwarding.

###### 10.19.3.1.8.3	Immediate call forwarding of an ad hoc group call

Figure 10.19.3.1.8.3-1 below illustrates the ad hoc group call setup procedure initiated by an authorized user when immediate call forwarding of the ad hoc group call is applied.

Pre-conditions:

1.	MCPTT client 3 is authorized to use ad hoc group call forwarding and has immediate call forwarding enabled towards MCPTT client 4.

2.	The number of participants being invited for the ad hoc group call is within the limit.

Figure 10.19.3.1.8.3-1: Immediate call forwarding of an ad hoc group call

1.	The user at MCPTT client 1 would like to initiate an ad hoc group call. The MCPTT client 1 initiates the ad hoc group call by sending the ad hoc group call request containing either a list of participants or criteria or an ad hoc group ID from an ad hoc group emergency alert to the MCPTT server.

2.	If the ad hoc group call is supported, the MCPTT server verifies whether the user at MCPTT client 1 is authorized to initiate an ad hoc group call.

3.	The MCPTT server sends the ad hoc group call request return message to MCPTT client 1.

4.	The MCPTT server 1 determines the list of participants when the ad hoc group call is determined by criteria. The MCPTT server checks if any of the invited MCPTT clients have call forwarding enabled. The MCPTT server detects that MCPTT client 3 has immediate call forwarding configured and enabled and checks the redirection counter of MCPTT client 3. The redirection counter is below the limit.

5.	The MCPTT server increments the redirection counter and determines the new target.

6a-6b.	The MCPTT server sends the ad hoc group call requests towards the MCPTT clients of the invited users based on step 1 and as determined in step 5.

7a-7b.	The receiving MCPTT clients notify their corresponding MCPTT user about the incoming ad hoc group call request with the information of the MCPTT group ID of the ad hoc group.

8a-8b.	The receiving MCPTT clients may accept or reject the ad hoc group call requests and send ad hoc group call responses to the MCPTT server.

9.	The MCPTT server sends the ad hoc group call response to MCPTT client 1 through the signalling path to inform about successful call establishment.

10.	The MCPTT server may notify the initiating MCPTT user of all MCPTT users joining or leaving the MCPTT ad hoc group call.

11.	MCPTT client 1, MCPTT client 2, and MCPTT client 4 establish media plane and floor control resources.

NOTE:	Step 11 can occur any time following step 9 if the conditions to proceed with the call are met.

If the ad hoc group call was initiated using criteria, the criteria is still applied to MCPTT client 3 and determines the participation of MCPTT client 4 to the ad hoc group call.

###### 10.19.3.1.8.4	Call forwarding of an ad hoc group call in case of no answer

Figure 10.19.3.1.8.4-1 below illustrates the ad hoc group call setup procedure initiated by an authorized user when call forwarding of the ad hoc group call in case of no answer is applied.

Pre-conditions:

1.	MCPTT client 3 is authorized to use ad hoc group call forwarding and has call forwarding no answer enabled with destination of MCPTT client 4.

2.	The number of participants being invited for the ad hoc group call is within the limit.

Figure 10.19.3.1.8.4-1: Call forwarding of an ad hoc group call in case of no answer

1.	The user at MCPTT client 1 would like to initiate an ad hoc group call. The MCPTT client 1 initiates the ad hoc group call by sending the ad hoc group call request containing either a list of participants or criteria or an ad hoc group ID from an ad hoc group emergency alert to the MCPTT server.

2.	If the ad hoc group call is supported, the MCPTT server verifies whether the user at MCPTT client 1 is authorized to initiate an ad hoc group call.

3.	The MCPTT server sends the ad hoc group call request return message to MCPTT client 1.

4a-4b.	The MCPTT server 1 determines the list of participants when the ad hoc group call is determined by criteria. The MCPTT server sends the ad hoc group call requests towards the MCPTT clients of the invited users.

5a-5b.	The receiving MCPTT clients notify their corresponding MCPTT user about the incoming ad hoc group call request with the information of the MCPTT group ID for the ad hoc group.

6a.	The receiving MCPTT clients may accept or reject the ad hoc group call requests and send ad hoc group call responses to the MCPTT server. The response may also contain a functional alias of the responding MCPTT user, which is verified (valid and activated for the user) by the MCPTT server.

7.	The MCPTT client 3 does not accept the incoming the ad hoc group call request within the specified time interval.

8.	The MCPTT server checks that MCPTT client 3 has call forwarding no answer enabled and that the redirection counter for MCPTT client 3 is below the limit.

9.	The MCPTT server increments the redirection counter MCPTT client 3.

10.	The MCPTT server sends an ad hoc group call request towards MCPTT client 4.

11.	The user at MCPTT client 4 is notifed about the incoming ad hoc group call request with the information of the MCPTT group ID for the ad hoc group

12.	The receiving MCPTT client 4 may accept or reject the ad hoc group call requests and sends an ad hoc group call response to the MCPTT server.

13.	The MCPTT server sends the ad hoc group call response to MCPTT client 1 through the signalling path to inform about successful call establishment.

14.	The MCPTT server may notify the initiating MCPTT user of all MCPTT users joining or leaving the MCPTT ad hoc group call.

15.	MCPTT client 1, MCPTT client 2 and MCPTT client 4 establish media plane and floor control resources.

NOTE:	Step 15 can occur any time following step 13 if the conditions to proceed with the call are met.

If the ad hoc group call was initiated using criteria, the criteria is still applied to MCPTT client 3 and determines the participation of MCPTT client 4 to the ad hoc group call.

###### 10.19.3.1.8.5	Call forwarding of an ad hoc group call based on manual user input

Figure 10.19.3.1.8.5-1 below illustrates the ad hoc group call setup procedure initiated by an authorized user when call forwarding of the ad hoc group call based on manual user input is applied.

Pre-conditions:

1.	MCPTT client 3 is authorized to use ad hoc group call forwarding.

2.	The number of participants being invited for the ad hoc group call is within the limit.

Figure 10.19.3.1.8.5-1: Call forwarding of an ad hoc group call based on manual user input

1.	User at MCPTT client 1 would like to initiate an ad hoc group call. The MCPTT client 1 initiates the ad hoc group call by sending the ad hoc group call request containing either a list of participants or criteria or an ad hoc group ID from an ad hoc group emergency alert to the MCPTT server.

2.	If the ad hoc group call is supported, the MCPTT server verifies whether the user at MCPTT client 1 is authorized to initiate an ad hoc group call.

3.	The MCPTT server sends the ad hoc group call request return message to MCPTT client 1.

4a-4b.	The MCPTT server 1 determines the list of participants when the ad hoc group call is determined by criteria. The MCPTT server sends the ad hoc group call requests towards the MCPTT clients of the invited users.

5a-5b.	The receiving MCPTT clients notify their corresponding MCPTT user about the incoming ad hoc group call request with the information of the MCPTT group ID for the ad hoc group.

6a.	The receiving MCPTT clients may accept or reject the ad hoc group call requests and send ad hoc group call responses to the MCPTT server.

7.	The user at MCPTT client 3 requests the call to be forwarded based on manual input.

8.	MCPTT client 3 sends an ad hoc group call forwarding request to the MCPTT server.

9.	The MCPTT server checks that MCPTT client 3 is authorized to perform call forwarding based on manual input, and that the redirection counter for MCPTT client 3 is below the limit.

10.	The MCPTT server increments the redirection counter MCPTT client 3.

11.	The MCPTT server sends an ad hoc group call request towards the MCPTT client 4.

12.	The user at MCPTT client 4 is notifed about the incoming ad hoc group call request with the information of the MCPTT group ID for the ad hoc group.

13.	The receiving MCPTT client 4 may accept or reject the ad hoc group call request and sends an ad hoc group call response to the MCPTT server.

14.	The MCPTT server sends an ad hoc group call forwarding response to MCPTT client 3 to indicate whether the forwarding request was successful.

15.	The MCPTT server sends the ad hoc group call response to MCPTT client 1 through the signalling path to inform about successful call establishment.

16.	The MCPTT server may notify the initiating MCPTT user of all MCPTT users joining or leaving the MCPTT ad hoc group call.

17.	MCPTT client 1, MCPTT client 2 and MCPTT client 4 establish media plane and floor control resources.

NOTE:	Step 17 can occur any time following step 15 if the conditions to proceed with the call are met.

If the ad hoc group call was initiated using criteria, the criteria is still applied to MCPTT client 3 and determines the participation of MCPTT client 4 to the ad hoc group call.

###### 10.19.3.1.8.6	Handling of multiple ad hoc group call forwardings

If the target of the ad hoc group call forwarding has also call forwarding activated the additional forwardings are performed sequentially. However, to avoid excessive numbers of forwardings and to avoid indefinite loops in the forwarding a limit for immediate forwardings is defined in the service configuration. For forwarding of ad hoc group calls this limit is used for all call forwarding types. The MCPTT server keeps track of the number of forwardings, if the limit is exceeded, the call forwarding towards the MCPTT client will be omitted.

* * * End of Changes * * *