Source:	Audio SWG Chair

Title:	Report on Audio SWG Call June 4, 2025

Agenda Item:	5.1

# Executive Summary

The Audio SWG (46 participants, see Annex B) met on June 4, 2025, 15:00-16:56 CEST (56 minutes beyond the initially scheduled end time). The meeting was dedicated to FS_ULBC.

The meeting is summarized as follows:

- There were 6 contributions (S4aA250030-S4aA250037 including revisions) on channel simulation parameters and one document (S4aA250038) summarizing the differences between the other inputs and the reasons for such differences. The latter also listed common and additionally proposed TBS and bitrate values from S4aA250030, S4aA250031, S4aA250035, and S4aA250037. All documents were noted, but there were a few agreements as follows.

- It was agreed to add references to TR 36.763 on GEO satellite parameters for simulation calibration and Link budget results for GEO satellite case into the draft TR based on the proposal in S4aA250035.

- Further, it was agreed to start simulations with the common numbers of the proposals, listed in black text in S4aA250038. The new proposed values (blue numbers in S4aA250038) will be discussed further in the next meeting.

  - The agreement does not exclude any other configurations, and additional values may be considered later.

# 1.1 	Opening of the session and registration of documents

The Audio SWG Chairman, Mr. Tomas Toftgård (Ericsson), opened the call on June 4, 2025, 15:00 CEST. The agenda with Tdoc allocation was presented and approved (see Annex A). Notes were taken by Thomas Stockhammer (with assistance of Copilot).

# 1.2                  IPR and antitrust reminder

The attention of the delegates to the meeting of this SWG Group was drawn to the fact that 3GPP Individual Members have the obligation under the IPR Policies of their respective Organizational Partners to inform their respective Organizational Partners of Essential IPRs they become aware of.

The delegates were asked to take note that they were thereby invited:

- to investigate whether their organization or any other organization owns IPRs which were, or were likely to become Essential in respect of the work of 3GPP.

- to notify their respective Organizational Partners of all potential IPRs, e.g., for ETSI, by means of the IPR Information Statement and the Licensing declaration forms.

The attention of the delegates to the meeting was drawn to the fact that 3GPP activities were subject to all applicable antitrust and competition laws and that compliance with said laws was therefore required by any participant of the meeting, including the Chair and Vice-Chairs and were invited to seek any clarification needed with their legal counsel. The leadership would conduct the present meeting with impartiality and in the interests of 3GPP. Delegates were reminded that timely submission of work items in advance of TSG/WG/SWG meetings was important to allow for full and fair consideration of such matters.

# 1.3                  Reports/Liaisons from other groups/meetings

None.

# 1.4                  Release 18 and earlier matters

None.

# 1.5                  IVAS_Codec_Ph2 (EVS Codec Extension for Immersive Voice and Audio Services, Phase 2)

None.

# 1.6                  ATIAS_Ph2 (Terminal Audio quality performance and Test methods for Immersive Audio Services, Phase 2)

None.

# 1.7                  DaCAS (Diverse audio CApturing system for Smartphone devices)

None.

# 1.8                  FS_ACAPI (Study on Audio Codec APIs)

None.

# 1.9	FS_ULBC (Study on Ultra Low Bitrate Speech Codec)

| S4aA250030 | Parameter updates on end to end simulation | vivo |
| --- | --- | --- |

Email discussion:

| 回复: [1.9 FS_ULBC, 030, Wednesday June 4, 1400 CEST] Parameter updates on end to end simulation | Ruonan Yang <RuonanYang@VIVO.COM> | Wed, 4 Jun 2025 07:17:37 +0000 |
| --- | --- | --- |
| Re: [1.9 FS_ULBC, 030, Wednesday June 4, 1400 CEST] Parameter updates on end to end simulation | Bruhn, Stefan <Stefan.Bruhn@DOLBY.COM> | Tue, 3 Jun 2025 21:02:21 +0000 |
| [1.9 FS_ULBC, 030, Wednesday June 4, 1400 CEST] Parameter updates on end to end simulation | Tomas Toftgård <tomas.toftgard@ERICSSON.COM> | Mon, 2 Jun 2025 06:16:20 +0000 |

Presenter: Ruonan Yang (vivo)

- Link Budget Analysis: Ruonan presented a detailed link budget analysis based on GEO parameters and handheld UE parameters, focusing on different UE transmit power considerations.

- TBS Determination: Ruonan discussed the TBS (Transport Block Size) determination, providing an initial analysis and highlighting the importance of evaluating 15 kHz SCS values.

- Link Budget Calculation: Ruonan illustrated the relationship between SNR and blur, emphasizing the need to determine the target SNR and corresponding BLER values.

- Additional Losses: Ruonan mentioned considering additional losses, such as 0dB and 3dB loss due to polarization.

- UE Transmit Power: Ruonan provided a table listing CNR values based on different UE transmit power levels, noting that 33 dBm of UE power was not agreed upon in RAN4.

- Duration Time: Ruonan highlighted the duration time without repetition consideration for TBS, suggesting that many TBS values may not suit the bounding time.

- Presentation aimed to delete the brackets in the current pCR, proposing that the TBS in the current tables are valid.

Comments / questions:

- Huan-Yu:

  - Observation 1: We do not share the view that 3.75 kHz cannot be used.

  - Simulation conclusion is different than ours. Need to analyse.

    - R: Table is just initial analysis. Duration is w/o repetition.

- Stefan B:

  - Question from mail – what do we mean with target BLER/SNR? We understand the limitation to transmit power. We consider that only a subset of parameters can be changed, but not the target BLER.

    - R: Relation is just a figure for illustration. Target BLER and Target SNR cannot be matched at the same time.

  - Is there any beamforming in UE?

- Liangping:

  - Table 2-2 Entry: Liangping pointed out that the last second entry in Table 2-2, which is grayed out, is considered infeasible due to the criteria of splitting time evenly between uplink and downlink. They suggested that an even split is not necessary and provided an example where 64 milliseconds for uplink and 4 milliseconds for downlink would still be feasible.

  - Link Budget Analysis: Liangping agreed with the paper's assumption that the loss should be 0 dB instead of 2.2 dB, as the latter is latitude-dependent and can lead to underestimating the available CNR. They emphasized the importance of considering both factors together to avoid grossly underestimating the available CNR.

- Peng:

  - Peng noted that the TBS determination section mentioned 2 / π BPSK and 16 QAM, but stated that the system does not support 16 QAM.

  - Peng questioned the link budget analysis table, specifically the inclusion of UE power classes 26, 29, 31, and 33 dBm, as only 23 dBm is specified for NB-IoT NTN UE Power class.

  - Peng expressed disagreement with the observation regarding 3.75 kHz, stating that the simulation results need to be double-checked or validated.

- Rashid:

  - Only 23 dBm was defined for S bands and that the other power classes are high power UEs, which are not compatible with the rest of the link budget table. Rashid also noted that the 33 dBm and 31 dBm values are directional antennas and not compatible with the 0 dBi antenna gain. They suggested using only one UE class, specifically the 23 dBm for the table, to avoid misleading CNR values.

- Thomas on chat: A key issue of all of the discussion is the following: We need to have a system design and we need to identify the open questions, that we cannot answer here. Those need to be formulated to be answered by RAN and SA2/CT people. Can we make sure that we collect these questions?

  - Jiayi: PD could be one option for open questions

  - Thomas Yes, but someone (Jiayi/Liangping need to collect those open issues)

Decision: SA4aA250030 is noted.

| S4aA250031 | On the TBS for NB-IoT NTN GEO simulation | Fraunhofer IIS |
| --- | --- | --- |

Email discussion:

| [1.9 FS_ULBC, 031, Wednesday June 4, 1400 CEST] On the TBS for NB-IoT NTN GEO simulation | Tomas Toftgård <tomas.toftgard@ERICSSON.COM> | Mon, 2 Jun 2025 06:16:21 +0000 |
| --- | --- | --- |

Presenter:  Markus Schnell (Fraunhofer)

- Markus proposed additional transport block sizes for channel simulations, targeting three aspects: hybrid rates, lowest bitrates, and balanced bitrate spacing.

- He emphasized that the highest bitrates are always below 4.75 kbps, which corresponds to AMR rates. He clarified that AMR is not useful for service continuation compared to EVS quality.

- Markus suggested considering higher bitrates if the channel can transport them, as well as ultra-low bitrate coding for enhanced capacity and robustness.

- He proposed adding a bitrate between 1 kbps and 2 kbps to achieve a well-balanced set of bitrates.

Comments / questions:

- Huan-yu:

  - Huan-yu expressed agreement with the conclusion to extend the bitrate up and down, acknowledging its potential usefulness at this stage of the project.

  - However, Huan-yu disagreed with the analysis that led to this conclusion, specifically the assumption that bitrates of 4-5 kilobits per second could deliver near-transparent quality to users. They cautioned against promising such quality targets to the industry, as it might be misleading.

- Dong:

  - Dong expressed that the expectation for users using satellite calls is to have lower quality due to the challenging conditions in the GEO application scenario. They believe it is not necessary to deliver the same high quality as other services like EVS in this context.

  - Dong emphasized that there should not be pre-requirements for codecs to achieve high quality in low bitrate ranges, as it is unrealistic to expect the same quality as higher bitrate services in such challenging conditions.

- Stefan B:

  - Stefan commented on the understanding of target block error rate (BLER) and target signal-to-noise ratio (SNR) in the context of the link budget analysis. He emphasized that the analysis should consider the limitations of transmit power and the resulting SNR at the satellite, which affects the BLER. He also questioned the assumption of a 2% BLER used in the analysis and highlighted the need to optimize parameters like bundling time and modulation schemes to maximize throughput.

- Markus Schnell responds:

  - The purpose of the channel simulations is to determine the block error rate for different bit rates.

  - He acknowledged the concerns about quality expectations but clarified that the simulations would help decide if certain bit rates are useful for the service. He also mentioned that the document does not need to make any quality promises and that the analysis should proceed without any statements about the final quality.

  - Markus referenced Nokia's listening test results, which indicated that starting at certain bit rates, the quality is close to EVS, but he agreed that the block error rate must be considered.

Disposition:

- Jiayi suggests to first look at Liangping’s document before agreeing

- Document is parked initially but then noted.

Decision: SA4aA250031 is noted.

| S4aA250032 | [FS_ULBC] GEO satellite channel parameters for FS_ULBC | Samsung R&D Institute UK |
| --- | --- | --- |

Email discussion:

| Re: [1.9 FS_ULBC, 032, Wednesday June 4, 1400 CEST] GEO satellite channel parameters for FS_ULBC | Hakju Ryan Lee <hakju00.lee@SAMSUNG.COM> | Wed, 4 Jun 2025 11:28:03 +0900 |
| --- | --- | --- |
| Re: [1.9 FS_ULBC, 032, Wednesday June 4, 1400 CEST] GEO satellite channel parameters for FS_ULBC | Liangping Ma <lpma@QTI.QUALCOMM.COM> | Wed, 4 Jun 2025 00:53:53 +0000 |
| [1.9 FS_ULBC, 032, Wednesday June 4, 1400 CEST] GEO satellite channel parameters for FS_ULBC | Tomas Toftgård <tomas.toftgard@ERICSSON.COM> | Mon, 2 Jun 2025 06:16:22 +0000 |

Presenter: Ryan Lee (Samsung)

- Ryan presented the results from RAN1, emphasizing that no new values are proposed in this document. The values are derived from TR 36.763.

- The document highlights the poor scenario in GEO satellite, with most cases showing target SNR values under 0 dB.

- Ryan proposed to record the results from TR 36.763 in the permanent document, without adding new observations.

- Ryan addressed a comment from Liangping regarding the loss factor in the link budget analysis, stating that the worst-case scenario should be considered to secure minimum performance.

Comments / questions:

- Liangping:

  - Downlink Signal Range: Liangping noted that the provided range is highly dependent on subcarrier spacing and the number of tones used. He emphasized that it is insufficient to provide a single value without specifying the corresponding uplink spectrum allocation.

  - Voice Over GEO Consideration: He mentioned that the IoT NTN study did not consider voice over GEO, which is a more demanding use case. He suggested that the link budget analysis should be more detailed to accommodate this.

  - Latitude-Dependent Loss: Liangping pointed out that the assumed 2.2 dB loss in TR 36.763 should be latitude-dependent. He argued that for latitudes above 20°, this value should be zero, which would significantly impact the link budget analysis.

  - Revision of Link Budget Analysis: He recommended revising the link budget analysis in TR 36.763 to reflect the latitude-dependent loss, ensuring a more accurate estimation of available CNR.

- Stefan B.:

  - Comment on Observation 2: Stefan questioned the assumption that bitrates less than one kbit per second or several hundred bits per second would be a primary consideration. He asked Ryan to comment on this based on their simulations and the block error rate (BLER) they would get.

  - Ryan admitted that the table was referred from another input and used to show how the target CNR value might affect the target bitrate. He mentioned that the other input assumed a 2% BLER.

  - Further Clarification: Stefan pointed out that the sensitivity of the codecs being considered is not yet known, so providing a bitrate number under the assumption of a 2% BLER might not be the right approach.

  - Ryan agreed that the table was just an example to illustrate the relationship between CNR and bitrate and that further discussion would be needed.

- Ruonan:

  - Ruonan commented on the link budgeting analysis of additional loss consideration. They mentioned that some companies consider a 3 dB additional loss due to the antenna gain difference between the edge of the beam and the center. Ruonan suggested that a better scenario should be considered, proposing that the additional loss should be 0 dB. They believe that if a better case is considered, the additional loss should be different from the TR and should be 0 dB.

Disposition:

- The group agreed to include references to the tables 1 and 2 from TR 36.763 in the permanent document, rather than including the entire tables. This was suggested to ensure clarity and avoid redundancy.

- A revision is expected to address the agreements.

Decision: SA4aA250032 is noted.

| S4aA250033 | On the remaining parameters for the RAN simulation for ULBC over NTN | Qualcomm Innovation Center Inc |
| --- | --- | --- |

SA4aA250033 is revised to SA4aA250036.

| S4aA250036 | On the remaining parameters for the RAN simulation for ULBC over NTN | Qualcomm Innovation Center Inc |
| --- | --- | --- |

SA4aA250036 is revised to SA4aA250037.

| S4aA250037 | On the remaining parameters for the RAN simulation for ULBC over NTN | Qualcomm Innovation Center Inc |
| --- | --- | --- |

Email discussion:

| Re: [1.9 FS_ULBC, 033, 036, Wednesday June 4, 1400 CEST] On the remaining parameters for the RAN simulation for ULBC over NTN | Liangping Ma <lpma@QTI.QUALCOMM.COM> | Wed, 4 Jun 2025 06:09:12 +0000 |
| --- | --- | --- |
| Re: [1.9 FS_ULBC, 033, 036, Wednesday June 4, 1400 CEST] On the remaining parameters for the RAN simulation for ULBC over NTN | lilei (CL) <lilei348@HUAWEI.COM> | Wed, 4 Jun 2025 02:55:17 +0000 |
| Re: [1.9 FS_ULBC, 033, 036, Wednesday June 4, 1400 CEST] On the remaining parameters for the RAN simulation for ULBC over NTN | Wang Bin 王宾 <wangbin23@XIAOMI.COM> | Wed, 4 Jun 2025 02:01:57 +0000 |
| Re: [1.9 FS_ULBC, 033, 036, Wednesday June 4, 1400 CEST] On the remaining parameters for the RAN simulation for ULBC over NTN | Liangping Ma <lpma@QTI.QUALCOMM.COM> | Tue, 3 Jun 2025 22:53:21 +0000 |
| Re: [1.9 FS_ULBC, 033, 036, Wednesday June 4, 1400 CEST] On the remaining parameters for the RAN simulation for ULBC over NTN | Liangping Ma <lpma@QTI.QUALCOMM.COM> | Tue, 3 Jun 2025 22:49:24 +0000 |
| Re: [1.9 FS_ULBC, 033, 036, Wednesday June 4, 1400 CEST] On the remaining parameters for the RAN simulation for ULBC over NTN | Bruhn, Stefan <Stefan.Bruhn@DOLBY.COM> | Tue, 3 Jun 2025 21:02:19 +0000 |
| Re: [1.9 FS_ULBC, 033, 036, Wednesday June 4, 1400 CEST] On the remaining parameters for the RAN simulation for ULBC over NTN | Wang Bin 王宾 <wangbin23@XIAOMI.COM> | Tue, 3 Jun 2025 08:48:07 +0000 |
| Re: [1.9 FS_ULBC, 033, 036, Wednesday June 4, 1400 CEST] On the remaining parameters for the RAN simulation for ULBC over NTN | lilei (CL) <lilei348@HUAWEI.COM> | Tue, 3 Jun 2025 07:26:46 +0000 |
| [1.9 FS_ULBC, 033, 036, Wednesday June 4, 1400 CEST] On the remaining parameters for the RAN simulation for ULBC over NTN | Tomas Toftgård <tomas.toftgard@ERICSSON.COM> | Mon, 2 Jun 2025 06:16:22 +0000 |

Presenter: Liangping Ma (Qualcomm)

- Link Budget Analysis: Liangping provided a detailed link budget analysis to derive the Carrier-to-Noise Ratio (CNR) values. This analysis included verification of calculations rather than quoting numbers from other Technical Reports (TRs).

- Frame Structure: The presentation included an FDD frame structure example, specifically an 80 milliseconds frame for uplink using 64 milliseconds to maximize energy received on the satellite.

- Uplink Signal Value: The available signal value for 3.75 kHz was noted to be 2.9 dB, which is 0.3 dB higher than the value from TR 36.763.

- Spreadsheet for Calculations: A spreadsheet was provided to show how the CNR values were derived, considering different subcarrier spacing for uplink and the number of tones.

- UE Antenna Gain: The analysis considered UE antenna gain values of 0 dBi and -5.5 dBi.

- Formula for CNR Calculation: Liangping presented a formula for deriving CNR values, taking into account bandwidth and UE transmission power.

- Detailed Assumptions: All assumptions on parameters were listed to ensure clarity in the derivation of values.

- Compliance with Standards: The frame structure was designed to be compliant with current standards, considering constraints on time gaps between control and data channels.

Comments / questions:

- Peng:

  - Channel Model: Peng suggested using the channel model derived specifically for IoT NTN from 36.102 instead of the more generic 38.811. Liangping responded that the channel model in 36.102 is derived from 38.811 and believes it is acceptable to continue using 38.811.

  - Power Class: Peng questioned the inclusion of power classes other than 23 dBm, stating that 23 dBm is the standard for MB IoT scenarios. Liangping argued that actual deployments could exceed 23 dBm and provided examples of higher power classes.

  - Bandwidth Parameter: Peng asked why other bandwidth parameters were included for uplink when 3.75 kHz was previously mentioned. Liangping explained that higher UE transmission power could enhance capacity, making it reasonable to consider more bandwidth in uplink scenarios.

  - Liangping emphasized that the channel model from 38.811 is suitable and that the power class and bandwidth parameters should reflect real-world deployment scenarios, which may exceed the standard 23 dBm.

  - Peng maintained that the standard practice is to use 23 dBm and questioned the necessity of simulating other parameters.

  - Both agreed to continue discussing the differences in channel models offline.

- Ruonan:

  - Ruonan questioned the inclusion of -5.5 dBi for UE antenna gain and the consideration of multiple tone locations for uplink, suggesting that focusing on single tone uplink might be more efficient. They expressed concern that evaluating all combinations in the table would significantly increase the workload.

  - Liangping clarified that the -5.5 dBi accounts for the loss due to the UE's internal antenna structure, which was considered in previous studies but might be conservative.

  - Liangping explained that multiple tone locations are included to accommodate scenarios where higher transmission power from the UE could enhance capacity. They noted that using more bandwidth in uplink could help maintain a lower channel coding rate, especially for larger TBS proposals and higher UE transmission power.

- Huan-yu:

  - Huan-yu suggested keeping both 2.3° and 12.5° elevation angles in the simulation to consider the worst-case scenario for maintaining acceptable quality.

  - They recommended including a range for UE power, starting from 23 dBm and considering higher values to observe channel characteristics.

  - They agreed with the 2.2 dB and 0 dB loss for centralization loss.

  - They expressed that the partitioning of uplink and downlink time should be split as proposed but noted that the specific numbers might not be accurate due to unknown bit rates and directions.

  - They questioned the TBS calculation, suggesting that the target SNR is not agreed upon and the calculated TBS might be questionable.

  - They mentioned the importance of using short PDCP headers to save payload bytes.

  - Discussion and Response from Liangping:

    - Liangping clarified that the 12.5° elevation angle was used in their proposal, similar to Huawei's simulation parameters. They noted that 2.3° is an extreme case and might not be practical due to potential obstructions.

    - They agreed that the difference between 2.3° and 12.5° is minimal (.2 DB) and should not significantly impact the simulation outcome.

    - Liangping acknowledged the importance of considering a range of UE power values and agreed with the dependent synch condition loss values (2.2 dB and 0 dB).

    - They emphasized the need to agree on some TBS values to enable companies to run simulations and cross-check results.

    - Liangping appreciated the comment on using short PDCP headers for codec efficiency.

- Stefan Bruhn:

  - Stefan suggested having a high-level description of how the simulation would be run, including configuration parameters, optimization parameters, and result parameters.

  - He asked for clarification on the assumed one millisecond guard time between switching uplink and downlink transmission, questioning if this short time period is confirmed to work.

  - Liangping agreed to provide an example of how to pick parameters for the simulation and add it in a future contribution.

  - He confirmed that the one millisecond guard time conforms with the existing NB-IoT specification, explaining that it is the required time for RF change from one frequency to another

Disposition:

- The document is parked for now.

Decision: SA4aA250037 is noted.

| S4aA250034 | On channel simulation parameter setting for ULBC | HuaWei Technologies Co., Ltd |
| --- | --- | --- |

Email discussion:

| FW: [1.9 FS_ULBC, 034, Wednesday June 4, 1400 CEST] On channel simulation parameter setting for ULBC | Tomas Toftgård <tomas.toftgard@ERICSSON.COM> | Wed, 4 Jun 2025 06:20:57 +0000 |
| --- | --- | --- |
| [1.9 FS_ULBC, 034, Wednesday June 4, 1400 CEST] On channel simulation parameter setting for ULBC | Tomas Toftgård <tomas.toftgard@ERICSSON.COM> | Mon, 2 Jun 2025 06:16:23 +0000 |

Presenter: Huan-yu Su (Huawei)

- Objective: Highlight the need for detailed parameter mapping in radio simulations to ensure accurate and representative results.

- Key Points:

  - Parameter Sensitivity: Emphasized that many parameters influence simulation outcomes, making it crucial to document all details for cross-verification.

  - Proposed Tables: Introduced tables for uplink and downlink to map dependent parameters, including UE speed and other factors affecting transmission performance.

  - Verification and Confidence: Stressed the importance of having verified and representative simulation results to avoid misleading conclusions in ULBC codec design.

  - Implementation: Suggested that the group consider these tables and populate them to identify the most representative scenarios for simulations.

- Huan-yu's proposal aims to enhance the reliability and accuracy of simulation results by ensuring all relevant parameters are meticulously documented and verified.

Comments / questions:

- Stefan Bruhn

  - Stefan expressed concern about the assumption of a 2% block error rate (BLER) for the UE. He emphasized the need for a range of different BLERs at different bit rates, as the codec designer would ultimately decide the target bitrate and robustness.

  - Huan-yu acknowledged Stefan's concern and explained that the 2% BLER might seem low for a 20-millisecond payload system but is considered high for a system bundling over 100 milliseconds of speech payload.

  - Huan-yu also addressed the elevation angle concern, noting that the difference between 2.3° and 12.5° elevation angles results in a .2 dB difference, which should not significantly impact the simulation outcome.

  - Additionally, Huan-yu agreed with the inclusion of different UE power levels and the consideration of dependent synch condition loss (2.2 dB and 0 dB).

- Liangping

  - Liangping expressed concern about the proposed target block error rate (BLER) of 2%, suggesting it is too limited for real-world scenarios. He noted that the current permanent document (PD) already includes BLER targets of 1%, 2%, 6%, and 10%.

  - He agreed with the proposed elevation angle of 12.5°, which aligns with Qualcomm's proposal, resulting in a free space path loss of 190.6 dB.

Decision: SA4aA250034 is noted.

| S4aA250035 | On TBS values for simulation model | Beijing Xiaomi Mobile Software |
| --- | --- | --- |

Email discussion:

| Re: [1.9 FS_ULBC, 035, Wednesday June 4, 1400 CEST] On TBS values for simulation model | Wang Bin 王宾 <wangbin23@XIAOMI.COM> | Wed, 4 Jun 2025 02:07:50 +0000 |
| --- | --- | --- |
| Re: [1.9 FS_ULBC, 035, Wednesday June 4, 1400 CEST] On TBS values for simulation model | Liangping Ma <lpma@QTI.QUALCOMM.COM> | Tue, 3 Jun 2025 23:17:51 +0000 |
| [1.9 FS_ULBC, 035, Wednesday June 4, 1400 CEST] On TBS values for simulation model | Tomas Toftgård <tomas.toftgard@ERICSSON.COM> | Mon, 2 Jun 2025 06:19:26 +0000 |

Presenter:  Wang Bin

- Purpose: The contribution aims to propose candidate TBS (Transport Block Size) values.

- Analysis: Various possible TBS values were analyzed based on different bundling times.

- Findings: In Table 2-5, the TBS value of 320 milliseconds was found invalid, while other TBS values were valid.

- Conclusion: The proposal suggests extending the TBS value range and correcting the PHY and Codec bit rate for the TBS value of 600.

Comments / questions:

- Liangping:

  - Liangping expressed concerns about the proposed TBS values, particularly the value of 504, which would require higher subcarrier spacing and higher UE power.

  - He highlighted the need to limit the size of the table due to the time-consuming nature of generating traces, mentioning that it would take 80 days to finish the simulation for the uplink alone.

  - Liangping suggested removing very low codec bit rate values like 104 and 136, as they seem unnecessary.

  - Wang Bin acknowledged the concern about the number of TBS values and the time required for evaluation.

  - He proposed that different companies could evaluate different TBS values to distribute the workload.

  - Markus suggested removing the 10% block error rate (BLER) from the simulations to reduce the number of parameters, as it may not be necessary.

  - Liangping responded that the 10% BLER is included to ensure the codec is robust under extreme real-world scenarios.

Decision: SA4aA250035 is noted.

| S4aA250038 | Calibration of contributions on RAN simulation assumptions for ULBC | Qualcomm Innovation Center Inc |
| --- | --- | --- |

Presenter: Liangping Ma (Qualcomm)

- Consistency in Calculations: Liangping highlighted that the calculations across different companies are consistent, though there are differences in assumptions, particularly regarding the simulation loss of 2.2 dB.

- New TBS Values: The document introduces newly proposed Transport Block Size (TBS) values, doubling the number of columns and potentially increasing the simulation time. Liangping emphasized the need to agree on a subset of TBS values to manage the simulation workload effectively.

- Common Values: The black values in the table represent common values seen across all companies, while the blue ones are newly proposed values.

Comments / questions:

- Anssi:

  - Anssi expressed concern about the feasibility of using a 320 millisecond bundling time for conversational use, suggesting that it adds significant delay. They proposed dropping the 320 millisecond bundling time from the considerations.

  - Thomas suggested not dropping the 320 millisecond bundling time but instead adding a footnote or some form of prioritization to indicate its lower priority.

  - Stefan Bruhn agreed with not skipping the 320 millisecond bundling time immediately. He proposed optimizing certain parameters on shorter frame intervals and then selecting the best possible parameters for longer simulations, potentially across companies.

- Stefan:

  - Stefan raised a concern about the assumed guard times between uplink and downlink transmissions, specifically questioning the feasibility of the one millisecond guard time assumed in the figure. He asked if this short time period is confirmed to work or if it still needs to be looked into.

  - Liangping confirmed that the one millisecond guard time conforms with the existing NB-IoT specification, which requires this time for the RF team to switch from one frequency to another. He also mentioned that the three milliseconds is the minimum amount of time needed to satisfy the requirement, ensuring compliance with the 3GPP standard.

- Amy

  - Amy expressed concern about the TBS values that provide codec bit rates larger than 4.75 Kbps, stating that such high bit rates are not necessary for the current application scenario.

  - Amy emphasized that the simulation work should not be divided based on TBS size alone, as companies may need to simulate nearly all TBS sizes and allow for cross-checking.

  - Tomas suggested keeping the black numbers for simulations and putting the blue ones in brackets for further discussion. Amy agreed but highlighted the need to avoid documenting unnecessary high bit rates.

- Markus Schnell:

  - Markus Schnell commented that he is not happy with agreeing only on the black numbers in the table, as it would not improve the document. He emphasized the need for an equally spaced number of rates and noted the high number of values between 2.4 and 4.5 kilobits per second, while there are very few low rates. He suggested that the group should agree on some of the new numbers to ensure a balanced set of bitrates.

- Stefan Bruhn:

  - Stefan Bruhn commented that it might be desirable to have a high-level description of how the simulation would be run, including parameters to optimize and result parameters. He also questioned the feasibility of the one millisecond guard time between uplink and downlink transmission, asking if this short time period is confirmed to work or needs further investigation.

- Discussion on Agreed Numbers and Disagreements:

  - Agreement on Black Numbers:

    - There was no strong objection on the black numbers in the tables, which were common across all companies' proposals. These numbers can be considered safe bets for simulations. However, there were concers only agreeing on these black numbers.

  - Disagreement on Additional Numbers:

    - There was a disagreement on adding new numbers (blue columns) to the tables. Some participants felt that more time was needed to align on these additional values.

    - Concerns were raised about the feasibility of higher TBS values and the need for more detailed simulations to validate them.

  - Proposal to Drop Lowest Values:

    - There was a proposal to drop the lowest TBS values (e.g., 104, 136) due to their low codec bit rates and the extensive simulation time required. However, this was met with resistance, and some participants suggested keeping these values in brackets for further discussion.

  - Concerns on High Bit Rates:

    - Some participants expressed concerns about including TBS values that result in codec bit rates higher than 4.75 kbps, arguing that these are not relevant for the current GEO application scenario.

  - Gentleman's Agreement:

    - A suggestion was made to have a gentleman's agreement to start simulations with the black numbers while keeping the additional numbers in brackets for further discussion. This approach aimed to ensure progress without stalling the work.

  - Key Takeaways:

    - Agreement on black numbers for simulations.

    - Disagreement on additional numbers, requiring more time for alignment.

    - Proposal to drop lowest TBS values met with resistance.

    - Concerns about high bit rates for GEO applications.

    - Gentleman's agreement suggested to proceed with black numbers and discuss additional values later.

- Here are the positions of various participants on the numbers discussed during the meeting:

  - Liangping: Proposed to agree on the black numbers in the tables and start simulations with them. Suggested that the new TBS values and previous ones have the same weight.

  - Markus Schnell: Expressed concern about only agreeing on the black numbers, emphasizing the need for an equally spaced number of rates. Suggested targeting lower BLER first for simulations.

  - Amy: Concerned about TBS values providing codec bit rates larger than 4.75 kbps, suggesting that such values are not suitable for GEO application scenarios.

  - Stefan Bruhn: Suggested agreeing on putting the black numbers into the meeting minutes as safe bets that allows Qualcomm to start simulations and not deciding at this meeting on the other numbers. Emphasized the need for design freedom for the codec.

  - Peng: Preferred to get more time to align on the numbers, suggesting that the current table does not include other potential TBS values proposed by other companies.

  - Wang: Clarified that the new TBS values and previous ones should have the same weight and not be prioritized differently.

Disposition:

- We agree that we start the simulations with the black numbers.

- Other TBS and parameters are not excluded for the future, but more time is needed on discussing those.

- The participants agreed to start simulations with the black numbers (existing values) for transport block sizes.

- The new proposed values (blue numbers) will be discussed further in the next meeting.

- Discussion on which channel model to use, e.g. NTN-TDL-C5 channel model as specified in TS 36.102, will continue offline.

- The agreement does not exclude any other configurations, and additional values may be considered later.

Decision: SA4aA250038 is noted.

# 1.10	Other Rel-19 matters including TEI

None.

# 1.11	Close of the session

The chairman thanked the participants. Next Audio SWG call on June 16 (DaCAS), next for ULBC is on June 18. The meeting was closed on June 4, 16:56 CEST.

# Annex A – Agenda

| 1 | Audio SWG |  |
| --- | --- | --- |
| 1.1 | Opening of the session and registration of documents |  |
| 1.2 | IPR and antitrust reminder | IPR/Anti-trust reminder |
| 1.3 | Reports/Liaisons from other groups/meetings |  |
| 1.4 | Release 18 and earlier matters |  |
| 1.5 | IVAS_Codec_Ph2 (EVS Codec Extension for Immersive Voice and Audio Services, Phase 2) |  |
| 1.6 | ATIAS_Ph2 (Terminal Audio quality performance and Test methods for Immersive Audio Services, Phase 2) |  |
| 1.7 | DaCAS (Diverse audio CApturing system for Smartphone devices) |  |
| 1.8 | FS_ACAPI (Study on Audio Codec APIs) |  |
| 1.9 | FS_ULBC (Study on Ultra Low Bitrate Speech Codec) | 030n 031n 032n 033->036->037n 034n 035n 038n |
| 1.10 | Other Rel-19 matters including TEI |  |
| 1.11 | Close of the session |  |

Legend for Tdocs:

- Color: not-yet processed, processed, late, withdrawn, moved to a different A.I., under email agreement

- a agreed, app approved, e endorsed, m missing, n noted np, not pursued, pp postponed, r revised

# Annex B – Participants

Table 1 - Registered participants

| NAME | Organization Represented |
| --- | --- |
| Amy Zhang | vivo Mobile Communication Co., |
| Anssi Rämö | Nokia Corporation |
| Bin Wang | Beijing Xiaomi Mobile Software |
| Eleni Fotopoulou | Fraunhofer IIS |
| Elmira Ramazanirend | VODAFONE Group Plc |
| Eric Yip | Samsung Electronics Co., Ltd |
| Erik Norvell | Ericsson LM |
| Frederic Gabin | Dolby France SAS |
| Gilles Teniou | Tencent |
| Hakju Ryan Lee | Samsung R&D Institute UK |
| Hiroyuki Ehara | Panasonic Holdings Corporation |
| Huan-yu Su | HuaWei Technologies Co., Ltd |
| Lasse Laaksonen | Nokia Corporation |
| Lei Li | Huawei Tech. Japan, K.K. |
| Lufeng Han | Spreadtrum |
| Liangping Ma | Qualcomm Innovation Center Inc |
| Marek Szczerba | Philips International B.V. |
| Markus Multrus | Fraunhofer IIS |
| Markus Schnell | Fraunhofer IIS |
| Milan Jelinek | VoiceAge Corporation |
| Peng Tan | Skylo Technologies |
| Rishabh Tyagi | Dolby Laboratories Inc. |
| Srikanth Korse | Nokia Corporation |
| Stefan Bruhn | Dolby Laboratories Inc. |
| Stefan Doehla | Fraunhofer IIS |
| Stephane Villette | Qualcomm Technologies |
| Thomas Stockhammer | Qualcomm Germany |
| Tomas Jansson Toftgård | Ericsson LM |
| ZISIS IASON SKORDILIS | Qualcomm Incorporated |

Table 2 - Additional participants

NAME

Andre Schevciw

DONG (WANG)-vivo

Jiayi Xu-CMCC

Khouli, Rashid

LI_JIAWEI

Noboru Harada

OPPO - Dong Wang

RAGOT Stéphane INNOV/IT-S

Ruonan Yang

Shao Chengjun

Sujeet Mate (Nokia)

Tommy Vaillancourt

V. Atti

Vaclav Eksler

Vladimir Malenovsky

Xiaodong Sun(vivo)

Zhe Wang - Huawei

Tomas Toftgård, Email: tomas.toftgard@ericsson.com