3GPP TSG-SA4 Meeting #132 	S4-251354

# Fukuoka, Japan,  19. - 23. May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.502 | CR | 0040 | rev | 2 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Aligning Stage 2 Session Announcement with Stage 3 on target service area |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19, 5MBUSA |  |  |  |  |  | Date: |  |  | 13.5.2025 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The stage 3 of the MBS User Data Ingest Session API allows supporting an external target service area for MBS Application providers outside of the trust domain. The Availability Information (TS 26.517 Table 5.2.9-1) allows inclusion of a Target Service Area into Service Announcement and refers to Table 4.5.8-1 of this specification for a detailed stage 2 definition. TS 23.247, Clause 6.11 allows insertion of Cell Id Lists, TAIs or geographical area information into service announcement. However, Table 4.5.8-1 of this specification does not include a Target Service Area. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Insertion of a clarification, that an MBS Application Service Provider outside of the trust domain may provide the target service area description in form of a geographical area or a civic address. Insertion of a Target service area Parameter including a description into the MBS Distribution Session Announcement parameter table. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent specifications may lead to interoperability issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5.2, 4.5.6, 4.5.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S4-251065, S4-250911 |  |  |  |  |  |  |  |  |

**** First Change ****

### 4.5.2	Static information model

Figure 4.5.2-1 shows how the different service and session concepts depicted in figure 4.5.1-1 above relate to each other. In this figure:

1.	The MBS Application Provider provisions the parameters of a new MBS User Service by invoking the Nmbsf service either directly, or via the NEF. This specifies which of the Service announcement modes are to be used to advertise the MBS User Service, as well as descriptive metadata for inclusion in the MBS User Service Announcement.

2.	The MBS Application Provider provisions a number of time-bound MBS User Data Ingest Sessions within the scope of the MBS User Service by invoking the Nmbsf service either directly, or via an equivalent Nnef service provided by the NEF. Each MBS User Data Ingest Session includes the details of one or more MBS Distribution Sessions.

-	To indicate that it has a restricted MBS service area (i.e. corresponding to a local MBS Service, as defined in clause 6.2.2 of TS 23.247 [5]), an MBS Distribution Session may specify one or more Target service areas. In line with [5], MBS data is not transmitted outside the MBS service area derived from the indicated Target service areas.

-	To provision location-dependent variants of an MBS User Service (see clause 6.2.3 of TS 23.247 [5]), a number of MBS Distribution Sessions conveying different MBS data may be provisioned within the scope of the same MBS User Service by setting the Location-dependent service flag on the MBS Distribution Sessions of each variant. Location-dependent MBS Distribution Session variants shall have the same MBS Session Identifier, but they shall have disjoint Target service areas.

-	When the Multiplexed service flag is set on the MBS Distribution Session, all MBS Distribution Sessions with an identical (or empty) set of Target service areas shall be multiplexed onto the same MBS Session. The MBS Session Identifier shall be the same for all MBS Distribution Sessions within the multiplex. This feature may be combined with the Location-dependent service flag, in which case each location-dependent multiplex of MBS Distribution Sessions is mapped into a separate MBS Session.

-	The MBS Application Provider may set the Transport security protection flag to indicate that transport security protection (as specified in annex W of TS 33.501 [18]) is required for the MBS Distribution Session. When the flag is set, the MBSSF chooses between the control plane or user plane security procedure.

-	The MBS Application Provider may indicate in Target UE classes whether a broadcast MBS Distribution Session is intended for consumption by UEs of reduced capability ("NR RedCap UE" as defined in clause 6.19 of TS 23.247 [5]), by reduced capability UEs and full capability UEs, or by full capability UEs only.

The MBSF provisions additional MBS Distribution Session parameters (denoted in table 4.5.6-1 as assigned by the MBSF) and exposes some of them back to the MBS Application Provider (as indicated by NOTE 1 to table 4.5.6-1).

NOTE 1:	The MBSF typically allocates an MBS Session Identifier, such as a Temporary Mobile Group Identity (TMGI) for each MBS Distribution session (see step 4 below) as a side-effect of provisioning, but it is also possible for the Nmbsf service invoker to nominate a particular value during this provisioning step if TMGI allocations are managed externally to the MBSF.

3.	The MBS Application Provider may additionally provision an MBS Consumption Reporting Configuration within the scope of the MBS User Service by invoking the Nmbsf service either directly, or via the NEF.

NOTE 2:	Reception reporting for MBS User Services is for future study.

Shortly before the current time enters the time window of a provisioned MBS User Data Ingest Session:

4.	The MBSF provisions an MBS Session in the MBS System by invoking the Nmbsmf service on the MB-SMF (see clause 9 of TS 23.247 [5]) to allocate a TMGI (if one has not already been allocated) for each MBS Distribution Session and to create an MBS Session Context for each one. The parameters of the MBS Session Context shall be populated as specified in clause 4.5.9. In response, the MB-SMF provides the MB-UPF ingest information (specifically, the MB-UPF tunnel endpoint address and traffic flow information to be used by the MBSTF) to the MBSF.

5.	The MBSF provisions an MBS Distribution Session in the MBSTF by invoking the Nmbstf service at reference point Nmb2 using the parameters from the newly created MBS Session Context.

6.	Using the parameters from the MBS Distribution Session and from the newly created MBS Session Context, the MBSF compiles an MBS User Service Announcement to advertise the availability of the MBS User Service and makes this service access information available by one or more of the Service announcement modes provisioned in the MBS User Service.

NOTE 1	Square brackets after a parameter name indicate multiplicity; parameter names rendered in italics with parentheses are optional. See the following clauses for details.

NOTE 2:	Parameters and entities not exposed to the MBS Application Provider via the Nmbsf service at reference point Nmb10 are annotated with the dagger symbol †.

NOTE 3:	MBS Session Identifier is defined by clause 6.5.1 of TS 23.247 [5] as a Temporary Mobile Group Identity (TMGI) or a Source-Specific Multicast (SSM) IP address.

Figure 4.5.2-1: MBS User Services static information model

**** Next change ****

### 4.5.6	MBS Distribution Session parameters

This entity models an MBS Distribution Session, as provisioned by the MBS Application Provider and as managed by the MBSF. This MBSF subsequently uses this information to provision a corresponding MBS Distribution Session in the MBSTF.

The following parameters assigned by the MBS Application Provider may be updated by the MBS Application Provider at any time:

-	Target service areas,

NOTE:	When the MBS Application Provider is outside the trust domain these may be provided in form of a list of geographical area(s) or civic address(es).

-	MBS Frequency Selection Area (FSA) Identifier (applicable only to broadcast Service type),

-	QoS information,

-	Target UE classes as defined in clause 6.19 of TS 23.247 [5].

With the exception of the MBS Session Identifier (which is immutable after initial assignment) and the Location-dependent service flag (which is immutable after creation), all other parameters assigned by the MBS Application Provider may be updated by the MBS Application Provider when the MBS Distribution Session is in the INACTIVE state.

The baseline parameters for an MBS Distribution Session that are common to all distribution methods are listed in table 4.5.6-1 below. All parameters are exposed to the MBS Application Provider except where noted otherwise.

**** Next Change ****

### 4.5.8	MBS Distribution Session Announcement parameters

This entity models an MBS Distribution Session Announcement, which is compiled by the MBSF and used to advertise the current or imminent availability of an MBS Distribution Session in the MBS System. The baseline parameters for an MBS Distribution Session Announcement are listed in table 4.5.8-1 below:

Table 4.5.8-1: Baseline parameters of MBS Distribution Session Announcement entity

| Parameter | Cardinality | Assigner | Description |
| --- | --- | --- | --- |
| MBS Session Identifier | 1..1 | MB-SMF | The Temporary Mobile Group Identity (TMGI) or Source-Specific Multicast (SSM) IP address of the MBS Distribution Session from which this announcement is derived. |
| Target service areas | 0..* | MBS Application Provider or MBSF | The set of regions comprising the MBS service area in which this MBS Distribution Session is targeted. |

| MBS Frequency Selection Area (FSA) Identifier | 0..1 | MBS Application Provider or MB-SMF | (Broadcast MBS Session only.) Identifies a preconfigured area within which, and in proximity to, the cell(s) are announcing the MBS FSA ID and the associated frequency corresponding to this MBS Distribution Session Announcement (see NOTE). |
| --- | --- | --- | --- |
| Radio parameters | 0..* | MBSF | (Broadcast MBS Session only.) Radio transmission parameters of this MBS Distribution Session in its target service areas. The values are obtained by the MBSF from the OAM using the MBS Frequency Selection Area (FSA) Identifier (see above) as the lookup key. |
| Target UE classes | 0..* | MBS Application Provider | (Broadcast MBS Session only.) Indicates whether the MBS Distribution Session described by this announcement is suitable for consumption by NR RedCap UEs and/or non-NR RedCap UEs as defined in clause 6.19 of TS 23.247 [5]. |

| Distribution method | 1..1 | MBS Application Provider | The distribution method (as defined in clause 6) of the MBS Distribution Session from which this announcement is derived. |
| --- | --- | --- | --- |
| Session Description parameters | 1..* | MBSF | Additional parameters needed to receive the MBS Distribution Session from which this announcement is derived, including relevant User Plane traffic flow parameters. |
| Service protection description | 0..1 | MBSSF | The security parameters for the MBS Distribution Session (see clause W.4.2 of TS 33.501), including: - Which form of transport security protection is in force, whether UICC key management (see TS 33.246 [19]) is selected and/or whether 2G GBA security (see TS 33.246 [19]) is selected. - The MBS Session Key (MSK) identifier and key domain. - The address of the key management server (FQDN of the MBSSF) when user plane security is in force. |

| Time service endpoints | 0..N | MBS AS | A set of endpoints provided by the MBS AS and used by the MBS Client to synchronise its clock with the needed precision. |
| --- | --- | --- | --- |

NOTE: Used to guide frequency selection by the UE for a broadcast MBS Session.

The following session announcement parameters are additionally relevant when Distribution method above indicates the Object Distribution Method:

Table 4.5.8-2: Additional MBS Distribution Session Announcement parameters for Object Distribution Method

| Parameter | Cardinality | Assigner | Description |
| --- | --- | --- | --- |
| Object distribution schedule | 0..1 | MBS Application Provider | A schedule indicating when individual objects are to be delivered on the corresponding MBS Distribution Session. Present only when this information has been provided in the Object acquisition identifiers of the corresponding MBS Distribution Session (see table 4.5.6-2). |
| Object distribution base URL | 0..1 |  | A URL prefix substituted by the MBSTF Client with the Object repair base URL when repairing objects not received completely intact from the corresponding MBS Distribution Session. Present only when object repair is provisioned for the corresponding MBS Distribution Session. |
| Object repair base URL | 0..1 | MBSF | The base URL of the MBS AS to be used for object repair of the corresponding MBS Distribution Session. Present only when object repair is provisioned for the corresponding MBS Distribution Session. |