3GPP TSG-SA2 Meeting #170	 S2-2506216

Gothenburg, Sweden 25th – 29th August 2025	(revision of)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6336 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | On clarification of sharing information at NF/node level from OAM among EIF exposure subscriptions. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2025-07-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The EIF imports NF level information from OAM. This information needs not be imported independently for each UE for which the EIF needs it, hence the text should be clearer that this info can be shared/reused. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarified the reuse of OAM node level information can be across UEs not for a single UE as the current text seems to confusingly imply. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusing specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.51.2.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

##### 5.51.2.2.3	Energy Consumption information collection from OAM

Table 5.51.2.2.3-1 provides the list of information received from OAM. EIF requests the energy consumption and data volume information per NF level from OAM by providing the serving gNB ID(s) and (I-)UPF ID(s) received from SMFs.

NOTE 1:	The management consumer in EIF is configured with information to retrieve the energy related information from OAM as defined in TS 28.554 [209].

The node-level reporting interval from the OAM is the network-wide configurable starting time and interval T which is the same as data volume reporting interval from SMFs.

Table 5.51.2.2.3-1: Information from OAM for user-plane energy consumption calculation

| Information | Description |
| --- | --- |
| gNB energy consumption | The Energy consumed by a gNB over the configured time interval T based on clause 6.7.3.4.2 of TS 28.554 [209]. |
| gNB data volume | The UL/DL data volume handled by a gNB over the configured time interval T based on clause 6.7.1.1 of TS 28.554 [209]. |
| UPF energy consumption | The Energy consumed by a UPF over the configure time interval T based on clause 6.7.3.1 of TS 28.554 [209]. |
| UPF data volume | Data volume consumed at a UPF. |

When the serving gNB and/or the (I-)UPF(s) of the UE are changed, the serving gNB ID and (I-)UPF ID(s) will be updated to the EIF through SMF(s). And then, the EIF requests the energy consumption and data volume information from OAM for the updated serving gNB ID and (I-)UPF ID(s).

NOTE 2:	The Node-level Energy Consumption information received from OAM could be used by EIF for all the UEs being served by the same NF or Node. If the EIF has subscriptions from one or more EIF Service Consumers for the same UE or different UEs, the Node-level Energy Consumption information for one NF or Node is requested from OAM only once for all the subscriptions and it is used by the EIF for the calculations of the information required to provide the energy consumption information to these one or more EIF Service Consumer that subscribed to the EIF.

NOTE 3:	In this Release, only the energy related information of user plane communication is supported and that of control plane signalling is not supported.

The EIF acts as the enforcement point for user consent for collection and processing of energy-related information depending on local regulations. For support of user consent (for collection and processing of energy-related information), the EIF communicates with the UDM as specified in Annex V of TS 33.501 [29].

End of changes