3GPP TSG-SA3 Meeting #123	S3-252929

Goteborg, Sweden, 25 – 29 August 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.545 | CR | 0022 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Changes based on RAN3 input |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_Femto_Sec |  |  |  |  |  | Date: |  |  | 2025-08-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | RAN3 sent a LS to SA3 pointing out two issues: CAG ID verification and access mode. This contribution provides the necessary modifications to align with RAN# updates. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Deletion of “operating in closed access mode”, and modification of the CAG ID verification requirements to make it mandatory for each CAG ID for each Femto cell. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misaligned specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************Start of Change************

## 5.8	CAG ID verification

The NR Femto GW shall verify that all UE associated messages from a Femto cell  can be mapped to a specific CAG ID and that this CAG ID is allowed for the identity of the originating Femto. In the absence of a NR Femto GW, the AMF shall perform this CAG ID verification.

One NR Femto can host multiple Femto cells. Cells may belong to PNI-NPN. Hence, the CAGID verification shall be applied to all the CAG IDs mapping to individual Femto cells. The existing CAG concept defined for PNI-NPN in TS 23.501 [2] clause 5.30.3 shall be re-used for Femtocell access control.

NOTE 1: 	The CAG ID being verified may be explicitly present in the message as an information element or may be mapped by other means.

NOTE 2:	The above requirement implies that the network has to ensure that all UE-associated messages from Femto cell are subject to the above verification, even if a (rogue) message from a compromised Femto would not indicate to originate from a NR Femto.

************End of Change************