3GPP TSG-SA6 Meeting #68	S6-253430

# Gothenburg, Sweden, 25th - 29th August 2025 	                        (revision of S6-253252)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0681 | rev | 1 | Current version: | 20.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to referenced figure numbers |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Airbus |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC_Ph2-MC |  |  |  |  |  | Date: |  |  | 2025-08-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Wrong figure numbers referred to in the text. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the references to figure numbers. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lower TS quality. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.18.2.1, 10.18.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

10.18.2.1	Store target group configuration at the GMS

The procedure for store target group configurations at the group management server is described in figure 10.18.2.1-1.

A recording admin shall use this procedure to set/modify/remove target group(s) for recording.

Pre-conditions:

-	The recording admin service user has performed user authentication and service authorization with the IdMS and downloaded its own service profile from CMS.

-	The UE has secure access to the group management server.

Figure 10.18.2.1-1: Store target group configuration at group management server

1.	The group management client – triggered by a recording admin user - sends a store target group configuration request to the group management server.

2.	The group management server checks the authorization of the originator (MCRec ID) for this operation.

3.	The group management server stores the updated group configuration data.

4.	The group management server provides a store group configuration response indicating success or failure.

10.18.2.2	Update target MC service user profile data to the CMS

The procedure for update target MC service user profile data to CMS is illustrated in figure 10.18.2.2-1.

A recording admin shall use this procedure to set/modify/remove target MC service user (s) for recording.

Pre-conditions:

-	The recording admin service user has performed user authentication and service authorization with the IdMS and downloaded its own service profile from CMS.

-	The UE has secure access to the configuration management server.

Figure 10.18.2.2-1: Update target MC service user profile data to the CMS

1.	The configuration management client – triggered by a recording admin user - sends the update target MC service user profile data request to the configuration management server.

2.	The configuration management server checks that the target MC service user is under the requestor’s (MCRec ID) authority.

3.  The configuration management server stores the received target MC service user profile data. If this target MC service ID is associated with multiple MC service user profiles, the same update must be done by CMS to all these profiles.

4.	The configuration management server sends update MC service user profile data response to the configuration management client.

# * * * * End of changes * * * *