3GPP TSG-SA WG6 Meeting #68	S6-253427

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253329)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.379 | CR | 0482 | rev | 1 | Current version: | 19.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Ambient listening call pause and resume |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon, China BroadNet |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC_Ph2-MC |  |  |  |  |  | Date: |  |  | 2025-04-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | SA1 introduce new requirements about ambient listening call being pause and resume as described in [R6.16.2.2.1-005]. Ambient listening now is a “sendonly/recvonly” call between the listened to user and the listening user. This CR is proposed to use the SDP negotiation mechanism to change the media stream to ‘inactive’ and the media will be paused. When the new call is terminated, the media stream will be changed back to “sendonly/recvonly” and the ambient call is resumed. Such mechaism is similar to the IMS call HOLD. Using this mechansim can make the switch of “pause” and “resume” operations quickly without releasing and restoring the signalling plane. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce a new pause and resume procedure of ambient listening call. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambient listening call is terminated by other incoming call and the ambient listening is stopped. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New {10.14.3.x}, A.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /**************************** First changes **********************/

#### 10.14.3.x	Ambient listening call pause and resume due to a new call

Figure 10.14.3.x-1 illustrates the high-level procedure for ambient listening call being "paused" and "resumed" via the SDP re-negotiation. When a new call is detected at the "listened to" MCPTT client, the "listened to" MCPTT client sends the ambient listening call pause message to change the media plane to inactive i.e., only keep the signalling plane without media plane data exchange. After that, the media is paused. Later when the new call is detected as terminated, the "listened to" MCPTT client sends the ambient listening call resume message to change the media plane to active, i.e., resume the media plane data exchange. With this procedure, the ambient listening call is automatically resumed without any interactions with the "listened to" user. In order to utilize the network resource efficiently, the network resource used by this paused ambient listening call may be released based on local configuration (e.g., a bearer hold timer) at the MC service server.

Pre-conditions:

-	There is an ongoing ambient listening call between MCPTT client 1 and MCPTT client 2. The call is either a remotely initiated ambient listening call or a locally initiated ambient listening call.

-	MCPTT user 1 is the "listening" user at MCPTT client 1, and MCPTT user 2 is the "listened to" user at MCPTT client 2.

Figure 10.14.3.x-1: Ambient listening call pause and resume

1.	The MCPTT client 2 used by the "listened to" user detects that a new call is initiated or coming in.

1a.	The MCPTT client 2 receives a new group call request or a private call request.

1b.	The MCPTT client 2 sends out a new group call request or a private call request.

2.	The MCPTT client 2 sends the ambient listening call pause request to the MCPTT server to change the media plane to inactive.

3.	The MCPTT server sends the ambient listening call pause request to the MCPTT client 1 to de-activate the media plane.

Optionally, it may include a reason information indicating that a new call is occurred at the "listened to" user in step 2 and step 3.

If a bearer hold timer is configured, the MCPTT server starts the timer and releases the network resource used by this paused ambient listening call if the timer expires before the call resumes.

4.	MCPTT client 1 sends ambient listening call pause response to the MCPTT server.

5.	MCPTT server returns ambient listening call pause response to the MCPTT client 2.

NOTE:	Step 5 can be performed after step 2 without waiting for step 4.

6.	 The ambient listening call is paused without no more media plane data exchange.

7.	The new group call or private call is established and the media plane is also established. The MCPTT client 1 starts exchanging media data in the new call.

8.	The MCPTT client 2 used by the "listened to" user detects the new call is terminated via the following ways:

8a.	The MCPTT client 2 receives a new group call release request or a private call release request.

8b.	The MCPTT client 2 sends out the group call release request, group call leave request or a private call release request.

9.	The new call is terminated.

10.	Based on the detected call termination event, the MCPTT client 2 sends the ambient listening call resume request to the MCPTT server automatically without any interactions with the "listened to" user, to activate the paused media plane.

Optionally, it may include an SDP offer, if the bearer hold timer has expired, the bearers had been released before step 9 or the media plane is de-activated in step 2 to step 6.

11.	The MCPTT server sends the ambient listening call resume request to the MCPTT client 1 with the aim to re-activate the media plane or set up the bearer.

12.	The MCPTT client 2 sends the ambient listening call resume response to the MCPTT server.

13.	The MCPTT server sends the ambient listening call resume response to the MCPTT client 1.

14.	The media plane is resumed.

15.	The MCPTT server sends the floor granted to the MCPTT client 2.

16.	The MCPTT server sends the floor taken to the MCPTT client 1.

17.	The MCPTT client 2 starts sending media to the MCPTT client 1.

# /************************* 2nd changes **********************/

A.5	MCPTT service configuration data

The general aspects of MC service configuration are specified in 3GPP TS 23.280 [16]. The MCPTT service configuration data is stored in the MCPTT server.

Tables A.5-1 and A.5-2 describe the configuration data required to support the use of on-network MCPTT service. Tables A.5-1 and A.5-3 describe the configuration data required to support the use of off-network MCPTT service. Data in tables A.5-1and A.5-3 can be configured offline using the CSC-11 reference point.

Table A.5-1: MCPTT service configuration data (on and off network)

| Reference | Parameter description | MCPTT UE | MCPTT Server | Configuration management server |
| --- | --- | --- | --- | --- |
| [R-5.2.2-001] of 3GPP TS 22.280 [17] | Levels of group hierarchy for group-broadcast groups (Bc1) | Y | Y | Y |
| [R-5.2.3-001] of 3GPP TS 22.280 [17] | Levels of user hierarchy for user-broadcast groups (Bc2) | Y | Y | Y |
| [R-5.8-002] of 3GPP TS 22.280 [17] | Minimum length (Nc3) of an alphanumeric identifier (i.e. alias) assigned by an MCPTT administrator. | Y | N | Y |

Table A.5-2: MCPTT service configuration data (on-network)

| Reference | Parameter description | MCPTT UE | MCPTT Server | Configuration management server |
| --- | --- | --- | --- | --- |
| [R-5.7.2.3.2-002] of 3GPP TS 22.179 [2] | Timeout value for the cancellation of an in-progress emergency for an on-network private call | N | Y | Y |
| [R-5.7.2.1.2-002] of 3GPP TS 22.280 [17] | Time limit for an in-progress emergency related to an on-network MCPTT group | N | Y | Y |
| [R-5.6.5-004] of 3GPP TS 22.179 [2] | Max on-network private call (with floor control) duration | N | Y | Y |
| [R-6.2.4-003] of 3GPP TS 22.179 [2] | Hang timer for private calls | N | Y | Y |
| [R-6.7.2-008] of 3GPP TS 22.280 [17] | Max duration of private call (without floor control) | N | Y | Y |
| [R-6.2.3.3.1-001] of 3GPP TS 22.179 [2] | Hierarchy of participant rights to override | N | Y | Y |
| [R-6.2.3.5-002] of 3GPP TS 22.179 [2] | Transmit time limit from a single request to transmit in a group or private call transmission | N | Y | Y |
| [R-6.2.3.5-003], [R-6.2.3.5-004] of 3GPP TS 22.179 [2] | Configuration of warning time before time limit of transmission is reached (on-network) | N | Y | Y |
| [R-6.2.4-005] of 3GPP TS 22.179 [2] | Configuration of warning time before call hang time (on-network) | N | Y | Y |
| [R-6.2.3.2-006] of 3GPP TS 22.179 [2] | Depth of floor control queue | N | Y | Y |
| [R-6.2.3.2-012] of 3GPP TS 22.179 [2] | Max time for a user's floor control request to be queued | N | Y | Y |
| [R-5.13-001] of 3GPP TS 22.280 [17] | Protect confidentiality of signalling (see NOTE 1) | Y | Y | Y |
| [R-5.13-001] of 3GPP TS 22.280 [17] | Protect integrity of signalling (see NOTE 1) | Y | Y | Y |
| [R-5.13-001] of 3GPP TS 22.280 [17] | Use signalling protection between MCPTT servers (see NOTE 1) | N | Y | Y |
| [R-5.13-001] of 3GPP TS 22.280 [17] | Use floor control protection between MCPTT servers (see NOTE 1) | N | Y | Y |
|  | List of functional alias identities |  |  |  |
| [R-5.9a-005] of 3GPP TS 22.280 [17] | > Functional alias | N | Y | Y |
| [R-5.9a-016] of 3GPP TS 22.280 [17] | > Communication priority (see NOTE 2) | N | Y | Y |
| [R-5.9a-005] of 3GPP TS 22.280 [17] | > Limit number of simultaneous activations | N | Y | Y |
| [R-5.9a-005] of 3GPP TS 22.280 [17] | > This functional alias can be taken over | N | Y | Y |
|  | > List of users |  |  |  |
| [R-5.9a-005] of 3GPP TS 22.280 [17] | >> MCPTT ID | N | Y | Y |
| [R-5.6.3-015], [R-6.7.4-016] of 3GPP TS 22.179 [2] | Max number immediate forwardings | N | Y | Y |
| [R-5.10-001a] of 3GPP TS 22.280 [17] | Maximum number of successful simultaneous service authorizations of clients from a user | N | Y | Y |
|  | Ad hoc group call configurations |  |  |  |
| [R-6.15.5.3-005] of 3GPP TS 22.280 [17] | > Support of ad hoc group call (enabled/disabled) (see NOTE 3) | Y | Y | Y |
| [R-6.15.5.3-002] of 3GPP TS 22.280 [17] | > Maximum number of participants allowed to participate in an ad hoc group call | Y | Y | Y |
| [R-6.15.5.3-004] of 3GPP TS 22.280 [17] | > Hang timer for ad hoc group call | N | Y | Y |
|  | > Maximum duration for ad hoc group call | Y | Y | Y |
|  | > List of preferred voice codecs for ad hoc group call | Y | Y | Y |
| Subclause 6.2.3.7.13 of 3GPP TS 22.179 [2] | > Multi-talker floor control supported | Y | Y | Y |
| Subclause 7.4.2.3.5 and subclause 7.4.2.3.6 | > Audio mixing is performed in the UE or in the network to support multi-talker control | Y | Y | Y |
| [R-6.2.3.7.2-005] of 3GPP TS 22.179 [2] | > Maximum number of simultaneous talkers applicable for multi-talker control | N | Y | Y |
| [R-6.16.2.2.1-005] of 3GPP TS 22.179 [2] | Bearer hold timer after the ambient listening call is being paused | Y | Y | Y |
| NOTE 1: Security mechanisms are specified in 3GPP TS 33.180 [19]. NOTE 2: The usage of this parameter by the MCPTT server is up to implementation. NOTE 3: If the support for ad hoc group call is disabled by the MC system then all other configurations related to ad hoc group call are not applicable. |  |  |  |  |

Table A.5-3: MCPTT service configuration data (off-network)

| Reference | Parameter description | MCPTT UE | MCPTT Server | Configuration management server |
| --- | --- | --- | --- | --- |
| [R-5.7.2.3.2-002] of 3GPP TS 22.179 [2] | Timeout value for the cancellation of an in-progress emergency for an off-network private call | Y | N | Y |
| [R-5.7.2.1.2-002] of 3GPP TS 22.280 [17] | Time limit for an in-progress emergency related to an off-network MCPTT group | Y | N | Y |
| [R-5.6.5-004] of 3GPP TS 22.179 [2] | Max off-network private call (with floor control) duration | Y | N | Y |
| [R-7.4-002] of 3GPP TS 22.179 [2] [R-7.4-003] of 3GPP TS 22.280 [17] | Hang timer for private calls in off-network | Y | N | Y |
| [R-7.3.3-001], [R-7.3.3-002], [R-7.3.3-003] of 3GPP TS 22.179 [2] | Priority hierarchy for floor control override in off-network | Y | N | Y |
| [R-7.3.5-001], [R-7.3.5-002], [R-7.3.5-003] of 3GPP TS 22.179 [2] | Transmit time limit from a single request to transmit in a group or private call. | Y | N | Y |
| [R-7.3.5-001], [R-7.3.5-004] of 3GPP TS 22.179 [2] | Configuration of warning time before time limit of transmission is reached (off-network) | Y | N | Y |
| [R-7.4-004] of 3GPP TS 22.280 [17] | Configuration of warning time before hang time is reached (off-network) | Y | N | Y |
| [R-7.7-001], [R-7.7-003] of 3GPP TS 22.280 [17] [R-7.7-002] of 3GPP TS 22.179 [2] | Default ProSe Per-Packet priority (as specified in 3GPP TS 23.303 [7]) values |  |  |  |
|  | > MCPTT private call signalling | Y | N | Y |
|  | > MCPTT private call media | Y | N | Y |
|  | > MCPTT Emergency private call signalling | Y | N | Y |
|  | > MCPTT Emergency private call media | Y | N | Y |
| [R-7.15-001], [R-7.7-003] of 3GPP TS 22.280 [17] | Configuration of metadata to log | Y | N | Y |