3GPP TSG-SA WG6 Meeting #68	S6-253479

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253321)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.282 | CR | 0388 | rev | 1 | Current version: | 19.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to the information flow tables in clause 7.14.2.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon, China BroadNet |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | FRMCS_Ph5 |  |  |  |  |  | Date: |  |  | 2025-08-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In clause 7.14.2.1.1, there are two information flow tables in clause 7.14.2.1.1. However, the table 7.14.2.1.1-2 lacks introduction and is not aligned with the drafting rule. This CR is proposed to move the table 7.14.2.1.1-2 to a new clause. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Move the table 7.14.2.1.1-2 to a new clause. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Table and introduction are not aligned which impact the TS quality. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.14.2.1.1, new 7.14.2.1.1a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /**************************** First changes **********************/

7.14.2.1.1	MCData IPcon point-to-point request (MCData client to MCData server)

Table 7.14.2.1.1-1 describes the information flow of the MCData IPcon point-to-point request sent from the MCData client to the MCData server.

Table 7.14.2.1.1-1: MCData IPcon point-to-point request (MCData client to MCData server)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The MCData identity of the originator MCData user; |
| Functional alias | O | The associated functional alias of the originator MCData user; |
| MCData ID | O (NOTE 2) | The MCData identity of the target MCData client IP connectivity is requested. |
| Functional alias | O (NOTE 2) | The functional alias of the target MCData client. |
| SDP offer | M | Offered media parameters describing the requested characteristics of the IP tunnel between the MCData UEs. |
| Requested Priority (NOTE 3) | O | Application priority level requested for this communication. |
| Location Information | O (NOTE 1) | Actual location information of the originating MCData user; |
| Time Limit | O | Proposed time limit of the requested IP connectivity (1min- infinite); |
| Establishment reason | O | IP connectivity establishment reason |
| Application data (see NOTE 4) | O | Application specific information that is communicated to the recipient |
| NOTE 1: This information contains the latest available location information of the requesting MCData user that may be different to the latest available location information in the MC system. NOTE 2: At least one identity shall be present. If both are present the MCData ID shall be used to route the request and the functional alias is just for information. NOTE 3: The predefined priority of the MC service user is applied by the MCData server if the requested priority is not present or not accepted by the MCData server. NOTE 4: How the MCData client gets the content of this information element is outside the scope of the present document. |  |  |

7.14.2.1.1a	MCData IPcon point-to-point request (MCData server to MCData client)

Table 7.14.2.1.1a-1 describes the information flow of the MCData IPcon point-to-point request sent from the MCData server to the MCData client.

Table 7.14.2.1.1a-1: MCData IPcon point-to-point request (MCData server to MCData client)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The MCData identity of the originator MCData user; |
| MCData ID | M | The MCData identity of the target MCData client IP connectivity is requested. |
| SDP offer | M | Offered media parameters describing the requested characteristics of the IP tunnel between the MCData UEs. |
| Location Information | O (NOTE 1) | Actual location information of the originating MCData user; |
| Time Limit | O | Proposed time limit of the requested IP connectivity (1min- infinite); |
| Establishment reason | O | IP connectivity establishment reason |
| Application data (see NOTE 2) | O | Application specific information that is communicated to the recipient |
| NOTE 1: This information contains the latest available location information of the requesting MCData user. NOTE 2: How the MCData client gets the content of this information element is outside the scope of the present document. |  |  |