3GPP TSG-SA WG6 Meeting #68	S6-253476

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253066)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.379 | CR | 0476 | rev | 1 | Current version: | 19.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Ad hoc group call leave in single MCPTT system |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | UIC, NOKIA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | FRMCS_Ph5 |  |  |  |  |  | Date: |  |  | 2025-08-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The information flow ad hoc group call leave request from the MCPTT client to the MCPTT server is defined in 10.19.2.13a, but the corresponding procedure is missing. Same applies to the information flow Ad hoc group call leave response from the MCPTT server to the MCPTT client, defined in 10.19.2.14a. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding missing ad hoc group call leave procedure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MCPTT users can not leave an ongoing ad hoc group call. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.19.2.13a, 10.19.2.14a, 10.19.3.1.7 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | None. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** Change #1*****************************************************************************

#### 10.19.2.13a	Ad hoc group call leave request (MCPTT client – MCPTT server)

Table 10.19.2.13a-1 describes the information flow ad hoc group call leave request from the MCPTT client to the MCPTT server.

Table 10.19.2.13a-1: Ad hoc group call leave request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the ad hoc group call participant leaving the call |
| Functional alias | O | The functional alias of the ad hoc group call participant leaving the call |
| MCPTT ad hoc group ID | M | The MCPTT group ID of ad hoc group call on which call is on-going |
| Reason to leave | M | Carries the reason of why the MCPTT client is leaving ongoing ad hoc group call (e.g. Due to migration). |

***** End of Change #1 ***********************************************************************

***** Change #2*****************************************************************************

#### 10.19.2.14a	Ad hoc group call leave response (MCPTT server – MCPTT client)

Table 10.19.2.14a-1 describes the information flow Ad hoc group call leave response from the MCPTT server to the MCPTT client.

Table 10.19.2.14a-1: Ad hoc group call leave response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the ad hoc group call participant leaving the call |
| Functional alias | O | The functional alias of the ad hoc group call participant leaving the call |
| MCPTT ad hoc group ID | M | The MCPTT group ID of ad hoc group call on which call is on-going |
| Result | M | Result of the ad hoc group call leave request – success or failure. |

***** End of Change #2 ***********************************************************************

***** Change #3*****************************************************************************

##### 10.19.3.1.7	Leave ad hoc group call

Figure 10.19.3.1.7-1 below illustrates the leave of an ad hoc group call procedure by an authorized user.

Figure 10.19.3.1.7-1: Leave ad hoc group call by an authorized user

1.	It is assumed that MCPTT users on MCPTT client 1, client 2 and client 3 are already part of the ongoing ad hoc group call.

2.	The MCPTT user at the MCPTT client 1 requests to leave an ad hoc group call. If the MCPTT user of MCPTT client 1 has selected a functional alias, then the ad hoc group call leave request contains that functional alias.

3.	If functional alias is present, the MCPTT server checks whether the provided functional alias is allowed to be used and has been activated for the MCPTT user at the MCPTT client 1. The MCPTT server checks whether the MCPTT user on MCPTT client 1 is authorized to leave the ad hoc group call based on local policy.

If authorised, the MCPTT server removes the MCPTT user on MCPTT client 1 from the ad hoc group call and marks it so that the MCPTT server will not add it back to the ad hoc group call based on the criteria.

NOTE: 	The MCPTT user on MCPTT client 1 being removed can be added back by an authorized user via the modify ad hoc group call participants request.

The MCPTT server may release the ongoing ad hoc group call e.g., last participant leaving, second last participant leaving.

If not authorized, the MCPTT server rejects the request to leave as specified in step 4.

4.	The MCPTT server sends ad hoc group call leave response to MCPTT client 1, containing the result of whether the ad hoc group call leave request is authorized or not. If the ad hoc group call leave request is not authorized, the MCPTT server and MCPTT client 1 shall not proceed with the rest of the steps.

5.	The MCPTT server may notify the participants about the change of the participants of the on-going ad hoc group call.

***** End of Change #3 ***********************************************************************