3GPP TSG-SA5 Meeting #162	S5-253533

# Stor-Göteborg, Sweden, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0723 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.552 Fix and adjust some formatting issues in 5.1.1.27 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom Corporation Ltd. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph4 |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There are formatting issues in clause 5.1.1.27. Additionally, it would be better to optimise some numbering of clauses for a clearer document structure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Fix some formatting issues and fold paging measurements in NTN Deployments into a subdirectory. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Formatting error may cause confusion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.27.2, 5.1.1.27.3, 5.1.1.27.4, 5.1.1.27.5, 5.1.1.27.9, 5.1.1.27.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

5.1.1.27.2	Number of NG-RAN Initiated paging records received by the gNB-CU

a) This measurement provides numbeof NR RAN Initiated paging records received by the gNB-CU.

b) CC.

c) Reception of a RAN PAGING message from NR RAN (See in TS 38.304 [37] and TS 38.423 [13]).

d)  A single integer value.

e)  PAG.ReceivedNbrRanIntiated.

f)	GNBCUCPFunction

g)	Valid for packet switched traffic

h)	5GS

5.1.1.27.3	Number of paging records received by the NRCellDU

a)	This measurement provides number of paging records received by gNB-DU which shall perform paging of the UE in cells which belong to cells as indicated in the Paging Cell List IE (See in TS 38.473 [6]).

b)	CC.

c)	Reception of a PAGING message from gNB-CU, (See in TS 38.473 [6]).

d)	A single integer value.

e)	PAG.ReceivedNbr.

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

5.1.1.27.4	Number of CN Initiated paging records discarded at the gNB-CU

a)	This measurement provides number of CN Initiated paging records discarded at the gNB-CU.

b)	CC.

c) Reception of a PAGING message from AMF, (See in TS 38.413 [11]) that is discarded at the gNB-CU.

d)	A single integer value.

e)	PAG.DiscardedNbrCnInitiated

f)	GNBCUCPFunction

g)	Valid for packet switched traffic

h)	5GS

5.1.1.27.5	Number of NG-RAN Initiated paging records discarded at the gNB-CU

a)	This measurement provides number of NG-RAN Initiated paging records discarded at the gNB-CU.

b)	CC.

c)	Reception of a RAN PAGING message from NG-RAN (See in TS 38.304 [37] and TS 38.423 [13]) that is discarded at the gNB-CU.

d)	A single integer value.

e)	PAG.DiscardedNbrRanInitiated

f)	GNBCUCPFunction

g)	Valid for packet switched traffic

h)	5GS

next change

5.1.1.27.Y Paging measurements in NTN Deployments

5.1.1.27.Y.1	Distribution of Instantaneous Paging Load for Core Initiated Page in NTN Deployments

a)	This measurement provides distribution of the instantaneous paging load applicable for core-initiated paging in NTN deployments. The measurement is provided per TAI and only for NTN.

b)	CC

c)	Each sample is obtained with an internal sampling period (e.g. one second) as the number of UEs in Idle state in the list of TAIs (comprising the Registration Area) in which a page is sent out by the Core Network as specified in TS 38.331 [20], TS 38.300 [49], TS 38.401 [66] and TS 23.501 [4]. The number of UEs in Idle is obtained using the running count of UEs which are incremented whenever a UE is moved from RRC Connected to RRC Idle in a given TAI and summing up the counts per TAI across all the TAIs in which the page is being sent.

d)	Each measurement is an integer.

e)	The measurement name has the form PAG.PagingLoadInstantCore.Bin, where Bin indicates a data volume range which is vendor specific.

f)	NRCellCU

GNBCUCPFunction

g)	Valid for packet switched traffic.

h)	5GS

NOTE:	This measurement is applicable to Earth-fixed and quasi-Earth-fixed cell NTN deployments.

5.1.1.27.Y.2	Distribution of Instantaneous Paging Load for RAN Initiated Page in NTN Deployments

a)	This measurement provides distribution of the of instantaneous paging load applicable to RAN initiated paging in NTN deployments. The measurement is provided per RNA and only for NTN.

b)	CC

c)	Each sample is obtained with an internal sampling period (e.g. one second) as the number of UEs in RRC-INACTIVE state for each RAN based Notification Area (RNA) in the RAN as specified in TS 38.300 [49], TS 38.401 [66] and TS 23.501 [4]. The number of UEs in RRC-INACTIVE is obtained using the running count of UEs which is incremented whenever a UE is moved from RRC Connected to RRC INACTIVE in a given RNA.

d)	Each measurement is an integer.

e)	The measurement name has the form PAG.PagingLoadInstantRAN.Bin, where Bin indicates a data volume range which is vendor specific.

f)	NRCellCU

GNBCUCPFunction

g)	Valid for packet switched traffic.

h)	5GS

NOTE:	This measurement is applicable to Earth-fixed and quasi-Earth-fixed cell NTN deployments.

End of changes