# 3GPP TSG-SA Meeting #109	SP-251122

Beijing, China, September 16 – 19, 2025

# Source:	Nokia

# Title:	pCR TR 22.850: Updates to terminology, evaluation and conclusions

# Document for:	Approval

# Agenda Item:	8

# 1	Introduction

This contribution proposes to:

- update ML to AI/ML in some places where “ML” occurrences were still available,

- update definitions for horizontal federated learning and vertical federated learning,

- progress evaluation and conclusion parts from TR 22.850.

# 2	Detailed proposal

The following changes are proposed to be included into TR 22.850.

***** START OF CHANGES *****

# 6	Analysis on AI/ML across 3GPP

#### 6.2.1.1	Analysis on ML model

The term 'ML model' has been defined differently by SA WG5, SA WG6 and RAN WG1, as illustrated in Table 6.2.1.1-1.

Table 6.2.1.1-1: Definition of ML model as defined across 3GPP WGs

| TSG (TS/TR) | ML model |
| --- | --- |
| SA WG5 TS 28.105 [9] | A manageable representation of an ML model algorithm. NOTE 1: An ML model algorithm is a mathematical algorithm through which running a set of input data can generate a set of inference output. NOTE 2: An ML model algorithm is proprietary and not in scope for standardization and therefore not treated in this specification. NOTE 3: An ML model may include metadata. Metadata may include e.g. information related to the trained model and applicable runtime context. |
| SA WG6 TR TS 23.482 [34] | According to TS 28.105 [9], mathematical algorithm that can be "trained" by data and human expert input as examples to replicate a decision an expert would make when provided that same information. |
| RAN WG1 TR 38.843 [3] | A data driven algorithm that applies AI/ML techniques to generate a set of outputs based on a set of inputs. |

The following unified definition for 'AI/ML model' is proposed:

AI/ML model:  A mathematical algorithm that applies AI/ML techniques to generate a set of outputs based on a set of inputs. It may include metadata which consists of, e.g. information related to the model and applicable runtime context.

NOTE:	An AI/ML model can be managed, stored and transferred as artifacts, which may be containers, images, or proprietary file formats.

***** NEXT CHANGE *****

#### 6.2.1.2	Analysis on ML model training

The term 'ML model training' has been defined differently by SA WG5, SA WG6 and RAN WG1, as illustrated in Table 6.2.1.2-1. RAN WG3 follows the definition of SA WG5.

Table 6.2.1.2-1: Definition of ML model training as defined across 3GPP WGs

| TSG (TS/TR) | ML model training |
| --- | --- |
| SA WG5 TS 28.105 [9] | A process performed by an ML training function to take training data, run it through an ML model algorithm, derive the associated loss and adjust the parameterization of that ML model iteratively based on the computed loss and generate the trained ML model. |
| SA WG6 TR TS 23.482 [34] | According to TS 28.105 [9], ML model training includes capabilities of an ML training function or service to take data, run it through an ML model, derive the associated loss and adjust the parameterization of that ML model based on the computed loss. |
| RAN WG1 TR 38.843 [3] | A process to train an AI/ML Model [by learning the input/output relationship] in a data driven manner and obtain the trained AI/ML Model for inference. |
| RAN WG3 TS 38.300 [11] | AI/ML Model Training follows the definition of the "ML model training" as specified in clause 3.1 of TS 28.105 [9]. |
| RAN WG3 TS 38.401 | AI/ML Model Training follows the definition of the "ML model training" as specified in clause 3.1 of TS 28.105 [9]. |

The following unified definition for 'AI/ML model training' is proposed:

AI/ML model training: A process to train an AI/ML Model by learning the input/output relationship in a data driven manner and obtain the trained AI/ML Model for e.g. inference.

***** NEXT CHANGE *****

#### 6.2.1.5	Analysis on ML model inference

The term 'ML model inference' has been defined differently by SA WG5, SA WG6 and RAN WG1, as illustrated in Table 6.2.1.5-1. RAN WG3 follows the definition of SA WG5.

Table 6.2.1.5-1: Definition of ML model inference as defined across 3GPP WGs

| TSG (TS/TR) | ML model inference |
| --- | --- |
| SA WG5 TS 28.105 [9] | A process of running a set of input data through a trained ML model to produce set of output data, such as predictions. |
| SA WG6 TR TS 23.482 [34] | According to TS 28.105 [9], ML model training includes capabilities of an ML model inference function that employs an ML model and/or AI decision entity to conduct inference. |
| RAN WG1 TR 38.843 [3] | A process of using a trained AI/ML model to produce a set of outputs based on a set of inputs. |
| RAN WG3 TS 38.300 [11] | AI/ML Model Inference follows the definition of the "AI/ML inference" as defined in clause 3.1 of TS 28.105 [9]. |
| RAN WG3 TS 38.401 | AI/ML Model Inference follows the definition of the "AI/ML inference" as defined in clause 3.1 of TS 28.105 [9]. |

The following unified definition for 'AI/ML model inference' is proposed:

AI/ML model inference: A process of running a set of inputs through a trained AI/ML model to produce a set of outputs.

***** NEXT CHANGE *****

#### 6.2.1.6	Analysis on ML model activation & ML model de-activation

The term 'ML model activation' and 'ML model deactivation' have been defined by RAN WG1, as illustrated in Table 6.2.1.6-1. SA WG5 mentions the terms ML activation and ML deactivation several times in TS 28.105 [9] but does not provide a definition. It defines instead AI/ML activation/deactivation of the scope of an inference function.

Table 6.2.1.6-1: Definition of ML model activation & ML model de-activation as defined across 3GPP WGs

| TSG (TS/TR) | ML model activation & ML model de-activation |
| --- | --- |
| SA WG5 TS 28.105 [9] | AI/ML activation: a process of enabling the inference capability of an AI/ML inference function. AI/ML deactivation: a process of disabling the inference capability of an AI/ML inference function. |
| RAN WG1 TR 38.843 [3] | ML Model activation: enable an AI/ML model for a specific AI/ML-enabled feature. ML Model deactivation: disable an AI/ML model for a specific AI/ML-enabled feature. |

The following unified definition for 'AI/ML model activation' and 'AI/ML model deactivation' is proposed:

AI/ML model activation: A process to enable an AI/ML model for a specific AI/ML-enabled feature.

AI/ML model deactivation: A process to disable an AI/ML model for a specific AI/ML-enabled feature.

Editor's note:	Further analysis is required to determine whether ML model activation and deactivation should be associated specifically with inference capability (according to TS 28.105 [9]) or with the ML model more broadly (according to TR 38.843 [3]).

***** NEXT CHANGE *****

#### 6.2.1.8	Analysis on ML model lifecycle management

SA WG5 describes the ML model lifecycle in clause 4a.0 of TS 28.105 [9] and ML model lifecycle management capabilities for ML model training, ML model testing, ML inference emulation as optional, ML model deployment and AI/ML inference in clause 6.1 of TS 28.105 [9]. The terms 'ML model-based lifecycle management', 'ML-enabled functionality' and 'Functionality-based lifecycle management' have been defined by RAN1, as illustrated in Table 6.2.1.8-1.

Table 6.2.1.8-1: Definitions of ML model-based lifecycle management, ML-enabled functionality and Functionality-based lifecycle management as defined across 3GPP WGs

| TSG (TS/TR) | ML model lifecycle management / Functionality-based lifecycle management |
| --- | --- |
| RAN WG1 TR 38.843 [3] | ML model-based lifecycle management: Operates based on identified logical models, where a model may be associated with specific configurations/conditions associated with UE capability of an AI/ML-enabled Feature / Feature Group and additional conditions (e.g. scenarios, sites and datasets) as determined/identified between UE-side and NW-side. The models are identified at the Network and Network/UE may activate/deactivate/select/switch individual AI/ML models via model ID. (ML-enabled) Functionality: An AI/ML-enabled Feature/Feature Group enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability. Functionality-based lifecycle management: Signaling procedure where network indicates activation/deactivation/fallback/switching of AI/ML functionality via 3GPP signaling (e.g. RRC, MAC-CE, DCI); operates based on, at least, one configuration of AI/ML-enabled Feature/FG or specific configurations of an AI/ML-enabled Feature / Feature Group. |
| SA WG5 TS 28.105 [9] | ML model training management: enables requesting, consuming and controlling ML model training and re-training processes. It includes training performance management and policy setting for producer-initiated training. ML model testing management: allows requesting and receiving ML model testing results, selecting performance metrics and triggering model re-training based on test performance. ML model loading management: supports triggering, controlling and monitoring the ML model loading process as part of model deployment. AI/ML inference management: allows managing inference functions and/or ML model(s), including activation/deactivation, output parameter configuration, performance monitoring and triggering model updates if necessary. |

The following unified definition for 'AI/ML model lifecycle management' is proposed:

AI/ML model lifecycle management: The management capabilities allowing a producer or consumer to manage different phases of the AI/ML model lifecycle as defined in clause 6.2.1.7.

The following definition for 'Functionality-based lifecycle management' is proposed for adoption by all 3GPP RAN Working Groups:

Functionality-based lifecycle management: Signalling procedure where network indicates activation/deactivation/fallback/switching of AI/ML functionality via 3GPP signalling (e.g. RRC, MAC-CE, DCI); operates based on, at least, one configuration of AI/ML-enabled Feature / Feature Group or specific configurations of an AI/ML-enabled Feature/FG.

NOTE 1:	In the context of RAN1, RAN2 and RAN4, functionality-based lifecycle management does not consider training, testing and maintenance phases and consider them as implementation-specific.

NOTE 2:	Applicability of Functionality-based lifecycle management definition to/in TSG SA WGs is optional.

Editor's note:	The following analyses on the key differences between ML Model LC and LCM is to be revised and possibly relocated to different clause in this TR.

Key differences between ML Model lifecycle (LC) and ML Model lifecycle management (LCM)

TS 28.105 [9] defines both ML model lifecycle (LC) and ML model lifecycle management (LCM) within the scope of AI/ML management in 3GPP networks. The key differences between the two are:

ML model lifecycle (LC) describes the essential steps (phases) an ML model undergoes, from training to inference. It consists of:

-	ML model training (e.g. initial training & re-training).

-	ML model testing.

-	ML emulation.

-	ML model deployment (including ML model loading).

-	AI/ML inference.

ML model lifecycle management (LCM) focuses on the management capabilities that control and optimize each phase of the ML model lifecycle. LCM enables functionalities such as:

-	Training management (e.g. triggering re-training, setting policies).

-	Testing management (e.g. evaluating performance, determining retraining needs).

-	Deployment management (including ML model loading).

-	Inference management (e.g. monitoring inference results, managing AI/ML inference functions).

LCM, as specified in TS 28.105 [9], encompasses the full lifecycle management of both ML models and AI/ML inference functions. This means that LCM does not only manage the ML model while inference remains a separate process; rather, it ensures a unified management approach that includes both:

-	ML model lifecycle management covers the entire lifecycle of the ML model itself, including its training, validation, deployment and inference.

-	AI/ML inference function lifecycle management is also part of LCM, ensuring that inference operations are properly activated, configured, monitored and optimized.

For example, LCM in TS 28.105 [9] enables not just the deployment of an ML model but also the continuous management of its inference functions, such as their activation, configuration and real-time monitoring. This differs from a narrower view of lifecycle (LC), which only considers inference as a step where the ML model is applied, without addressing its ongoing management.

***** NEXT CHANGE *****

### 6.2.2	Analysis on Federated Learning

The term 'Horizontal Federated Learning' and 'Vertical Federated Learning' have been defined in SA WG2 and RAN WG1 as well as SA WG5 defines 'Federated Learning', as illustrated in Table 6.2.2-1.

Table 6.2.2-1: Definition of Federated Learning as defined across 3GPP WGs

| TSG (TS/TR) | Federated Learning |
| --- | --- |
| SA WG2 TR 23.700-84 [4] | Horizontal Federated Learning: A federated learning technique without exchanging/sharing local data set, wherein the local data set in different FL clients for local model training have the same feature space for different samples (e.g. UE IDs). |
| SA WG2 TR 23.700-84 [4] | Vertical Federated Learning: A federated learning technique without exchanging/sharing local data set, wherein the local data set in different VFL Participant for local model training have different feature spaces for the same samples (e.g. UE IDs). |
| RAN WG1 TR 38.843 [3] | Federated Learning: A machine learning technique that trains an AI/ML model across multiple decentralized edge nodes (e.g. UEs, gNBs) each performing local model training using local data samples. The technique requires multiple interactions of the model, but no exchange of local data samples. |
| 3GPP SA5 TS 28.105 [9] | Federated Learning: a distributed machine learning approach where the ML model is trained collaboratively by multiple ML training functions. This includes multiple FL clients, which perform training on local data, and one FL server, which aggregates model outcomes from the clients iteratively without exchanging data samples. |

The definition of Federated Learning provided by RAN WG1 appears to only apply to Horizontal Federated Learning, as the phrase "each performing local model training using local data samples" implies that the data samples at individual nodes are distinct. The key difference between Horizontal Federated Learning and Vertical Federated Learning lies in the characteristics of the local datasets:

-	Horizontal Federated Learning: Local datasets have the same features but different samples.

-	Vertical Federated Learning: local data sets have different features but share same samples.

The definition of Federated Learning provided by SA WG5 highlights the collaborative training process among multiple FL participants, including an FL server and FL clients, without specifying the characteristics of the client datasets. This broader definition facilitates a more comprehensive understanding of both Horizontal Federated Learning (HFL) and Vertical Federated Learning (VFL) that are already defined in the specifications and also offers greater flexibility across more general scenarios, see definitions in clause 5.2.1.2.

The terms "distributed learning" and "federated learning" are often used together as "distributed/federated learning" in SA WG1 TS 22.261 [6]. "Distributed learning" typically refers to a broader set of learning techniques including "federated learning". Although the two terms are related, they are not identical and should be used appropriately based on the context.

The following unified definition for 'Federated Learning' is proposed:

Federated Learning: A distributed machine learning approach where the AI/ML model(s) are collaboratively trained by multiple participants, including one acting as an FL server and multiple acting as FL clients, iteratively without exchanging data samples.

The following unified definition for 'Horizontal Federated Learning' is proposed:

Horizontal Federated Learning: A federated learning technique without exchanging/sharing local data set, wherein the local data set in different clients for local model training have the same feature space for different samples.

The following unified definition for 'Vertical Federated Learning' is proposed:

Vertical Federated Learning: A federated learning technique without exchanging/sharing local data set, wherein the local data set in different clients for local model training have different feature spaces for the same samples.

***** NEXT CHANGE *****

# 7	Overall Evaluation

The study reviewed AI/ML-related activities across TSG SA, TSG RAN, and TSG CT Working Groups, to identify potential misalignment in AI/ML terminology and AI/ML feature descriptions across Working Groups.

The analysis of AI/ML terminology revealed that several AI/ML terms were used with overlapping or inconsistent meanings across Working Groups. To address this, a set of unified terminologies has been developed during the study, providing consistent definitions for key concepts such as AI/ML model, AI/ML model training, AI/ML model inference, AI/ML model lifecycle management, Functionality-based lifecycle management, Federated Learning (including Horizontal and Vertical), Transfer Learning.

It is also agreed that the term AI/ML is to be used as a unified definition encompassing “AI/ML”, “AI”, and “ML” in all corresponding ML-related TRs/TSs.

Adoption of these unified terms will ensure that AI/ML concepts are interpreted and applied consistently across all Working Groups, thereby improving clarity and specification quality.

The unified terminologies listed below are the agreed outcome of this study:

-	AI/ML model: A mathematical algorithm that applies AI/ML techniques to generate a set of outputs based on a set of inputs. It may include metadata which consists of, e.g. information related to the model and applicable runtime context.

-	AI/ML model training: A process to train an AI/ML Model by learning the input/output relationship in a data driven manner and obtain the trained AI/ML Model for e.g. inference.

-	AI/ML model inference: A process of running a set of inputs through a trained AI/ML model to produce a set of outputs.

-	AI/ML model lifecycle management: The management capabilities allowing a producer or consumer to manage different phases of the AI/ML model lifecycle as defined in clause 6.2.1.7.

-	Functionality-based lifecycle management: Signalling procedure where network indicates activation/deactivation/fallback/switching of AI/ML functionality via 3GPP signalling (e.g. RRC, MAC-CE, DCI); operates based on, at least, one configuration of AI/ML-enabled Feature / Feature Group or specific configurations of an AI/ML-enabled Feature/FG.

-	Federated Learning: A distributed machine learning approach where the AI/ML model(s) are collaboratively trained by multiple participants, including one acting as an FL server and multiple acting as FL clients, iteratively without exchanging data samples.

-	Horizontal Federated Learning: A federated learning technique without exchanging/sharing local data set, wherein the local data set in different clients for local model training have the same feature space for different samples.

-	Vertical Federated Learning: A federated learning technique without exchanging/sharing local data, wherein the local data set in different clients for local model training have different feature spaces for the same samples.

-	Transfer Learning: A machine learning technique where the knowledge acquired from training one or more ML models is leveraged to enhance the performance or accelerate the training of another ML model.

-	The term "output" is proposed as unified term since output may include decision or prediction or statistic or recommendation.

-	The term "AI/ML" is to be used as a unified definition encompassing "AI/ML", "AI" and "ML" in all corresponding ML related TRs/TSs.

The analysis of AI/ML-related features identified that services for AI/ML model training, AI/ML model inference and analytics and AI/ML performance evaluation are specified across the SA2, SA5 and SA6 Working Groups. The analysis of AI/ML-related features also identified that services for data collection for AI/ML are specified across the SA2, SA6 and RAN3 Working Groups. No detailed analysis was progressed on potential misalignment, inconsistencies or overlap between these services.

***** NEXT CHANGE *****

# 8	Conclusions

The term "AI/ML" is to be used as a unified definition encompassing "AI/ML", "AI" and "ML" in all corresponding ML related TRs/TSs.

The term "output" is proposed as unified term since output may include decision or prediction or statistic or recommendation.

The following unified definitions are proposed for adoption by WGs, and will be documented in a CR to 3GPP TR 21.905:

-	AI/ML model: A mathematical algorithm that applies AI/ML techniques to generate a set of outputs based on a set of inputs. It may include metadata which consists of, e.g. information related to the model and applicable runtime context.

-	AI/ML model training: A process to train an AI/ML Model by learning the input/output relationship in a data driven manner and obtain the trained AI/ML Model for e.g. inference.

-	AI/ML model inference: A process of running a set of inputs through a trained AI/ML model to produce a set of outputs.

-	AI/ML model lifecycle management: The management capabilities allowing a producer or consumer to manage different phases of the AI/ML model lifecycle as defined in clause 6.2.1.7.

-	Functionality-based lifecycle management: Signalling procedure where network indicates activation/deactivation/fallback/switching of AI/ML functionality via 3GPP signalling (e.g. RRC, MAC-CE, DCI); operates based on, at least, one configuration of AI/ML-enabled Feature / Feature Group or specific configurations of an AI/ML-enabled Feature/FG.

-	Federated Learning: A distributed machine learning approach where the AI/ML model(s) are collaboratively trained by multiple participants, including one acting as an FL server and multiple acting as FL clients, iteratively without exchanging data samples.

-	Horizontal Federated Learning: A federated learning technique without exchanging/sharing local data set, wherein the local data set in different clients for local model training have the same feature space for different samples.

-	Vertical Federated Learning: A federated learning technique without exchanging/sharing local data, wherein the local data set in different clients for local model training have different feature spaces for the same samples.

-	Transfer Learning: A machine learning technique where the knowledge acquired from training one or more ML models is leveraged to enhance the performance or accelerate the training of another ML model.

***** END OF CHANGES *****