3GPP TSG-WG SA2 Meeting #171	S2-2509548

# Wuhan, China, Oct 13 – 17, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.401 | CR | 3908 | rev | 5 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarify PDN connection handling for split MME |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3-ARC |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 1. It was agreed in Annex O.2 for split MME that the PDN connection establishment during the Attach procedure is handled based on implementation. However, the current text may cause misinterpretation that the whole attach procedure is handled based on implementation. 2. In case of S&F mode change, the satellite is supposed to be ‘operating in’ S&F mode and may cease ‘operating in’ S&F mode. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Clarify that it is the PDN connection handling that is up to implementation. 2. Change ‘supporting’ to ‘operating’ |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misunderstanding of specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.13.9.2, Annex O.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 4.13.9.2	Transition between S&F mode and non S&F mode

If a UE is attached to a PLMN using satellite operating in S&F mode, the UE follows the existing AS and NAS procedures to access a satellite that is not operating in S&F mode (either TN or NTN eNodeB) and vice versa.

In order to ensure smooth idle mode transition from S&F mode to non S&F mode, if the old (S&F capable) MME in inter MME mobility has the knowledge that the UE context is not up to date (e.g. based on implementation) then the old (S&F capable) MME rejects the MM context transfer.

If a satellite has no feeder link connectivity, it can only operate in S&F Mode. When a satellite operating in S&F mode obtains a feeder link, then, the satellite may cease to operate in S&F Mode depending on operator policy and regulatory requirement.

# * * * * Next change * * * *

# O.2	Model A: Split MME architecture

In this architecture option (see Figure O.2-1):

1.	eNB is onboard the satellite.

2.	MME is split into two functions:

a.	MME-onboard: the MME part which is onboard the satellite. MME-onboard is in charge of (1) handling the S1 interface with the onboard eNB and (2) handling the NAS protocol signalling from/to UEs via the onboard eNB.

b.	MME-ground: the MME part which is on the ground network. MME-ground is in charge of handling the rest of interfaces towards other CN functions (e.g. S6a towards HSS, SGd towards SMS-GMSC/IWMSC /SMS Router, T6a towards SCEF, T6ai towards IWF-SCEF, S11 towards SGW).

Figure O.2-1: "Split-MME" architecture for supporting Store and Forward Satellite operation for SMS and CP CIoT services

The split-MME architecture has below principles:

1.	An MME-ground is associated with at least one MME-onboard. An MME-onboard is associated with a Satellite ID identifier. The MME-ground together with the associated MME-onboard(s) behave jointly as a single MME entity.

2.	How MME-onboard(s) interacts with MME-ground and how synchronization of the UE context between them is done is outside the scope of 3GPP.

NOTE 1:	Based on implementation, the UE context can be uploaded on a subset of MME-onboard(s) or all MME-onboard(s) associated with MME-ground.

NOTE 2:	As a deployment option, the S&F Wait Timer (see clause 4.13.9.1) takes into account the time to synchronize the UE context between MME-ground and MME-onboard(s)

3.	When a UE initiates a MO procedure that needs an interaction with a core network node on the ground, the MME-onboard stores the MO procedure transaction if the feeder link is not available and synchronizes with the MME-ground when the feeder link becomes available. The MME-ground executes the procedure with the ground network nodes and syncs back the UE context with the MME-onboard when the feeder link becomes available.

4.	The MO data is stored in the MME-onboard when the service link is available and the feeder link is unavailable, and transferred to the MME-ground when the feeder link becomes available. The MT data is stored in the MME-ground when the feeder link is unavailable and transferred to the MME-onboard when the feeder link becomes available. The MME-ground determines the satellite through which to send MT data and the MT data is sent to the respective MME-onboard and stored in the MME-onboard when the feeder link is available, and transferred to the UE when service link becomes available.

5.	For MO SMS, if the feeder link is not available upon reception of the MO SMS the MME-onboard can store the MO-SMS and can immediately send the delivery report (i.e. RP-ACK) to the UE i.e. as if the MO-SMS has already been successfully delivered to the Service Centre (SC). Once the feeder link is established the MO-SMS is forwarded to the SC. Subsequently the SC sends the RP-ACK to the UE. The MME-ground can also discard the RP-ACK received from the SC.

6.	To support UE location verification on satellite, the E-SMLC, if needed, can be deployed on satellite to perform the verification of UE location functionality.

7.	For the monitoring event which allows the SCS/AS to be notified of Store and Forward Satellite operation (see clauses 5.6.1.11 and 5.6.3.10 of TS 23.682 [74]) the SCS/AS communicates with HSS and MME-ground to configure and/or delete Monitoring Event.

8.	For data transfer, only Control Plane CIoT EPS Optimization applies.

9.	During attach procedure, if a UE indicated Control Plane CIoT EPS Optimisation supported in Preferred Network Behaviour, and the UE included the ESM message container,  the PDN connection is handled based on implementation between the MME-onboard and ground CN entities. For example, before or after the successful authentication and Security Mode Command procedure, after the MME-onboard rejects the initial attach of the UE (see clause 4.13.9), the MME-onboard interacts with the MME-ground when the feeder link is available. Then, the MME-ground performs the PDN connection establishment procedure (steps 12-16 of Figure 5.3.2.1-1) with SGW. The MME-ground synchronizes with the MME-onboards regarding the default PDN connection establishment procedure results. After establishing the service link, when the UE initiates the attach procedure with a new MME-onboard again, the new MME-onboard sends Attach Accept message after the successful establishment of the PDN connection.

# * * * * End of changes * * * *