3GPP TSG SA WG 1 Meeting #111 	S1-253113

25-29 August 2025, Goteborg, Sweden

Source:	CEWiT

pCR Title:	PCR on use case for positioning service for RAN optimization

Draft Spec:	3GPP TR 22.870v0.3.1

Agenda item:	8.1.5

Document for:	Agreement

Contact:	Ankita Lachhwani ankita.lachhwani@cewit.org.in

Abhijeet A. Masal abhijeetmasal@cewit.org.in

Abstract: This contribution proposes a new use case for Positioning service for RAN optimization

* * * First Change * * * *

## 5.7 	6G enhancements of legacy/existing services and capabilities

Editor’s Note:  Text to be provided.  This clause to address potential (new) requirements for new (6G) aspects of existing (5G and earlier) services or capabilities.

### 5.7.X	Positioning service for RAN optimization

#### 5.7.X.1	Description

The following is introduced in clause 5.4.2 Support of legacy service requirements in this document:

The 6G system shall be able to support the following services:

-	Location and positioning services, ref TS 22.261 [14], TS 22.071 [61],

Positioning is a critical capability in mobile communication technologies, driven both by regulatory requirements and commercial service demands. In domains such as Location-Based Services (LBS) and eHealth, high positioning accuracy is essential to enable advanced services and applications in both outdoor and indoor environments.

Legacy positioning services already support multiple external clients—capabilities that remain relevant for 6G. Beyond service enablement, positioning can also play a key role in 6G Radio Access Network (RAN) optimization.

Few examples are

- Self-Organizing Networks (SON) encompass functionalities like self-configuration, self-optimization, and self-healing, which include Automatic neighbor list creation, effective interference mitigation, mobility configuration selection, automatic rerouting when a cell is down etc. In this context, UE location information can significantly improve SON decision-making by enabling UE-specific or UE-group-specific automation for configuration, optimization, and healing processes.

- In 6G, the communcation will be more contextual and data driven. UE location provides additional dimension to such data for the predication of the functionalities like beam prediction, mobility prediction, time-frequency resource allocation etc using AI/ML

- User Equipment (UE) location can complement radio measurement data to enhance relay infrastructure performance such as relays, network-controlled repeaters (NCRs), or reconfigurable intelligent surfaces (RIS) particularly for mitigating coverage holes.

Other scenarios where UE positioning is could be useful for RAN optimization,

- Mobility & handover prediction

- Configuring geograaphy specific mobility event configurations

- UE grouping for multi user MIMO

- Beam selection & beam prediction

To enable above mentioned features, 6G RAN should be allowed to consume the Loction and positioning services. 6G RAN shall support the mechanism to request the UE(s) positioning information from 6G core networks. The 6G RAN shall be able use the position-related data in compliance with authentication and UE security regulatory requirements. The 6G system shall control the authorization of such services to the 6G RAN.

#### 5.7.X.2 Existing features partly or fully covering the use case functionality

The 5G requirement on positioning service [14] include that the 5G network is able to request UE to provide its postioning related data. The corresponding 5G requirement is as below:

The 5G network shall be able to request the UE to provide its position-related-data on request together with the accuracy of its position triggered by an event or periodically and to request the UE to stop providing its position-related data periodically.

In 5G, this requirement was partially fulfilled as the mechanism to request and receive UE’s position-related data was enabled for  core network, however not enabled for 5G RAN. But the RAN should be able to receive UE position related data for its resource optimization directly from UE, which is not addressed in the 5GS.

#### 5.7.X.3 Potential New Requirements

[PR 5.7.X.3-1] Subject to regulation, operator(s) policies and user consent, 6G RAN shall be able to request the UE positioning information as a location client from 6G core network.

[PR 5.7.X.3-2] Subject to regulation, operator(s) policies and user consent, 6G RAN shall be able to receive the positioning information together with the accuracy of its position from UE(s), subject to system configuration.

[PR 5.7.X.3-3] Subject to regulation, operator(s) policies and user consent, 6G RAN shall be authorized by the 6G-CN to request/consume the UE positioning information.

[PR 5.7.X.3-4] 6G RAN shall be able use the UE positioning information in compliance security regulatory requirements for 6G RAN optimization.