3GPP TSG SA WG 1 Meeting #112 	S1-254327266r1

17-21 November 2025, Dallas, USA	(revision of 4266r1, merge of 4122)

Source:	Nokia (Moderator), CATT

pCR Title:	Pseudo-CR on consolidated performance requirements for Ubiqiutous Connectivity

Draft Spec:	3GPP 22.870

Agenda item:	8.1.10

Document for:	Approval

Contact:	feifei.lou@nokia.com

Abstract: this contribution provides the consolidation of performance requirements in clause 8 ubiquitous connectivity.

1. Introduction

The consolidated performance requirements for clause 8 are categorised into two parts.

The consolidated performance requirements for satellite-based positioning services covers the following use cases and PRs. The scenarios in the KPI table are sorted from less demanding to more demanding cases.

- [PR 8.5.6-1] from use case on resilient positioning in satellite networks

- [PR 8.7.6-2] from use case on low-energy positioning in satellite networks

- [PR 8.10.6-1] from use case on hybrid TN and NTN positioning

- [PR 8.11.6-2] from use case on hybrid NTN and GNSS positioning

The consolidated performance requirements for satellite access covers the following use cases and PRs.

- [PR 8.4.6-2] from use case on service continuity for wearable mobile devices

- [PR 8.6.6-2] from use case on disaster relief

- [PR 8.8.6-2] from use case on global mobile video

- [PR 8.9.6-2] from use case on low-altitude logistics supported by NTN

The following PR is not consolidated due to FFS.

- [PR 8.2.6-1] from use case on ubiquitous and resilient network

2. Reason for Change

To provide the consolidation of performance requirements in clause 8 ubiquitous connectivity.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 v0.4.0.

* * * First Change * * * *

All new texts

## 8.x	Consolidated performance requirements for Ubiquitous Connectivity

### 8.x.1	Consolidated performance requirements for satellite-based positioning services

Table 8.x.1-1: Consolidated performance requirements for positioning services

| Positioning technology | Scenario | Accuracy (95 % confidence level) |  | Positioning service availability | Positioning service latency | Environment of use | MAX UE speed | UE type | Others |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Horizontal Accuracy | Vertical Accuracy |  |  |  |  |  |  |
| Satellite-based positioning | Airplane en-route (note 2) | [50] m | [50] m | [99] % | [1] s | Outdoor | [1500] km/h | Airplane mounted | - |
|  | Airplane landing (note 2) | [10] m | [10] m | [99] % | [1] s | Outdoor | [350] km/h | Airplane mounted | - |
|  | UAV positioning (note 2) | [1] m | [1] m | [99] % | [1] s | Outdoor | [160] km/h | UAV mounted | - |
| Hybrid TN and NTN Hybrid non-3GPP technologies (e.g. GNSS) and 3GPP technologies | Containers on board | [100] m | N/A | [95] % | N/A | Outdoor | [37] km/h | Container mounted | Battery life expectancy (note 1) 12 years |
|  | Pallets | [100] m | N/A | [95] % | N/A | Outdoor | [100] km/h | Vehicle mounted | Battery life expectancy (note 1) 7 years |
|  | Wagons | [100] m | N/A | [95] % | N/A | Outdoor | [350] km/h | Train mounted | Battery life expectancy (note 1) 20 years |
| Hybrid NTN and GNSS positioning | UE with partly obstructed sky | [10] m | [3] m | [95] % | [1] s | Outdoor rural and urban | [250] km/h | Handheld, vehicle mounted or IoT | - |
|  | Maritime | [3] m | [3] m | [99] % | [1] s | Outdoor rural | TBD | Maritime mounted | - |
| Hybrid TN and NTN positioning | UAV facility monitoring (note 3) | [3] m | [3] m | [99] % | [0,1 – 0,5] s | Outdoor | [160] km/h | UAV mounted | - |
|  | Airplane taxiway (note 3, 4) | [1] m | N/A | [99] % | [0,1 – 0,5] s | Outdoor | [100] km/h | Airplane mounted | - |
| NOTE 1: Battery life expectancy is to be assumed in all coverage conditions and is based on typical message size value and typical frequency, e.g. the number of messages per day by a typical user. NOTE 2: Positioning services are provided with 3GPP technologies, independently of non-3GPP positioning technologies (e.g. GNSS). Multiple satellites can be used to support 3GPP positioning technologies. NOTE 3: Requirements for Airplane taxiway and UAV facility monitoring are in [25]. NOTE 4: Airplane taxiway refers to an airplane on ground during taxi operations. |  |  |  |  |  |  |  |  |  |

Editor’s Note: maritime mounted UE speed is FFS.

### 8.x.2	Consolidated performance requirements for satellite access

Table 8.x.2-1: Consolidated performance requirements for satellite access

| Scenario | Experienced data rate (DL) | Experienced data rate (UL) | Peak data rate (DL) | Peak data rate (UL) | Area traffic capacity (DL) (note 1) | Area traffic capacity (UL) (note 1) | Overall user density | Activity factor | UE speed | UE type | Reliability | Service availability | End-to-end latency | Others |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NOTE 1: Area capacity is averaged over a satellite beam. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[CPR 8.x.2-1]: The 6G system with satellite access, shall support SMS delivery to a high density of UEs, i.e. up to [1000] devices/km2.

[CPR 8.x.2-2] The 6G system with satellite access shall be able to support communication service for UEs (e.g. UAV) at the altitudes from 0 to 3 km via satellite access and/or terrestrial access.

* * * End of changes * * * *