3GPP TSG-SA WG1 Meeting #112 	S1-254324

17-21 November 2025, DALLAS, USA	(revision of S1-254023r2)

Title:	Consolidation of KPI requirements on immersive section

Agenda Item:

Source:	Moderator

Contact:	Qun Wei, weiqun5@chinaunicom.cn

Abstract: Prepare and propose a way for consolidation on KPI requirements on immersive section.

1. Introduction

Based on the meeting preparations, this document only provides the consolidation way forward and discussion regarding the immersive KPIs, based on TR 22.870 and do not include performance requirements with editor’s note.

2. Reason for Change

To provide communication performance requirements contribution and reflect key points regarding the performance requirements of immersive sections.

3. Proposal

It is proposed to agree the following changes to new version of 3GPP TR 22.870.

Discussion Part

Besides the basic template consolidation, the following comments have been taken into consideration for the latest updates：

General：

CM1: It needs to be clarified whether immersive and AI KPI definitions (already in agreed KPI tables) can be directly used during the consolidation work.

CM2: Some of this definition already exists in other specs can we just point to those specs instead of introducing the definitions here. Also, it would help see which definitions new and which ones are not New.

CM3: Which existing KPI table of TR 22.870 can be merged this time?

• Comments: Need to list merged table at the beginning.

• Tables with EN need to put into Part II.

CM4: consolidate the immersive KPIs and AI KPIs in separate documents.

CM5: Putting existing use cases and definitions in separate files to simplify consolidation.

CM6: “Characteristic parameter”, “Influence quantity”, are suggested to be removed from the tables at this point.

CM7：number of UEs is missing for table y.1-1. “Other KPIs” is yellow. Q: Do we need a separate column for the "number of UEs"? This parameter is only applicable to UC 9.6 and, not every other use cases?

UC 9.12 Personalized interactive immersive guided tour

CM1: the word “lag” in the synchronization threshold should be either defined or removed, change to delay? Or advanced?

CM2: Probably consider aligning the terminology for "lag" and "delay". Changing "lag" to "delay" would be an option, for the sake of consistency with the term in 22.261.

UC 9.2 Immersive Gaming

CM1: note A-6 is not referenced by the table.

Change1: note A-3 can be merged to table. Need author and companies further check.

UC 9.6 Immersive Gaming Immersive media content production via the wireless link

Change1: note B-1 can be merged to table. Need author and companies further check.

UC 9.7 Use case on multiple application media synchronization

CM1：UC 9.7 is probably separate table to explain what KPIs mean, as it is distinct from UCs 9.12 and 9.3.

* * * Begin of changes * *

Y Consolidated potential KPIs

y.1 KPIs for immersive communication service

Editor's Note: the following potential performance requirements from immersive session are considered for consolidation.

- TR 22.870 0.4.1 [PR 9.2.6-1] (Table 9.2.6-1)

- TR 22.870 0.4.1 [PR 9.3.6-1] (Table 9.3.6-1)

- TR 22.870 0.4.1 [PR 9.3.6-4] (Table 9.3.6-2)

- TR 22.870 0.4.1 [PR 9.5.6-5] (Table 9.5.6-1)

- TR 22.870 0.4.1 [PR 9.6.6-3] (Table 9.6.6-1)

- TR 22.870 0.4.1 [PR 9.7.2-1] (Table 9.7.2-1)

- TR 22.870 0.4.1 [PR 9.9.6-2] (Table 9.9.6-1)

- TR 22.870 0.4.1 [PR 9.12.6-6] (Table 9.12.6-1)

The following potential performance requirements from AI session are considered for consolidation.

- TR 22.870 0.4.1 [PR 6.10.6-6] (Table 6.10.6-1)

- TR 22.870 0.4.1 [PR 6.26.6-1] (Table 6.26.6-1)

- TR 22.870 0.4.1 [PR 6.48.6-1] (Table 6.48.6-1)

- TR 22.870 0.4.1 [PR 6.49.6-1] (Table 6.49.6-1)

The following potential performance requirements are excluded due to the remaining ENs and will be taken into consideration after the ENs are cleaned up:

- TR 22.870 0.4.1 [PR 9.8.6-2] (Table 9.8.6-1)

- TR 22.870 0.4.1 [PR 9.18.6-1] (Table 9.18.6-1)

- TR 22.870 0.4.1 [PR 6.38.6-3] (Table 6.38.6-1)

[CPR y.1-1] The 6G system including IMS shall support various immersive communication services with the following KPIs.

Editor’s Note: 	Unless stated otherwise, the definition and understanding "Max allowed end-to-end latency", "Service bit rate: user-experienced data rate ", "Reliability", "# of UEs", "UE Speed", "Service Area", and "Positioning accuracy" refers to [22.261] and [22.256].

Table y.1-1: Performance requirements for immersive communication services

| Use Cases | Max allowed end-to-end latency | Service bit rate: user-experienced data rate |  | Reliability | UE Speed | Service Area | Other KPIs |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Downlink | Uplink |  |  |  |  |
| Immersive Gaming (UC 9.2 A) | Compute flows: [5 - 20ms] Conversational and game state flows: [50 - 100ms] Streaming flows: [200 - 300ms] | (note A-6) Player/Cheerleader: (note A-1) [640 Mbps] (8K, 120fps, compression ratio of 300 and 8 bits per color) [240 Mbps] (8K, 120fps, compression ratio of 400 and 12 bits per pixel) [200 Mbps] (8K, 90fps, compression ratio of 400 and 12 bits per pixel) Spectator: (note A-3) [320 Mbps] (2D 8K video, 120fps, compression ratio of 300 and 8 bits per color) [120 Mbps] (2D 8K video, 120fps, compression ratio of 400 and 12 bits per pixel) [100 Mbps] (2D 8K video, 90fps, compression ratio of 400 and 12 bits per pixel) | (note A-6) Player/Cheerleader: (note A-2) [100 Mbps] Spectator: (note A-4) [10 Mbps] | [99.9 – 99.99 %] | Stationary, pedestrian | [38 m x 15 m] (note A-5) | Positioning accuracy: [≤10 cm] |
| Immersive media content production via the wireless link (UC 9.6 B) | [100ms] (note B-1) | (note B-2) | (note B-2) [2.65 Gbps] (4K, 60fps, 10 bits per color, compression ratio of 4) [1.1 Gbps] (4K, 60fps, 10 bits per color, compression ratio of 10) [7.96 Gbps] (8K, 60fps, 10 bits per color, compression ratio of 6) [4.8 Gbps] (8K, 60fps, 10 bits per color, compression ratio of 10) | [99,99 %] | Stationary, Pedestrian | 30 m x 30 m | # of UEs: 4 (note B-3) |
| Seamless Immersive Reality in Education (UC 9.5 D) | Split rendering：[< 10ms] Voice：[< 50ms] Collaboration：[< 150ms] | [< 250 Mbps] | [< 250 Mbps] | N/A | Pedestrian | N/A | Area traffic capacity: Indoor: [< 250 Mb/s/m2] Outdoor (Wide Area) : [< 20 Mb/s/m2] Positioning accuracy: Horizontal: [≤ 10cm] Vertical: [≤ 10cm] |
| Mixed Reality gaming (UC 9.9 E) | [20ms] (note E-1) | [50 Mbps] (note E-2) | [50-100 Mbps] (note E-3) | N/A | 5 km/h | N/A | Communication service availability: Dense Urban [99%] Urban [99%] Rural [98%] Overall user density: Dense Urban [25,000 /km2] Urban [1000-10,000 /km2] Rural [100 /km2] Activity factor: 2% (note E-4) |
| NOTE A-1: It is important to note that the data rates may change under different assumptions. Data Rate = Video resolution * (Bits per color * 3) * Refresh rate * # of eye buffer / (compression ratio), with the following assumptions: 2 eye buffers; 8K video resolution; refresh rate of 120 fps or 90 fps. The frame rate of 120 fps is assumed as it has been shown that such high frame rate helps reduce the probability of simulator sickness [15]. 90 fps is the typical frame rate in current display device. compression ratio of 300 and 8 bits per color; or compression ratio of 400 and 12 bits per pixel [160]. A resolution of 8K (8192 x 4320) per eye can help to remove graphics pixelation and provide good XR user experience [159]. High refresh rates (e.g. 120 fps) are very correlated and inter prediction between frames increase compression. Some codecs (e.g. MV-HEVC) may further drop the bitrate requirement. NOTE A-2: Significantly higher compared to 5G to enable sensor sharing for split computation. NOTE A-3: DL data rate for a spectator is assumed to be 2D 8K video. NOTE A-4: UL tracking for a spectator not as intensive as the player/cheerleader. NOTE A-5: Average basketball court is 28 m x 15 m (adding 5 m on both sides for the spectator seating) to make a 38 m x 15 m basketball gymnasium. NOTE A-6: The provided values are targeted values and not strict requirements. NOTE B-1: One-way delay that is from camera to holographic player. NOTE B-2: This bit rate is assuming to use the JPEG-XS [352] pre-compression encoding method to ensure both high-quality images and high encoding efficiency, while also realizing lower encoding delays [161]. Which is intended as a targeted value per UE and not a strict requirement. NOTE B-3: 4 cameras are used in simple stage [353], more cameras may be needed for condition of larger number of performers, with complex stage and lighting conditions. It depends on the deployment. NOTE E-1: The local encoding and processing on the glasses is assumed. NOTE E-2: Remote or split rendering for gaming applications, such as rendering 360-degree video tiles in the cloud and transmitting the images to device. NOTE E-3: Envisioned 1080P resolution 2 grayscale cameras 1920 x 1080 x 8 bit, a depth sensor 1920 x 1080 x 8 bit with 60 FPS means 1920 * 1080 * 24 * 60 bps = 3Gbps uncompressed. Depending on compression ratios that would lead to 50 Mbps to 100 Mbps NOTE E-4: Activity factor 2% is based on 20% uptake of XR service and 10% activity factor for the XR application. |  |  |  |  |  |  |  |

Editor’s Note: 	“UC 9.2 A”, the "A" has no technical meaning and serves solely to maintain correspondence with the respective use case and NOTEs during the consolidation work.

OP1: Keep lag and delay within the same table.

Table y.1-2: Performance requirements for media synchronization for single applications

| Use Cases | Max allowed end-to-end latency | Service bit rate: user-experienced data rate | Synchronization threshold | Other KPIs |
| --- | --- | --- | --- | --- |
| Personalized interactive immersive guided tour (UC 9.12 A) (note A-1) (note A-2) (note A-3) | Audio (UL/DL): [10ms] | Audio UL/DL: [5-512 kbps] | Audio-to-haptics lag: [25ms]; Haptics-to-audio lag: [12ms] | N/A |
|  | Immersive video (DL): [200-300ms] | Immersive video DL: [10-20 Mbps] | Visual-to-haptics lag: [20ms]; Haptics-to-visual lag: [30ms] |  |
|  | Avatar between remote guide and UEs: [20ms] | Avatar: [0.1-30] Mbps (depending on the format) Avatar animation: 2 Mbps uncompressed. 1 Mbps compressed | Audio-to-avatar lag: [25ms]; Avatar-to-audio lag: [12ms] |  |
|  | Pose & action data (UL): [5ms] | Pose & action data UL: [100 – 400 kbps] | Avatar-to-haptics lag: [20ms]; Haptics-to-avatar lag: [30ms] |  |
|  | Environment sensing data (UL): [5ms] | Environment sensing data UL: [10 – 50 Mbps] | Pose-to-visual lag: [50ms] (pose UL, visual DL) Visual-to-pose lag: [20ms] (visual DL, pose UL) |  |
|  | Haptic (DL): [5ms] | Haptic DL: [0.25 – 160 kbps] for parametric compressed format [up to 6400 kbps] for sample format. See. TR 26.854 [165] Table 5.4-1. | Audio-to-pose lag: [50ms] Pose-to-audio lag: [20ms] |  |
| Multi-party call with deterministic user experience (UC 9.3 C) | N/A | [>=30Mbps] (note C-1) | - In the range of [125ms to 5ms] for audio delayed - In the range of [45ms to 5ms] for advanced audio (note C-2) | Max. mouth-to-ear delay: [100ms] (note C-3) Max. duration of consecutive packet losses: [100ms] (note C-4) Availability: [99%] (note C-5) UE speed: up to [500 km/h] (note C-6) |
| NOTE A-1: Synchronization threshold values vary for active versus passive engagement scenarios. Scenarios other than the ones listed may require other synchronization thresholds for the same media combinations. NOTE A-2: “Media X to media Y lag” refers to the positive time difference between the reference media X component and the specified media Y component. For example, an “audio-to-haptics lag” of 25ms means that haptics media arriving within 25ms after the audio is acceptable. NOTE A-3: Delay, Packet loss, Update rate, Packet size and Throughput for each media type based on TR 26.854 [165] Table 10.3-1. NOTE B-1: As defined in TS 22.261 [14] clause 6.43.1. NOTE B-2: As defined in TS 22.261 [14] clause 7.6.1. NOTE C-1: It is derived based on 4K 60 fps video encoded with HEVC [103]. NOTE C-2: As defined in TS 22.261 [14] clause 7.6.1. NOTE C-3: One-way delay [102]. NOTE C-4: It refers to the capability of recovering the missing audio packets as long as 100ms, based on the assumption of 20ms voice samples encapsulated into one audio packet [101]. NOTE C-5: It means the probability to provide the above KPIs during the time that a user intends to use the above services. NOTE C-6: It is to consider the high-speed train scenario as in TS 22.261 [14] clause 7.1, which is intended as a targeted value and not a strict requirement. |  |  |  |  |

OP2: Split the deterministic KPI.

Table y.1-2: Performance requirements for media synchronization for single applications

| Use Cases | Max allowed end-to-end latency | Service bit rate: user-experienced data rate | Synchronization threshold | Other KPIs |
| --- | --- | --- | --- | --- |
| Personalized interactive immersive guided tour (UC 9.12 A) (note A-1) (note A-2) (note A-3) | Audio (UL/DL): [10ms] | Audio UL/DL: [5-512 kbps] | Audio-to-haptics lag: [25ms]; Haptics-to-audio lag: [12ms] | N/A |
|  | Immersive video (DL): [200-300ms] | Immersive video DL: [10-20 Mbps] | Visual-to-haptics lag: [20ms]; Haptics-to-visual lag: [30ms] |  |
|  | Avatar between remote guide and UEs: [20ms] | Avatar: [0.1-30] Mbps (depending on the format) Avatar animation: 2 Mbps uncompressed. 1 Mbps compressed | Audio-to-avatar lag: [25ms]; Avatar-to-audio lag: [12ms] |  |
|  | Pose & action data (UL): [5ms] | Pose & action data UL: [100 – 400 kbps] | Avatar-to-haptics lag: [20ms]; Haptics-to-avatar lag: [30ms] |  |
|  | Environment sensing data (UL): [5ms] | Environment sensing data UL: [10 – 50 Mbps] | Pose-to-visual lag: [50ms] (pose UL, visual DL) Visual-to-pose lag: [20ms] (visual DL, pose UL) |  |
|  | Haptic (DL): [5ms] | Haptic DL: [0.25 – 160 kbps] for parametric compressed format [up to 6400 kbps] for sample format. See. TR 26.854 [165] Table 5.4-1. | Audio-to-pose lag: [50ms] Pose-to-audio lag: [20ms] |  |
| Multi-party call with deterministic user experience (UC 9.3 C) | N/A | [>=30Mbps] (note C-1) | - In the range of [125ms to 5ms] for audio delayed - In the range of [45ms to 5ms] for advanced audio (note C-2) | Max. mouth-to-ear delay: [100ms] (note C-3) Max. duration of consecutive packet losses: [100ms] (note C-4) Availability: [99%] (note C-5) UE speed: up to [500 km/h] (note C-6) |
| NOTE A-1: Synchronization threshold values vary for active versus passive engagement scenarios. Scenarios other than the ones listed may require other synchronization thresholds for the same media combinations. NOTE A-2: “Media X to media Y lag” refers to the positive time difference between the reference media X component and the specified media Y component. For example, an “audio-to-haptics lag” of 25ms means that haptics media arriving within 25ms after the audio is acceptable. NOTE A-3: Delay, Packet loss, Update rate, Packet size and Throughput for each media type based on TR 26.854 [165] Table 10.3-1. NOTE C-1: It is derived based on 4K 60 fps video encoded with HEVC [103]. NOTE C-2: As defined in TS 22.261 [14] clause 7.6.1. NOTE C-3: One-way delay [102]. NOTE C-4: It refers to the capability of recovering the missing audio packets as long as 100ms, based on the assumption of 20ms voice samples encapsulated into one audio packet [101]. NOTE C-5: It means the probability to provide the above KPIs during the time that a user intends to use the above services. NOTE C-6: It is to consider the high-speed train scenario as in TS 22.261 [14] clause 7.1, which is intended as a targeted value and not a strict requirement. |  |  |  |  |

Editor’s Note: 	“UC 9.12 A”, the "A" has no technical meaning and serves solely to maintain correspondence with the respective use case and NOTEs during the consolidation work.

Editor’s Note: The word “lag” of UC 9.12 needs to be clarified.

[PR 9.3.6-4]: The 6G system including IMS shall be able to provide deterministic user experience for multi-party call with the following KPI:

Table y.1-X:  Performance requirements for multi-party call with deterministic user experience

| Use case | Max. mouth-to-ear delay | Audio-video synchronisation thresholds | Max. duration of consecutive packet losses | Throughput (UL and DL) | Availability | UE speed |
| --- | --- | --- | --- | --- | --- | --- |
| multi-party call (UC 9.3) | [100ms] (note 1) | [- in the range of [125 ms to 5 ms] for audio delayed - in the range of [45 ms to 5 ms] for audio advanced] (note 2) | [100 ms] (note 3) | [>=30Mbps] (note 4) | [99%] (note 5) | up to [500km/h] (note 6) |
| NOTE 1: one-way delay [102]. NOTE 2: as defined in TS 22.261 [14] clause 7.6.1. NOTE 3: it is referring to the capable of recovering the missing audio packets as long as 100ms, based on the assumption of 20ms voice samples encapsulated into one audio packet [101]. NOTE 4: it is derived based on 4K 60 fps video encoded with HEVC [103]. NOTE 5: it means the probability to provide the above KPIs during the time that a user intends to use the above services. NOTE 6: it is to consider the high-speed train scenario as in TS 22.261 [14] clause 7.1, which is intended as a targeted value and not a strict requirement. |  |  |  |  |  |  |

[CPR y.1-2]	Subject to operator policy, the 6G system including IMS shall support the synchronization of independent traffic flows of one or more applications, to be delivered to more than one device (i.e. UE or tethered devices).

NOTE:	It is assumed that there is association between the applications whose traffic flows are synchronized and that the association is known to the 6G system.

Table y.1-3: Performance requirements for media synchronization for multiple applications

| Use case | Audio-Haptic synchronization thresholds | Video- Haptic synchronization thresholds | Audio-video synchronisation thresholds |
| --- | --- | --- | --- |
| Remotely controlled repair (UC 9.7) | - In the range of [50ms to 0ms] for audio delayed (NOTE 1) - In the range of [25ms to 0ms] for audio advanced (NOTE 1) | - In the range of [15ms to 0ms] for video delayed (NOTE 1) - In the range of [50ms to 0ms] for video advanced (NOTE 1) | - In the range of [125ms to 5ms] for audio delayed (NOTE 2) - In the range of [45ms to 5ms] for audio advanced] (NOTE 2) |
| NOTE 1: as defined in TS 22.261 [14] clause 6.43.1. NOTE 2: as defined in TS 22.261 [14] clause 7.6.1. |  |  |  |

* * * End of changes * * *