SA WG2 Meeting #172	S2-2510135

17 – 21 November, 2025, Dallas, USA	(revision of S2-250xxxx)

Source:	CATT

Title:	Update the agreed principles and add conclusion for KI#4

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC/Rel-20

Abstract of the contribution: This pCR is proposed to update the agreed principles and add conclusion for KI#4.

- Introduction

This pCR is proposed to update the agreed principles and add conclusion for KI#4

**** First Change ****

7.1.4	Agreed Principles for KI#4: Sensing Data and the Associated Information Collection and Transport

The following principles are agreed for KI#4:

-	Based on the received sensing service requirements from the Sensing Function, the Sensing Entity (gNB) can determine how to perform the relevant sensing measurement to collect the Sensing data.

-	Sensing Entity can support one time, periodical Sensing Data to the Sensing Function.

Editor's note:	The type (one time, periodical, etc.) and content of Sensing Data that will be provided by gNB needs to be coordinated with RAN WGs in the normative phase.

-	Sensing data can be collected from one or multiple gNBs by Sensing Function for the same sensing service request.

-	The Sensing Function can generate sensing results based on the information provided by the Sensing Entity (gNB) node(s).

-	The sensing data is transferred from Sensing Entity (gNB) to Sensing Function via a P2P interface.

NOTE x:	The protocol stack of the P2P interface needs further coordination with RAN WG3.

NOTE:	The architecture aspect of this KI's interim conclusions will be aligned with KI#1 conclusions.

**** Second Change ****

8.1.x	Conclusions for KI#4: Sensing Data and the Associated Information Collection and Transport

The following conclusions for Key Issue #4 of sensing data and the associated information collection and transport are made:

-	Based on the received sensing service requirements from the Sensing Function, the Sensing Entity (gNB) can determine how to perform the relevant sensing measurement to collect the Sensing data.

-	Sensing Entity can support one time, periodical Sensing Data to the Sensing Function.

Editor's note:	The type (one time, periodical, etc.) and content of Sensing Data that will be provided by gNB needs to be coordinated with RAN WGs in the normative phase.

-	Sensing data can be collected from one or multiple gNBs by Sensing Function for the same sensing service request.

-	The Sensing Function can generate sensing results based on the information provided by the Sensing Entity (gNB) node(s).

-	The sensing data is transferred from Sensing Entity (gNB) to Sensing Function via a P2P interface.

NOTE x:	The protocol stack of the P2P interface needs further coordination with RAN WG3.

NOTE:	The architecture aspect of this KI's interim conclusions will be aligned with KI#1 conclusions.

**** End of Changes ****