3GPP TSG-SA WG2#172	S2-2510405

Dallas, USA, 17-21 November 2025	(was S2-2508xxxx)

Source:	Qualcomm Incorporated

Title:	KI#4: Interim conclusions

Document for:	Discussion/Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC/ Rel-20

Abstract of the contribution: Interim conclusions on KI#4: Location service for IMS emergency call and regulatory services over NB-IoT NTN.

# 1.	Discussion

Following solution has been agreed on KI#4: Location service for IMS emergency call and regulatory services over NB-IoT NTN.

- Solution #26: Support for location service for emergency call over NB-IoT NTN.

Based on the solutions outlined above, it is recommended that interim conclusions regarding KI#4 be formulated.

# 2.	Text proposal

It is proposed to agree the following changes vs. TS 23.700-19:

# >>>>BEGINNING OF CHANGES (all new text) <<<<

## 8.x	Interim conclusions for Key Issue #4

The following interim conclusions for Key Issue #4 Location service for IMS emergency call and regulatory services over NB-IoT NTN are made:

-	When initiating emergency call over NB-IoT NTN, the UE reports its location information to the IMS Core during session establishment.

-	When requested by the MME, the UE provides its coarse location information to the MME, which then forwards the data to the GMLC/LRF.

-	If the location information is not provided by UE or insufficient for PSAP discovery, the IMS Core retrieves the UE location information from GMLC/LRF.

# >>>>END OF CHANGES<<<<