3GPP TSG-SA WG2 Meeting #172	S2-2510799

# November 17-21, 2025 Dallas, USA

Source:	Skylo Technologies

Title:	FS_SMS2EC_ARC: Sol#10 updates to resolve EN

Agenda item:	20.8.1

Document for:	Agreement

Work Item / Release: FS_SMS2EC_ARC/Rel-20

Abstract of the contribution: This paper updates Sol#10 to resolve EN

# 1. Discussion

The EN in Sol#10 is addressed in this update.

In SMS over SGs deployment in EPS, SMS support is based on the connectionless SGs reference point between the MME and the MSC Server and use of NAS signalling between UE and the MME, i.e. no CS Fallback is performed for SMS. The SMS protocol entities are reused from the existing MSUE and MSC implementations as specified in clause 8.2 of TS 23.272 [7].

The procedure to transport SMS to EC described in clause 6.10.2.1 can be reused for EPS deployment in SGs option with the following changes as depicted in Figure 6.10.2.1a-1:

- In Step 0, the UE is EPS/IMSI attached and the NAS signalling based SMS transfer is chosen by the UE

- In Step 3a, the MME forwards the SMS to EC message to the MSC/VLR in an Uplink Unitdata message. And steps 3b-3d are performed as defined in TS 23.272 [7]. The SMS to EC message is forwarded to the appropriate SMSC.

- In Step 6, the Delivery Report is sent to UE via MSC/VLR and MME.

Additionally, we have updated the terminology for clarity. The previously used term, 'emergency message,' will now be referred to as an 'SMS to EC message'.

# 2 Proposal

It is proposed to include the following changes in TR 23.700-65 V0.2.0.

* * * First Change * * * *

## 6.10	Solution #10: SMS over NAS in 4G/5G to Support SMS Delivery to the Emergency Response Centre

### 6.10.0	High level principles

To support SMS delivery to Emergency Response Centres via NAS, the following high-level solution principles apply for both EPS and 5GS:

-	The UE may include an explicit indication towards MME/AMF to indicate the MO data containing SMS to Emergency Response Centre (EC).

-	For cases where the UE cannot detect the SMS to EC, the message is transmitted as a normal SMS, with identification of SMS to EC performed by the network.

-	Upon detection of a SMS to EC, the MME/AMF may trigger UE location retrieval.

-	Location information may be provided together with the SMS towards the SMSC/SMS Router for delivery to the Emergency Response Centre. The functionalities of SMS Router are specified in TS 23.040 [13], TS 23.204 [8] and TS 23.272 [7].

-	The MME in EPS or SMSF in 5GS forwards the SMS TPDU to the appropriate SMS-GMSC/SMS-IWMSC/SMS Router for onward routing.

-	The SMSC, based on user data header information in TPDU described in Solution 1, forwards the SMS to the operator's dedicated emergency service network or directly to the Emergency Response Centre, in accordance with local regulatory requirements.

These principles apply to both roaming and non-roaming scenarios. Callback from the Emergency Response Centre to the UE is supported by reusing the regular SMS delivery service. The SMS from the Emergency Response Centre is delivered via the standard SMS path through the serving PLMN's SMSC.

### 6.10.1	Description

In EPS, the UE delivers the SMS to EC using SMS over NAS. If the UE is in idle mode and detects the intent to send an SMS to EC, it initiates the UE-triggered Service Request procedure. If both the UE and the MME support Control Plane (CP) CIoT EPS Optimisation for SMS transfer, the UE initiates a Control Plane Service Request (CPSR).

When CP CIoT EPS Optimisation is used, the UE encapsulates the SMS-SUBMIT TPDU in a CPSR message towards the MME. Otherwise, the SMS to EC is encapsulated within the Uplink NAS Transport message. The UE may include an explicit SMS to EC indicator in the NAS message to inform the MME that the payload contains a SMS to EC in the SMS TPDU.

If the SMS is identified as a SMS to EC, the UE may include an emergency indication. The MME may use this indication to trigger UE location retrieval using EPC Network Induced Location Request (EPC-NI-LR) as specified in clause 9.1.17 of TS 23.271 [18] and/or bypass applicable barring checks. The MME then forwards the SMS TPDU, along with the obtained location information, to the SMS-GMSC/SMS-IWMSC/SMS Router over the SGd interface using the MO-Forward-Short-Message-Request procedure. The SMSC subsequently delivers the SMS to the Emergency Response Centre in accordance with local regulatory requirements.

In 5GS, the UE delivers the SMS to EC via an Uplink NAS Transport message towards the AMF. When the SMS is identified as a SMS to EC, the UE includes an indication in the NAS message. The AMF may use this indication to bypass barring checks and/or to trigger UE location retrieval using the 5GC Network Induced Location Request (5GC-NI-LR) as specified in clause 6.10 of TS 23.273 [17].

The AMF forwards the SMS to the serving SMSF using the Nsmsf_SMService_UplinkSMS operation. The SMSF examines the RP-DA, and forwards it to the appropriate SMS-GMSC/SMS-IWMSC/SMS Router, which delivers the message to the SMSC. The SMSC, based on information contained in SMS user data header, transfers the SMS to the Emergency Response Centre as required by local regulation.

Callback from the Emergency Response Centre to the UE is supported using the regular SMS delivery service. The Emergency Response Centre sends the SMS via the operator's SMSC and the SMS is delivered to the UE over the standard downlink SMS path.

### 6.10.2	Procedures

#### 6.10.2.1	SMS in MME for 4G

Figure 6.10.2.1-1 shows the procedures for SMS delivery to the Emergency Response Centre via SMS over NAS using the SMS in MME architecture option as described in Annex C of TS 23.272 [7].

Figure 6.10.2.1-1: Procedures for SMS to EC Message Delivery via SMS over NAS in EPC

0.	The UE attaches to the network using either Initial attach or Emergency Attach.

1.	If the UE is in idle mode and detects the intent to send a SMS to EC, it initiates the UE-triggered Service Request procedure. If both the UE and the MME support Control Plane (CP) CIoT EPS Optimisation for SMS transfer, the UE initiates a Control Plane Service Request (CPSR).

When CP CIoT EPS Optimisation is used, the CPSR message carries the SMS-SUBMIT TPDU directly. Otherwise, in step 1a, the SMS is encapsulated within the Uplink NAS Transport message.

The UE may include an explicit SMS to EC indicator in the NAS message to inform the MME that the payload contains an SMS to EC message in the SMS TPDU.

2.	If the UE does not know its own location, the UE may retrieve its location information from the network. Or based on the indication described in step 1, the MME may trigger UE location retrieval using EPC Network Induced Location Request (EPC-NI-LR) in step 2a as specified in clause 9.1.17 of TS 23.271 [18] and/or bypass applicable barring checks.

3.	The MME selects the target SMS-GMSC/SMS-IWMSC/SMS Router and forwards the SMS TPDU along with the location information using the MO-Forward-Short-Message-Request (OFR) message over the SGd interface, as specified in TS 29.338 [16].

4.	The SMS-GMSC/SMS-IWMSC/SMS Router, based on the RP-DA to route the message to the appropriate SMSC.

5.	The SMSC, based on the SMS-TPDU user data header information as described in Solution 1, forwards the message to the operator's dedicated emergency service network. For example:

-	Advanced Mobile Location (AML) over NAS/SGd, defined in GSMA NG.119 [11], allows the visited SMSC to send the SMS directly to the PSAP or Emergency Call Centre network.

-	Text-to-9-1-1, as specified in J-STD-110 [10], enables the SMSC to forward the SMS via the SMPP protocol to the Text Control Centre (TCC), which functions as a gateway to the PSAP.

6.	The Delivery Report is sent to UE.

#### 6.10.2.1a	SMS over SGs for 4G

In SMS over SGs deployment in EPS, SMS support is based on the connectionless SGs reference point between the MME and the MSC Server and use of NAS signalling between UE and the MME, i.e. no CS Fallback is performed for SMS. The SMS protocol entities are reused from the existing MSUE and MSC implementations as specified in clause 8.2 of TS 23.272 [7].

The procedure to transport SMS to EC described in clause 6.10.2.1 can be reused for EPS deployment in SGs option with the following changes as depicted in Figure 6.10.2.1a-1:

- In Step 0, the UE is EPS/IMSI attached and the NAS signalling based SMS transfer is chosen by the UE

- In Step 3a, the MME forwards the SMS to EC message to the MSC/VLR in an Uplink Unitdata message. And steps 3b-3d are performed as defined in TS 23.272 [7]. The SMS to EC message is forwarded to the appropriate SMSC.

- In Step 6, the Delivery Report is sent to UE via MSC/VLR and MME.

Figure 6.10.2.1a-1: Procedures for SMS to EC Message Delivery via SMS over SGs

#### 6.10.2.2	SMS over NAS for 5G

Figure 6.10.2.2-1 shows the procedures for SMS delivery to the Emergency Response Centre using SMS over NAS for 5G.

Figure 6.10.2.2-1: Procedures for SMS to EC Message Delivery via SMS over NAS in 5G

0.	The UE follows the procedures specified in clause 4.13.3 of TS 23.502 [3] for registration with SMS over NAS support. During the Registration procedure in 5GS, the UE includes an "SMS supported" indication in the Registration Request to indicate its capability for SMS over NAS transport.

1.	If the UE is in CM-IDLE state and intends to send an uplink SMS message, the UE and the network perform the UE-triggered Service Request procedure as defined in clause 4.2.3.2 of TS 23.502 [3] to establish a NAS signalling connection to the AMF.

If the UE is in CM-CONNECTED state, this step is skipped and the procedure continues with step 2.

2.	The UE builds the SMS message.

For a SMS to EC, the UE encapsulates the SMS in an Uplink NAS Transport message and includes a SMS to EC indication. The Uplink NAS Transport message is then sent to the AMF.

For a SMS to EC that is not detectable by the UE (e.g. based on number or configuration), the SMS is encapsulated in an Uplink NAS Transport message without the emergency indication and sent to the AMF.

3.	If the UE does not know its own location, the UE may retrieve its location information from the network. Or based on the SMS to EC indication, the AMF may trigger UE location retrieval, using the 5GC Network Induced Location Request (5GC-NI-LR) in step 3a, as specified in clause 6.10 of TS 23.273 [17] and/or bypass applicable barring checks.

4.	The AMF forwards the SMS to the appropriate SMSF serving SMS to EC message by invoking the Nsmsf_SMService_UplinkSMS service operation.

5.	The SMSF inspects the user data header contained in TPDU.

If the message is identified as a SMS to EC, the SMSF forwards it to the appropriate SMS-GMSC/SMS-IWMSC/SMS Router.

The SMSF then returns the SMS-Submit Report to the AMF using the Namf_Communication_N1N2MessageTransfer service operation. The AMF delivers the report to the UE via Downlink NAS Transport as described in clause 4.13.3.3 of TS 23.502 [3].

6.	The SMS-GMSC/SMS-IWMSC/SMS Router, based on the RP-DA route the message to the appropriate SMSC.

7.	The SMSC, based on the user data header information contained in TPDU, forwards the message to the operator's dedicated emergency service network. For example:

-	Advanced Mobile Location (AML) over NAS/SGd, defined in GSMA NG.119 [11], enables the visited SMSC to deliver the SMS directly to the PSAP or Emergency Call Centre network.

-	Text-to-9-1-1, as specified in J-STD-110 [10], enables the SMSC to forward the message via the SMPP protocol to the Text Control Centre (TCC), which functions as a gateway to the PSAP.

8.	The Submit Report is sent to UE.

### 6.10.3	Impacts to Services, Entities and Interfaces

MME:

-	MME uses SMS to EC indication to trigger location request.

AMF:

-	AMF uses SMS to EC indication to trigger location request.

UE:

-	UE indicates SMS to EC  indication towards MME/AMF.

SMSC:

-	SMSC uses user data header information contained in TPDU to forward the message to a proper emergency service network.

* * * End of Changes * * * *