3GPP WG-SA2 Meeting #170 	S2-2506594

Goteborg, Sweden, August 25 – 29, 2025

# Source:	China Unicom

# Title:	New WID on eRG without USIM

# Document for:	Information

# Agenda Item:	30.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Support of eRG without USIM

Acronym:	TEI20_non-USIM_eRG

Unique identifier:	TBD

Potential target Release:	Rel-20

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | x |  | x |  |
| No | x |  | x |  | x |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| x | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| Resident_Ph2 | SA1 | 1050107 | Resident Phase 2 |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

3	Justification

SA1 TS22.261 subclause 6.38.2.2 has specified that the 5G network shall provide support to authenticate an eRG without a USIM to access the 5G core network via wireline access only.

More and more non-3GPP devices support network connectivity capabilities, and the non-3GPP devices and UEs belonging to the same home can form a group to enable uniform service control, charging, or support interaction between the UE and the non-3GPP devices via 5GC. For example, a human user may create a  home network that includes various non-3GPP devices, such as Network Attached Storage (NAS), home surveillance cameras, Networked Printers, and VR/XR devices etc. A mobile phone (i.e. a remote UE) located outside the home network, e.g. a family member’s mobile phone, could be authorized to access these non-3GPP devices via cellular network when needed.

To achieve this, the first step is to register the residential gateway with 5GC via wireline access network. The 3GPP, in collaboration with BBF, has specified Wireline/Wireless Convergence architecture (5WWC) since Rel-16, enabling the 5G core network to support the wireline access, which is more applicable to wireline-wireless convergence operational models. However, customers are often reluctant to replace their existing wireline residential gateways, which typically have no USIMs equipped. Even with solutions like FN-RG that retain the legacy residential gateway(RG), new functional requirements are required on the existing BNG in wireline network. Given operator policies and challenges associated with network renovation, such as upgrading a large number of existing network devices and interfaces, these requirements hinder the development of certain smart home industries.

Therefore, this WID aims to define the non-USIM evolved Residential Gateway (eRG) for accessing 5GC/EPC based on the enhancement of ePDG, which has no impact on the legacy wireline access network. Since ePDG is only the non-3GPP access gateway deployed on the network, enhancing it provides the greatest benefits for operators.

4	Objective

The objective of this WID is to define the following areas:

Work Tasks 1:  Support the non-USIM eRG access to 5GC/EPC via ePDG without impact on wireline access network.

- WT#1.1: Define the new SUPI for non-USIM eRG.

- WT#1.2: Enhance the ePDG to support the non-USIM eRG access by reusing the IPSec procedure.

NOTE 1: The authentication procedures for non-USIM eRG need to be studied by SA WG3.

NOTE 2: There is no impact on BBF related aspects.

5	Expected Output and Time scale

| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| --- | --- | --- | --- | --- | --- |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 23.501 | The definition and feature description of eRG without USIM |  | Dependent on TEI20 WI plan dates |

This TEI20_WID requires 1 TU.

6	Work item Rapporteur(s)

7	Work item leadership

SA2

8	Aspects that involve other WGs

Security aspects will be covered by SA3.

Charging aspects will be covered by SA5.

9	Supporting Individual Members

Supporting IM name

China Unicom

Huawei

HiSilicon

CATT

Verizon Wireless