SA WG2 Meeting #170	S2-2506877

25 – 29 Aug, 2025, Goteborg , SE

Source:	Google, AT&T

Title:	[WT#1.4] Scope and Key issues for supporting messaging service in 6G

Document for:	Agreement

Agenda Item:	20.6.1

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution covers the messaging services aspects for WT#1.4 in 6G study, which provides an overview of current architecture limitations, and proposes scope and key issues for WT#1.4 to support messaging service in 6G systems.

# 1. Justifications

Editor's Note: This justification section is not included in the TR; it is intended solely to justify the scope of the proposed WT.

In this contribution, we propose to include messaging services in WT#1.4 scope and corresponding key issues for 6G study as outlined in SP-250806 [x1]. We demonstrate the urgent need for architectural enhancements to support a lightweight messaging service in 6G for accommodating applications with extensible features and effectively leverage 6G capabilities while addressing fundamental protocol inefficiencies and security limitations.

Currently, in 5GS, the following messaging services are supported:

- SMS over NAS: Legacy SMS delivery mechanism using NAS signaling as defined in 3GPP TS 23.040 [x2] and TS 23.540 [x3].

- SMS over IMS (SMS over IP): IP-based SMS using SIP signaling over IMS infrastructure as defined in 3GPP TS 23.204 [x4] and related IMS specifications.

- MSGin5G: designed primarily for massive IoT device communication, supporting point-to-point, application-to-point, group and broadcast message delivery as defined in TS 23.554 [x5]. This service enables thing-to-thing and person-to-thing communication with gateway functionality for legacy 3GPP and non-3GPP message delivery.

- IMS-based Messaging using RCS (Rich Communication Services): Rich Communication Services built on IMS infrastructure implementing the GSMA Universal Profile specification [x6], providing enhanced messaging capabilities including multimedia sharing, group chat, presence indicators, and read receipts using SIP/MSRP protocols [x7].

To meet ITU IMT-2030 timeline and design goals of interoperability and substitutability, the 6G architecture must evolve to support lightweight messaging service to accommodate extensible application features while leveraging advanced 6G capabilities including enhanced network capability exposure, adaptive QoS provisioning, etc.

## Current Messaging Architecture Limitations

For person-to-person messaging services, the landscape has evolved dramatically from simple text-based SMS and multimedia MMS toward IMS based RCS, which has emerged as the successor to legacy messaging. However, current messaging architecture faces critical limitations that prevent it from meeting modern user expectations, fast reaction to market requirements, customization to meet local challenges, extensibility and regulatory requirements.

While SMS remains globally interoperable since its first specification release (3GPP TS23.040, June 1999) and IMS based messaging using RCS [x6] continuously gains more global adoptions, adapting SMS/RCS to modern communication and regulatory needs have highlighted several critical architectural and functional challenges:

- Protocol Inefficiencies: Current RCS implementations rely heavily on legacy protocols that suffer from inherent limitations if requiring persistent connection, e.g.

  - SIP (for signaling/session management)/MSRP (for message/media transfer) dependency causes battery consumption due to persistent connections required by MSRP.

  - Complexity overhead compared to modern lightweight protocols used by OTT services.

  - Inefficient signaling mechanisms not optimized for mobile data usage patterns.

- Security and Privacy Gaps: Current messaging services lack robust security mechanisms. For example:

  - SMS by default lacks strong authentication, making it vulnerable to spoofing, phishing, and interception risks.

  - Legacy RCS supports client to server encryption failing to meet modern privacy expectations for End-to-End Encryption (E2EE).

  - Regulatory and consumer expectations for privacy, authentication (e.g., 2FA), and anti-spam protections exceed current protocol capabilities.

- Limited and Complicated Interoperability: Current systems are constrained by:

  - MSISDN-only identities, preventing cross-platform messaging with non-cellular identities

  - None or complex interworking with OTT messaging services

  - Lack of seamless integration with modern identity frameworks

- Emergency Services Limitations: Current SMS lacks reliable routing to appropriate local emergency centers (PSAPs). While a Rel-20 5G-A study has been approved for SMS routing to emergency centers, comprehensive solutions are still required to enable location-based routing, priority handling, and cross-network interoperability.

- Feature Gap with OTT Applications: users expect rich, interactive, and secure communication experiences comparable to OTT messaging applications, including real-time chat, high-quality media sharing, advanced group functionalities, message reactions, replies, edit/delete capabilities, and critically, robust E2EE as standard. Current 5GS messaging architecture cannot efficiently support these advanced features.

- Lack of Extensibility: Current messaging services lack extensibility to react to fast moving market requirements. Long lead time to develop, test and deploy new messaging-based services often misses the market window and yields new revenue opportunity to OTT messaging services.

- Lack of Customization: Messaging services are ubiquitous and universal in nature. But local or regional competition and regulatory requirements will need to be considered, hence a simple way to augment the global base messaging services is needed.

## 6G Opportunities

6GS presents unprecedented opportunities to enable a lightweight messaging service for future mobile network generations while supporting SMS as a fallback mechanism. To meet ITU IMT-2030 timeline and design goals of 6G system for interoperability and substitutability, the 6G architecture must effectively evolve while leveraging advanced 6G capabilities including enhanced network exposure, and optimized lightweight protocol handling for 6G messaging approaches.

Figure 1.1-1 shows high-level end-to-end messaging service diagram, which demonstrates the on-going standardization activities that can contribute to this 6G messaging work task in 6G study. The 6GS is expected to evolve with a light weight protocol to accommodate applications with extensible features, using universal User and Network Interface (UNI) protocol, and supporting interoperable routing over Network-to-Network Interface (NNI).

Figure 1.1-1: high-level end to end 6G messaging service

## 1.2 Summary

Architectural Motivation: 6GS presents unprecedented opportunities to enable a lightweight messaging service for future mobile network generations and effectively leverage future 6G architecture capabilities while supporting SMS as a fallback mechanism.

Gap Analysis: Current 3GPP architecture lacks lightweight protocols support for accommodating modern messaging application features, e.g. reactions, replies, message edit/delete with extensible framework, non-MSISDN identities, robust E2EE, and seamless interoperability with OTT services, etc. 6GS architecture requires adaptations to efficiently support lightweight protocol for messaging application features using universal UNI protocol, moving away from legacy IMS-only approaches.

Technical Impact: Following impacts are expected to be covered to resolve this gap:

- Support lightweight messaging routing and delivery without SIP/MSRP based on extensible UNI messaging protocol optimized for 6G architecture

- Enhance network capability exposure for messaging application server using universal UNI protocol

- Support both MSISDN and Non-MSISDN identities

- Support E2EE messages identification and routing

- Support QoS provisioning for messages with mixed media types, e.g. text, image, emoji, video.

- Support extensibility and local customization

- Enhance emergency messaging capabilities with interoperable routing and priority handling.

* * * Start of Change (All New Texts) * * * *

# Annex A.1.4. WT#X Scope for Messaging Services

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

WT 1.4 covers the study of lightweight messaging service for 6G architecture, including at least the following:

- Whether and how to enable a lightweight messaging service in 6G for accommodating extensible features, e.g. End-to-End Encryption between clients, Non-MSISDN based identifier, etc., for applications using universal UNI protocol and achieve seamless interoperability between carrier-based and OTT messaging services.

- Whether and how to enable emergency messaging capabilities with interoperable routing and priority handling for emergency scenarios.

This topic is scoped by the WT descriptions in Table A.1.4-1.

Table A.1.4-1: Work tasks for Enhanced Messaging Services

| Work Task ID | Work Task(s) | Work Task Description |
| --- | --- | --- |
| WT_#1.4.1 | Messaging Service Architecture [NOTE 5] | 1.1: Define architecture requirements to support lightweight messaging services without SIP/MSRP [NOTE 1] 1.2: Study enhancement for network capability exposure for messaging services [NOTE 2] 1.3: Study support for both MSISDN and Non-MSISDN identities 1.4: Study support for E2EE messages identification and routing [NOTE 3] 1.5: Study QoS provisioning for lightweight messaging service with mixed media types, e.g. text, image, emoji, video. 1.6: Study how to support messaging service extensibility and customization. |
| WT_#1.4.2 | Support for Emergency Messaging | 2.1: Enhance network capabilities with interoperable routing and priority handling for emergency scenarios [NOTE 4] |
| NOTE 1: this is to support universal UNI protocol based on GSMA recommendation. NOTE 2: this sub-task can coordinate and leverage WT#1.2 about network capability exposure enhancement. NOTE 3: This is to ensure network provides secure transport for E2EE traffic while maintaining end-to-end security principles. NOTE 4: this is subject to operator policies and regulatory requirements. NOTE 5: this work task assumes SMS as a fallback mechanism. |  |  |

* * * Second Change (All New Texts) * * * *

# 5.X. Key Issue #4: Enhanced Messaging Services Architecture for 6G

Editor's Note: This clause defines the potential scope of KI(s) and is part of the TR.

Description: This key issue focuses on designing a lightweight messaging service enabled with extensible application features that addresses current limitations to effectively leverage 6G architecture capabilities while supporting SMS as a fallback mechanism.

This key issue focuses on following aspects:

- enabling a lightweight messaging service in 6G for accommodating extensible features, e.g. End-to-End Encryption between clients, Non-MSISDN based identifier, etc., for applications using universal UNI protocol and achieve seamless interoperability between carrier-based and OTT messaging services.

- enabling the messaging service in 6G to support extensibility and customization

- enabling emergency messaging capabilities with interoperable routing and priority handling for emergency scenarios.

Solutions for this key issue will describe, based on the WT scope described in Annex A.1.4:

- What are architecture requirements to support lightweight messaging service without SIP/MSRP for applications using universal UNI protocol.

- Whether and how to enhance network capability exposure for messaging services without compromising E2EE principles.

- Whether and how to support E2EE messages identification and routing for applications using advanced security mechanisms (e.g., MLS) while maintaining network efficiency.

- Whether and how to support diverse messaging identities, i.e. Non-MSISDN, and cross-platform interoperability,

- Whether and how to provision QoS for messages with mixed media types, e.g. text, image, emoji, video.

- Whether and how to support messaging service extensibility to meet new market requirements in a timely manner.

- Whether and how to support messaging service provider to customize messaging service in a simple way without changing the basic messaging service architecture.

- Whether and how to enhance network capabilities with interoperable routing and priority handling for emergency scenarios.

* * * Third Change (All New Texts) * * * *

Reference:

[x1]: SP-250806 – SID: Study on Architecture for 6G System

[x2]: TS23.040: Technical realization of the Short Message Service (SMS)

[x3]: 3GPP TS 23.540: 5G System: Technical realization of Service Based Short Message Service; Stage 2

[x4]: TS 23.204: Support of Short Message Service (SMS) over generic 3GPP Internet Protocol (IP) access; Stage 2

[x5]: TS23.554: Application architecture for MSGin5G Service; Stage 2

[x6]: RCS Universal Profile Service Definition Document Version 3.1 21 July 2025

[x7]: IETF RFC 9420: Messaging layer security (MLS)

* * * End of Changes * * * *