3GPP TSG-SA WG2 Meeting #170	S2-2506959

Stor-Göteborg, SE, Aug 25 – 29, 2025

Source:	China Telecom, ZTE, Huawei, Hisilicon

Title:	KI#2: New solution: Suppression of CAT service for IMS voice call via NB-IoT GEO

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4-ARC / Rel-20

Abstract of the contribution: The contribution proposes a solution to optimize the IMS call setup procedure for voice over GEO satellite NTN, focusing on minimizing signaling and simplifying service interactions to reduce call setup time under low bitrate conditions.

1. Discussion

In conventional terrestrial networks, some early media services, e.g., CAT (Customized Alerting Tones) service require multiple rounds of SDP negotiation as specified in annex A.5 in 3GPP TS 24.182 [x]. Firstly, the voice codec must be negotiated between the MO UE and the CAT AS, and then again between the MO UE and the MT UE when the MT UE answers the phone via SIP UPDATE or re-INVITE messages. Therefore, the call setup procedure requires extra time for the exchange of multiple messages, which is acceptable for terrestrial networks with relatively negligible latency. However, when a UE accesses the networks via a GEO satellite, this significantly impacts call setup latency. Furthermore, the CAT services that play music for the MO user typically require a high-speed codec. When transformed to a low-bitrate codec (e.g.1 kbps), the music might not sound right or pleasant.

It is proposed that CAT service, should be suppressed when using IMS voice communication via a GEO satellite connected to the EPC.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-19.

* * * Start of Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[6]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[7]	3GPP TS 23.167: "IP Multimedia Subsystem (IMS) emergency sessions".

[8]	3GPP TS 22.261: "Service requirements for the 5G system".

[9]	3GPP TS 22.101: "Service aspects; Service principles".

[x]	3GPP TS 24.182: " IP Multimedia Subsystem (IMS) Customized Alerting Tones (CAT)".

* * * Second of Change * * * *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 |
| X |  | X |  |  |

* * * Third of Change * * * *

6.X	Solution #X: Suppression of CAT service for GEO NB-IoT NTN Access with Call Setup Time Constraints

### 6.X.1	High level principles

This solution is to address Key Issue #2: IMS enhancement for GEO NB-IoT NTN access and gives a solution on how to suppress the certain early media service, i.e., CAT (Customized Alerting Tones) service when either the MO UE or the MT UE accesses IMS network via NB-IoT over GEO.

The following principles apply to IMS enhancement for this scenario:

- During the MO/MT call setup procedure, the P-CSCF identifies the UE's RAT type as NB-IoT (GEO). Based on this information, it inserts RAT type and Tone Suppression Indication in the SIP INVITE or SIP 18X messages (SIP 183 or SIP 180 messages) before forwarding them to the peer IMS network.

- When receiving the RAT type (i.e. NB-IoT (GEO)) and Tone Suppression Indication, the originating or terminating home IMS network (depending on which IMS network provides the CAT service) shall not trigger or handle the CAT service.

### 6.X.2	Description

For the MO/MT call setup procedures, the P-CSCF receives either a SIP INVITE or a SIP 18X message and identifies the RAT type of the MO or MT UE. There are three possible alternatives depending on how the P-CSCF obtains the RAT type of its serving UE:

- The P-CSCF obtains the RAT type from the PCRF, which may obtain it from the PGW.

- The P-CSCF derives the RAT type from the P-Access-Network-Info field in the SIP INVITE or SIP 18X messages sent by the UE, which may be unreliable.

- The P-CSCF obtains the RAT type from the PCRF during the SIP registration procedure and subscribes to the RAT type change event.

The P-CSCF decides to suppress the CAT service when the RAT type is NB-IoT (GEO). It then forwards SIP INVITE or SIP 18X messages adding the RAT type in the P-Access-Network-Info field and the Tone Suppression Indication in the P-Early-Media or other field, to the peer IMS network.

When receiving the RAT type (i.e. NB-IoT (GEO)) and the Tone Suppression Indication, the IMS network that provides the CAT service (S-CSCF or CAT AS) shall not trigger or handle the CAT services.

### 6.X.3	Procedures

6.X.3.1	Mobile Originating Procedure

The following figure depicts the IMS Mobile origination call setup procedure, where the MO UE accesses the network via NB-IoT over GEO, and MO UE or MT UE may have subscribed to the CAT service.

Figure 1: Mobile Originating Procedure

- UE A sends a SIP INVITE request, containing an initial SDP offer and optional RAT type.

- P-CSCF A determines the RAT type of UE A and establishes the bearer, if necessary. There are three possible alternatives depending on how P-CSCF A obtains the RAT type of UE A:

- P-CSCF A obtains the RAT type from the PCRF, which may obtain it from the PGW.

- P-CSCF A derives the RAT type from the P-Access-Network-Info field in the SIP INVITE messages sent by UE A.

- P-CSCF A obtains the RAT type from the PCRF during the SIP registration procedure and subscribes to the RAT type change event.

- P-CSCF A decides to suppress the CAT service when the RAT type is NB-IoT (GEO). It then forwards SIP INVITE messages adding the RAT type (i.e. NB-IoT (GEO)) in the P-Access-Network-Info field and the Tone Suppression Indication in the P-Early-Media or other field, to the originating home IMS network (S-CSCF A).

- When the RAT type (i.e. NB-IoT (GEO)) and Tone Suppression Indication are received, S-CSCF A determines to suppress the certain early media service, i.e., CAT service if UE A has subscribed to the CAT service, S-CSCF A may either choose: a) not to trigger CAT AS A; or b) to trigger CAT AS A. Based on the parameters received, the CAT AS A shall skip the CAT service logic.

- S-CSCF A forwards the SIP INVITE message to S-CSCF B containing the RAT type (i.e. NB-IoT (GEO)), Tone Suppression Indication and SDP A.

- When the RAT type (i.e. NB-IoT (GEO)) and Tone Suppression Indication are received, S-CSCF B determines to suppress the certain early media service, i.e., CAT service if the UE B has subscribed to the CAT service, S-CSCF B may either choose: a) not to trigger CAT AS B; or b) to trigger CAT AS B. Based on the parameters received, CAT AS B shall skip the CAT service logic.

- S-CSCF B forwards the SIP INVITE message to UE B containing the SDP A.

- UE B returns a SIP 180 or 183 message containing an SDP answer (SDP B), which shall not be triggered to or handled by the CAT AS. The message is then forwarded to UE A.

- The procedure is continued and the IMS session is established as specified in TS 23.228.

6.X.3.2	Mobile Terminating Procedure

The following figure depicts the IMS Mobile termination procedure, where the MT UE accesses the network via NB-IoT over GEO, and MO UE or MT UE may have subscribed to the CAT service.

Figure 2: Mobile Terminating Procedure

- UE A sends a SIP INVITE request containing an initial SDP offer (SDP A) to S-CSCF A.

- If UE A has subscribed to the CAT service, S-CSCF A triggers the CAT service as normal and forwards the SIP INVITE request to CAT AS A, based on the initial filter criteria (iFC) for UE A. CAT AS A then performs the service process and sends the SIP INVITE message containing SDP A back to the S-CSCF A as specified in annex A.5 in 3GPP TS 24.182 [x]

- The S-CSCF A forwards the SIP INVITE message containing the SDP A to the S-CSCF B.

- If UE B has subscribed to the CAT service, S-CSCF B triggers the CAT service as normal and forwards the SIP INVITE request to CAT AS B, based on the initial filter criteria (iFC) for UE B. CAT AS B then performs the service process and sends the SIP INVITE message containing SDP A back to S-CSCF B as specified in annex A.5 in 3GPP TS 24.182 [x].

- S-CSCF B forwards the SIP INVITE message containing the SDP A to UE B.

- UE B returns a SIP 180 or 183 message containing an SDP answer (SDP B) and an optional RAT type.

- P-CSCF B determines the RAT type of UE B and establishes the bearer, if necessary. There are three possible alternatives depending on how P-CSCF B obtains the RAT type of UE B:

- P-CSCF B obtains the RAT type from the PCRF, which may obtain it from the PGW.

- P-CSCF B derives the RAT type from the P-Access-Network-Info field in the SIP 18X messages sent by UE B.

- P-CSCF B obtains the RAT type from the PCRF during the SIP registration procedure and subscribes to the RAT type change event.

- P-CSCF B decides to suppress the CAT service when the RAT type is NB-IoT (GEO). It then forwards the SIP 18X message adding the RAT type (i.e. NB-IoT (GEO)) in the P-Access-Network-Info field and the Tone Suppression Indication in the P-Early-Media or other field, to S-CSCF B. S-CSCF B forwards the SIP 18X message to CAT AS B based on the Via field, provided that CAT AS B for UE B has previously been triggered during the SIP INVITE request procedure.

- CAT AS B shall skip the CAT service logic based on the RAT type (i.e. NB-IoT (GEO)) and Tone Suppression Indication.

- CAT AS B forwards the SIP 18X message containing the RAT type (i.e. NB-IoT (GEO)), Tone Suppression Indication and SDP B, back to S-CSCF B.

- S-CSCF B forwards this SIP 18X message to S-CSCF A. S-CSCF A then forwards the SIP 18X message to CAT AS A based on the Via field, provided that CAT AS A for UE A has previously been triggered during the SIP INVITE request procedure.

- CAT AS A shall skip the CAT service logic based on the RAT type (i.e. NB-IoT (GEO)) and Tone Suppression Indication.

- CAT AS A forwards the SIP 18X message containing the SDP B back to S-CSCF A.

- S-CSCF A forwards this SIP 18X message to UE A.

- The procedure is continued and the IMS session is established as specified in TS 23.228.

### 6.X.4	Impacts to Services, Entities and Interfaces

P-CSCF:

- Support for obtaining the RAT type of the its serving UE during the MO/MT call setup procedures.

- Support for identifying the UE's RAT type as NB-IoT (GEO) and adding the RAT type (i.e. NB-IoT (GEO)) and the Tone Suppression Indication to SIP INVITE or 18X messages forwarded to the peer IMS network.

S-CSCF/AS:

- The IMS network that provides the CAT service (S-CSCF or CAT AS) shall not trigger or handle CAT services when it receives RAT type (i.e. NB-IoT (GEO)) and the Tone Suppression Indication in SIP INVITE or 18X messages.

* * * End of Change * * * *